Name : 



   Badge number :                                                   .           
Self-assessment exam ( Server /Storage )

1. There was a report from customer that system has randomly rebooting with out any detail log. And they forward to us the Following DSET log files. Pls verify what the possible root cause of these issues. (25 mark )
[image: image1]

2. There was constant failover failure in cluster mode in semi-conductor line.

The configuration is as below what would be first thing would you guided to customer based on DSET log files? (25 mark)

[image: image16.emf]C:\Documents  and Settings\Gong_Jae_Kim\Desktop\Exam_Q2\Server exam\DSET Report (050331).zip



[image: image2.emf]C:\Documents  and Settings\Gong_Jae_Kim\Desktop\Exam_Q2\Server exam\exam DSET.zip


3. Describe how to retrieve TTY log from system when there is PV220s attached 
What course of action is needed to get the valid log files from system. (20 mark )
4. Describe what these code implies in TTY log ? (15mark)
0xf0
0x08
0xf1
0x02
0xFD

0xF5
5. Find out details from the attached TTY log. (15 mark )
Type of controller : 
Firmware version of controller :
Amount of cache :
Physical devices attached : 
Firmware of those devices : 
PV 220s you will be able to see the ZEMM firmware : 

[image: image3.emf]C:\Documents  and Settings\Gong_Jae_Kim\Desktop\Exam_Q2\Server exam\LSI_logs.txt


� EMBED Package  ���








[image: image4.emf]C:\Documents  and Settings\Gong_Jae_Kim\Desktop\Exam_Q2\Server exam\DSET Report (050331).zip

[image: image5.png]PowerVault
2208 or 2215

pr

server.

host-based RAID controler or HBA

cluster

| N—

bus
configuration
switch

positions



[image: image6.emf]CWVWB1S

[image: image7.emf]JR3XB1S

[image: image8.emf]3DY381S

[image: image9.emf]PERC 4/DC

[image: image10.emf]PERC 3/DC

[image: image11.emf]6WVWB1S

[image: image12.emf]HR3XB1S

[image: image13.emf]3CY381S

[image: image14.emf]PERC 4/DC

[image: image15.emf]PERC 3/DC

_1235630712/exam DSET.zip


3dy381s.zip




DSET Report for ss {L4-PS2 SvcTag-3DY381S-PowerEdge 2600}.zip






READ ME before viewing this report!.txt





=======================================================

Instructions for the proper viewing of this DSET report

=======================================================





1) Decompress ALL files within this report into one folder using any ZIP utility 



2) When prompted enter the password "dell" (no quotes, all lowercase letters) to decrypt

   the report contents during decompression



3) Then open "dsetreport.hta" in Internet Explorer 5.0 or newer (which must support

   viewing XML data)





NOTE: Do NOT open the ZIP report file using the built-in Windows XP/2003 .ZIP expansion

      utility then double-clicking "dsetreport.hta". Other GUI support files must also

      be decompressed into the folder in order for the DSET graphical web interface to

      display properly. The DSET .ZIP report file must be entirely decompressed to a

      folder first before opening "dsetreport.hta".



NOTE: Dsetreport.hta is considered an "HTML Application" for purposes of secure local

      operation on Windows. Double-clicking the file will launch the HTML rendering engine

      of Windows which looks very similar to Internet Explorer. Do not rename the file

      extension to ".htm" as unpredictable behavior may result.



NOTE: Netscape and Mozilla are NOT supported at this time for displaying the DSET

      graphical interface



NOTE: Various log and configuration files can be found categorized within the "log"

      folder of this report








dsetreport.hta









p0-3DY381S.TXT




TTY History for HA(0) -Bus 0x0a Device 0x06

���������������������������������������q���





T0: LSI Logic MegaRAID i960 firmware loaded


T0: Firmware version 351S build on May 31 2005 at 21:05:31


T0: Board is type 1000/1960/1028/0518





T0: Can_flush=0


T0: DRAM SIZE=128 MB


T0: DRAM_ALT sig invalid from previous boot


T0: FLUSH_ON_SYSTEM_RESET=0





T0: (C) LSI Logic Corporation 2002


T0: MegaRAID Series 518 firmware version 351S


T0: WAIT FOR BIOS....


T0: BIOS UP!


T0: Enabling data cache


T0: EepromInit: Family=14, SN=e7efc9010000


T0: EepromInit: Board SN not programmed





T0: Environment data:


T0: VALIDATION=None





T0: MFC data:


T0:     Vendor/DeviceID=1000/1960


T0:     SubVendor/SubDevice=1028/0518


T0:     OEM=Dell, clusterDisable=0, flexRaidDisable=0


T0:     rebuildRate=30, stripeSize=128, flushTime=4


T0:     cachedIo=1, writeBack=1, readAhead=0


T0:     channelBase='0', smartMode=6, alarmDisable=0


T0:     fastInitDisable=0, coercion=128M, disablePredictiveFail=0


T0:     disableWebBios=1, disableCtrlM=0, writeThroughWhenBatteryBad=1


T0:     zcrConfig=Undefined, keepSafteFailedStatus=0, autoHotSpareRestore=0


T0:     variableChkConRate=0, enableNvramDiskMgmtChange=0


T0:     dirtyLedShowsDriveActivity=0, disableConsChkRestoration=0


T0:     biosContinueOnError=0, biosAutoConfig=Prompt


T0:     disableRandomDriveDeletion=0





T0: DISK_CACHE_ADDR=d0db4800


T0: MEM_END_ADDR=d7fffff0


T0: Found MPT LVD 1030(1/5/1) at f7ff0004/0, mapped to 8bff0000


T0: Found MPT LVD 1030(1/5/0) at f7fd0004/0, mapped to 8bfd0000


T0: Total LSI MPT Chips found 2 


T0: LSI_InitMPT : start_index 0 totalLSIMPTChips 2


T0: Set to Get f/w features chip 0 


T0: 	Verifying Image Signatures...VERIFIED


T0: 	Verifying image check sum... VERIFIED


T0: The FW version being loaded is MPTFW-01.03.35.00-IT


T0: NextImageHeaderOffset=9c70, ExtImageSize=818


T0: FW download complete... Expecting LSI FW to start excute and come to ready state


T3: MISM CHN_STATE_MPT_GET_FW_FEAT chip 1 


T3: PRESENT SCSI_ID = 0


T3: Changing scsiId to  6


T3: Check IOC FACTS chip 1


T3: MISM CHN_STATE_MPT_OPERATIONAL chip 1 


T3: MISM: Reply frame size 60 start addr d05258c0 


T3: fe reply free frames posted


T3: MISM CHN_STATE_MPT_INIT_BUS_RST chip 1 


T3: MPT_Poll: chip 0 CHN_STATE_MPT_WAIT


T3: MISM CHN_STATE_MPT_GET_FW_FEAT chip 0 


T3: PRESENT SCSI_ID = 0


T3: Changing scsiId to  6


T3: Check IOC FACTS chip 0


T3: MISM CHN_STATE_MPT_OPERATIONAL chip 0 


T3: MISM: Reply frame size 60 start addr d052b880 


T3: fe reply free frames posted


T3: MISM CHN_STATE_MPT_INIT_BUS_RST chip 0 


T4: MPT_Poll: chip 1 CHN_STATE_MPT_INIT_BUS_RST 


T6: MPT_Poll: chip 0 CHN_STATE_MPT_INIT_BUS_RST 


T8: DISM: Queued!


T8: MPT_ProcessIo Reply Fr 2 EVENT_NOTIFICATION


T8: MPI_EVENT_EVENT_CHANGE


T8: MPT_SetIocPageParameters: After Write CoalescingDepth=1, Timeout=0, Flags=1 


T8: MPT_ProcessIo Reply Fr 2 EVENT_NOTIFICATION


T8: MPI_EVENT_EVENT_CHANGE


T8: MPT_SetIocPageParameters: After Write CoalescingDepth=1, Timeout=0, Flags=1 


T8: DISM: Cluster peer is 0x7


T8: DISM: oper_chn_bmap is 1, operating_chn: 0


T11: DISM_ProcessPprState: DomainVal done on all disks


T13: DISM: Complete!!!





T13: Physical device info:


T13: ID  NVRState  Vendor    Product           Rev    6   7  56


T13: --  --------  --------  ----------------  ----  --  --  --


T14: 00  Online    FUJITSU   MAP3735NC         5608  01  3a  0f


T14: 01  Online    FUJITSU   MAP3735NC         5608  01  3a  0f


T14: 02  Online    SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 03  HotSpare  SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 04  Online    SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 05  Online    SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 07  Ready     DELL      PERC 4/DC         350O  00  00  00


T14: 08  HotSpare  FUJITSU   MAT3073NC         5704  01  3a  0d


T14: 0f  Ready     DELL      PV22XS            E.17  40  00  00





T19: batteryinit: TransBBU pack present


T19: TBBU: TBBU h/w . Cache not dirty


T19: Verifying config struct at Addr e0001400


T19: NVRAM checksum OK - reading configuration


T19: DISK_CACHE_ADDR=d0db4800


T19: MEM_END_ADDR=d7fffff0


T19: Memory End d7fffff0


T19: Total memory available for disk cache: 724b3f0


T19: Total Number of Cache Lines 1824


T19: SS 128: mrs=5  lc=1824 ldc=1  ps=1 cm=ff ba=0 LDs: 0


T19: LD  0: L=5  SS=128  Size=10eec000  NL=1824  Status=2  DT=255  BT=512


T19:        span 0: sBlk=00000000, nBlk=043bb000, dev=00-01-02-05-04


T19: can_flush = 0


T19: No Reconst:Checking drive info


T19: MIGRATE: 40LD or 8ld new drive  ch 0 tgt 0


T19: REF drive found at ch 0 tgt 0


T19: Attempting to perform drive roaming


T19: NOT Flushing Cache


T19: RMW: NVRAM structure valid - checking for active RMWs


T19: Inside  SymRequestPoolInit


T19: Memory Pages USed=100


T19: Memory Pages USed=164


T19: RequestQ ADDR=d0210000


T19: exit   SymRequestPoolInit


T19: inside   SymMsgFifoInit


T19: InboundFreeQEnd=d0200000


T19: exit   SymMsgFifoInit


T19: inside   SymMsgFifoInit


T19: exit  SymMsgFifoInit


T19: Memory Pages USed=34a


T19: Memory Pages USed=34e


T19: Memory Pages USed=34f


T19: Memory Pages USed=374


T19: found 1 logical drives


T19: logDrv=0  Size=10eec000


09/29 14:29:53: Time established at T45


09/29 14:29:53: BIOS CALL FOR DRV ROAMING : 55


09/29 14:29:53: drive roaming not done


09/29 14:30:24: HOST ISR: Sync cmd 0 arrived


09/29 14:31:31: LD 2 RdlongSize=22a #diff=16


09/29 14:31:31: LD 3 RdlongSize=22a #diff=16


09/29 14:31:32: LD 4 RdlongSize=22a #diff=16


09/29 14:31:32: LD 5 RdlongSize=22a #diff=16


09/29 14:31:32: LD 8 RdlongSize=23a #diff=6


09/29 14:32:10: LD 2 RdlongSize=22a #diff=16


09/29 14:32:10: LD 3 RdlongSize=22a #diff=16


09/29 14:32:10: LD 4 RdlongSize=22a #diff=16


09/29 14:32:10: LD 5 RdlongSize=22a #diff=16


09/29 14:32:10: LD 8 RdlongSize=23a #diff=6


09/29 14:32:24: LD 2 RdlongSize=22a #diff=16


09/29 14:32:24: LD 3 RdlongSize=22a #diff=16


09/29 14:32:24: LD 4 RdlongSize=22a #diff=16


09/29 14:32:24: LD 5 RdlongSize=22a #diff=16


09/29 14:32:25: LD 8 RdlongSize=23a #diff=6


�������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������









PO-3DY381S.TXT


TTY History for HA(0) -Bus 0x0a Device 0x06

������������������������������������

TTY History for HA(0) -Bus 0x0a Device 0x06

���������������������������������������q���





T0: LSI Logic MegaRAID i960 firmware loaded


T0: Firmware version 351S build on May 31 2005 at 21:05:31


T0: Board is type 1000/1960/1028/0518





T0: Can_flush=0


T0: DRAM SIZE=128 MB


T0: DRAM_ALT sig invalid from previous boot


T0: FLUSH_ON_SYSTEM_RESET=0





T0: (C) LSI Logic Corporation 2002


T0: MegaRAID Series 518 firmware version 351S


T0: WAIT FOR BIOS....


T0: BIOS UP!


T0: Enabling data cache


T0: EepromInit: Family=14, SN=e7efc9010000


T0: EepromInit: Board SN not programmed





T0: Environment data:


T0: VALIDATION=None





T0: MFC data:


T0:     Vendor/DeviceID=1000/1960


T0:     SubVendor/SubDevice=1028/0518


T0:     OEM=Dell, clusterDisable=0, flexRaidDisable=0


T0:     rebuildRate=30, stripeSize=128, flushTime=4


T0:     cachedIo=1, writeBack=1, readAhead=0


T0:     channelBase='0', smartMode=6, alarmDisable=0


T0:     fastInitDisable=0, coercion=128M, disablePredictiveFail=0


T0:     disableWebBios=1, disableCtrlM=0, writeThroughWhenBatteryBad=1


T0:     zcrConfig=Undefined, keepSafteFailedStatus=0, autoHotSpareRestore=0


T0:     variableChkConRate=0, enableNvramDiskMgmtChange=0


T0:     dirtyLedShowsDriveActivity=0, disableConsChkRestoration=0


T0:     biosContinueOnError=0, biosAutoConfig=Prompt


T0:     disableRandomDriveDeletion=0





T0: DISK_CACHE_ADDR=d0db4800


T0: MEM_END_ADDR=d7fffff0


T0: Found MPT LVD 1030(1/5/1) at f7ff0004/0, mapped to 8bff0000


T0: Found MPT LVD 1030(1/5/0) at f7fd0004/0, mapped to 8bfd0000


T0: Total LSI MPT Chips found 2 


T0: LSI_InitMPT : start_index 0 totalLSIMPTChips 2


T0: Set to Get f/w features chip 0 


T0: 	Verifying Image Signatures...VERIFIED


T0: 	Verifying image check sum... VERIFIED


T0: The FW version being loaded is MPTFW-01.03.35.00-IT


T0: NextImageHeaderOffset=9c70, ExtImageSize=818


T0: FW download complete... Expecting LSI FW to start excute and come to ready state


T3: MISM CHN_STATE_MPT_GET_FW_FEAT chip 1 


T3: PRESENT SCSI_ID = 0


T3: Changing scsiId to  6


T3: Check IOC FACTS chip 1


T3: MISM CHN_STATE_MPT_OPERATIONAL chip 1 


T3: MISM: Reply frame size 60 start addr d05258c0 


T3: fe reply free frames posted


T3: MISM CHN_STATE_MPT_INIT_BUS_RST chip 1 


T3: MPT_Poll: chip 0 CHN_STATE_MPT_WAIT


T3: MISM CHN_STATE_MPT_GET_FW_FEAT chip 0 


T3: PRESENT SCSI_ID = 0


T3: Changing scsiId to  6


T3: Check IOC FACTS chip 0


T3: MISM CHN_STATE_MPT_OPERATIONAL chip 0 


T3: MISM: Reply frame size 60 start addr d052b880 


T3: fe reply free frames posted


T3: MISM CHN_STATE_MPT_INIT_BUS_RST chip 0 


T4: MPT_Poll: chip 1 CHN_STATE_MPT_INIT_BUS_RST 


T6: MPT_Poll: chip 0 CHN_STATE_MPT_INIT_BUS_RST 


T8: DISM: Queued!


T8: MPT_ProcessIo Reply Fr 2 EVENT_NOTIFICATION


T8: MPI_EVENT_EVENT_CHANGE


T8: MPT_SetIocPageParameters: After Write CoalescingDepth=1, Timeout=0, Flags=1 


T8: MPT_ProcessIo Reply Fr 2 EVENT_NOTIFICATION


T8: MPI_EVENT_EVENT_CHANGE


T8: MPT_SetIocPageParameters: After Write CoalescingDepth=1, Timeout=0, Flags=1 


T8: DISM: Cluster peer is 0x7


T8: DISM: oper_chn_bmap is 1, operating_chn: 0


T11: DISM_ProcessPprState: DomainVal done on all disks


T13: DISM: Complete!!!





T13: Physical device info:


T13: ID  NVRState  Vendor    Product           Rev    6   7  56


T13: --  --------  --------  ----------------  ----  --  --  --


T14: 00  Online    FUJITSU   MAP3735NC         5608  01  3a  0f


T14: 01  Online    FUJITSU   MAP3735NC         5608  01  3a  0f


T14: 02  Online    SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 03  HotSpare  SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 04  Online    SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 05  Online    SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 07  Ready     DELL      PERC 4/DC         350O  00  00  00


T14: 08  HotSpare  FUJITSU   MAT3073NC         5704  01  3a  0d


T14: 0f  Ready     DELL      PV22XS            E.17  40  00  00





T19: batteryinit: TransBBU pack present


T19: TBBU: TBBU h/w . Cache not dirty


T19: Verifying config struct at Addr e0001400


T19: NVRAM checksum OK - reading configuration


T19: DISK_CACHE_ADDR=d0db4800


T19: MEM_END_ADDR=d7fffff0


T19: Memory End d7fffff0


T19: Total memory available for disk cache: 724b3f0


T19: Total Number of Cache Lines 1824


T19: SS 128: mrs=5  lc=1824 ldc=1  ps=1 cm=ff ba=0 LDs: 0


T19: LD  0: L=5  SS=128  Size=10eec000  NL=1824  Status=2  DT=255  BT=512


T19:        span 0: sBlk=00000000, nBlk=043bb000, dev=00-01-02-05-04


T19: can_flush = 0


T19: No Reconst:Checking drive info


T19: MIGRATE: 40LD or 8ld new drive  ch 0 tgt 0


T19: REF drive found at ch 0 tgt 0


T19: Attempting to perform drive roaming


T19: NOT Flushing Cache


T19: RMW: NVRAM structure valid - checking for active RMWs


T19: Inside  SymRequestPoolInit


T19: Memory Pages USed=100


T19: Memory Pages USed=164


T19: RequestQ ADDR=d0210000


T19: exit   SymRequestPoolInit


T19: inside   SymMsgFifoInit


T19: InboundFreeQEnd=d0200000


T19: exit   SymMsgFifoInit


T19: inside   SymMsgFifoInit


T19: exit  SymMsgFifoInit


T19: Memory Pages USed=34a


T19: Memory Pages USed=34e


T19: Memory Pages USed=34f


T19: Memory Pages USed=374


T19: found 1 logical drives


T19: logDrv=0  Size=10eec000


09/29 14:29:53: Time established at T45


09/29 14:29:53: BIOS CALL FOR DRV ROAMING : 55


09/29 14:29:53: drive roaming not done


09/29 14:30:24: HOST ISR: Sync cmd 0 arrived


09/29 14:31:31: LD 2 RdlongSize=22a #diff=16


09/29 14:31:31: LD 3 RdlongSize=22a #diff=16


09/29 14:31:32: LD 4 RdlongSize=22a #diff=16


09/29 14:31:32: LD 5 RdlongSize=22a #diff=16


09/29 14:31:32: LD 8 RdlongSize=23a #diff=6


09/29 14:32:10: LD 2 RdlongSize=22a #diff=16


09/29 14:32:10: LD 3 RdlongSize=22a #diff=16


09/29 14:32:10: LD 4 RdlongSize=22a #diff=16


09/29 14:32:10: LD 5 RdlongSize=22a #diff=16


09/29 14:32:10: LD 8 RdlongSize=23a #diff=6


09/29 14:32:24: LD 2 RdlongSize=22a #diff=16


09/29 14:32:24: LD 3 RdlongSize=22a #diff=16


09/29 14:32:24: LD 4 RdlongSize=22a #diff=16


09/29 14:32:24: LD 5 RdlongSize=22a #diff=16


09/29 14:32:25: LD 8 RdlongSize=23a #diff=6


�������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������









tty-3DY381S.log




TTY History for HA(0) -Bus 0x0a Device 0x06

���������������������������������������q���





T0: LSI Logic MegaRAID i960 firmware loaded


T0: Firmware version 351S build on May 31 2005 at 21:05:31


T0: Board is type 1000/1960/1028/0518





T0: Can_flush=0


T0: DRAM SIZE=128 MB


T0: DRAM_ALT sig invalid from previous boot


T0: FLUSH_ON_SYSTEM_RESET=0





T0: (C) LSI Logic Corporation 2002


T0: MegaRAID Series 518 firmware version 351S


T0: WAIT FOR BIOS....


T0: BIOS UP!


T0: Enabling data cache


T0: EepromInit: Family=14, SN=e7efc9010000


T0: EepromInit: Board SN not programmed





T0: Environment data:


T0: VALIDATION=None





T0: MFC data:


T0:     Vendor/DeviceID=1000/1960


T0:     SubVendor/SubDevice=1028/0518


T0:     OEM=Dell, clusterDisable=0, flexRaidDisable=0


T0:     rebuildRate=30, stripeSize=128, flushTime=4


T0:     cachedIo=1, writeBack=1, readAhead=0


T0:     channelBase='0', smartMode=6, alarmDisable=0


T0:     fastInitDisable=0, coercion=128M, disablePredictiveFail=0


T0:     disableWebBios=1, disableCtrlM=0, writeThroughWhenBatteryBad=1


T0:     zcrConfig=Undefined, keepSafteFailedStatus=0, autoHotSpareRestore=0


T0:     variableChkConRate=0, enableNvramDiskMgmtChange=0


T0:     dirtyLedShowsDriveActivity=0, disableConsChkRestoration=0


T0:     biosContinueOnError=0, biosAutoConfig=Prompt


T0:     disableRandomDriveDeletion=0





T0: DISK_CACHE_ADDR=d0db4800


T0: MEM_END_ADDR=d7fffff0


T0: Found MPT LVD 1030(1/5/1) at f7ff0004/0, mapped to 8bff0000


T0: Found MPT LVD 1030(1/5/0) at f7fd0004/0, mapped to 8bfd0000


T0: Total LSI MPT Chips found 2 


T0: LSI_InitMPT : start_index 0 totalLSIMPTChips 2


T0: Set to Get f/w features chip 0 


T0: 	Verifying Image Signatures...VERIFIED


T0: 	Verifying image check sum... VERIFIED


T0: The FW version being loaded is MPTFW-01.03.35.00-IT


T0: NextImageHeaderOffset=9c70, ExtImageSize=818


T0: FW download complete... Expecting LSI FW to start excute and come to ready state


T3: MISM CHN_STATE_MPT_GET_FW_FEAT chip 1 


T3: PRESENT SCSI_ID = 0


T3: Changing scsiId to  6


T3: Check IOC FACTS chip 1


T3: MISM CHN_STATE_MPT_OPERATIONAL chip 1 


T3: MISM: Reply frame size 60 start addr d05258c0 


T3: fe reply free frames posted


T3: MISM CHN_STATE_MPT_INIT_BUS_RST chip 1 


T3: MPT_Poll: chip 0 CHN_STATE_MPT_WAIT


T3: MISM CHN_STATE_MPT_GET_FW_FEAT chip 0 


T3: PRESENT SCSI_ID = 0


T3: Changing scsiId to  6


T3: Check IOC FACTS chip 0


T3: MISM CHN_STATE_MPT_OPERATIONAL chip 0 


T3: MISM: Reply frame size 60 start addr d052b880 


T3: fe reply free frames posted


T3: MISM CHN_STATE_MPT_INIT_BUS_RST chip 0 


T4: MPT_Poll: chip 1 CHN_STATE_MPT_INIT_BUS_RST 


T6: MPT_Poll: chip 0 CHN_STATE_MPT_INIT_BUS_RST 


T8: DISM: Queued!


T8: MPT_ProcessIo Reply Fr 2 EVENT_NOTIFICATION


T8: MPI_EVENT_EVENT_CHANGE


T8: MPT_SetIocPageParameters: After Write CoalescingDepth=1, Timeout=0, Flags=1 


T8: MPT_ProcessIo Reply Fr 2 EVENT_NOTIFICATION


T8: MPI_EVENT_EVENT_CHANGE


T8: MPT_SetIocPageParameters: After Write CoalescingDepth=1, Timeout=0, Flags=1 


T8: DISM: Cluster peer is 0x7


T8: DISM: oper_chn_bmap is 1, operating_chn: 0


T11: DISM_ProcessPprState: DomainVal done on all disks


T13: DISM: Complete!!!





T13: Physical device info:


T13: ID  NVRState  Vendor    Product           Rev    6   7  56


T13: --  --------  --------  ----------------  ----  --  --  --


T14: 00  Online    FUJITSU   MAP3735NC         5608  01  3a  0f


T14: 01  Online    FUJITSU   MAP3735NC         5608  01  3a  0f


T14: 02  Online    SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 03  HotSpare  SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 04  Online    SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 05  Online    SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 07  Ready     DELL      PERC 4/DC         350O  00  00  00


T14: 08  HotSpare  FUJITSU   MAT3073NC         5704  01  3a  0d


T14: 0f  Ready     DELL      PV22XS            E.17  40  00  00





T19: batteryinit: TransBBU pack present


T19: TBBU: TBBU h/w . Cache not dirty


T19: Verifying config struct at Addr e0001400


T19: NVRAM checksum OK - reading configuration


T19: DISK_CACHE_ADDR=d0db4800


T19: MEM_END_ADDR=d7fffff0


T19: Memory End d7fffff0


T19: Total memory available for disk cache: 724b3f0


T19: Total Number of Cache Lines 1824


T19: SS 128: mrs=5  lc=1824 ldc=1  ps=1 cm=ff ba=0 LDs: 0


T19: LD  0: L=5  SS=128  Size=10eec000  NL=1824  Status=2  DT=255  BT=512


T19:        span 0: sBlk=00000000, nBlk=043bb000, dev=00-01-02-05-04


T19: can_flush = 0


T19: No Reconst:Checking drive info


T19: MIGRATE: 40LD or 8ld new drive  ch 0 tgt 0


T19: REF drive found at ch 0 tgt 0


T19: Attempting to perform drive roaming


T19: NOT Flushing Cache


T19: RMW: NVRAM structure valid - checking for active RMWs


T19: Inside  SymRequestPoolInit


T19: Memory Pages USed=100


T19: Memory Pages USed=164


T19: RequestQ ADDR=d0210000


T19: exit   SymRequestPoolInit


T19: inside   SymMsgFifoInit


T19: InboundFreeQEnd=d0200000


T19: exit   SymMsgFifoInit


T19: inside   SymMsgFifoInit


T19: exit  SymMsgFifoInit


T19: Memory Pages USed=34a


T19: Memory Pages USed=34e


T19: Memory Pages USed=34f


T19: Memory Pages USed=374


T19: found 1 logical drives


T19: logDrv=0  Size=10eec000


09/29 14:29:53: Time established at T45


09/29 14:29:53: BIOS CALL FOR DRV ROAMING : 55


09/29 14:29:53: drive roaming not done


09/29 14:30:24: HOST ISR: Sync cmd 0 arrived


09/29 14:31:31: LD 2 RdlongSize=22a #diff=16


09/29 14:31:31: LD 3 RdlongSize=22a #diff=16


09/29 14:31:32: LD 4 RdlongSize=22a #diff=16


09/29 14:31:32: LD 5 RdlongSize=22a #diff=16


09/29 14:31:32: LD 8 RdlongSize=23a #diff=6


09/29 14:32:10: LD 2 RdlongSize=22a #diff=16


09/29 14:32:10: LD 3 RdlongSize=22a #diff=16


09/29 14:32:10: LD 4 RdlongSize=22a #diff=16


09/29 14:32:10: LD 5 RdlongSize=22a #diff=16


09/29 14:32:10: LD 8 RdlongSize=23a #diff=6


09/29 14:32:24: LD 2 RdlongSize=22a #diff=16


09/29 14:32:24: LD 3 RdlongSize=22a #diff=16


09/29 14:32:24: LD 4 RdlongSize=22a #diff=16


09/29 14:32:24: LD 5 RdlongSize=22a #diff=16


09/29 14:32:25: LD 8 RdlongSize=23a #diff=6


�������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������TTY History for HA(0) -Bus 0x0a Device 0x06

������������������������������������

TTY History for HA(0) -Bus 0x0a Device 0x06

���������������������������������������q���





T0: LSI Logic MegaRAID i960 firmware loaded


T0: Firmware version 351S build on May 31 2005 at 21:05:31


T0: Board is type 1000/1960/1028/0518





T0: Can_flush=0


T0: DRAM SIZE=128 MB


T0: DRAM_ALT sig invalid from previous boot


T0: FLUSH_ON_SYSTEM_RESET=0





T0: (C) LSI Logic Corporation 2002


T0: MegaRAID Series 518 firmware version 351S


T0: WAIT FOR BIOS....


T0: BIOS UP!


T0: Enabling data cache


T0: EepromInit: Family=14, SN=e7efc9010000


T0: EepromInit: Board SN not programmed





T0: Environment data:


T0: VALIDATION=None





T0: MFC data:


T0:     Vendor/DeviceID=1000/1960


T0:     SubVendor/SubDevice=1028/0518


T0:     OEM=Dell, clusterDisable=0, flexRaidDisable=0


T0:     rebuildRate=30, stripeSize=128, flushTime=4


T0:     cachedIo=1, writeBack=1, readAhead=0


T0:     channelBase='0', smartMode=6, alarmDisable=0


T0:     fastInitDisable=0, coercion=128M, disablePredictiveFail=0


T0:     disableWebBios=1, disableCtrlM=0, writeThroughWhenBatteryBad=1


T0:     zcrConfig=Undefined, keepSafteFailedStatus=0, autoHotSpareRestore=0


T0:     variableChkConRate=0, enableNvramDiskMgmtChange=0


T0:     dirtyLedShowsDriveActivity=0, disableConsChkRestoration=0


T0:     biosContinueOnError=0, biosAutoConfig=Prompt


T0:     disableRandomDriveDeletion=0





T0: DISK_CACHE_ADDR=d0db4800


T0: MEM_END_ADDR=d7fffff0


T0: Found MPT LVD 1030(1/5/1) at f7ff0004/0, mapped to 8bff0000


T0: Found MPT LVD 1030(1/5/0) at f7fd0004/0, mapped to 8bfd0000


T0: Total LSI MPT Chips found 2 


T0: LSI_InitMPT : start_index 0 totalLSIMPTChips 2


T0: Set to Get f/w features chip 0 


T0: 	Verifying Image Signatures...VERIFIED


T0: 	Verifying image check sum... VERIFIED


T0: The FW version being loaded is MPTFW-01.03.35.00-IT


T0: NextImageHeaderOffset=9c70, ExtImageSize=818


T0: FW download complete... Expecting LSI FW to start excute and come to ready state


T3: MISM CHN_STATE_MPT_GET_FW_FEAT chip 1 


T3: PRESENT SCSI_ID = 0


T3: Changing scsiId to  6


T3: Check IOC FACTS chip 1


T3: MISM CHN_STATE_MPT_OPERATIONAL chip 1 


T3: MISM: Reply frame size 60 start addr d05258c0 


T3: fe reply free frames posted


T3: MISM CHN_STATE_MPT_INIT_BUS_RST chip 1 


T3: MPT_Poll: chip 0 CHN_STATE_MPT_WAIT


T3: MISM CHN_STATE_MPT_GET_FW_FEAT chip 0 


T3: PRESENT SCSI_ID = 0


T3: Changing scsiId to  6


T3: Check IOC FACTS chip 0


T3: MISM CHN_STATE_MPT_OPERATIONAL chip 0 


T3: MISM: Reply frame size 60 start addr d052b880 


T3: fe reply free frames posted


T3: MISM CHN_STATE_MPT_INIT_BUS_RST chip 0 


T4: MPT_Poll: chip 1 CHN_STATE_MPT_INIT_BUS_RST 


T6: MPT_Poll: chip 0 CHN_STATE_MPT_INIT_BUS_RST 


T8: DISM: Queued!


T8: MPT_ProcessIo Reply Fr 2 EVENT_NOTIFICATION


T8: MPI_EVENT_EVENT_CHANGE


T8: MPT_SetIocPageParameters: After Write CoalescingDepth=1, Timeout=0, Flags=1 


T8: MPT_ProcessIo Reply Fr 2 EVENT_NOTIFICATION


T8: MPI_EVENT_EVENT_CHANGE


T8: MPT_SetIocPageParameters: After Write CoalescingDepth=1, Timeout=0, Flags=1 


T8: DISM: Cluster peer is 0x7


T8: DISM: oper_chn_bmap is 1, operating_chn: 0


T11: DISM_ProcessPprState: DomainVal done on all disks


T13: DISM: Complete!!!





T13: Physical device info:


T13: ID  NVRState  Vendor    Product           Rev    6   7  56


T13: --  --------  --------  ----------------  ----  --  --  --


T14: 00  Online    FUJITSU   MAP3735NC         5608  01  3a  0f


T14: 01  Online    FUJITSU   MAP3735NC         5608  01  3a  0f


T14: 02  Online    SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 03  HotSpare  SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 04  Online    SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 05  Online    SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 07  Ready     DELL      PERC 4/DC         350O  00  00  00


T14: 08  HotSpare  FUJITSU   MAT3073NC         5704  01  3a  0d


T14: 0f  Ready     DELL      PV22XS            E.17  40  00  00





T19: batteryinit: TransBBU pack present


T19: TBBU: TBBU h/w . Cache not dirty


T19: Verifying config struct at Addr e0001400


T19: NVRAM checksum OK - reading configuration


T19: DISK_CACHE_ADDR=d0db4800


T19: MEM_END_ADDR=d7fffff0


T19: Memory End d7fffff0


T19: Total memory available for disk cache: 724b3f0


T19: Total Number of Cache Lines 1824


T19: SS 128: mrs=5  lc=1824 ldc=1  ps=1 cm=ff ba=0 LDs: 0


T19: LD  0: L=5  SS=128  Size=10eec000  NL=1824  Status=2  DT=255  BT=512


T19:        span 0: sBlk=00000000, nBlk=043bb000, dev=00-01-02-05-04


T19: can_flush = 0


T19: No Reconst:Checking drive info


T19: MIGRATE: 40LD or 8ld new drive  ch 0 tgt 0


T19: REF drive found at ch 0 tgt 0


T19: Attempting to perform drive roaming


T19: NOT Flushing Cache


T19: RMW: NVRAM structure valid - checking for active RMWs


T19: Inside  SymRequestPoolInit


T19: Memory Pages USed=100


T19: Memory Pages USed=164


T19: RequestQ ADDR=d0210000


T19: exit   SymRequestPoolInit


T19: inside   SymMsgFifoInit


T19: InboundFreeQEnd=d0200000


T19: exit   SymMsgFifoInit


T19: inside   SymMsgFifoInit


T19: exit  SymMsgFifoInit


T19: Memory Pages USed=34a


T19: Memory Pages USed=34e


T19: Memory Pages USed=34f


T19: Memory Pages USed=374


T19: found 1 logical drives


T19: logDrv=0  Size=10eec000


09/29 14:29:53: Time established at T45


09/29 14:29:53: BIOS CALL FOR DRV ROAMING : 55


09/29 14:29:53: drive roaming not done


09/29 14:30:24: HOST ISR: Sync cmd 0 arrived


09/29 14:31:31: LD 2 RdlongSize=22a #diff=16


09/29 14:31:31: LD 3 RdlongSize=22a #diff=16


09/29 14:31:32: LD 4 RdlongSize=22a #diff=16


09/29 14:31:32: LD 5 RdlongSize=22a #diff=16


09/29 14:31:32: LD 8 RdlongSize=23a #diff=6


09/29 14:32:10: LD 2 RdlongSize=22a #diff=16


09/29 14:32:10: LD 3 RdlongSize=22a #diff=16


09/29 14:32:10: LD 4 RdlongSize=22a #diff=16


09/29 14:32:10: LD 5 RdlongSize=22a #diff=16


09/29 14:32:10: LD 8 RdlongSize=23a #diff=6


09/29 14:32:24: LD 2 RdlongSize=22a #diff=16


09/29 14:32:24: LD 3 RdlongSize=22a #diff=16


09/29 14:32:24: LD 4 RdlongSize=22a #diff=16


09/29 14:32:24: LD 5 RdlongSize=22a #diff=16


09/29 14:32:25: LD 8 RdlongSize=23a #diff=6


�������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������









tty-3DY381S_0.log


TTY History for HA(0) -Bus 0x0a Device 0x06

������������������������������������

TTY History for HA(0) -Bus 0x0a Device 0x06

���������������������������������������q���





T0: LSI Logic MegaRAID i960 firmware loaded


T0: Firmware version 351S build on May 31 2005 at 21:05:31


T0: Board is type 1000/1960/1028/0518





T0: Can_flush=0


T0: DRAM SIZE=128 MB


T0: DRAM_ALT sig invalid from previous boot


T0: FLUSH_ON_SYSTEM_RESET=0





T0: (C) LSI Logic Corporation 2002


T0: MegaRAID Series 518 firmware version 351S


T0: WAIT FOR BIOS....


T0: BIOS UP!


T0: Enabling data cache


T0: EepromInit: Family=14, SN=e7efc9010000


T0: EepromInit: Board SN not programmed





T0: Environment data:


T0: VALIDATION=None





T0: MFC data:


T0:     Vendor/DeviceID=1000/1960


T0:     SubVendor/SubDevice=1028/0518


T0:     OEM=Dell, clusterDisable=0, flexRaidDisable=0


T0:     rebuildRate=30, stripeSize=128, flushTime=4


T0:     cachedIo=1, writeBack=1, readAhead=0


T0:     channelBase='0', smartMode=6, alarmDisable=0


T0:     fastInitDisable=0, coercion=128M, disablePredictiveFail=0


T0:     disableWebBios=1, disableCtrlM=0, writeThroughWhenBatteryBad=1


T0:     zcrConfig=Undefined, keepSafteFailedStatus=0, autoHotSpareRestore=0


T0:     variableChkConRate=0, enableNvramDiskMgmtChange=0


T0:     dirtyLedShowsDriveActivity=0, disableConsChkRestoration=0


T0:     biosContinueOnError=0, biosAutoConfig=Prompt


T0:     disableRandomDriveDeletion=0





T0: DISK_CACHE_ADDR=d0db4800


T0: MEM_END_ADDR=d7fffff0


T0: Found MPT LVD 1030(1/5/1) at f7ff0004/0, mapped to 8bff0000


T0: Found MPT LVD 1030(1/5/0) at f7fd0004/0, mapped to 8bfd0000


T0: Total LSI MPT Chips found 2 


T0: LSI_InitMPT : start_index 0 totalLSIMPTChips 2


T0: Set to Get f/w features chip 0 


T0: 	Verifying Image Signatures...VERIFIED


T0: 	Verifying image check sum... VERIFIED


T0: The FW version being loaded is MPTFW-01.03.35.00-IT


T0: NextImageHeaderOffset=9c70, ExtImageSize=818


T0: FW download complete... Expecting LSI FW to start excute and come to ready state


T3: MISM CHN_STATE_MPT_GET_FW_FEAT chip 1 


T3: PRESENT SCSI_ID = 0


T3: Changing scsiId to  6


T3: Check IOC FACTS chip 1


T3: MISM CHN_STATE_MPT_OPERATIONAL chip 1 


T3: MISM: Reply frame size 60 start addr d05258c0 


T3: fe reply free frames posted


T3: MISM CHN_STATE_MPT_INIT_BUS_RST chip 1 


T3: MPT_Poll: chip 0 CHN_STATE_MPT_WAIT


T3: MISM CHN_STATE_MPT_GET_FW_FEAT chip 0 


T3: PRESENT SCSI_ID = 0


T3: Changing scsiId to  6


T3: Check IOC FACTS chip 0


T3: MISM CHN_STATE_MPT_OPERATIONAL chip 0 


T3: MISM: Reply frame size 60 start addr d052b880 


T3: fe reply free frames posted


T3: MISM CHN_STATE_MPT_INIT_BUS_RST chip 0 


T4: MPT_Poll: chip 1 CHN_STATE_MPT_INIT_BUS_RST 


T6: MPT_Poll: chip 0 CHN_STATE_MPT_INIT_BUS_RST 


T8: DISM: Queued!


T8: MPT_ProcessIo Reply Fr 2 EVENT_NOTIFICATION


T8: MPI_EVENT_EVENT_CHANGE


T8: MPT_SetIocPageParameters: After Write CoalescingDepth=1, Timeout=0, Flags=1 


T8: MPT_ProcessIo Reply Fr 2 EVENT_NOTIFICATION


T8: MPI_EVENT_EVENT_CHANGE


T8: MPT_SetIocPageParameters: After Write CoalescingDepth=1, Timeout=0, Flags=1 


T8: DISM: Cluster peer is 0x7


T8: DISM: oper_chn_bmap is 1, operating_chn: 0


T11: DISM_ProcessPprState: DomainVal done on all disks


T13: DISM: Complete!!!





T13: Physical device info:


T13: ID  NVRState  Vendor    Product           Rev    6   7  56


T13: --  --------  --------  ----------------  ----  --  --  --


T14: 00  Online    FUJITSU   MAP3735NC         5608  01  3a  0f


T14: 01  Online    FUJITSU   MAP3735NC         5608  01  3a  0f


T14: 02  Online    SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 03  HotSpare  SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 04  Online    SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 05  Online    SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 07  Ready     DELL      PERC 4/DC         350O  00  00  00


T14: 08  HotSpare  FUJITSU   MAT3073NC         5704  01  3a  0d


T14: 0f  Ready     DELL      PV22XS            E.17  40  00  00





T19: batteryinit: TransBBU pack present


T19: TBBU: TBBU h/w . Cache not dirty


T19: Verifying config struct at Addr e0001400


T19: NVRAM checksum OK - reading configuration


T19: DISK_CACHE_ADDR=d0db4800


T19: MEM_END_ADDR=d7fffff0


T19: Memory End d7fffff0


T19: Total memory available for disk cache: 724b3f0


T19: Total Number of Cache Lines 1824


T19: SS 128: mrs=5  lc=1824 ldc=1  ps=1 cm=ff ba=0 LDs: 0


T19: LD  0: L=5  SS=128  Size=10eec000  NL=1824  Status=2  DT=255  BT=512


T19:        span 0: sBlk=00000000, nBlk=043bb000, dev=00-01-02-05-04


T19: can_flush = 0


T19: No Reconst:Checking drive info


T19: MIGRATE: 40LD or 8ld new drive  ch 0 tgt 0


T19: REF drive found at ch 0 tgt 0


T19: Attempting to perform drive roaming


T19: NOT Flushing Cache


T19: RMW: NVRAM structure valid - checking for active RMWs


T19: Inside  SymRequestPoolInit


T19: Memory Pages USed=100


T19: Memory Pages USed=164


T19: RequestQ ADDR=d0210000


T19: exit   SymRequestPoolInit


T19: inside   SymMsgFifoInit


T19: InboundFreeQEnd=d0200000


T19: exit   SymMsgFifoInit


T19: inside   SymMsgFifoInit


T19: exit  SymMsgFifoInit


T19: Memory Pages USed=34a


T19: Memory Pages USed=34e


T19: Memory Pages USed=34f


T19: Memory Pages USed=374


T19: found 1 logical drives


T19: logDrv=0  Size=10eec000


09/29 14:29:53: Time established at T45


09/29 14:29:53: BIOS CALL FOR DRV ROAMING : 55


09/29 14:29:53: drive roaming not done


09/29 14:30:24: HOST ISR: Sync cmd 0 arrived


09/29 14:31:31: LD 2 RdlongSize=22a #diff=16


09/29 14:31:31: LD 3 RdlongSize=22a #diff=16


09/29 14:31:32: LD 4 RdlongSize=22a #diff=16


09/29 14:31:32: LD 5 RdlongSize=22a #diff=16


09/29 14:31:32: LD 8 RdlongSize=23a #diff=6


09/29 14:32:10: LD 2 RdlongSize=22a #diff=16


09/29 14:32:10: LD 3 RdlongSize=22a #diff=16


09/29 14:32:10: LD 4 RdlongSize=22a #diff=16


09/29 14:32:10: LD 5 RdlongSize=22a #diff=16


09/29 14:32:10: LD 8 RdlongSize=23a #diff=6


09/29 14:32:24: LD 2 RdlongSize=22a #diff=16


09/29 14:32:24: LD 3 RdlongSize=22a #diff=16


09/29 14:32:24: LD 4 RdlongSize=22a #diff=16


09/29 14:32:24: LD 5 RdlongSize=22a #diff=16


09/29 14:32:25: LD 8 RdlongSize=23a #diff=6


�������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������TTY History for HA(0) -Bus 0x0a Device 0x06

������������������������������������

TTY History for HA(0) -Bus 0x0a Device 0x06

���������������������������������������q���





T0: LSI Logic MegaRAID i960 firmware loaded


T0: Firmware version 351S build on May 31 2005 at 21:05:31


T0: Board is type 1000/1960/1028/0518





T0: Can_flush=0


T0: DRAM SIZE=128 MB


T0: DRAM_ALT sig invalid from previous boot


T0: FLUSH_ON_SYSTEM_RESET=0





T0: (C) LSI Logic Corporation 2002


T0: MegaRAID Series 518 firmware version 351S


T0: WAIT FOR BIOS....


T0: BIOS UP!


T0: Enabling data cache


T0: EepromInit: Family=14, SN=e7efc9010000


T0: EepromInit: Board SN not programmed





T0: Environment data:


T0: VALIDATION=None





T0: MFC data:


T0:     Vendor/DeviceID=1000/1960


T0:     SubVendor/SubDevice=1028/0518


T0:     OEM=Dell, clusterDisable=0, flexRaidDisable=0


T0:     rebuildRate=30, stripeSize=128, flushTime=4


T0:     cachedIo=1, writeBack=1, readAhead=0


T0:     channelBase='0', smartMode=6, alarmDisable=0


T0:     fastInitDisable=0, coercion=128M, disablePredictiveFail=0


T0:     disableWebBios=1, disableCtrlM=0, writeThroughWhenBatteryBad=1


T0:     zcrConfig=Undefined, keepSafteFailedStatus=0, autoHotSpareRestore=0


T0:     variableChkConRate=0, enableNvramDiskMgmtChange=0


T0:     dirtyLedShowsDriveActivity=0, disableConsChkRestoration=0


T0:     biosContinueOnError=0, biosAutoConfig=Prompt


T0:     disableRandomDriveDeletion=0





T0: DISK_CACHE_ADDR=d0db4800


T0: MEM_END_ADDR=d7fffff0


T0: Found MPT LVD 1030(1/5/1) at f7ff0004/0, mapped to 8bff0000


T0: Found MPT LVD 1030(1/5/0) at f7fd0004/0, mapped to 8bfd0000


T0: Total LSI MPT Chips found 2 


T0: LSI_InitMPT : start_index 0 totalLSIMPTChips 2


T0: Set to Get f/w features chip 0 


T0: 	Verifying Image Signatures...VERIFIED


T0: 	Verifying image check sum... VERIFIED


T0: The FW version being loaded is MPTFW-01.03.35.00-IT


T0: NextImageHeaderOffset=9c70, ExtImageSize=818


T0: FW download complete... Expecting LSI FW to start excute and come to ready state


T3: MISM CHN_STATE_MPT_GET_FW_FEAT chip 1 


T3: PRESENT SCSI_ID = 0


T3: Changing scsiId to  6


T3: Check IOC FACTS chip 1


T3: MISM CHN_STATE_MPT_OPERATIONAL chip 1 


T3: MISM: Reply frame size 60 start addr d05258c0 


T3: fe reply free frames posted


T3: MISM CHN_STATE_MPT_INIT_BUS_RST chip 1 


T3: MPT_Poll: chip 0 CHN_STATE_MPT_WAIT


T3: MISM CHN_STATE_MPT_GET_FW_FEAT chip 0 


T3: PRESENT SCSI_ID = 0


T3: Changing scsiId to  6


T3: Check IOC FACTS chip 0


T3: MISM CHN_STATE_MPT_OPERATIONAL chip 0 


T3: MISM: Reply frame size 60 start addr d052b880 


T3: fe reply free frames posted


T3: MISM CHN_STATE_MPT_INIT_BUS_RST chip 0 


T4: MPT_Poll: chip 1 CHN_STATE_MPT_INIT_BUS_RST 


T6: MPT_Poll: chip 0 CHN_STATE_MPT_INIT_BUS_RST 


T8: DISM: Queued!


T8: MPT_ProcessIo Reply Fr 2 EVENT_NOTIFICATION


T8: MPI_EVENT_EVENT_CHANGE


T8: MPT_SetIocPageParameters: After Write CoalescingDepth=1, Timeout=0, Flags=1 


T8: MPT_ProcessIo Reply Fr 2 EVENT_NOTIFICATION


T8: MPI_EVENT_EVENT_CHANGE


T8: MPT_SetIocPageParameters: After Write CoalescingDepth=1, Timeout=0, Flags=1 


T8: DISM: Cluster peer is 0x7


T8: DISM: oper_chn_bmap is 1, operating_chn: 0


T11: DISM_ProcessPprState: DomainVal done on all disks


T13: DISM: Complete!!!





T13: Physical device info:


T13: ID  NVRState  Vendor    Product           Rev    6   7  56


T13: --  --------  --------  ----------------  ----  --  --  --


T14: 00  Online    FUJITSU   MAP3735NC         5608  01  3a  0f


T14: 01  Online    FUJITSU   MAP3735NC         5608  01  3a  0f


T14: 02  Online    SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 03  HotSpare  SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 04  Online    SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 05  Online    SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 07  Ready     DELL      PERC 4/DC         350O  00  00  00


T14: 08  HotSpare  FUJITSU   MAT3073NC         5704  01  3a  0d


T14: 0f  Ready     DELL      PV22XS            E.17  40  00  00





T19: batteryinit: TransBBU pack present


T19: TBBU: TBBU h/w . Cache not dirty


T19: Verifying config struct at Addr e0001400


T19: NVRAM checksum OK - reading configuration


T19: DISK_CACHE_ADDR=d0db4800


T19: MEM_END_ADDR=d7fffff0


T19: Memory End d7fffff0


T19: Total memory available for disk cache: 724b3f0


T19: Total Number of Cache Lines 1824


T19: SS 128: mrs=5  lc=1824 ldc=1  ps=1 cm=ff ba=0 LDs: 0


T19: LD  0: L=5  SS=128  Size=10eec000  NL=1824  Status=2  DT=255  BT=512


T19:        span 0: sBlk=00000000, nBlk=043bb000, dev=00-01-02-05-04


T19: can_flush = 0


T19: No Reconst:Checking drive info


T19: MIGRATE: 40LD or 8ld new drive  ch 0 tgt 0


T19: REF drive found at ch 0 tgt 0


T19: Attempting to perform drive roaming


T19: NOT Flushing Cache


T19: RMW: NVRAM structure valid - checking for active RMWs


T19: Inside  SymRequestPoolInit


T19: Memory Pages USed=100


T19: Memory Pages USed=164


T19: RequestQ ADDR=d0210000


T19: exit   SymRequestPoolInit


T19: inside   SymMsgFifoInit


T19: InboundFreeQEnd=d0200000


T19: exit   SymMsgFifoInit


T19: inside   SymMsgFifoInit


T19: exit  SymMsgFifoInit


T19: Memory Pages USed=34a


T19: Memory Pages USed=34e


T19: Memory Pages USed=34f


T19: Memory Pages USed=374


T19: found 1 logical drives


T19: logDrv=0  Size=10eec000


09/29 14:29:53: Time established at T45


09/29 14:29:53: BIOS CALL FOR DRV ROAMING : 55


09/29 14:29:53: drive roaming not done


09/29 14:30:24: HOST ISR: Sync cmd 0 arrived


09/29 14:31:31: LD 2 RdlongSize=22a #diff=16


09/29 14:31:31: LD 3 RdlongSize=22a #diff=16


09/29 14:31:32: LD 4 RdlongSize=22a #diff=16


09/29 14:31:32: LD 5 RdlongSize=22a #diff=16


09/29 14:31:32: LD 8 RdlongSize=23a #diff=6


09/29 14:32:10: LD 2 RdlongSize=22a #diff=16


09/29 14:32:10: LD 3 RdlongSize=22a #diff=16


09/29 14:32:10: LD 4 RdlongSize=22a #diff=16


09/29 14:32:10: LD 5 RdlongSize=22a #diff=16


09/29 14:32:10: LD 8 RdlongSize=23a #diff=6


09/29 14:32:24: LD 2 RdlongSize=22a #diff=16


09/29 14:32:24: LD 3 RdlongSize=22a #diff=16


09/29 14:32:24: LD 4 RdlongSize=22a #diff=16


09/29 14:32:24: LD 5 RdlongSize=22a #diff=16


09/29 14:32:25: LD 8 RdlongSize=23a #diff=6


�������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������










hr3xb1s.zip




DSET Report for ss {L4-LC1 SvcTag-HR3XB1S-PowerEdge 2600}.zip






READ ME before viewing this report!.txt





=======================================================

Instructions for the proper viewing of this DSET report

=======================================================





1) Decompress ALL files within this report into one folder using any ZIP utility 



2) When prompted enter the password "dell" (no quotes, all lowercase letters) to decrypt

   the report contents during decompression



3) Then open "dsetreport.hta" in Internet Explorer 5.0 or newer (which must support

   viewing XML data)





NOTE: Do NOT open the ZIP report file using the built-in Windows XP/2003 .ZIP expansion

      utility then double-clicking "dsetreport.hta". Other GUI support files must also

      be decompressed into the folder in order for the DSET graphical web interface to

      display properly. The DSET .ZIP report file must be entirely decompressed to a

      folder first before opening "dsetreport.hta".



NOTE: Dsetreport.hta is considered an "HTML Application" for purposes of secure local

      operation on Windows. Double-clicking the file will launch the HTML rendering engine

      of Windows which looks very similar to Internet Explorer. Do not rename the file

      extension to ".htm" as unpredictable behavior may result.



NOTE: Netscape and Mozilla are NOT supported at this time for displaying the DSET

      graphical interface



NOTE: Various log and configuration files can be found categorized within the "log"

      folder of this report








dsetreport.hta









po-HR3XB1S.TXT




TTY History for HA(0) -Bus 0x02 Device 0x02

���������������������������������������‰���





T0: LSI Logic MegaRAID i960 firmware loaded


T0: Firmware version 351S build on May 31 2005 at 21:05:31


T0: Board is type 1000/1960/1028/0518





T0: Can_flush=0


T0: DRAM SIZE=128 MB


T0: DRAM_ALT sig invalid from previous boot


T0: FLUSH_ON_SYSTEM_RESET=0





T0: (C) LSI Logic Corporation 2002


T0: MegaRAID Series 518 firmware version 351S


T0: WAIT FOR BIOS....


T0: BIOS UP!


T0: Enabling data cache


T0: EepromInit: Family=14, SN=5a2f80010000


T0: EepromInit: Board SN not programmed





T0: Environment data:


T0: VALIDATION=None





T0: MFC data:


T0:     Vendor/DeviceID=1000/1960


T0:     SubVendor/SubDevice=1028/0518


T0:     OEM=Dell, clusterDisable=0, flexRaidDisable=0


T0:     rebuildRate=30, stripeSize=128, flushTime=4


T0:     cachedIo=1, writeBack=1, readAhead=0


T0:     channelBase='0', smartMode=6, alarmDisable=0


T0:     fastInitDisable=0, coercion=128M, disablePredictiveFail=0


T0:     disableWebBios=1, disableCtrlM=0, writeThroughWhenBatteryBad=1


T0:     zcrConfig=Undefined, keepSafteFailedStatus=0, autoHotSpareRestore=0


T0:     variableChkConRate=0, enableNvramDiskMgmtChange=0


T0:     dirtyLedShowsDriveActivity=0, disableConsChkRestoration=0


T0:     biosContinueOnError=0, biosAutoConfig=Prompt


T0:     disableRandomDriveDeletion=0





T0: CM: 00 ff 00 00


T0: DISK_CACHE_ADDR=d0db4800


T0: MEM_END_ADDR=d7fffff0


T0: Found MPT LVD 1030(1/5/1) at f7ff0004/0, mapped to 8bff0000


T0: Found MPT LVD 1030(1/5/0) at f7fd0004/0, mapped to 8bfd0000


T0: Total LSI MPT Chips found 2 


T0: LSI_InitMPT : start_index 0 totalLSIMPTChips 2


T0: Set to Get f/w features chip 0 


T0: 	Verifying Image Signatures...VERIFIED


T0: 	Verifying image check sum... VERIFIED


T0: The FW version being loaded is MPTFW-01.03.35.00-IT


T0: NextImageHeaderOffset=9c70, ExtImageSize=818


T0: FW download complete... Expecting LSI FW to start excute and come to ready state


T3: MISM CHN_STATE_MPT_GET_FW_FEAT chip 1 


T3: PRESENT SCSI_ID = 0


T3: Changing scsiId to  6


T3: Check IOC FACTS chip 1


T3: MISM CHN_STATE_MPT_OPERATIONAL chip 1 


T3: MISM: Reply frame size 60 start addr d05258c0 


T3: fe reply free frames posted


T3: MISM CHN_STATE_MPT_INIT_BUS_RST chip 1 


T3: MPT_Poll: chip 0 CHN_STATE_MPT_WAIT


T3: MISM CHN_STATE_MPT_GET_FW_FEAT chip 0 


T3: PRESENT SCSI_ID = 0


T3: Changing scsiId to  6


T3: Check IOC FACTS chip 0


T3: MISM CHN_STATE_MPT_OPERATIONAL chip 0 


T3: MISM: Reply frame size 60 start addr d052b880 


T3: fe reply free frames posted


T3: MISM CHN_STATE_MPT_INIT_BUS_RST chip 0 


T4: MPT_Poll: chip 1 CHN_STATE_MPT_INIT_BUS_RST 


T6: MPT_Poll: chip 0 CHN_STATE_MPT_INIT_BUS_RST 


T8: DISM: Queued!


T8: MPT_ProcessIo Reply Fr 2 EVENT_NOTIFICATION


T8: MPI_EVENT_EVENT_CHANGE


T8: MPT_SetIocPageParameters: After Write CoalescingDepth=1, Timeout=0, Flags=1 


T8: MPT_ProcessIo Reply Fr 2 EVENT_NOTIFICATION


T8: MPI_EVENT_EVENT_CHANGE


T8: MPT_SetIocPageParameters: After Write CoalescingDepth=1, Timeout=0, Flags=1 


T8: DISM: Cluster peer is 0x7


T8: CM: 09 ff 01 00


T8: DISM: oper_chn_bmap is 1, operating_chn: 0


T9: CM: 01 ff 00 00


T11: DISM_ProcessPprState: DomainVal done on all disks


T14: DISM: Complete!!!





T14: Physical device info:


T14: CM: 01 ff 00 00


T14: ID  NVRState  Vendor    Product           Rev    6   7  56


T14: --  --------  --------  ----------------  ----  --  --  --


T14: 00  Online    SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 01  Online    SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 02  Online    SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 03  Online    FUJITSU   MAT3073NC         5704  01  3a  0d


T14: 04  Online    SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 05  HotSpare  FUJITSU   MAT3073NC         5704  01  3a  0d


T14: 07  Ready     DELL      PERC 4/DC         351S  01  00  00


T14: 08  Online    FUJITSU   MAT3073NC         5704  01  3a  0d


T14: 09  Online    FUJITSU   MAP3367NC         5605  01  3a  0f


T14: 0a  Online    SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 0f  Ready     DELL      PV22XS            E.17  40  00  00





T19: batteryinit: TransBBU pack present


T20: TBBU: TBBU h/w . Cache not dirty


T20: Verifying config struct at Addr e0001400


T20: NVRAM checksum OK - reading configuration


T20: DISK_CACHE_ADDR=d0db4800


T20: MEM_END_ADDR=d7fffff0


T20: Memory End d7fffff0


T20: Total memory available for disk cache: 724b3f0


T20: Total Number of Cache Lines 1824


T20: SS 128: mrs=2  lc=1824 ldc=1  ps=1 cm=ff ba=0 LDs: 0


T20: LD  0: L=1  SS=128  Size=10ef0000  NL=1824  Status=2  DT=255  BT=512


T20:        span 0: sBlk=00000000, nBlk=043bc000, dev=00-01


T20:        span 1: sBlk=00000000, nBlk=043bc000, dev=03-02


T20:        span 2: sBlk=00000000, nBlk=043bc000, dev=04-0a


T20:        span 3: sBlk=00000000, nBlk=043bc000, dev=09-08


T20: CM: 01 ff 00 00


T20: CM: 01 ff 00 00


T20: can_flush = 0


T20: No Reconst:Checking drive info


T20: MIGRATE: 40LD or 8ld new drive  ch 0 tgt 0


T20: REF drive found at ch 0 tgt 0


T20: Attempting to perform drive roaming


T20: NOT Flushing Cache


T20: RMW: NVRAM structure valid - checking for active RMWs


T20: Inside  SymRequestPoolInit


T20: Memory Pages USed=100


T20: Memory Pages USed=164


T20: RequestQ ADDR=d0210000


T20: exit   SymRequestPoolInit


T20: inside   SymMsgFifoInit


T20: InboundFreeQEnd=d0200000


T20: exit   SymMsgFifoInit


T20: inside   SymMsgFifoInit


T20: exit  SymMsgFifoInit


T20: Memory Pages USed=34a


T20: Memory Pages USed=34e


T20: Memory Pages USed=34f


T20: Memory Pages USed=374


T20: found 1 logical drives


T20: logDrv=0  Size=10ef0000


T20: CM: 0b ff 00 00


09/29 12:44:01: Time established at T46


09/29 12:44:01: BIOS CALL FOR DRV ROAMING : 55


09/29 12:44:01: drive roaming not done


09/29 12:44:44: HOST ISR: Sync cmd 0 arrived


�������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������









tty-HR3XB1S.log


TTY History for HA(0) -Bus 0x02 Device 0x02

������������������������������������

TTY History for HA(0) -Bus 0x02 Device 0x02

���������������������������������������‰���





T0: LSI Logic MegaRAID i960 firmware loaded


T0: Firmware version 351S build on May 31 2005 at 21:05:31


T0: Board is type 1000/1960/1028/0518





T0: Can_flush=0


T0: DRAM SIZE=128 MB


T0: DRAM_ALT sig invalid from previous boot


T0: FLUSH_ON_SYSTEM_RESET=0





T0: (C) LSI Logic Corporation 2002


T0: MegaRAID Series 518 firmware version 351S


T0: WAIT FOR BIOS....


T0: BIOS UP!


T0: Enabling data cache


T0: EepromInit: Family=14, SN=5a2f80010000


T0: EepromInit: Board SN not programmed





T0: Environment data:


T0: VALIDATION=None





T0: MFC data:


T0:     Vendor/DeviceID=1000/1960


T0:     SubVendor/SubDevice=1028/0518


T0:     OEM=Dell, clusterDisable=0, flexRaidDisable=0


T0:     rebuildRate=30, stripeSize=128, flushTime=4


T0:     cachedIo=1, writeBack=1, readAhead=0


T0:     channelBase='0', smartMode=6, alarmDisable=0


T0:     fastInitDisable=0, coercion=128M, disablePredictiveFail=0


T0:     disableWebBios=1, disableCtrlM=0, writeThroughWhenBatteryBad=1


T0:     zcrConfig=Undefined, keepSafteFailedStatus=0, autoHotSpareRestore=0


T0:     variableChkConRate=0, enableNvramDiskMgmtChange=0


T0:     dirtyLedShowsDriveActivity=0, disableConsChkRestoration=0


T0:     biosContinueOnError=0, biosAutoConfig=Prompt


T0:     disableRandomDriveDeletion=0





T0: CM: 00 ff 00 00


T0: DISK_CACHE_ADDR=d0db4800


T0: MEM_END_ADDR=d7fffff0


T0: Found MPT LVD 1030(1/5/1) at f7ff0004/0, mapped to 8bff0000


T0: Found MPT LVD 1030(1/5/0) at f7fd0004/0, mapped to 8bfd0000


T0: Total LSI MPT Chips found 2 


T0: LSI_InitMPT : start_index 0 totalLSIMPTChips 2


T0: Set to Get f/w features chip 0 


T0: 	Verifying Image Signatures...VERIFIED


T0: 	Verifying image check sum... VERIFIED


T0: The FW version being loaded is MPTFW-01.03.35.00-IT


T0: NextImageHeaderOffset=9c70, ExtImageSize=818


T0: FW download complete... Expecting LSI FW to start excute and come to ready state


T3: MISM CHN_STATE_MPT_GET_FW_FEAT chip 1 


T3: PRESENT SCSI_ID = 0


T3: Changing scsiId to  6


T3: Check IOC FACTS chip 1


T3: MISM CHN_STATE_MPT_OPERATIONAL chip 1 


T3: MISM: Reply frame size 60 start addr d05258c0 


T3: fe reply free frames posted


T3: MISM CHN_STATE_MPT_INIT_BUS_RST chip 1 


T3: MPT_Poll: chip 0 CHN_STATE_MPT_WAIT


T3: MISM CHN_STATE_MPT_GET_FW_FEAT chip 0 


T3: PRESENT SCSI_ID = 0


T3: Changing scsiId to  6


T3: Check IOC FACTS chip 0


T3: MISM CHN_STATE_MPT_OPERATIONAL chip 0 


T3: MISM: Reply frame size 60 start addr d052b880 


T3: fe reply free frames posted


T3: MISM CHN_STATE_MPT_INIT_BUS_RST chip 0 


T4: MPT_Poll: chip 1 CHN_STATE_MPT_INIT_BUS_RST 


T6: MPT_Poll: chip 0 CHN_STATE_MPT_INIT_BUS_RST 


T8: DISM: Queued!


T8: MPT_ProcessIo Reply Fr 2 EVENT_NOTIFICATION


T8: MPI_EVENT_EVENT_CHANGE


T8: MPT_SetIocPageParameters: After Write CoalescingDepth=1, Timeout=0, Flags=1 


T8: MPT_ProcessIo Reply Fr 2 EVENT_NOTIFICATION


T8: MPI_EVENT_EVENT_CHANGE


T8: MPT_SetIocPageParameters: After Write CoalescingDepth=1, Timeout=0, Flags=1 


T8: DISM: Cluster peer is 0x7


T8: CM: 09 ff 01 00


T8: DISM: oper_chn_bmap is 1, operating_chn: 0


T9: CM: 01 ff 00 00


T11: DISM_ProcessPprState: DomainVal done on all disks


T14: DISM: Complete!!!





T14: Physical device info:


T14: CM: 01 ff 00 00


T14: ID  NVRState  Vendor    Product           Rev    6   7  56


T14: --  --------  --------  ----------------  ----  --  --  --


T14: 00  Online    SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 01  Online    SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 02  Online    SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 03  Online    FUJITSU   MAT3073NC         5704  01  3a  0d


T14: 04  Online    SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 05  HotSpare  FUJITSU   MAT3073NC         5704  01  3a  0d


T14: 07  Ready     DELL      PERC 4/DC         351S  01  00  00


T14: 08  Online    FUJITSU   MAT3073NC         5704  01  3a  0d


T14: 09  Online    FUJITSU   MAP3367NC         5605  01  3a  0f


T14: 0a  Online    SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 0f  Ready     DELL      PV22XS            E.17  40  00  00





T19: batteryinit: TransBBU pack present


T20: TBBU: TBBU h/w . Cache not dirty


T20: Verifying config struct at Addr e0001400


T20: NVRAM checksum OK - reading configuration


T20: DISK_CACHE_ADDR=d0db4800


T20: MEM_END_ADDR=d7fffff0


T20: Memory End d7fffff0


T20: Total memory available for disk cache: 724b3f0


T20: Total Number of Cache Lines 1824


T20: SS 128: mrs=2  lc=1824 ldc=1  ps=1 cm=ff ba=0 LDs: 0


T20: LD  0: L=1  SS=128  Size=10ef0000  NL=1824  Status=2  DT=255  BT=512


T20:        span 0: sBlk=00000000, nBlk=043bc000, dev=00-01


T20:        span 1: sBlk=00000000, nBlk=043bc000, dev=03-02


T20:        span 2: sBlk=00000000, nBlk=043bc000, dev=04-0a


T20:        span 3: sBlk=00000000, nBlk=043bc000, dev=09-08


T20: CM: 01 ff 00 00


T20: CM: 01 ff 00 00


T20: can_flush = 0


T20: No Reconst:Checking drive info


T20: MIGRATE: 40LD or 8ld new drive  ch 0 tgt 0


T20: REF drive found at ch 0 tgt 0


T20: Attempting to perform drive roaming


T20: NOT Flushing Cache


T20: RMW: NVRAM structure valid - checking for active RMWs


T20: Inside  SymRequestPoolInit


T20: Memory Pages USed=100


T20: Memory Pages USed=164


T20: RequestQ ADDR=d0210000


T20: exit   SymRequestPoolInit


T20: inside   SymMsgFifoInit


T20: InboundFreeQEnd=d0200000


T20: exit   SymMsgFifoInit


T20: inside   SymMsgFifoInit


T20: exit  SymMsgFifoInit


T20: Memory Pages USed=34a


T20: Memory Pages USed=34e


T20: Memory Pages USed=34f


T20: Memory Pages USed=374


T20: found 1 logical drives


T20: logDrv=0  Size=10ef0000


T20: CM: 0b ff 00 00


09/29 12:44:01: Time established at T46


09/29 12:44:01: BIOS CALL FOR DRV ROAMING : 55


09/29 12:44:01: drive roaming not done


09/29 12:44:44: HOST ISR: Sync cmd 0 arrived


�������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������









tty-HR3XB1Stty.log


TTY History for HA(0) -Bus 0x02 Device 0x02

������������������������������������

TTY History for HA(0) -Bus 0x02 Device 0x02

���������������������������������������‰���





T0: LSI Logic MegaRAID i960 firmware loaded


T0: Firmware version 351S build on May 31 2005 at 21:05:31


T0: Board is type 1000/1960/1028/0518





T0: Can_flush=0


T0: DRAM SIZE=128 MB


T0: DRAM_ALT sig invalid from previous boot


T0: FLUSH_ON_SYSTEM_RESET=0





T0: (C) LSI Logic Corporation 2002


T0: MegaRAID Series 518 firmware version 351S


T0: WAIT FOR BIOS....


T0: BIOS UP!


T0: Enabling data cache


T0: EepromInit: Family=14, SN=5a2f80010000


T0: EepromInit: Board SN not programmed





T0: Environment data:


T0: VALIDATION=None





T0: MFC data:


T0:     Vendor/DeviceID=1000/1960


T0:     SubVendor/SubDevice=1028/0518


T0:     OEM=Dell, clusterDisable=0, flexRaidDisable=0


T0:     rebuildRate=30, stripeSize=128, flushTime=4


T0:     cachedIo=1, writeBack=1, readAhead=0


T0:     channelBase='0', smartMode=6, alarmDisable=0


T0:     fastInitDisable=0, coercion=128M, disablePredictiveFail=0


T0:     disableWebBios=1, disableCtrlM=0, writeThroughWhenBatteryBad=1


T0:     zcrConfig=Undefined, keepSafteFailedStatus=0, autoHotSpareRestore=0


T0:     variableChkConRate=0, enableNvramDiskMgmtChange=0


T0:     dirtyLedShowsDriveActivity=0, disableConsChkRestoration=0


T0:     biosContinueOnError=0, biosAutoConfig=Prompt


T0:     disableRandomDriveDeletion=0





T0: CM: 00 ff 00 00


T0: DISK_CACHE_ADDR=d0db4800


T0: MEM_END_ADDR=d7fffff0


T0: Found MPT LVD 1030(1/5/1) at f7ff0004/0, mapped to 8bff0000


T0: Found MPT LVD 1030(1/5/0) at f7fd0004/0, mapped to 8bfd0000


T0: Total LSI MPT Chips found 2 


T0: LSI_InitMPT : start_index 0 totalLSIMPTChips 2


T0: Set to Get f/w features chip 0 


T0: 	Verifying Image Signatures...VERIFIED


T0: 	Verifying image check sum... VERIFIED


T0: The FW version being loaded is MPTFW-01.03.35.00-IT


T0: NextImageHeaderOffset=9c70, ExtImageSize=818


T0: FW download complete... Expecting LSI FW to start excute and come to ready state


T3: MISM CHN_STATE_MPT_GET_FW_FEAT chip 1 


T3: PRESENT SCSI_ID = 0


T3: Changing scsiId to  6


T3: Check IOC FACTS chip 1


T3: MISM CHN_STATE_MPT_OPERATIONAL chip 1 


T3: MISM: Reply frame size 60 start addr d05258c0 


T3: fe reply free frames posted


T3: MISM CHN_STATE_MPT_INIT_BUS_RST chip 1 


T3: MPT_Poll: chip 0 CHN_STATE_MPT_WAIT


T3: MISM CHN_STATE_MPT_GET_FW_FEAT chip 0 


T3: PRESENT SCSI_ID = 0


T3: Changing scsiId to  6


T3: Check IOC FACTS chip 0


T3: MISM CHN_STATE_MPT_OPERATIONAL chip 0 


T3: MISM: Reply frame size 60 start addr d052b880 


T3: fe reply free frames posted


T3: MISM CHN_STATE_MPT_INIT_BUS_RST chip 0 


T4: MPT_Poll: chip 1 CHN_STATE_MPT_INIT_BUS_RST 


T6: MPT_Poll: chip 0 CHN_STATE_MPT_INIT_BUS_RST 


T8: DISM: Queued!


T8: MPT_ProcessIo Reply Fr 2 EVENT_NOTIFICATION


T8: MPI_EVENT_EVENT_CHANGE


T8: MPT_SetIocPageParameters: After Write CoalescingDepth=1, Timeout=0, Flags=1 


T8: MPT_ProcessIo Reply Fr 2 EVENT_NOTIFICATION


T8: MPI_EVENT_EVENT_CHANGE


T8: MPT_SetIocPageParameters: After Write CoalescingDepth=1, Timeout=0, Flags=1 


T8: DISM: Cluster peer is 0x7


T8: CM: 09 ff 01 00


T8: DISM: oper_chn_bmap is 1, operating_chn: 0


T9: CM: 01 ff 00 00


T11: DISM_ProcessPprState: DomainVal done on all disks


T14: DISM: Complete!!!





T14: Physical device info:


T14: CM: 01 ff 00 00


T14: ID  NVRState  Vendor    Product           Rev    6   7  56


T14: --  --------  --------  ----------------  ----  --  --  --


T14: 00  Online    SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 01  Online    SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 02  Online    SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 03  Online    FUJITSU   MAT3073NC         5704  01  3a  0d


T14: 04  Online    SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 05  HotSpare  FUJITSU   MAT3073NC         5704  01  3a  0d


T14: 07  Ready     DELL      PERC 4/DC         351S  01  00  00


T14: 08  Online    FUJITSU   MAT3073NC         5704  01  3a  0d


T14: 09  Online    FUJITSU   MAP3367NC         5605  01  3a  0f


T14: 0a  Online    SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 0f  Ready     DELL      PV22XS            E.17  40  00  00





T19: batteryinit: TransBBU pack present


T20: TBBU: TBBU h/w . Cache not dirty


T20: Verifying config struct at Addr e0001400


T20: NVRAM checksum OK - reading configuration


T20: DISK_CACHE_ADDR=d0db4800


T20: MEM_END_ADDR=d7fffff0


T20: Memory End d7fffff0


T20: Total memory available for disk cache: 724b3f0


T20: Total Number of Cache Lines 1824


T20: SS 128: mrs=2  lc=1824 ldc=1  ps=1 cm=ff ba=0 LDs: 0


T20: LD  0: L=1  SS=128  Size=10ef0000  NL=1824  Status=2  DT=255  BT=512


T20:        span 0: sBlk=00000000, nBlk=043bc000, dev=00-01


T20:        span 1: sBlk=00000000, nBlk=043bc000, dev=03-02


T20:        span 2: sBlk=00000000, nBlk=043bc000, dev=04-0a


T20:        span 3: sBlk=00000000, nBlk=043bc000, dev=09-08


T20: CM: 01 ff 00 00


T20: CM: 01 ff 00 00


T20: can_flush = 0


T20: No Reconst:Checking drive info


T20: MIGRATE: 40LD or 8ld new drive  ch 0 tgt 0


T20: REF drive found at ch 0 tgt 0


T20: Attempting to perform drive roaming


T20: NOT Flushing Cache


T20: RMW: NVRAM structure valid - checking for active RMWs


T20: Inside  SymRequestPoolInit


T20: Memory Pages USed=100


T20: Memory Pages USed=164


T20: RequestQ ADDR=d0210000


T20: exit   SymRequestPoolInit


T20: inside   SymMsgFifoInit


T20: InboundFreeQEnd=d0200000


T20: exit   SymMsgFifoInit


T20: inside   SymMsgFifoInit


T20: exit  SymMsgFifoInit


T20: Memory Pages USed=34a


T20: Memory Pages USed=34e


T20: Memory Pages USed=34f


T20: Memory Pages USed=374


T20: found 1 logical drives


T20: logDrv=0  Size=10ef0000


T20: CM: 0b ff 00 00


09/29 12:44:01: Time established at T46


09/29 12:44:01: BIOS CALL FOR DRV ROAMING : 55


09/29 12:44:01: drive roaming not done


09/29 12:44:44: HOST ISR: Sync cmd 0 arrived


�������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������










jr3xb1s.zip




d-JR3XB1S.log




TTY History for HA(0) -Bus 0x02 Device 0x02

���������������������������������������CX�� @T0


(C) LSI Logic 2002 @T0


Megaraid Series 0 firmware version 350O @T0


Build date: Jun  1 2004 at 15:44:00 @T0


Board type: 1000/1960/1028/0518 @T0


DRAM_ALT sig invalid from previous boot @T0


FLUSH_ON_SYSTEM_RESET=0 @T0


WAIT FOR BIOS.... @T0


bus 1 dev 5 function 1 @T0


reg 0 value ffffff01 @T0


reg 1 value f7ff0004 @T0


reg 2 value 0 @T0


reg 3 value f7fe0004 @T0


reg 4 value 0 @T0


reg 5 value 0 @T0


bus 1 dev 5 function 0 @T0


reg 0 value fffffe01 @T0


reg 1 value f7fd0004 @T0


reg 2 value 0 @T0


reg 3 value f7fc0004 @T0


reg 4 value 0 @T0


reg 5 value 0 @T0


SCSI chip is on the secondary bus @T0


Found MPT LVD 30 at f7ff0004 @T0


 isp 0 membaseaddr 8bff0000 iobaseaddr 9001ff00  @T0


SCSI chip is on the secondary bus @T0


Found MPT LVD 30 at f7fd0004 @T0


 isp 1 membaseaddr 8bfd0000 iobaseaddr 9001fe00  @T0


BIOS UP! @T0


Can_flush=0 DRAM SIZE=128 MB @T0


* RST * @T0


Enabling data cache @T0


 @T0


Environment data: @T0


VALIDATION=None @T0


VERSIONS=CTLM_U821,BIOS_1.09,APP_350O,DBGR_N/A,BTBL_1.7.6DELL @T0


 @T0


pciDebug = d0008abc @T0


calling init_scsi @T0


DISK_CACHE_ADDR=d0db4c00  @T0


MEM_END_ADDR=d7fffff0  @T0


Total LSI MPT Chips found 2  @T0


LSI_InitMPT : start_index 0 totalLSIMPTChips 2 @T0


 Set to Get f/w features chip 0  @T0


	Verifying Image Signatures...VERIFIED @T0


	Verifying image check sum... VERIFIED @T0


BaseAddr 9001fe00 chip 1 @T0


Checking Diagnostic Register write access...	Enabled. @T0


reset adapter bit cleared @T0


Complete. @T0


Checking Diagnostic Register write access...	Enabled. @T0


The FW version being loaded is MPTFW-01.03.35.00-IT @T0


NextImageHeaderOffset 9c70 @T0


ExtImage Size 818 @T0


Diag, Register disabled DIAGNOSTIC_REG 131 @T0


FW download complete... Expecting LSI FW to start excute and come to ready state @T0


 For this sys doorbell reg bit 28 should be set  @T0


 MISM CHN_STATE_MPT_GET_FW_FEAT chip 1  @T3


 Check IOC FACTS chip 1  @T3


 MISM CHN_STATE_MPT_OPERATIONAL chip 1  @T3


 MISM: Reply frame size 60 start addr d054ec3c  @T3


fe reply free frames posted @T3


 MISM CHN_STATE_MPT_INIT_BUS_RST chip 1  @T3


MPT_Poll: chip 0 CHN_STATE_MPT_WAIT @T3


 MISM CHN_STATE_MPT_GET_FW_FEAT chip 0  @T3


 Check IOC FACTS chip 0  @T3


 MISM CHN_STATE_MPT_OPERATIONAL chip 0  @T3


 MISM: Reply frame size 60 start addr d0554be0  @T3


fe reply free frames posted @T3


 MISM CHN_STATE_MPT_INIT_BUS_RST chip 0  @T3


MPT_Poll: chip 1 CHN_STATE_MPT_INIT_BUS_RST  @T3


 MISM CHN_STATE_MPT_INIT_BUS_RLY chip 1  @T3


 MISM CHN_STATE_MPT_READY chip 1  @T6


cmdBufferAddr = f854e9c0, ioIdx = 0 @T6


cmdBufferAddr = f854ea58, ioIdx = 1 @T6


CommandBufferPost Post: Request = d0020200 @T6


DISM: Queued! @T6


MPT_Poll: chip 0 CHN_STATE_MPT_INIT_BUS_RST  @T6


 MISM CHN_STATE_MPT_INIT_BUS_RLY chip 0  @T6


 MISM CHN_STATE_MPT_READY chip 0  @T9


cmdBufferAddr = f854e6c0, ioIdx = 0 @T9


cmdBufferAddr = f854e758, ioIdx = 1 @T9


CommandBufferPost Post: Request = d0020200 @T9


DISM: Queued! @T9


 MPT_ProcessIo Reply Fr 2 EVENT_NOTIFICATION @T9


MPI_EVENT_EVENT_CHANGE @T9


 MPT_ProcessIo Reply Fr 2 EVENT_NOTIFICATION @T9


MPI_EVENT_EVENT_CHANGE @T9


DISM: Cluster peer is 0x6 @T9


DISM: CR8 SAFTE at chan 0 id f @T11


DISM_ProcessPprState: DomainVal done on all disks @T13


DISM: Complete!!! @T15


bbuDebugFlags = d000d94c @T15


batteryinit: TransBBU pack present @T20


TBBU: TBBU h/w . Cache not dirty @T20


Veirfyin config struct at Addr e0001400  @T20


NVRAM checksum OK - reading configuration @T20


DISK_CACHE_ADDR=d0db4c00  @T20


MEM_END_ADDR=d7fffff0  @T20


Memory End d7fffff0 @T20


Total Number of Cache Lines 1822 @T20


L 5   SS 128   Size 10eec000   N 1822   Status 2   DT  1293   BT 1755 @T20


NVF: Updating NV e0004f82 with data 0 @T20


can_flush = 0 @T21


No Reconst:Checking drive info @T21


 @T21


REF drive found at ch 0 tgt 0  @T21


comparing disk 1 with nvram @T21


src chn 0  drvptr d048e08c dst chn 0 nvramptr d04d044c @T21


check config okay.... @T21


comparing disk 2 with nvram @T21


src chn 0  drvptr d048e08c dst chn 0 nvramptr d04d044c @T21


check config okay.... @T21


comparing disk 3 with nvram @T21


src chn 0  drvptr d048e08c dst chn 0 nvramptr d04d044c @T21


check config okay.... @T21


comparing disk 4 with nvram @T21


src chn 0  drvptr d048e08c dst chn 0 nvramptr d04d044c @T21


check config okay.... @T21


comparing disk 5 with nvram @T21


src chn 0  drvptr d048e08c dst chn 0 nvramptr d04d044c @T21


check config okay.... @T21


comparing disk 8 with nvram @T21


src chn 0  drvptr d048e08c dst chn 0 nvramptr d04d044c @T21


check config okay.... @T21


Attempting to perform drive roaming @T21


NOT Flushing Cache @T21


check to start Patrol read @T21


BIOS CALL FOR DRV ROAMING : 55 @7/1 16:8:1


drive roaming not done @7/1 16:8:1


HOST ISR: Sync cmd 0 arrived @7/1 16:8:39


MPT_Rec: INQ Error - Negotiating LD[6] pRfm d00ada40 @7/1 16:9:18


MPT_Rec: INQ Error - Negotiating LD[6] pRfm d00af600 @7/1 16:9:25


MPT_Rec: INQ Error - Negotiating LD[6] pRfm d00b0800 @7/1 16:9:26


MPT_Rec: INQ Error - Negotiating LD[6] pRfm d00bbc00 @7/1 16:10:1


MPT_Rec: INQ Error - Negotiating LD[6] pRfm d0091c00 @7/1 16:10:9


 MPT_ProcessIo Reply Fr cb EVENT_NOTIFICATION @7/7 14:53:42


 MPT_ProcessIo Reply Fr cc EVENT_ACK @7/7 14:53:42


TMM:Setting Peer id to 6 @7/7 14:53:47


TMM:oper_chn_bmap is 0x1, operating_chn: 0x0 @7/7 14:53:47


TMM:Setting Peer id to 6 @7/7 14:53:58


TMM:oper_chn_bmap is 0x1, operating_chn: 0x0 @7/7 14:53:58


MPT_Rec: INQ Error - Negotiating LD[6] pRfm d00970c0 @7/14 13:30:52


 MPT_ProcessIo Reply Fr 3e EVENT_NOTIFICATION @7/24 0:50:16


LD Check: FCM_StartScsiBusReset @7/24 0:50:16


StartScsiBusReset(chip 0):Send 0 msgs to LdWaitQ @7/24 0:50:16


 MPT_ProcessIo Reply Fr 3f EVENT_ACK @7/24 0:50:16


CQM4: calling FCM_CompleteScsiBusReset chip 0 port 0 @7/24 0:50:17


LD Check: FCM_CompleteScsiBusReset @7/24 0:50:17


 MPT_ProcessIo Reply Fr 41 EVENT_NOTIFICATION @7/24 0:50:17


LD Check: FCM_StartScsiBusReset @7/24 0:50:17


StartScsiBusReset(chip 0):Send 0 msgs to LdWaitQ @7/24 0:50:17


CQM2: calling FCM_CompleteScsiBusReset chip 0 port 0 @7/24 0:50:17


LD Check: FCM_CompleteScsiBusReset @7/24 0:50:17


 MPT_ProcessIo Reply Fr 43 EVENT_ACK @7/24 0:50:17


LD Unlocked... No Lock... Done @7/24 0:50:32


 MPT_ProcessIo Reply Fr aa EVENT_NOTIFICATION @9/29 2:12:19


LD Check: FCM_StartScsiBusReset @9/29 2:12:19


StartScsiBusReset(chip 0):Send 0 msgs to LdWaitQ @9/29 2:12:19


CQM2: calling FCM_CompleteScsiBusReset chip 0 port 0 @9/29 2:12:19


LD Check: FCM_CompleteScsiBusReset @9/29 2:12:19


 MPT_ProcessIo Reply Fr b4 EVENT_ACK @9/29 2:12:19


LD Unlocked... No Lock... Done @9/29 2:12:24


TMM:Setting Peer id to 6 @9/29 2:12:24


TMM:oper_chn_bmap is 0x1, operating_chn: 0x0 @9/29 2:12:24


TMM:Setting Peer id to 6 @9/29 2:12:35


TMM:oper_chn_bmap is 0x1, operating_chn: 0x0 @9/29 2:12:35


 MPT_ProcessIo Reply Fr 1b EVENT_NOTIFICATION @9/29 2:21:14


LD Check: FCM_StartScsiBusReset @9/29 2:21:14


StartScsiBusReset(chip 0):Send 0 msgs to LdWaitQ @9/29 2:21:14


CQM2: calling FCM_CompleteScsiBusReset chip 0 port 0 @9/29 2:21:14


LD Check: FCM_CompleteScsiBusReset @9/29 2:21:14


 MPT_ProcessIo Reply Fr 28 EVENT_ACK @9/29 2:21:14


TMM:Setting Peer id to 6 @9/29 2:21:21


TMM:oper_chn_bmap is 0x1, operating_chn: 0x0 @9/29 2:21:21


LD Unlocked... No Lock... Done @9/29 2:21:28


TMM:Setting Peer id to 6 @9/29 2:21:31


TMM:oper_chn_bmap is 0x1, operating_chn: 0x0 @9/29 2:21:31


 MPT_ProcessIo Reply Fr 48 EVENT_NOTIFICATION @9/29 2:22:43


LD Check: FCM_StartScsiBusReset @9/29 2:22:43


StartScsiBusReset(chip 0):Send 0 msgs to LdWaitQ @9/29 2:22:43


CQM2: calling FCM_CompleteScsiBusReset chip 0 port 0 @9/29 2:22:43


LD Check: FCM_CompleteScsiBusReset @9/29 2:22:43


 MPT_ProcessIo Reply Fr 5b EVENT_ACK @9/29 2:22:43


LD Unlocked... No Lock... Done @9/29 2:22:48


TMM:Setting Peer id to 6 @9/29 2:22:51


TMM:oper_chn_bmap is 0x1, operating_chn: 0x0 @9/29 2:22:51


TMM:Setting Peer id to 6 @9/29 2:23:0


TMM:oper_chn_bmap is 0x1, operating_chn: 0x0 @9/29 2:23:0


LD Check: FCM_StartScsiBusReset @9/29 2:25:45


StartScsiBusReset(chip 0):Send 0 msgs to LdWaitQ @9/29 2:25:45


CQM2: calling FCM_CompleteScsiBusReset chip 0 port 0 @9/29 2:25:45


LD Check: FCM_CompleteScsiBusReset @9/29 2:25:45


 MPT_ProcessIo Reply Fr 8d EVENT_NOTIFICATION @9/29 2:25:45


LD Check: FCM_StartScsiBusReset @9/29 2:25:45


StartScsiBusReset(chip 0):Send 0 msgs to LdWaitQ @9/29 2:25:45


CQM2: calling FCM_CompleteScsiBusReset chip 0 port 0 @9/29 2:25:45


LD Check: FCM_CompleteScsiBusReset @9/29 2:25:45


 MPT_ProcessIo Reply Fr 8f EVENT_ACK @9/29 2:25:45


TMM:Setting Peer id to 6 @9/29 2:25:51


TMM:oper_chn_bmap is 0x1, operating_chn: 0x0 @9/29 2:25:51


LD Unlocked... No Lock... Done @9/29 2:26:0


TMM:Setting Peer id to 6 @9/29 2:26:0


TMM:oper_chn_bmap is 0x1, operating_chn: 0x0 @9/29 2:26:0


SCSI_PROTOCOL_ERR:REC: pRfm d00a0d80, LD[1] - Ioc d0559560, iocStatus 47 @9/29 2:29:2


 MPT_ProcessIo Reply Fr ca EVENT_NOTIFICATION @9/29 2:29:33


 MPT_ProcessIo Reply Fr cb EVENT_ACK @9/29 2:29:33


TMM:Setting Peer id to 6 @9/29 2:29:39


TMM:oper_chn_bmap is 0x1, operating_chn: 0x0 @9/29 2:29:39


TMM:Setting Peer id to 6 @9/29 2:29:48


TMM:oper_chn_bmap is 0x1, operating_chn: 0x0 @9/29 2:29:48


LD Check: FCM_StartScsiBusReset @9/29 2:33:54


StartScsiBusReset(chip 0):Send 0 msgs to LdWaitQ @9/29 2:33:54


CQM2: calling FCM_CompleteScsiBusReset chip 0 port 0 @9/29 2:33:54


LD Check: FCM_CompleteScsiBusReset @9/29 2:33:54


 MPT_ProcessIo Reply Fr 12 EVENT_NOTIFICATION @9/29 2:33:54


LD Check: FCM_StartScsiBusReset @9/29 2:33:54


StartScsiBusReset(chip 0):Send 0 msgs to LdWaitQ @9/29 2:33:54


CQM2: calling FCM_CompleteScsiBusReset chip 0 port 0 @9/29 2:33:54


LD Check: FCM_CompleteScsiBusReset @9/29 2:33:54


 MPT_ProcessIo Reply Fr 14 EVENT_ACK @9/29 2:33:54


TMM:Setting Peer id to 6 @9/29 2:33:59


TMM:oper_chn_bmap is 0x1, operating_chn: 0x0 @9/29 2:33:59


LD Unlocked... No Lock... Done @9/29 2:34:0


TMM:Setting Peer id to 6 @9/29 2:34:9


TMM:oper_chn_bmap is 0x1, operating_chn: 0x0 @9/29 2:34:9


SCSI_PROTOCOL_ERR:REC: pRfm d0099280, LD[5] - Ioc d05567a0, iocStatus 47 @9/29 2:37:15


�[2J @T0


(C) LSI Logic 2002 @T0


Megaraid Series 0 firmware version 350O @T0


Build date: Jun  1 2004 at 15:44:00 @T0


Board type: 1000/1960/1028/0518 @T0


FLUSH_ON_SYSTEM_RESET=0 @T0


WAIT FOR BIOS.... @T0


bus 1 dev 5 function 1 @T0


reg 0 value ffffff01 @T0


reg 1 value f7ff0004 @T0


reg 2 value 0 @T0


reg 3 value f7fe0004 @T0


reg 4 value 0 @T0


reg 5 value 0 @T0


bus 1 dev 5 function 0 @T0


reg 0 value fffffe01 @T0


reg 1 value f7fd0004 @T0


reg 2 value 0 @T0


reg 3 value f7fc0004 @T0


reg 4 value 0 @T0


reg 5 value 0 @T0


SCSI chip is on the secondary bus @T0


Found MPT LVD 30 at f7ff0004 @T0


 isp 0 membaseaddr 8bff0000 iobaseaddr 9001ff00  @T0


SCSI chip is on the secondary bus @T0


Found MPT LVD 30 at f7fd0004 @T0


 isp 1 membaseaddr 8bfd0000 iobaseaddr 9001fe00  @T0


BIOS UP! @T0


Can_flush=0 DRAM SIZE=128 MB @T0


* RST * @T0


Enabling data cache @T0


 @T0


Environment data: @T0


VALIDATION=None @T0


VERSIONS=CTLM_U821,BIOS_1.09,APP_350O,DBGR_N/A,BTBL_1.7.6DELL @T0


 @T0


pciDebug = d0008abc @T0


calling init_scsi @T0


DISK_CACHE_ADDR=d0db4c00  @T0


MEM_END_ADDR=d7fffff0  @T0


Total LSI MPT Chips found 2  @T0


LSI_InitMPT : start_index 0 totalLSIMPTChips 2 @T0


 Set to Get f/w features chip 0  @T0


	Verifying Image Signatures...VERIFIED @T0


	Verifying image check sum... VERIFIED @T0


BaseAddr 9001fe00 chip 1 @T0


Checking Diagnostic Register write access...	Enabled. @T0


reset adapter bit cleared @T0


Complete. @T0


Checking Diagnostic Register write access...	Enabled. @T0


The FW version being loaded is MPTFW-01.03.35.00-IT @T0


NextImageHeaderOffset 9c70 @T0


ExtImage Size 818 @T0


Diag, Register disabled DIAGNOSTIC_REG 131 @T0


FW download complete... Expecting LSI FW to start excute and come to ready state @T0


 For this sys doorbell reg bit 28 should be set  @T0


 MISM CHN_STATE_MPT_GET_FW_FEAT chip 1  @T3


 Check IOC FACTS chip 1  @T3


 MISM CHN_STATE_MPT_OPERATIONAL chip 1  @T3


 MISM: Reply frame size 60 start addr d054ec3c  @T3


fe reply free frames posted @T3


 MISM CHN_STATE_MPT_INIT_BUS_RST chip 1  @T3


MPT_Poll: chip 0 CHN_STATE_MPT_WAIT @T3


 MISM CHN_STATE_MPT_GET_FW_FEAT chip 0  @T3


 Check IOC FACTS chip 0  @T3


 MISM CHN_STATE_MPT_OPERATIONAL chip 0  @T3


 MISM: Reply frame size 60 start addr d0554be0  @T3


fe reply free frames posted @T3


 MISM CHN_STATE_MPT_INIT_BUS_RST chip 0  @T3


MPT_Poll: chip 1 CHN_STATE_MPT_INIT_BUS_RST  @T3


 MISM CHN_STATE_MPT_INIT_BUS_RLY chip 1  @T3


 MISM CHN_STATE_MPT_READY chip 1  @T6


cmdBufferAddr = f854e9c0, ioIdx = 0 @T6


cmdBufferAddr = f854ea58, ioIdx = 1 @T6


CommandBufferPost Post: Request = d0020200 @T6


DISM: Queued! @T6


MPT_Poll: chip 0 CHN_STATE_MPT_INIT_BUS_RST  @T6


 MISM CHN_STATE_MPT_INIT_BUS_RLY chip 0  @T6


 MISM CHN_STATE_MPT_READY chip 0  @T9


cmdBufferAddr = f854e6c0, ioIdx = 0 @T9


cmdBufferAddr = f854e758, ioIdx = 1 @T9


CommandBufferPost Post: Request = d0020200 @T9


DISM: Queued! @T9


 MPT_ProcessIo Reply Fr 2 EVENT_NOTIFICATION @T9


MPI_EVENT_EVENT_CHANGE @T9


 MPT_ProcessIo Reply Fr 2 EVENT_NOTIFICATION @T9


MPI_EVENT_EVENT_CHANGE @T9


setting pRfm flagin pRfm = d0081640, flag = 420, dev = 2 @T9


setting pRfm flagin pRfm = d00817c0, flag = 420, dev = 4 @T9


setting pRfm flagin pRfm = d0081880, flag = 420, dev = 5 @T9


DISM: Cluster peer is 0x6 @T9


DISM: CR8 SAFTE at chan 0 id f @T11


DISM_ProcessPprState: DomainVal done on all disks @T13


DISM: Complete!!! @T15


bbuDebugFlags = d000d94c @T15


batteryinit: TransBBU pack present @T20


TBBU: TBBU h/w . Cache not dirty @T20


Veirfyin config struct at Addr e0001400  @T20


NVRAM checksum OK - reading configuration @T20


DISK_CACHE_ADDR=d0db4c00  @T20


MEM_END_ADDR=d7fffff0  @T20


Memory End d7fffff0 @T20


Total Number of Cache Lines 1822 @T20


L 5   SS 128   Size 10eec000   N 1822   Status 2   DT  1293   BT 1755 @T20


NVF: Updating NV e0004f82 with data 0 @T20


can_flush = 0 @T20


No Reconst:Checking drive info @T20


 @T20


REF drive found at ch 0 tgt 0  @T20


comparing disk 1 with nvram @T20


src chn 0  drvptr d048e08c dst chn 0 nvramptr d04d044c @T20


check config okay.... @T20


comparing disk 2 with nvram @T20


src chn 0  drvptr d048e08c dst chn 0 nvramptr d04d044c @T21


check config okay.... @T21


comparing disk 3 with nvram @T21


src chn 0  drvptr d048e08c dst chn 0 nvramptr d04d044c @T21


check config okay.... @T21


comparing disk 4 with nvram @T21


src chn 0  drvptr d048e08c dst chn 0 nvramptr d04d044c @T21


check config okay.... @T21


comparing disk 5 with nvram @T21


src chn 0  drvptr d048e08c dst chn 0 nvramptr d04d044c @T21


check config okay.... @T21


comparing disk 8 with nvram @T21


src chn 0  drvptr d048e08c dst chn 0 nvramptr d04d044c @T21


check config okay.... @T21


Attempting to perform drive roaming @T21


NOT Flushing Cache @T21


check to start Patrol read @T21


BIOS CALL FOR DRV ROAMING : 55 @9/29 14:53:33


drive roaming not done @9/29 14:53:33


Flash: Waiting to Free all IO @9/29 14:54:30


Flash: All IO over - Allocating memory @9/29 14:54:30


Flash: Writing to Flash Rom... @9/29 14:55:8


Flash:  Ready to flash...disabling all serial port output @9/29 14:55:9


NVF: Updating NV e0004f6c with data fe @9/29 14:55:41








T0: LSI Logic MegaRAID i960 firmware loaded


T0: Firmware version 351S build on May 31 2005 at 21:05:31


T0: Board is type 1000/1960/1028/0518





T0: Can_flush=1


T0: DRAM SIZE=128 MB


T0: FLUSH_ON_SYSTEM_RESET=0





T0: (C) LSI Logic Corporation 2002


T0: MegaRAID Series 518 firmware version 351S


T0: WAIT FOR BIOS....


T0: BIOS UP!


T0: Enabling data cache


T0: EepromInit: Family=14, SN=c94954010000


T0: EepromInit: Board SN not programmed


T0: EepromInit: Family=14, SN=c94954010000


T0: EepromInit: Board SN not programmed





T0: Environment data:


T0: VALIDATION=None





T0: MFC data:


T0:     Vendor/DeviceID=1000/1960


T0:     SubVendor/SubDevice=1028/0518


T0:     OEM=Dell, clusterDisable=0, flexRaidDisable=0


T0:     rebuildRate=30, stripeSize=128, flushTime=4


T0:     cachedIo=1, writeBack=1, readAhead=0


T0:     channelBase='0', smartMode=6, alarmDisable=0


T0:     fastInitDisable=0, coercion=128M, disablePredictiveFail=0


T0:     disableWebBios=1, disableCtrlM=0, writeThroughWhenBatteryBad=1


T0:     zcrConfig=Undefined, keepSafteFailedStatus=0, autoHotSpareRestore=0


T0:     variableChkConRate=0, enableNvramDiskMgmtChange=0


T0:     dirtyLedShowsDriveActivity=0, disableConsChkRestoration=0


T0:     biosContinueOnError=0, biosAutoConfig=Prompt


T0:     disableRandomDriveDeletion=0





T0: PatrolReadInit Checksum invalid - initializing data


T0: CM: 00 ff 00 00


T0: DISK_CACHE_ADDR=d0db4800


T0: MEM_END_ADDR=d7fffff0


T0: Found MPT LVD 1030(1/5/1) at f7ff0004/0, mapped to 8bff0000


T0: Found MPT LVD 1030(1/5/0) at f7fd0004/0, mapped to 8bfd0000


T0: Total LSI MPT Chips found 2 


T0: LSI_InitMPT : start_index 0 totalLSIMPTChips 2


T0: Set to Get f/w features chip 0 


T0: 	Verifying Image Signatures...VERIFIED


T0: 	Verifying image check sum... VERIFIED


T0: The FW version being loaded is MPTFW-01.03.35.00-IT


T0: NextImageHeaderOffset=9c70, ExtImageSize=818


T0: FW download complete... Expecting LSI FW to start excute and come to ready state


T3: MISM CHN_STATE_MPT_GET_FW_FEAT chip 1 


T3: PRESENT SCSI_ID = 0


T3: Changing scsiId to  7


T3: Check IOC FACTS chip 1


T3: MISM CHN_STATE_MPT_OPERATIONAL chip 1 


T3: MISM: Reply frame size 60 start addr d05258c0 


T3: fe reply free frames posted


T3: MISM CHN_STATE_MPT_INIT_BUS_RST chip 1 


T3: MPT_Poll: chip 0 CHN_STATE_MPT_WAIT


T3: MISM CHN_STATE_MPT_GET_FW_FEAT chip 0 


T4: PRESENT SCSI_ID = 0


T4: Changing scsiId to  7


T4: Check IOC FACTS chip 0


T4: MISM CHN_STATE_MPT_OPERATIONAL chip 0 


T4: MISM: Reply frame size 60 start addr d052b880 


T4: fe reply free frames posted


T4: MISM CHN_STATE_MPT_INIT_BUS_RST chip 0 


T4: MPT_Poll: chip 1 CHN_STATE_MPT_INIT_BUS_RST 


T6: MPT_Poll: chip 0 CHN_STATE_MPT_INIT_BUS_RST 


T8: DISM: Queued!


T8: MPT_ProcessIo Reply Fr 2 EVENT_NOTIFICATION


T8: MPI_EVENT_EVENT_CHANGE


T8: MPT_SetIocPageParameters: After Write CoalescingDepth=1, Timeout=0, Flags=1 


T8: MPT_ProcessIo Reply Fr 2 EVENT_NOTIFICATION


T8: MPI_EVENT_EVENT_CHANGE


T8: MPT_SetIocPageParameters: After Write CoalescingDepth=1, Timeout=0, Flags=1 


T8: DISM: Cluster peer is 0x6


T8: CM: 09 ff 01 00


T8: DISM: oper_chn_bmap is 1, operating_chn: 0


T9: CM: 01 ff 00 00


T12: DISM_ProcessPprState: DomainVal done on all disks


T14: DISM: Complete!!!





T14: Physical device info:


T14: CM: 01 ff 00 00


T14: ID  NVRState  Vendor    Product           Rev    6   7  56


T14: --  --------  --------  ----------------  ----  --  --  --


T14: 00  Online    FUJITSU   MAP3735NC         5608  01  3a  0f


T14: 01  Online    FUJITSU   MAP3735NC         5608  01  3a  0f


T14: 02  Online    SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 03  HotSpare  SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 04  Online    SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 05  Online    SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 06  Ready     DELL      PERC 4/DC         351S  01  00  00


T14: 08  HotSpare  FUJITSU   MAT3073NC         5704  01  3a  0d


T14: 0f  Ready     DELL      PV22XS            E.17  40  00  00





T19: batteryinit: TransBBU pack present


T19: TBBU: TBBU h/w . Cache not dirty


T19: Verifying config struct at Addr e0001400


T19: NVRAM checksum OK - reading configuration


T19: DISK_CACHE_ADDR=d0db4800


T19: MEM_END_ADDR=d7fffff0


T19: Memory End d7fffff0


T19: Total memory available for disk cache: 724b3f0


T19: Total Number of Cache Lines 1824


T19: SS 128: mrs=5  lc=1824 ldc=1  ps=1 cm=ff ba=0 LDs: 0


T19: LD  0: L=5  SS=128  Size=10eec000  NL=1824  Status=2  DT=255  BT=512


T19:        span 0: sBlk=00000000, nBlk=043bb000, dev=00-01-02-05-04


T19: CM: 01 ff 00 00


T19: CM: 01 ff 00 00


T19: can_flush = 0


T19: No Reconst:Checking drive info


T19: MIGRATE: 40LD or 8ld new drive  ch 0 tgt 0


T19: REF drive found at ch 0 tgt 0


T19: Attempting to perform drive roaming


T19: AUTO_RES: NOT proceeding auto resume


T19: rebuildResume checksum is bad - initializing NVRAM structure


T19: NOT Flushing Cache


T19: RMW: NVRAM structure invalid - initializing


T19: Inside  SymRequestPoolInit


T19: Memory Pages USed=100


T19: Memory Pages USed=164


T19: RequestQ ADDR=d0210000


T19: exit   SymRequestPoolInit


T19: inside   SymMsgFifoInit


T19: InboundFreeQEnd=d0200000


T19: exit   SymMsgFifoInit


T19: inside   SymMsgFifoInit


T19: exit  SymMsgFifoInit


T19: Memory Pages USed=34a


T19: Memory Pages USed=34e


T19: Memory Pages USed=34f


T19: Memory Pages USed=374


T19: found 1 logical drives


T19: logDrv=0  Size=10eec000


T19: CM: 0b ff 00 00


T48: Ch <0> TM Wheel <0> Slots: [0]=0 [1]=0 [2]=1 [3]=0 [4]=0 [5]=0 Cur= [0]


T52: CQM: Timing wheel expired - Chip 0


T52: Ch <0> TM Wheel <0> Slots: [0]=0 [1]=0 [2]=1 [3]=0 [4]=0 [5]=0 Cur= [2]


T52: CQM_RecScanAllMsg (slot 2):Scanned 2, Disabled 0, Timeout 1 Msg


T52: LD Check: FCM_StartScsiBusReset


T52: StartScsiBusReset(chip 0, port 0):Send 0 msgs to LdWaitQ


T52: MISM: MPT_TaskMgmt chip 0 iob addr d0525190 task 4 ispMC=2 


T52: CQM4: calling FCM_CompleteScsiBusReset chip 0 port 0


T52: LD Check: FCM_CompleteScsiBusReset


T52: Recd TskMgmt Reply 6b IOCStatus 0 iob addr d0525190 type 4 chip 0 ispMC=0 


09/29 14:57:10: Time established at T57


09/29 14:57:10: BIOS CALL FOR DRV ROAMING : 55


09/29 14:57:10: drive roaming not done


09/29 14:57:11: PatrolReadTimer: Clearing PR completion bitmaps; next clear scheduled for 10/29 14:57:11


09/29 14:57:11: prDiskStart: starting Patrol Read on PD=00


09/29 14:57:11: prDiskStart: starting Patrol Read on PD=01


09/29 14:57:11: prDiskStart: starting Patrol Read on PD=02


09/29 14:57:11: prDiskStart: starting Patrol Read on PD=03


09/29 14:57:11: prDiskStart: starting Patrol Read on PD=04


09/29 14:57:11: prDiskStart: starting Patrol Read on PD=05


09/29 14:57:11: prDiskStart: starting Patrol Read on PD=08


09/29 14:57:11: Next PR scheduled to start at 09/29 16:00:00


09/29 14:57:17: LD Unlocked... No Lock... Done


09/29 14:57:49: HOST ISR: Sync cmd 0 arrived


09/29 14:59:25: MPT_ProcessIo Reply Fr c3 EVENT_NOTIFICATION


09/29 14:59:25: LD Check: FCM_StartScsiBusReset


09/29 14:59:25: StartScsiBusReset(chip 0, port 0):Send 0 msgs to LdWaitQ


09/29 14:59:25: CQM4: calling FCM_CompleteScsiBusReset chip 0 port 0


09/29 14:59:25: LD Check: FCM_CompleteScsiBusReset


09/29 14:59:25: MPT_ProcessIo Reply Fr c5 EVENT_ACK


09/29 14:59:57: LD Unlocked... No Lock... Done


�����������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������









DSET Report for ss {L4-PS1 SvcTag-JR3XB1S-PowerEdge 2600}.zip






READ ME before viewing this report!.txt





=======================================================

Instructions for the proper viewing of this DSET report

=======================================================





1) Decompress ALL files within this report into one folder using any ZIP utility 



2) When prompted enter the password "dell" (no quotes, all lowercase letters) to decrypt

   the report contents during decompression



3) Then open "dsetreport.hta" in Internet Explorer 5.0 or newer (which must support

   viewing XML data)





NOTE: Do NOT open the ZIP report file using the built-in Windows XP/2003 .ZIP expansion

      utility then double-clicking "dsetreport.hta". Other GUI support files must also

      be decompressed into the folder in order for the DSET graphical web interface to

      display properly. The DSET .ZIP report file must be entirely decompressed to a

      folder first before opening "dsetreport.hta".



NOTE: Dsetreport.hta is considered an "HTML Application" for purposes of secure local

      operation on Windows. Double-clicking the file will launch the HTML rendering engine

      of Windows which looks very similar to Internet Explorer. Do not rename the file

      extension to ".htm" as unpredictable behavior may result.



NOTE: Netscape and Mozilla are NOT supported at this time for displaying the DSET

      graphical interface



NOTE: Various log and configuration files can be found categorized within the "log"

      folder of this report








dsetreport.hta









tty-JR3XB1S.log


TTY History for HA(0) -Bus 0x02 Device 0x02

������������������������������������

TTY History for HA(0) -Bus 0x02 Device 0x02

���������������������������������������CX�� @T0


(C) LSI Logic 2002 @T0


Megaraid Series 0 firmware version 350O @T0


Build date: Jun  1 2004 at 15:44:00 @T0


Board type: 1000/1960/1028/0518 @T0


DRAM_ALT sig invalid from previous boot @T0


FLUSH_ON_SYSTEM_RESET=0 @T0


WAIT FOR BIOS.... @T0


bus 1 dev 5 function 1 @T0


reg 0 value ffffff01 @T0


reg 1 value f7ff0004 @T0


reg 2 value 0 @T0


reg 3 value f7fe0004 @T0


reg 4 value 0 @T0


reg 5 value 0 @T0


bus 1 dev 5 function 0 @T0


reg 0 value fffffe01 @T0


reg 1 value f7fd0004 @T0


reg 2 value 0 @T0


reg 3 value f7fc0004 @T0


reg 4 value 0 @T0


reg 5 value 0 @T0


SCSI chip is on the secondary bus @T0


Found MPT LVD 30 at f7ff0004 @T0


 isp 0 membaseaddr 8bff0000 iobaseaddr 9001ff00  @T0


SCSI chip is on the secondary bus @T0


Found MPT LVD 30 at f7fd0004 @T0


 isp 1 membaseaddr 8bfd0000 iobaseaddr 9001fe00  @T0


BIOS UP! @T0


Can_flush=0 DRAM SIZE=128 MB @T0


* RST * @T0


Enabling data cache @T0


 @T0


Environment data: @T0


VALIDATION=None @T0


VERSIONS=CTLM_U821,BIOS_1.09,APP_350O,DBGR_N/A,BTBL_1.7.6DELL @T0


 @T0


pciDebug = d0008abc @T0


calling init_scsi @T0


DISK_CACHE_ADDR=d0db4c00  @T0


MEM_END_ADDR=d7fffff0  @T0


Total LSI MPT Chips found 2  @T0


LSI_InitMPT : start_index 0 totalLSIMPTChips 2 @T0


 Set to Get f/w features chip 0  @T0


	Verifying Image Signatures...VERIFIED @T0


	Verifying image check sum... VERIFIED @T0


BaseAddr 9001fe00 chip 1 @T0


Checking Diagnostic Register write access...	Enabled. @T0


reset adapter bit cleared @T0


Complete. @T0


Checking Diagnostic Register write access...	Enabled. @T0


The FW version being loaded is MPTFW-01.03.35.00-IT @T0


NextImageHeaderOffset 9c70 @T0


ExtImage Size 818 @T0


Diag, Register disabled DIAGNOSTIC_REG 131 @T0


FW download complete... Expecting LSI FW to start excute and come to ready state @T0


 For this sys doorbell reg bit 28 should be set  @T0


 MISM CHN_STATE_MPT_GET_FW_FEAT chip 1  @T3


 Check IOC FACTS chip 1  @T3


 MISM CHN_STATE_MPT_OPERATIONAL chip 1  @T3


 MISM: Reply frame size 60 start addr d054ec3c  @T3


fe reply free frames posted @T3


 MISM CHN_STATE_MPT_INIT_BUS_RST chip 1  @T3


MPT_Poll: chip 0 CHN_STATE_MPT_WAIT @T3


 MISM CHN_STATE_MPT_GET_FW_FEAT chip 0  @T3


 Check IOC FACTS chip 0  @T3


 MISM CHN_STATE_MPT_OPERATIONAL chip 0  @T3


 MISM: Reply frame size 60 start addr d0554be0  @T3


fe reply free frames posted @T3


 MISM CHN_STATE_MPT_INIT_BUS_RST chip 0  @T3


MPT_Poll: chip 1 CHN_STATE_MPT_INIT_BUS_RST  @T3


 MISM CHN_STATE_MPT_INIT_BUS_RLY chip 1  @T3


 MISM CHN_STATE_MPT_READY chip 1  @T6


cmdBufferAddr = f854e9c0, ioIdx = 0 @T6


cmdBufferAddr = f854ea58, ioIdx = 1 @T6


CommandBufferPost Post: Request = d0020200 @T6


DISM: Queued! @T6


MPT_Poll: chip 0 CHN_STATE_MPT_INIT_BUS_RST  @T6


 MISM CHN_STATE_MPT_INIT_BUS_RLY chip 0  @T6


 MISM CHN_STATE_MPT_READY chip 0  @T9


cmdBufferAddr = f854e6c0, ioIdx = 0 @T9


cmdBufferAddr = f854e758, ioIdx = 1 @T9


CommandBufferPost Post: Request = d0020200 @T9


DISM: Queued! @T9


 MPT_ProcessIo Reply Fr 2 EVENT_NOTIFICATION @T9


MPI_EVENT_EVENT_CHANGE @T9


 MPT_ProcessIo Reply Fr 2 EVENT_NOTIFICATION @T9


MPI_EVENT_EVENT_CHANGE @T9


DISM: Cluster peer is 0x6 @T9


DISM: CR8 SAFTE at chan 0 id f @T11


DISM_ProcessPprState: DomainVal done on all disks @T13


DISM: Complete!!! @T15


bbuDebugFlags = d000d94c @T15


batteryinit: TransBBU pack present @T20


TBBU: TBBU h/w . Cache not dirty @T20


Veirfyin config struct at Addr e0001400  @T20


NVRAM checksum OK - reading configuration @T20


DISK_CACHE_ADDR=d0db4c00  @T20


MEM_END_ADDR=d7fffff0  @T20


Memory End d7fffff0 @T20


Total Number of Cache Lines 1822 @T20


L 5   SS 128   Size 10eec000   N 1822   Status 2   DT  1293   BT 1755 @T20


NVF: Updating NV e0004f82 with data 0 @T20


can_flush = 0 @T21


No Reconst:Checking drive info @T21


 @T21


REF drive found at ch 0 tgt 0  @T21


comparing disk 1 with nvram @T21


src chn 0  drvptr d048e08c dst chn 0 nvramptr d04d044c @T21


check config okay.... @T21


comparing disk 2 with nvram @T21


src chn 0  drvptr d048e08c dst chn 0 nvramptr d04d044c @T21


check config okay.... @T21


comparing disk 3 with nvram @T21


src chn 0  drvptr d048e08c dst chn 0 nvramptr d04d044c @T21


check config okay.... @T21


comparing disk 4 with nvram @T21


src chn 0  drvptr d048e08c dst chn 0 nvramptr d04d044c @T21


check config okay.... @T21


comparing disk 5 with nvram @T21


src chn 0  drvptr d048e08c dst chn 0 nvramptr d04d044c @T21


check config okay.... @T21


comparing disk 8 with nvram @T21


src chn 0  drvptr d048e08c dst chn 0 nvramptr d04d044c @T21


check config okay.... @T21


Attempting to perform drive roaming @T21


NOT Flushing Cache @T21


check to start Patrol read @T21


BIOS CALL FOR DRV ROAMING : 55 @7/1 16:8:1


drive roaming not done @7/1 16:8:1


HOST ISR: Sync cmd 0 arrived @7/1 16:8:39


MPT_Rec: INQ Error - Negotiating LD[6] pRfm d00ada40 @7/1 16:9:18


MPT_Rec: INQ Error - Negotiating LD[6] pRfm d00af600 @7/1 16:9:25


MPT_Rec: INQ Error - Negotiating LD[6] pRfm d00b0800 @7/1 16:9:26


MPT_Rec: INQ Error - Negotiating LD[6] pRfm d00bbc00 @7/1 16:10:1


MPT_Rec: INQ Error - Negotiating LD[6] pRfm d0091c00 @7/1 16:10:9


 MPT_ProcessIo Reply Fr cb EVENT_NOTIFICATION @7/7 14:53:42


 MPT_ProcessIo Reply Fr cc EVENT_ACK @7/7 14:53:42


TMM:Setting Peer id to 6 @7/7 14:53:47


TMM:oper_chn_bmap is 0x1, operating_chn: 0x0 @7/7 14:53:47


TMM:Setting Peer id to 6 @7/7 14:53:58


TMM:oper_chn_bmap is 0x1, operating_chn: 0x0 @7/7 14:53:58


MPT_Rec: INQ Error - Negotiating LD[6] pRfm d00970c0 @7/14 13:30:52


 MPT_ProcessIo Reply Fr 3e EVENT_NOTIFICATION @7/24 0:50:16


LD Check: FCM_StartScsiBusReset @7/24 0:50:16


StartScsiBusReset(chip 0):Send 0 msgs to LdWaitQ @7/24 0:50:16


 MPT_ProcessIo Reply Fr 3f EVENT_ACK @7/24 0:50:16


CQM4: calling FCM_CompleteScsiBusReset chip 0 port 0 @7/24 0:50:17


LD Check: FCM_CompleteScsiBusReset @7/24 0:50:17


 MPT_ProcessIo Reply Fr 41 EVENT_NOTIFICATION @7/24 0:50:17


LD Check: FCM_StartScsiBusReset @7/24 0:50:17


StartScsiBusReset(chip 0):Send 0 msgs to LdWaitQ @7/24 0:50:17


CQM2: calling FCM_CompleteScsiBusReset chip 0 port 0 @7/24 0:50:17


LD Check: FCM_CompleteScsiBusReset @7/24 0:50:17


 MPT_ProcessIo Reply Fr 43 EVENT_ACK @7/24 0:50:17


LD Unlocked... No Lock... Done @7/24 0:50:32


 MPT_ProcessIo Reply Fr aa EVENT_NOTIFICATION @9/29 2:12:19


LD Check: FCM_StartScsiBusReset @9/29 2:12:19


StartScsiBusReset(chip 0):Send 0 msgs to LdWaitQ @9/29 2:12:19


CQM2: calling FCM_CompleteScsiBusReset chip 0 port 0 @9/29 2:12:19


LD Check: FCM_CompleteScsiBusReset @9/29 2:12:19


 MPT_ProcessIo Reply Fr b4 EVENT_ACK @9/29 2:12:19


LD Unlocked... No Lock... Done @9/29 2:12:24


TMM:Setting Peer id to 6 @9/29 2:12:24


TMM:oper_chn_bmap is 0x1, operating_chn: 0x0 @9/29 2:12:24


TMM:Setting Peer id to 6 @9/29 2:12:35


TMM:oper_chn_bmap is 0x1, operating_chn: 0x0 @9/29 2:12:35


 MPT_ProcessIo Reply Fr 1b EVENT_NOTIFICATION @9/29 2:21:14


LD Check: FCM_StartScsiBusReset @9/29 2:21:14


StartScsiBusReset(chip 0):Send 0 msgs to LdWaitQ @9/29 2:21:14


CQM2: calling FCM_CompleteScsiBusReset chip 0 port 0 @9/29 2:21:14


LD Check: FCM_CompleteScsiBusReset @9/29 2:21:14


 MPT_ProcessIo Reply Fr 28 EVENT_ACK @9/29 2:21:14


TMM:Setting Peer id to 6 @9/29 2:21:21


TMM:oper_chn_bmap is 0x1, operating_chn: 0x0 @9/29 2:21:21


LD Unlocked... No Lock... Done @9/29 2:21:28


TMM:Setting Peer id to 6 @9/29 2:21:31


TMM:oper_chn_bmap is 0x1, operating_chn: 0x0 @9/29 2:21:31


 MPT_ProcessIo Reply Fr 48 EVENT_NOTIFICATION @9/29 2:22:43


LD Check: FCM_StartScsiBusReset @9/29 2:22:43


StartScsiBusReset(chip 0):Send 0 msgs to LdWaitQ @9/29 2:22:43


CQM2: calling FCM_CompleteScsiBusReset chip 0 port 0 @9/29 2:22:43


LD Check: FCM_CompleteScsiBusReset @9/29 2:22:43


 MPT_ProcessIo Reply Fr 5b EVENT_ACK @9/29 2:22:43


LD Unlocked... No Lock... Done @9/29 2:22:48


TMM:Setting Peer id to 6 @9/29 2:22:51


TMM:oper_chn_bmap is 0x1, operating_chn: 0x0 @9/29 2:22:51


TMM:Setting Peer id to 6 @9/29 2:23:0


TMM:oper_chn_bmap is 0x1, operating_chn: 0x0 @9/29 2:23:0


LD Check: FCM_StartScsiBusReset @9/29 2:25:45


StartScsiBusReset(chip 0):Send 0 msgs to LdWaitQ @9/29 2:25:45


CQM2: calling FCM_CompleteScsiBusReset chip 0 port 0 @9/29 2:25:45


LD Check: FCM_CompleteScsiBusReset @9/29 2:25:45


 MPT_ProcessIo Reply Fr 8d EVENT_NOTIFICATION @9/29 2:25:45


LD Check: FCM_StartScsiBusReset @9/29 2:25:45


StartScsiBusReset(chip 0):Send 0 msgs to LdWaitQ @9/29 2:25:45


CQM2: calling FCM_CompleteScsiBusReset chip 0 port 0 @9/29 2:25:45


LD Check: FCM_CompleteScsiBusReset @9/29 2:25:45


 MPT_ProcessIo Reply Fr 8f EVENT_ACK @9/29 2:25:45


TMM:Setting Peer id to 6 @9/29 2:25:51


TMM:oper_chn_bmap is 0x1, operating_chn: 0x0 @9/29 2:25:51


LD Unlocked... No Lock... Done @9/29 2:26:0


TMM:Setting Peer id to 6 @9/29 2:26:0


TMM:oper_chn_bmap is 0x1, operating_chn: 0x0 @9/29 2:26:0


SCSI_PROTOCOL_ERR:REC: pRfm d00a0d80, LD[1] - Ioc d0559560, iocStatus 47 @9/29 2:29:2


 MPT_ProcessIo Reply Fr ca EVENT_NOTIFICATION @9/29 2:29:33


 MPT_ProcessIo Reply Fr cb EVENT_ACK @9/29 2:29:33


TMM:Setting Peer id to 6 @9/29 2:29:39


TMM:oper_chn_bmap is 0x1, operating_chn: 0x0 @9/29 2:29:39


TMM:Setting Peer id to 6 @9/29 2:29:48


TMM:oper_chn_bmap is 0x1, operating_chn: 0x0 @9/29 2:29:48


LD Check: FCM_StartScsiBusReset @9/29 2:33:54


StartScsiBusReset(chip 0):Send 0 msgs to LdWaitQ @9/29 2:33:54


CQM2: calling FCM_CompleteScsiBusReset chip 0 port 0 @9/29 2:33:54


LD Check: FCM_CompleteScsiBusReset @9/29 2:33:54


 MPT_ProcessIo Reply Fr 12 EVENT_NOTIFICATION @9/29 2:33:54


LD Check: FCM_StartScsiBusReset @9/29 2:33:54


StartScsiBusReset(chip 0):Send 0 msgs to LdWaitQ @9/29 2:33:54


CQM2: calling FCM_CompleteScsiBusReset chip 0 port 0 @9/29 2:33:54


LD Check: FCM_CompleteScsiBusReset @9/29 2:33:54


 MPT_ProcessIo Reply Fr 14 EVENT_ACK @9/29 2:33:54


TMM:Setting Peer id to 6 @9/29 2:33:59


TMM:oper_chn_bmap is 0x1, operating_chn: 0x0 @9/29 2:33:59


LD Unlocked... No Lock... Done @9/29 2:34:0


TMM:Setting Peer id to 6 @9/29 2:34:9


TMM:oper_chn_bmap is 0x1, operating_chn: 0x0 @9/29 2:34:9


SCSI_PROTOCOL_ERR:REC: pRfm d0099280, LD[5] - Ioc d05567a0, iocStatus 47 @9/29 2:37:15


�[2J @T0


(C) LSI Logic 2002 @T0


Megaraid Series 0 firmware version 350O @T0


Build date: Jun  1 2004 at 15:44:00 @T0


Board type: 1000/1960/1028/0518 @T0


FLUSH_ON_SYSTEM_RESET=0 @T0


WAIT FOR BIOS.... @T0


bus 1 dev 5 function 1 @T0


reg 0 value ffffff01 @T0


reg 1 value f7ff0004 @T0


reg 2 value 0 @T0


reg 3 value f7fe0004 @T0


reg 4 value 0 @T0


reg 5 value 0 @T0


bus 1 dev 5 function 0 @T0


reg 0 value fffffe01 @T0


reg 1 value f7fd0004 @T0


reg 2 value 0 @T0


reg 3 value f7fc0004 @T0


reg 4 value 0 @T0


reg 5 value 0 @T0


SCSI chip is on the secondary bus @T0


Found MPT LVD 30 at f7ff0004 @T0


 isp 0 membaseaddr 8bff0000 iobaseaddr 9001ff00  @T0


SCSI chip is on the secondary bus @T0


Found MPT LVD 30 at f7fd0004 @T0


 isp 1 membaseaddr 8bfd0000 iobaseaddr 9001fe00  @T0


BIOS UP! @T0


Can_flush=0 DRAM SIZE=128 MB @T0


* RST * @T0


Enabling data cache @T0


 @T0


Environment data: @T0


VALIDATION=None @T0


VERSIONS=CTLM_U821,BIOS_1.09,APP_350O,DBGR_N/A,BTBL_1.7.6DELL @T0


 @T0


pciDebug = d0008abc @T0


calling init_scsi @T0


DISK_CACHE_ADDR=d0db4c00  @T0


MEM_END_ADDR=d7fffff0  @T0


Total LSI MPT Chips found 2  @T0


LSI_InitMPT : start_index 0 totalLSIMPTChips 2 @T0


 Set to Get f/w features chip 0  @T0


	Verifying Image Signatures...VERIFIED @T0


	Verifying image check sum... VERIFIED @T0


BaseAddr 9001fe00 chip 1 @T0


Checking Diagnostic Register write access...	Enabled. @T0


reset adapter bit cleared @T0


Complete. @T0


Checking Diagnostic Register write access...	Enabled. @T0


The FW version being loaded is MPTFW-01.03.35.00-IT @T0


NextImageHeaderOffset 9c70 @T0


ExtImage Size 818 @T0


Diag, Register disabled DIAGNOSTIC_REG 131 @T0


FW download complete... Expecting LSI FW to start excute and come to ready state @T0


 For this sys doorbell reg bit 28 should be set  @T0


 MISM CHN_STATE_MPT_GET_FW_FEAT chip 1  @T3


 Check IOC FACTS chip 1  @T3


 MISM CHN_STATE_MPT_OPERATIONAL chip 1  @T3


 MISM: Reply frame size 60 start addr d054ec3c  @T3


fe reply free frames posted @T3


 MISM CHN_STATE_MPT_INIT_BUS_RST chip 1  @T3


MPT_Poll: chip 0 CHN_STATE_MPT_WAIT @T3


 MISM CHN_STATE_MPT_GET_FW_FEAT chip 0  @T3


 Check IOC FACTS chip 0  @T3


 MISM CHN_STATE_MPT_OPERATIONAL chip 0  @T3


 MISM: Reply frame size 60 start addr d0554be0  @T3


fe reply free frames posted @T3


 MISM CHN_STATE_MPT_INIT_BUS_RST chip 0  @T3


MPT_Poll: chip 1 CHN_STATE_MPT_INIT_BUS_RST  @T3


 MISM CHN_STATE_MPT_INIT_BUS_RLY chip 1  @T3


 MISM CHN_STATE_MPT_READY chip 1  @T6


cmdBufferAddr = f854e9c0, ioIdx = 0 @T6


cmdBufferAddr = f854ea58, ioIdx = 1 @T6


CommandBufferPost Post: Request = d0020200 @T6


DISM: Queued! @T6


MPT_Poll: chip 0 CHN_STATE_MPT_INIT_BUS_RST  @T6


 MISM CHN_STATE_MPT_INIT_BUS_RLY chip 0  @T6


 MISM CHN_STATE_MPT_READY chip 0  @T9


cmdBufferAddr = f854e6c0, ioIdx = 0 @T9


cmdBufferAddr = f854e758, ioIdx = 1 @T9


CommandBufferPost Post: Request = d0020200 @T9


DISM: Queued! @T9


 MPT_ProcessIo Reply Fr 2 EVENT_NOTIFICATION @T9


MPI_EVENT_EVENT_CHANGE @T9


 MPT_ProcessIo Reply Fr 2 EVENT_NOTIFICATION @T9


MPI_EVENT_EVENT_CHANGE @T9


setting pRfm flagin pRfm = d0081640, flag = 420, dev = 2 @T9


setting pRfm flagin pRfm = d00817c0, flag = 420, dev = 4 @T9


setting pRfm flagin pRfm = d0081880, flag = 420, dev = 5 @T9


DISM: Cluster peer is 0x6 @T9


DISM: CR8 SAFTE at chan 0 id f @T11


DISM_ProcessPprState: DomainVal done on all disks @T13


DISM: Complete!!! @T15


bbuDebugFlags = d000d94c @T15


batteryinit: TransBBU pack present @T20


TBBU: TBBU h/w . Cache not dirty @T20


Veirfyin config struct at Addr e0001400  @T20


NVRAM checksum OK - reading configuration @T20


DISK_CACHE_ADDR=d0db4c00  @T20


MEM_END_ADDR=d7fffff0  @T20


Memory End d7fffff0 @T20


Total Number of Cache Lines 1822 @T20


L 5   SS 128   Size 10eec000   N 1822   Status 2   DT  1293   BT 1755 @T20


NVF: Updating NV e0004f82 with data 0 @T20


can_flush = 0 @T20


No Reconst:Checking drive info @T20


 @T20


REF drive found at ch 0 tgt 0  @T20


comparing disk 1 with nvram @T20


src chn 0  drvptr d048e08c dst chn 0 nvramptr d04d044c @T20


check config okay.... @T20


comparing disk 2 with nvram @T20


src chn 0  drvptr d048e08c dst chn 0 nvramptr d04d044c @T21


check config okay.... @T21


comparing disk 3 with nvram @T21


src chn 0  drvptr d048e08c dst chn 0 nvramptr d04d044c @T21


check config okay.... @T21


comparing disk 4 with nvram @T21


src chn 0  drvptr d048e08c dst chn 0 nvramptr d04d044c @T21


check config okay.... @T21


comparing disk 5 with nvram @T21


src chn 0  drvptr d048e08c dst chn 0 nvramptr d04d044c @T21


check config okay.... @T21


comparing disk 8 with nvram @T21


src chn 0  drvptr d048e08c dst chn 0 nvramptr d04d044c @T21


check config okay.... @T21


Attempting to perform drive roaming @T21


NOT Flushing Cache @T21


check to start Patrol read @T21


BIOS CALL FOR DRV ROAMING : 55 @9/29 14:53:33


drive roaming not done @9/29 14:53:33


Flash: Waiting to Free all IO @9/29 14:54:30


Flash: All IO over - Allocating memory @9/29 14:54:30


Flash: Writing to Flash Rom... @9/29 14:55:8


Flash:  Ready to flash...disabling all serial port output @9/29 14:55:9


NVF: Updating NV e0004f6c with data fe @9/29 14:55:41








T0: LSI Logic MegaRAID i960 firmware loaded


T0: Firmware version 351S build on May 31 2005 at 21:05:31


T0: Board is type 1000/1960/1028/0518





T0: Can_flush=1


T0: DRAM SIZE=128 MB


T0: FLUSH_ON_SYSTEM_RESET=0





T0: (C) LSI Logic Corporation 2002


T0: MegaRAID Series 518 firmware version 351S


T0: WAIT FOR BIOS....


T0: BIOS UP!


T0: Enabling data cache


T0: EepromInit: Family=14, SN=c94954010000


T0: EepromInit: Board SN not programmed


T0: EepromInit: Family=14, SN=c94954010000


T0: EepromInit: Board SN not programmed





T0: Environment data:


T0: VALIDATION=None





T0: MFC data:


T0:     Vendor/DeviceID=1000/1960


T0:     SubVendor/SubDevice=1028/0518


T0:     OEM=Dell, clusterDisable=0, flexRaidDisable=0


T0:     rebuildRate=30, stripeSize=128, flushTime=4


T0:     cachedIo=1, writeBack=1, readAhead=0


T0:     channelBase='0', smartMode=6, alarmDisable=0


T0:     fastInitDisable=0, coercion=128M, disablePredictiveFail=0


T0:     disableWebBios=1, disableCtrlM=0, writeThroughWhenBatteryBad=1


T0:     zcrConfig=Undefined, keepSafteFailedStatus=0, autoHotSpareRestore=0


T0:     variableChkConRate=0, enableNvramDiskMgmtChange=0


T0:     dirtyLedShowsDriveActivity=0, disableConsChkRestoration=0


T0:     biosContinueOnError=0, biosAutoConfig=Prompt


T0:     disableRandomDriveDeletion=0





T0: PatrolReadInit Checksum invalid - initializing data


T0: CM: 00 ff 00 00


T0: DISK_CACHE_ADDR=d0db4800


T0: MEM_END_ADDR=d7fffff0


T0: Found MPT LVD 1030(1/5/1) at f7ff0004/0, mapped to 8bff0000


T0: Found MPT LVD 1030(1/5/0) at f7fd0004/0, mapped to 8bfd0000


T0: Total LSI MPT Chips found 2 


T0: LSI_InitMPT : start_index 0 totalLSIMPTChips 2


T0: Set to Get f/w features chip 0 


T0: 	Verifying Image Signatures...VERIFIED


T0: 	Verifying image check sum... VERIFIED


T0: The FW version being loaded is MPTFW-01.03.35.00-IT


T0: NextImageHeaderOffset=9c70, ExtImageSize=818


T0: FW download complete... Expecting LSI FW to start excute and come to ready state


T3: MISM CHN_STATE_MPT_GET_FW_FEAT chip 1 


T3: PRESENT SCSI_ID = 0


T3: Changing scsiId to  7


T3: Check IOC FACTS chip 1


T3: MISM CHN_STATE_MPT_OPERATIONAL chip 1 


T3: MISM: Reply frame size 60 start addr d05258c0 


T3: fe reply free frames posted


T3: MISM CHN_STATE_MPT_INIT_BUS_RST chip 1 


T3: MPT_Poll: chip 0 CHN_STATE_MPT_WAIT


T3: MISM CHN_STATE_MPT_GET_FW_FEAT chip 0 


T4: PRESENT SCSI_ID = 0


T4: Changing scsiId to  7


T4: Check IOC FACTS chip 0


T4: MISM CHN_STATE_MPT_OPERATIONAL chip 0 


T4: MISM: Reply frame size 60 start addr d052b880 


T4: fe reply free frames posted


T4: MISM CHN_STATE_MPT_INIT_BUS_RST chip 0 


T4: MPT_Poll: chip 1 CHN_STATE_MPT_INIT_BUS_RST 


T6: MPT_Poll: chip 0 CHN_STATE_MPT_INIT_BUS_RST 


T8: DISM: Queued!


T8: MPT_ProcessIo Reply Fr 2 EVENT_NOTIFICATION


T8: MPI_EVENT_EVENT_CHANGE


T8: MPT_SetIocPageParameters: After Write CoalescingDepth=1, Timeout=0, Flags=1 


T8: MPT_ProcessIo Reply Fr 2 EVENT_NOTIFICATION


T8: MPI_EVENT_EVENT_CHANGE


T8: MPT_SetIocPageParameters: After Write CoalescingDepth=1, Timeout=0, Flags=1 


T8: DISM: Cluster peer is 0x6


T8: CM: 09 ff 01 00


T8: DISM: oper_chn_bmap is 1, operating_chn: 0


T9: CM: 01 ff 00 00


T12: DISM_ProcessPprState: DomainVal done on all disks


T14: DISM: Complete!!!





T14: Physical device info:


T14: CM: 01 ff 00 00


T14: ID  NVRState  Vendor    Product           Rev    6   7  56


T14: --  --------  --------  ----------------  ----  --  --  --


T14: 00  Online    FUJITSU   MAP3735NC         5608  01  3a  0f


T14: 01  Online    FUJITSU   MAP3735NC         5608  01  3a  0f


T14: 02  Online    SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 03  HotSpare  SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 04  Online    SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 05  Online    SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 06  Ready     DELL      PERC 4/DC         351S  01  00  00


T14: 08  HotSpare  FUJITSU   MAT3073NC         5704  01  3a  0d


T14: 0f  Ready     DELL      PV22XS            E.17  40  00  00





T19: batteryinit: TransBBU pack present


T19: TBBU: TBBU h/w . Cache not dirty


T19: Verifying config struct at Addr e0001400


T19: NVRAM checksum OK - reading configuration


T19: DISK_CACHE_ADDR=d0db4800


T19: MEM_END_ADDR=d7fffff0


T19: Memory End d7fffff0


T19: Total memory available for disk cache: 724b3f0


T19: Total Number of Cache Lines 1824


T19: SS 128: mrs=5  lc=1824 ldc=1  ps=1 cm=ff ba=0 LDs: 0


T19: LD  0: L=5  SS=128  Size=10eec000  NL=1824  Status=2  DT=255  BT=512


T19:        span 0: sBlk=00000000, nBlk=043bb000, dev=00-01-02-05-04


T19: CM: 01 ff 00 00


T19: CM: 01 ff 00 00


T19: can_flush = 0


T19: No Reconst:Checking drive info


T19: MIGRATE: 40LD or 8ld new drive  ch 0 tgt 0


T19: REF drive found at ch 0 tgt 0


T19: Attempting to perform drive roaming


T19: AUTO_RES: NOT proceeding auto resume


T19: rebuildResume checksum is bad - initializing NVRAM structure


T19: NOT Flushing Cache


T19: RMW: NVRAM structure invalid - initializing


T19: Inside  SymRequestPoolInit


T19: Memory Pages USed=100


T19: Memory Pages USed=164


T19: RequestQ ADDR=d0210000


T19: exit   SymRequestPoolInit


T19: inside   SymMsgFifoInit


T19: InboundFreeQEnd=d0200000


T19: exit   SymMsgFifoInit


T19: inside   SymMsgFifoInit


T19: exit  SymMsgFifoInit


T19: Memory Pages USed=34a


T19: Memory Pages USed=34e


T19: Memory Pages USed=34f


T19: Memory Pages USed=374


T19: found 1 logical drives


T19: logDrv=0  Size=10eec000


T19: CM: 0b ff 00 00


T48: Ch <0> TM Wheel <0> Slots: [0]=0 [1]=0 [2]=1 [3]=0 [4]=0 [5]=0 Cur= [0]


T52: CQM: Timing wheel expired - Chip 0


T52: Ch <0> TM Wheel <0> Slots: [0]=0 [1]=0 [2]=1 [3]=0 [4]=0 [5]=0 Cur= [2]


T52: CQM_RecScanAllMsg (slot 2):Scanned 2, Disabled 0, Timeout 1 Msg


T52: LD Check: FCM_StartScsiBusReset


T52: StartScsiBusReset(chip 0, port 0):Send 0 msgs to LdWaitQ


T52: MISM: MPT_TaskMgmt chip 0 iob addr d0525190 task 4 ispMC=2 


T52: CQM4: calling FCM_CompleteScsiBusReset chip 0 port 0


T52: LD Check: FCM_CompleteScsiBusReset


T52: Recd TskMgmt Reply 6b IOCStatus 0 iob addr d0525190 type 4 chip 0 ispMC=0 


09/29 14:57:10: Time established at T57


09/29 14:57:10: BIOS CALL FOR DRV ROAMING : 55


09/29 14:57:10: drive roaming not done


09/29 14:57:11: PatrolReadTimer: Clearing PR completion bitmaps; next clear scheduled for 10/29 14:57:11


09/29 14:57:11: prDiskStart: starting Patrol Read on PD=00


09/29 14:57:11: prDiskStart: starting Patrol Read on PD=01


09/29 14:57:11: prDiskStart: starting Patrol Read on PD=02


09/29 14:57:11: prDiskStart: starting Patrol Read on PD=03


09/29 14:57:11: prDiskStart: starting Patrol Read on PD=04


09/29 14:57:11: prDiskStart: starting Patrol Read on PD=05


09/29 14:57:11: prDiskStart: starting Patrol Read on PD=08


09/29 14:57:11: Next PR scheduled to start at 09/29 16:00:00


09/29 14:57:17: LD Unlocked... No Lock... Done


09/29 14:57:49: HOST ISR: Sync cmd 0 arrived


09/29 14:59:25: MPT_ProcessIo Reply Fr c3 EVENT_NOTIFICATION


09/29 14:59:25: LD Check: FCM_StartScsiBusReset


09/29 14:59:25: StartScsiBusReset(chip 0, port 0):Send 0 msgs to LdWaitQ


09/29 14:59:25: CQM4: calling FCM_CompleteScsiBusReset chip 0 port 0


09/29 14:59:25: LD Check: FCM_CompleteScsiBusReset


09/29 14:59:25: MPT_ProcessIo Reply Fr c5 EVENT_ACK


09/29 14:59:57: LD Unlocked... No Lock... Done


�����������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������









tty-JR3XB1S.txt.log


TTY History for HA(0) -Bus 0x02 Device 0x02

������������������������������������

TTY History for HA(0) -Bus 0x02 Device 0x02

���������������������������������������CX�� @T0


(C) LSI Logic 2002 @T0


Megaraid Series 0 firmware version 350O @T0


Build date: Jun  1 2004 at 15:44:00 @T0


Board type: 1000/1960/1028/0518 @T0


DRAM_ALT sig invalid from previous boot @T0


FLUSH_ON_SYSTEM_RESET=0 @T0


WAIT FOR BIOS.... @T0


bus 1 dev 5 function 1 @T0


reg 0 value ffffff01 @T0


reg 1 value f7ff0004 @T0


reg 2 value 0 @T0


reg 3 value f7fe0004 @T0


reg 4 value 0 @T0


reg 5 value 0 @T0


bus 1 dev 5 function 0 @T0


reg 0 value fffffe01 @T0


reg 1 value f7fd0004 @T0


reg 2 value 0 @T0


reg 3 value f7fc0004 @T0


reg 4 value 0 @T0


reg 5 value 0 @T0


SCSI chip is on the secondary bus @T0


Found MPT LVD 30 at f7ff0004 @T0


 isp 0 membaseaddr 8bff0000 iobaseaddr 9001ff00  @T0


SCSI chip is on the secondary bus @T0


Found MPT LVD 30 at f7fd0004 @T0


 isp 1 membaseaddr 8bfd0000 iobaseaddr 9001fe00  @T0


BIOS UP! @T0


Can_flush=0 DRAM SIZE=128 MB @T0


* RST * @T0


Enabling data cache @T0


 @T0


Environment data: @T0


VALIDATION=None @T0


VERSIONS=CTLM_U821,BIOS_1.09,APP_350O,DBGR_N/A,BTBL_1.7.6DELL @T0


 @T0


pciDebug = d0008abc @T0


calling init_scsi @T0


DISK_CACHE_ADDR=d0db4c00  @T0


MEM_END_ADDR=d7fffff0  @T0


Total LSI MPT Chips found 2  @T0


LSI_InitMPT : start_index 0 totalLSIMPTChips 2 @T0


 Set to Get f/w features chip 0  @T0


	Verifying Image Signatures...VERIFIED @T0


	Verifying image check sum... VERIFIED @T0


BaseAddr 9001fe00 chip 1 @T0


Checking Diagnostic Register write access...	Enabled. @T0


reset adapter bit cleared @T0


Complete. @T0


Checking Diagnostic Register write access...	Enabled. @T0


The FW version being loaded is MPTFW-01.03.35.00-IT @T0


NextImageHeaderOffset 9c70 @T0


ExtImage Size 818 @T0


Diag, Register disabled DIAGNOSTIC_REG 131 @T0


FW download complete... Expecting LSI FW to start excute and come to ready state @T0


 For this sys doorbell reg bit 28 should be set  @T0


 MISM CHN_STATE_MPT_GET_FW_FEAT chip 1  @T3


 Check IOC FACTS chip 1  @T3


 MISM CHN_STATE_MPT_OPERATIONAL chip 1  @T3


 MISM: Reply frame size 60 start addr d054ec3c  @T3


fe reply free frames posted @T3


 MISM CHN_STATE_MPT_INIT_BUS_RST chip 1  @T3


MPT_Poll: chip 0 CHN_STATE_MPT_WAIT @T3


 MISM CHN_STATE_MPT_GET_FW_FEAT chip 0  @T3


 Check IOC FACTS chip 0  @T3


 MISM CHN_STATE_MPT_OPERATIONAL chip 0  @T3


 MISM: Reply frame size 60 start addr d0554be0  @T3


fe reply free frames posted @T3


 MISM CHN_STATE_MPT_INIT_BUS_RST chip 0  @T3


MPT_Poll: chip 1 CHN_STATE_MPT_INIT_BUS_RST  @T3


 MISM CHN_STATE_MPT_INIT_BUS_RLY chip 1  @T3


 MISM CHN_STATE_MPT_READY chip 1  @T6


cmdBufferAddr = f854e9c0, ioIdx = 0 @T6


cmdBufferAddr = f854ea58, ioIdx = 1 @T6


CommandBufferPost Post: Request = d0020200 @T6


DISM: Queued! @T6


MPT_Poll: chip 0 CHN_STATE_MPT_INIT_BUS_RST  @T6


 MISM CHN_STATE_MPT_INIT_BUS_RLY chip 0  @T6


 MISM CHN_STATE_MPT_READY chip 0  @T9


cmdBufferAddr = f854e6c0, ioIdx = 0 @T9


cmdBufferAddr = f854e758, ioIdx = 1 @T9


CommandBufferPost Post: Request = d0020200 @T9


DISM: Queued! @T9


 MPT_ProcessIo Reply Fr 2 EVENT_NOTIFICATION @T9


MPI_EVENT_EVENT_CHANGE @T9


 MPT_ProcessIo Reply Fr 2 EVENT_NOTIFICATION @T9


MPI_EVENT_EVENT_CHANGE @T9


DISM: Cluster peer is 0x6 @T9


DISM: CR8 SAFTE at chan 0 id f @T11


DISM_ProcessPprState: DomainVal done on all disks @T13


DISM: Complete!!! @T15


bbuDebugFlags = d000d94c @T15


batteryinit: TransBBU pack present @T20


TBBU: TBBU h/w . Cache not dirty @T20


Veirfyin config struct at Addr e0001400  @T20


NVRAM checksum OK - reading configuration @T20


DISK_CACHE_ADDR=d0db4c00  @T20


MEM_END_ADDR=d7fffff0  @T20


Memory End d7fffff0 @T20


Total Number of Cache Lines 1822 @T20


L 5   SS 128   Size 10eec000   N 1822   Status 2   DT  1293   BT 1755 @T20


NVF: Updating NV e0004f82 with data 0 @T20


can_flush = 0 @T21


No Reconst:Checking drive info @T21


 @T21


REF drive found at ch 0 tgt 0  @T21


comparing disk 1 with nvram @T21


src chn 0  drvptr d048e08c dst chn 0 nvramptr d04d044c @T21


check config okay.... @T21


comparing disk 2 with nvram @T21


src chn 0  drvptr d048e08c dst chn 0 nvramptr d04d044c @T21


check config okay.... @T21


comparing disk 3 with nvram @T21


src chn 0  drvptr d048e08c dst chn 0 nvramptr d04d044c @T21


check config okay.... @T21


comparing disk 4 with nvram @T21


src chn 0  drvptr d048e08c dst chn 0 nvramptr d04d044c @T21


check config okay.... @T21


comparing disk 5 with nvram @T21


src chn 0  drvptr d048e08c dst chn 0 nvramptr d04d044c @T21


check config okay.... @T21


comparing disk 8 with nvram @T21


src chn 0  drvptr d048e08c dst chn 0 nvramptr d04d044c @T21


check config okay.... @T21


Attempting to perform drive roaming @T21


NOT Flushing Cache @T21


check to start Patrol read @T21


BIOS CALL FOR DRV ROAMING : 55 @7/1 16:8:1


drive roaming not done @7/1 16:8:1


HOST ISR: Sync cmd 0 arrived @7/1 16:8:39


MPT_Rec: INQ Error - Negotiating LD[6] pRfm d00ada40 @7/1 16:9:18


MPT_Rec: INQ Error - Negotiating LD[6] pRfm d00af600 @7/1 16:9:25


MPT_Rec: INQ Error - Negotiating LD[6] pRfm d00b0800 @7/1 16:9:26


MPT_Rec: INQ Error - Negotiating LD[6] pRfm d00bbc00 @7/1 16:10:1


MPT_Rec: INQ Error - Negotiating LD[6] pRfm d0091c00 @7/1 16:10:9


 MPT_ProcessIo Reply Fr cb EVENT_NOTIFICATION @7/7 14:53:42


 MPT_ProcessIo Reply Fr cc EVENT_ACK @7/7 14:53:42


TMM:Setting Peer id to 6 @7/7 14:53:47


TMM:oper_chn_bmap is 0x1, operating_chn: 0x0 @7/7 14:53:47


TMM:Setting Peer id to 6 @7/7 14:53:58


TMM:oper_chn_bmap is 0x1, operating_chn: 0x0 @7/7 14:53:58


MPT_Rec: INQ Error - Negotiating LD[6] pRfm d00970c0 @7/14 13:30:52


 MPT_ProcessIo Reply Fr 3e EVENT_NOTIFICATION @7/24 0:50:16


LD Check: FCM_StartScsiBusReset @7/24 0:50:16


StartScsiBusReset(chip 0):Send 0 msgs to LdWaitQ @7/24 0:50:16


 MPT_ProcessIo Reply Fr 3f EVENT_ACK @7/24 0:50:16


CQM4: calling FCM_CompleteScsiBusReset chip 0 port 0 @7/24 0:50:17


LD Check: FCM_CompleteScsiBusReset @7/24 0:50:17


 MPT_ProcessIo Reply Fr 41 EVENT_NOTIFICATION @7/24 0:50:17


LD Check: FCM_StartScsiBusReset @7/24 0:50:17


StartScsiBusReset(chip 0):Send 0 msgs to LdWaitQ @7/24 0:50:17


CQM2: calling FCM_CompleteScsiBusReset chip 0 port 0 @7/24 0:50:17


LD Check: FCM_CompleteScsiBusReset @7/24 0:50:17


 MPT_ProcessIo Reply Fr 43 EVENT_ACK @7/24 0:50:17


LD Unlocked... No Lock... Done @7/24 0:50:32


 MPT_ProcessIo Reply Fr aa EVENT_NOTIFICATION @9/29 2:12:19


LD Check: FCM_StartScsiBusReset @9/29 2:12:19


StartScsiBusReset(chip 0):Send 0 msgs to LdWaitQ @9/29 2:12:19


CQM2: calling FCM_CompleteScsiBusReset chip 0 port 0 @9/29 2:12:19


LD Check: FCM_CompleteScsiBusReset @9/29 2:12:19


 MPT_ProcessIo Reply Fr b4 EVENT_ACK @9/29 2:12:19


LD Unlocked... No Lock... Done @9/29 2:12:24


TMM:Setting Peer id to 6 @9/29 2:12:24


TMM:oper_chn_bmap is 0x1, operating_chn: 0x0 @9/29 2:12:24


TMM:Setting Peer id to 6 @9/29 2:12:35


TMM:oper_chn_bmap is 0x1, operating_chn: 0x0 @9/29 2:12:35


 MPT_ProcessIo Reply Fr 1b EVENT_NOTIFICATION @9/29 2:21:14


LD Check: FCM_StartScsiBusReset @9/29 2:21:14


StartScsiBusReset(chip 0):Send 0 msgs to LdWaitQ @9/29 2:21:14


CQM2: calling FCM_CompleteScsiBusReset chip 0 port 0 @9/29 2:21:14


LD Check: FCM_CompleteScsiBusReset @9/29 2:21:14


 MPT_ProcessIo Reply Fr 28 EVENT_ACK @9/29 2:21:14


TMM:Setting Peer id to 6 @9/29 2:21:21


TMM:oper_chn_bmap is 0x1, operating_chn: 0x0 @9/29 2:21:21


LD Unlocked... No Lock... Done @9/29 2:21:28


TMM:Setting Peer id to 6 @9/29 2:21:31


TMM:oper_chn_bmap is 0x1, operating_chn: 0x0 @9/29 2:21:31


 MPT_ProcessIo Reply Fr 48 EVENT_NOTIFICATION @9/29 2:22:43


LD Check: FCM_StartScsiBusReset @9/29 2:22:43


StartScsiBusReset(chip 0):Send 0 msgs to LdWaitQ @9/29 2:22:43


CQM2: calling FCM_CompleteScsiBusReset chip 0 port 0 @9/29 2:22:43


LD Check: FCM_CompleteScsiBusReset @9/29 2:22:43


 MPT_ProcessIo Reply Fr 5b EVENT_ACK @9/29 2:22:43


LD Unlocked... No Lock... Done @9/29 2:22:48


TMM:Setting Peer id to 6 @9/29 2:22:51


TMM:oper_chn_bmap is 0x1, operating_chn: 0x0 @9/29 2:22:51


TMM:Setting Peer id to 6 @9/29 2:23:0


TMM:oper_chn_bmap is 0x1, operating_chn: 0x0 @9/29 2:23:0


LD Check: FCM_StartScsiBusReset @9/29 2:25:45


StartScsiBusReset(chip 0):Send 0 msgs to LdWaitQ @9/29 2:25:45


CQM2: calling FCM_CompleteScsiBusReset chip 0 port 0 @9/29 2:25:45


LD Check: FCM_CompleteScsiBusReset @9/29 2:25:45


 MPT_ProcessIo Reply Fr 8d EVENT_NOTIFICATION @9/29 2:25:45


LD Check: FCM_StartScsiBusReset @9/29 2:25:45


StartScsiBusReset(chip 0):Send 0 msgs to LdWaitQ @9/29 2:25:45


CQM2: calling FCM_CompleteScsiBusReset chip 0 port 0 @9/29 2:25:45


LD Check: FCM_CompleteScsiBusReset @9/29 2:25:45


 MPT_ProcessIo Reply Fr 8f EVENT_ACK @9/29 2:25:45


TMM:Setting Peer id to 6 @9/29 2:25:51


TMM:oper_chn_bmap is 0x1, operating_chn: 0x0 @9/29 2:25:51


LD Unlocked... No Lock... Done @9/29 2:26:0


TMM:Setting Peer id to 6 @9/29 2:26:0


TMM:oper_chn_bmap is 0x1, operating_chn: 0x0 @9/29 2:26:0


SCSI_PROTOCOL_ERR:REC: pRfm d00a0d80, LD[1] - Ioc d0559560, iocStatus 47 @9/29 2:29:2


 MPT_ProcessIo Reply Fr ca EVENT_NOTIFICATION @9/29 2:29:33


 MPT_ProcessIo Reply Fr cb EVENT_ACK @9/29 2:29:33


TMM:Setting Peer id to 6 @9/29 2:29:39


TMM:oper_chn_bmap is 0x1, operating_chn: 0x0 @9/29 2:29:39


TMM:Setting Peer id to 6 @9/29 2:29:48


TMM:oper_chn_bmap is 0x1, operating_chn: 0x0 @9/29 2:29:48


LD Check: FCM_StartScsiBusReset @9/29 2:33:54


StartScsiBusReset(chip 0):Send 0 msgs to LdWaitQ @9/29 2:33:54


CQM2: calling FCM_CompleteScsiBusReset chip 0 port 0 @9/29 2:33:54


LD Check: FCM_CompleteScsiBusReset @9/29 2:33:54


 MPT_ProcessIo Reply Fr 12 EVENT_NOTIFICATION @9/29 2:33:54


LD Check: FCM_StartScsiBusReset @9/29 2:33:54


StartScsiBusReset(chip 0):Send 0 msgs to LdWaitQ @9/29 2:33:54


CQM2: calling FCM_CompleteScsiBusReset chip 0 port 0 @9/29 2:33:54


LD Check: FCM_CompleteScsiBusReset @9/29 2:33:54


 MPT_ProcessIo Reply Fr 14 EVENT_ACK @9/29 2:33:54


TMM:Setting Peer id to 6 @9/29 2:33:59


TMM:oper_chn_bmap is 0x1, operating_chn: 0x0 @9/29 2:33:59


LD Unlocked... No Lock... Done @9/29 2:34:0


TMM:Setting Peer id to 6 @9/29 2:34:9


TMM:oper_chn_bmap is 0x1, operating_chn: 0x0 @9/29 2:34:9


SCSI_PROTOCOL_ERR:REC: pRfm d0099280, LD[5] - Ioc d05567a0, iocStatus 47 @9/29 2:37:15


�[2J @T0


(C) LSI Logic 2002 @T0


Megaraid Series 0 firmware version 350O @T0


Build date: Jun  1 2004 at 15:44:00 @T0


Board type: 1000/1960/1028/0518 @T0


FLUSH_ON_SYSTEM_RESET=0 @T0


WAIT FOR BIOS.... @T0


bus 1 dev 5 function 1 @T0


reg 0 value ffffff01 @T0


reg 1 value f7ff0004 @T0


reg 2 value 0 @T0


reg 3 value f7fe0004 @T0


reg 4 value 0 @T0


reg 5 value 0 @T0


bus 1 dev 5 function 0 @T0


reg 0 value fffffe01 @T0


reg 1 value f7fd0004 @T0


reg 2 value 0 @T0


reg 3 value f7fc0004 @T0


reg 4 value 0 @T0


reg 5 value 0 @T0


SCSI chip is on the secondary bus @T0


Found MPT LVD 30 at f7ff0004 @T0


 isp 0 membaseaddr 8bff0000 iobaseaddr 9001ff00  @T0


SCSI chip is on the secondary bus @T0


Found MPT LVD 30 at f7fd0004 @T0


 isp 1 membaseaddr 8bfd0000 iobaseaddr 9001fe00  @T0


BIOS UP! @T0


Can_flush=0 DRAM SIZE=128 MB @T0


* RST * @T0


Enabling data cache @T0


 @T0


Environment data: @T0


VALIDATION=None @T0


VERSIONS=CTLM_U821,BIOS_1.09,APP_350O,DBGR_N/A,BTBL_1.7.6DELL @T0


 @T0


pciDebug = d0008abc @T0


calling init_scsi @T0


DISK_CACHE_ADDR=d0db4c00  @T0


MEM_END_ADDR=d7fffff0  @T0


Total LSI MPT Chips found 2  @T0


LSI_InitMPT : start_index 0 totalLSIMPTChips 2 @T0


 Set to Get f/w features chip 0  @T0


	Verifying Image Signatures...VERIFIED @T0


	Verifying image check sum... VERIFIED @T0


BaseAddr 9001fe00 chip 1 @T0


Checking Diagnostic Register write access...	Enabled. @T0


reset adapter bit cleared @T0


Complete. @T0


Checking Diagnostic Register write access...	Enabled. @T0


The FW version being loaded is MPTFW-01.03.35.00-IT @T0


NextImageHeaderOffset 9c70 @T0


ExtImage Size 818 @T0


Diag, Register disabled DIAGNOSTIC_REG 131 @T0


FW download complete... Expecting LSI FW to start excute and come to ready state @T0


 For this sys doorbell reg bit 28 should be set  @T0


 MISM CHN_STATE_MPT_GET_FW_FEAT chip 1  @T3


 Check IOC FACTS chip 1  @T3


 MISM CHN_STATE_MPT_OPERATIONAL chip 1  @T3


 MISM: Reply frame size 60 start addr d054ec3c  @T3


fe reply free frames posted @T3


 MISM CHN_STATE_MPT_INIT_BUS_RST chip 1  @T3


MPT_Poll: chip 0 CHN_STATE_MPT_WAIT @T3


 MISM CHN_STATE_MPT_GET_FW_FEAT chip 0  @T3


 Check IOC FACTS chip 0  @T3


 MISM CHN_STATE_MPT_OPERATIONAL chip 0  @T3


 MISM: Reply frame size 60 start addr d0554be0  @T3


fe reply free frames posted @T3


 MISM CHN_STATE_MPT_INIT_BUS_RST chip 0  @T3


MPT_Poll: chip 1 CHN_STATE_MPT_INIT_BUS_RST  @T3


 MISM CHN_STATE_MPT_INIT_BUS_RLY chip 1  @T3


 MISM CHN_STATE_MPT_READY chip 1  @T6


cmdBufferAddr = f854e9c0, ioIdx = 0 @T6


cmdBufferAddr = f854ea58, ioIdx = 1 @T6


CommandBufferPost Post: Request = d0020200 @T6


DISM: Queued! @T6


MPT_Poll: chip 0 CHN_STATE_MPT_INIT_BUS_RST  @T6


 MISM CHN_STATE_MPT_INIT_BUS_RLY chip 0  @T6


 MISM CHN_STATE_MPT_READY chip 0  @T9


cmdBufferAddr = f854e6c0, ioIdx = 0 @T9


cmdBufferAddr = f854e758, ioIdx = 1 @T9


CommandBufferPost Post: Request = d0020200 @T9


DISM: Queued! @T9


 MPT_ProcessIo Reply Fr 2 EVENT_NOTIFICATION @T9


MPI_EVENT_EVENT_CHANGE @T9


 MPT_ProcessIo Reply Fr 2 EVENT_NOTIFICATION @T9


MPI_EVENT_EVENT_CHANGE @T9


setting pRfm flagin pRfm = d0081640, flag = 420, dev = 2 @T9


setting pRfm flagin pRfm = d00817c0, flag = 420, dev = 4 @T9


setting pRfm flagin pRfm = d0081880, flag = 420, dev = 5 @T9


DISM: Cluster peer is 0x6 @T9


DISM: CR8 SAFTE at chan 0 id f @T11


DISM_ProcessPprState: DomainVal done on all disks @T13


DISM: Complete!!! @T15


bbuDebugFlags = d000d94c @T15


batteryinit: TransBBU pack present @T20


TBBU: TBBU h/w . Cache not dirty @T20


Veirfyin config struct at Addr e0001400  @T20


NVRAM checksum OK - reading configuration @T20


DISK_CACHE_ADDR=d0db4c00  @T20


MEM_END_ADDR=d7fffff0  @T20


Memory End d7fffff0 @T20


Total Number of Cache Lines 1822 @T20


L 5   SS 128   Size 10eec000   N 1822   Status 2   DT  1293   BT 1755 @T20


NVF: Updating NV e0004f82 with data 0 @T20


can_flush = 0 @T20


No Reconst:Checking drive info @T20


 @T20


REF drive found at ch 0 tgt 0  @T20


comparing disk 1 with nvram @T20


src chn 0  drvptr d048e08c dst chn 0 nvramptr d04d044c @T20


check config okay.... @T20


comparing disk 2 with nvram @T20


src chn 0  drvptr d048e08c dst chn 0 nvramptr d04d044c @T21


check config okay.... @T21


comparing disk 3 with nvram @T21


src chn 0  drvptr d048e08c dst chn 0 nvramptr d04d044c @T21


check config okay.... @T21


comparing disk 4 with nvram @T21


src chn 0  drvptr d048e08c dst chn 0 nvramptr d04d044c @T21


check config okay.... @T21


comparing disk 5 with nvram @T21


src chn 0  drvptr d048e08c dst chn 0 nvramptr d04d044c @T21


check config okay.... @T21


comparing disk 8 with nvram @T21


src chn 0  drvptr d048e08c dst chn 0 nvramptr d04d044c @T21


check config okay.... @T21


Attempting to perform drive roaming @T21


NOT Flushing Cache @T21


check to start Patrol read @T21


BIOS CALL FOR DRV ROAMING : 55 @9/29 14:53:33


drive roaming not done @9/29 14:53:33


Flash: Waiting to Free all IO @9/29 14:54:30


Flash: All IO over - Allocating memory @9/29 14:54:30


Flash: Writing to Flash Rom... @9/29 14:55:8


Flash:  Ready to flash...disabling all serial port output @9/29 14:55:9


NVF: Updating NV e0004f6c with data fe @9/29 14:55:41








T0: LSI Logic MegaRAID i960 firmware loaded


T0: Firmware version 351S build on May 31 2005 at 21:05:31


T0: Board is type 1000/1960/1028/0518





T0: Can_flush=1


T0: DRAM SIZE=128 MB


T0: FLUSH_ON_SYSTEM_RESET=0





T0: (C) LSI Logic Corporation 2002


T0: MegaRAID Series 518 firmware version 351S


T0: WAIT FOR BIOS....


T0: BIOS UP!


T0: Enabling data cache


T0: EepromInit: Family=14, SN=c94954010000


T0: EepromInit: Board SN not programmed


T0: EepromInit: Family=14, SN=c94954010000


T0: EepromInit: Board SN not programmed





T0: Environment data:


T0: VALIDATION=None





T0: MFC data:


T0:     Vendor/DeviceID=1000/1960


T0:     SubVendor/SubDevice=1028/0518


T0:     OEM=Dell, clusterDisable=0, flexRaidDisable=0


T0:     rebuildRate=30, stripeSize=128, flushTime=4


T0:     cachedIo=1, writeBack=1, readAhead=0


T0:     channelBase='0', smartMode=6, alarmDisable=0


T0:     fastInitDisable=0, coercion=128M, disablePredictiveFail=0


T0:     disableWebBios=1, disableCtrlM=0, writeThroughWhenBatteryBad=1


T0:     zcrConfig=Undefined, keepSafteFailedStatus=0, autoHotSpareRestore=0


T0:     variableChkConRate=0, enableNvramDiskMgmtChange=0


T0:     dirtyLedShowsDriveActivity=0, disableConsChkRestoration=0


T0:     biosContinueOnError=0, biosAutoConfig=Prompt


T0:     disableRandomDriveDeletion=0





T0: PatrolReadInit Checksum invalid - initializing data


T0: CM: 00 ff 00 00


T0: DISK_CACHE_ADDR=d0db4800


T0: MEM_END_ADDR=d7fffff0


T0: Found MPT LVD 1030(1/5/1) at f7ff0004/0, mapped to 8bff0000


T0: Found MPT LVD 1030(1/5/0) at f7fd0004/0, mapped to 8bfd0000


T0: Total LSI MPT Chips found 2 


T0: LSI_InitMPT : start_index 0 totalLSIMPTChips 2


T0: Set to Get f/w features chip 0 


T0: 	Verifying Image Signatures...VERIFIED


T0: 	Verifying image check sum... VERIFIED


T0: The FW version being loaded is MPTFW-01.03.35.00-IT


T0: NextImageHeaderOffset=9c70, ExtImageSize=818


T0: FW download complete... Expecting LSI FW to start excute and come to ready state


T3: MISM CHN_STATE_MPT_GET_FW_FEAT chip 1 


T3: PRESENT SCSI_ID = 0


T3: Changing scsiId to  7


T3: Check IOC FACTS chip 1


T3: MISM CHN_STATE_MPT_OPERATIONAL chip 1 


T3: MISM: Reply frame size 60 start addr d05258c0 


T3: fe reply free frames posted


T3: MISM CHN_STATE_MPT_INIT_BUS_RST chip 1 


T3: MPT_Poll: chip 0 CHN_STATE_MPT_WAIT


T3: MISM CHN_STATE_MPT_GET_FW_FEAT chip 0 


T4: PRESENT SCSI_ID = 0


T4: Changing scsiId to  7


T4: Check IOC FACTS chip 0


T4: MISM CHN_STATE_MPT_OPERATIONAL chip 0 


T4: MISM: Reply frame size 60 start addr d052b880 


T4: fe reply free frames posted


T4: MISM CHN_STATE_MPT_INIT_BUS_RST chip 0 


T4: MPT_Poll: chip 1 CHN_STATE_MPT_INIT_BUS_RST 


T6: MPT_Poll: chip 0 CHN_STATE_MPT_INIT_BUS_RST 


T8: DISM: Queued!


T8: MPT_ProcessIo Reply Fr 2 EVENT_NOTIFICATION


T8: MPI_EVENT_EVENT_CHANGE


T8: MPT_SetIocPageParameters: After Write CoalescingDepth=1, Timeout=0, Flags=1 


T8: MPT_ProcessIo Reply Fr 2 EVENT_NOTIFICATION


T8: MPI_EVENT_EVENT_CHANGE


T8: MPT_SetIocPageParameters: After Write CoalescingDepth=1, Timeout=0, Flags=1 


T8: DISM: Cluster peer is 0x6


T8: CM: 09 ff 01 00


T8: DISM: oper_chn_bmap is 1, operating_chn: 0


T9: CM: 01 ff 00 00


T12: DISM_ProcessPprState: DomainVal done on all disks


T14: DISM: Complete!!!





T14: Physical device info:


T14: CM: 01 ff 00 00


T14: ID  NVRState  Vendor    Product           Rev    6   7  56


T14: --  --------  --------  ----------------  ----  --  --  --


T14: 00  Online    FUJITSU   MAP3735NC         5608  01  3a  0f


T14: 01  Online    FUJITSU   MAP3735NC         5608  01  3a  0f


T14: 02  Online    SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 03  HotSpare  SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 04  Online    SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 05  Online    SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 06  Ready     DELL      PERC 4/DC         351S  01  00  00


T14: 08  HotSpare  FUJITSU   MAT3073NC         5704  01  3a  0d


T14: 0f  Ready     DELL      PV22XS            E.17  40  00  00





T19: batteryinit: TransBBU pack present


T19: TBBU: TBBU h/w . Cache not dirty


T19: Verifying config struct at Addr e0001400


T19: NVRAM checksum OK - reading configuration


T19: DISK_CACHE_ADDR=d0db4800


T19: MEM_END_ADDR=d7fffff0


T19: Memory End d7fffff0


T19: Total memory available for disk cache: 724b3f0


T19: Total Number of Cache Lines 1824


T19: SS 128: mrs=5  lc=1824 ldc=1  ps=1 cm=ff ba=0 LDs: 0


T19: LD  0: L=5  SS=128  Size=10eec000  NL=1824  Status=2  DT=255  BT=512


T19:        span 0: sBlk=00000000, nBlk=043bb000, dev=00-01-02-05-04


T19: CM: 01 ff 00 00


T19: CM: 01 ff 00 00


T19: can_flush = 0


T19: No Reconst:Checking drive info


T19: MIGRATE: 40LD or 8ld new drive  ch 0 tgt 0


T19: REF drive found at ch 0 tgt 0


T19: Attempting to perform drive roaming


T19: AUTO_RES: NOT proceeding auto resume


T19: rebuildResume checksum is bad - initializing NVRAM structure


T19: NOT Flushing Cache


T19: RMW: NVRAM structure invalid - initializing


T19: Inside  SymRequestPoolInit


T19: Memory Pages USed=100


T19: Memory Pages USed=164


T19: RequestQ ADDR=d0210000


T19: exit   SymRequestPoolInit


T19: inside   SymMsgFifoInit


T19: InboundFreeQEnd=d0200000


T19: exit   SymMsgFifoInit


T19: inside   SymMsgFifoInit


T19: exit  SymMsgFifoInit


T19: Memory Pages USed=34a


T19: Memory Pages USed=34e


T19: Memory Pages USed=34f


T19: Memory Pages USed=374


T19: found 1 logical drives


T19: logDrv=0  Size=10eec000


T19: CM: 0b ff 00 00


T48: Ch <0> TM Wheel <0> Slots: [0]=0 [1]=0 [2]=1 [3]=0 [4]=0 [5]=0 Cur= [0]


T52: CQM: Timing wheel expired - Chip 0


T52: Ch <0> TM Wheel <0> Slots: [0]=0 [1]=0 [2]=1 [3]=0 [4]=0 [5]=0 Cur= [2]


T52: CQM_RecScanAllMsg (slot 2):Scanned 2, Disabled 0, Timeout 1 Msg


T52: LD Check: FCM_StartScsiBusReset


T52: StartScsiBusReset(chip 0, port 0):Send 0 msgs to LdWaitQ


T52: MISM: MPT_TaskMgmt chip 0 iob addr d0525190 task 4 ispMC=2 


T52: CQM4: calling FCM_CompleteScsiBusReset chip 0 port 0


T52: LD Check: FCM_CompleteScsiBusReset


T52: Recd TskMgmt Reply 6b IOCStatus 0 iob addr d0525190 type 4 chip 0 ispMC=0 


09/29 14:57:10: Time established at T57


09/29 14:57:10: BIOS CALL FOR DRV ROAMING : 55


09/29 14:57:10: drive roaming not done


09/29 14:57:11: PatrolReadTimer: Clearing PR completion bitmaps; next clear scheduled for 10/29 14:57:11


09/29 14:57:11: prDiskStart: starting Patrol Read on PD=00


09/29 14:57:11: prDiskStart: starting Patrol Read on PD=01


09/29 14:57:11: prDiskStart: starting Patrol Read on PD=02


09/29 14:57:11: prDiskStart: starting Patrol Read on PD=03


09/29 14:57:11: prDiskStart: starting Patrol Read on PD=04


09/29 14:57:11: prDiskStart: starting Patrol Read on PD=05


09/29 14:57:11: prDiskStart: starting Patrol Read on PD=08


09/29 14:57:11: Next PR scheduled to start at 09/29 16:00:00


09/29 14:57:17: LD Unlocked... No Lock... Done


09/29 14:57:49: HOST ISR: Sync cmd 0 arrived


09/29 14:59:25: MPT_ProcessIo Reply Fr c3 EVENT_NOTIFICATION


09/29 14:59:25: LD Check: FCM_StartScsiBusReset


09/29 14:59:25: StartScsiBusReset(chip 0, port 0):Send 0 msgs to LdWaitQ


09/29 14:59:25: CQM4: calling FCM_CompleteScsiBusReset chip 0 port 0


09/29 14:59:25: LD Check: FCM_CompleteScsiBusReset


09/29 14:59:25: MPT_ProcessIo Reply Fr c5 EVENT_ACK


09/29 14:59:57: LD Unlocked... No Lock... Done


�����������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������










3cy381s.zip




DSET Report for ss {L4-LC2 SvcTag-3CY381S-PowerEdge 2600}.zip






READ ME before viewing this report!.txt





=======================================================

Instructions for the proper viewing of this DSET report

=======================================================





1) Decompress ALL files within this report into one folder using any ZIP utility 



2) When prompted enter the password "dell" (no quotes, all lowercase letters) to decrypt

   the report contents during decompression



3) Then open "dsetreport.hta" in Internet Explorer 5.0 or newer (which must support

   viewing XML data)





NOTE: Do NOT open the ZIP report file using the built-in Windows XP/2003 .ZIP expansion

      utility then double-clicking "dsetreport.hta". Other GUI support files must also

      be decompressed into the folder in order for the DSET graphical web interface to

      display properly. The DSET .ZIP report file must be entirely decompressed to a

      folder first before opening "dsetreport.hta".



NOTE: Dsetreport.hta is considered an "HTML Application" for purposes of secure local

      operation on Windows. Double-clicking the file will launch the HTML rendering engine

      of Windows which looks very similar to Internet Explorer. Do not rename the file

      extension to ".htm" as unpredictable behavior may result.



NOTE: Netscape and Mozilla are NOT supported at this time for displaying the DSET

      graphical interface



NOTE: Various log and configuration files can be found categorized within the "log"

      folder of this report








dsetreport.hta









po-3CY381S.txt




TTY History for HA(0) -Bus 0x0a Device 0x06

���������������������������������������9���





T0: LSI Logic MegaRAID i960 firmware loaded


T0: Firmware version 351S build on May 31 2005 at 21:05:31


T0: Board is type 1000/1960/1028/0518





T0: Can_flush=0


T0: DRAM SIZE=128 MB


T0: DRAM_ALT sig invalid from previous boot


T0: FLUSH_ON_SYSTEM_RESET=0





T0: (C) LSI Logic Corporation 2002


T0: MegaRAID Series 518 firmware version 351S


T0: WAIT FOR BIOS....


T0: BIOS UP!


T0: Enabling data cache


T0: EepromInit: Family=14, SN=94a5f2010000


T0: EepromInit: Board SN not programmed





T0: Environment data:


T0: VALIDATION=None





T0: MFC data:


T0:     Vendor/DeviceID=1000/1960


T0:     SubVendor/SubDevice=1028/0518


T0:     OEM=Dell, clusterDisable=0, flexRaidDisable=0


T0:     rebuildRate=30, stripeSize=128, flushTime=4


T0:     cachedIo=1, writeBack=1, readAhead=0


T0:     channelBase='0', smartMode=6, alarmDisable=0


T0:     fastInitDisable=0, coercion=128M, disablePredictiveFail=0


T0:     disableWebBios=1, disableCtrlM=0, writeThroughWhenBatteryBad=1


T0:     zcrConfig=Undefined, keepSafteFailedStatus=0, autoHotSpareRestore=0


T0:     variableChkConRate=0, enableNvramDiskMgmtChange=0


T0:     dirtyLedShowsDriveActivity=0, disableConsChkRestoration=0


T0:     biosContinueOnError=0, biosAutoConfig=Prompt


T0:     disableRandomDriveDeletion=0





T0: DISK_CACHE_ADDR=d0db4800


T0: MEM_END_ADDR=d7fffff0


T0: Found MPT LVD 1030(1/5/1) at f7ff0004/0, mapped to 8bff0000


T0: Found MPT LVD 1030(1/5/0) at f7fd0004/0, mapped to 8bfd0000


T0: Total LSI MPT Chips found 2 


T0: LSI_InitMPT : start_index 0 totalLSIMPTChips 2


T0: Set to Get f/w features chip 0 


T0: 	Verifying Image Signatures...VERIFIED


T0: 	Verifying image check sum... VERIFIED


T0: The FW version being loaded is MPTFW-01.03.35.00-IT


T0: NextImageHeaderOffset=9c70, ExtImageSize=818


T0: FW download complete... Expecting LSI FW to start excute and come to ready state


T3: MISM CHN_STATE_MPT_GET_FW_FEAT chip 1 


T3: PRESENT SCSI_ID = 0


T3: Changing scsiId to  7


T3: Check IOC FACTS chip 1


T3: MISM CHN_STATE_MPT_OPERATIONAL chip 1 


T3: MISM: Reply frame size 60 start addr d05258c0 


T3: fe reply free frames posted


T3: MISM CHN_STATE_MPT_INIT_BUS_RST chip 1 


T3: MPT_Poll: chip 0 CHN_STATE_MPT_WAIT


T3: MISM CHN_STATE_MPT_GET_FW_FEAT chip 0 


T3: PRESENT SCSI_ID = 0


T3: Changing scsiId to  7


T3: Check IOC FACTS chip 0


T3: MISM CHN_STATE_MPT_OPERATIONAL chip 0 


T3: MISM: Reply frame size 60 start addr d052b880 


T3: fe reply free frames posted


T3: MISM CHN_STATE_MPT_INIT_BUS_RST chip 0 


T4: MPT_Poll: chip 1 CHN_STATE_MPT_INIT_BUS_RST 


T6: MPT_Poll: chip 0 CHN_STATE_MPT_INIT_BUS_RST 


T8: DISM: Queued!


T8: MPT_ProcessIo Reply Fr 2 EVENT_NOTIFICATION


T8: MPI_EVENT_EVENT_CHANGE


T8: MPT_SetIocPageParameters: After Write CoalescingDepth=1, Timeout=0, Flags=1 


T8: MPT_ProcessIo Reply Fr 2 EVENT_NOTIFICATION


T8: MPI_EVENT_EVENT_CHANGE


T8: MPT_SetIocPageParameters: After Write CoalescingDepth=1, Timeout=0, Flags=1 


T8: DISM: Cluster peer is 0x6


T8: DISM: oper_chn_bmap is 1, operating_chn: 0


T11: DISM_ProcessPprState: DomainVal done on all disks


T14: DISM: Complete!!!





T14: Physical device info:


T14: ID  NVRState  Vendor    Product           Rev    6   7  56


T14: --  --------  --------  ----------------  ----  --  --  --


T14: 00  Online    SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 01  Online    SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 02  Online    SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 03  Online    FUJITSU   MAT3073NC         5704  01  3a  0d


T14: 04  Online    SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 05  HotSpare  FUJITSU   MAT3073NC         5704  01  3a  0d


T14: 06  Ready     DELL      PERC 4/DC         350O  00  00  00


T14: 08  Online    FUJITSU   MAT3073NC         5704  01  3a  0d


T14: 09  Online    FUJITSU   MAP3367NC         5605  01  3a  0f


T14: 0a  Online    SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 0f  Ready     DELL      PV22XS            E.17  40  00  00





T19: batteryinit: TransBBU pack present


T19: TBBU: TBBU h/w . Cache not dirty


T19: Verifying config struct at Addr e0001400


T19: NVRAM checksum OK - reading configuration


T20: DISK_CACHE_ADDR=d0db4800


T20: MEM_END_ADDR=d7fffff0


T20: Memory End d7fffff0


T20: Total memory available for disk cache: 724b3f0


T20: Total Number of Cache Lines 1824


T20: SS 128: mrs=2  lc=1824 ldc=1  ps=1 cm=ff ba=0 LDs: 0


T20: LD  0: L=1  SS=128  Size=10ef0000  NL=1824  Status=2  DT=255  BT=512


T20:        span 0: sBlk=00000000, nBlk=043bc000, dev=00-01


T20:        span 1: sBlk=00000000, nBlk=043bc000, dev=03-02


T20:        span 2: sBlk=00000000, nBlk=043bc000, dev=04-0a


T20:        span 3: sBlk=00000000, nBlk=043bc000, dev=09-08


T20: can_flush = 0


T20: No Reconst:Checking drive info


T20: MIGRATE: 40LD or 8ld new drive  ch 0 tgt 0


T20: REF drive found at ch 0 tgt 0


T20: Attempting to perform drive roaming


T20: NOT Flushing Cache


T20: RMW: NVRAM structure valid - checking for active RMWs


T20: Inside  SymRequestPoolInit


T20: Memory Pages USed=100


T20: Memory Pages USed=164


T20: RequestQ ADDR=d0210000


T20: exit   SymRequestPoolInit


T20: inside   SymMsgFifoInit


T20: InboundFreeQEnd=d0200000


T20: exit   SymMsgFifoInit


T20: inside   SymMsgFifoInit


T20: exit  SymMsgFifoInit


T20: Memory Pages USed=34a


T20: Memory Pages USed=34e


T20: Memory Pages USed=34f


T20: Memory Pages USed=374


T20: found 1 logical drives


T20: logDrv=0  Size=10ef0000


09/29 12:18:10: Time established at T45


09/29 12:18:10: BIOS CALL FOR DRV ROAMING : 55


09/29 12:18:10: drive roaming not done


09/29 12:18:40: HOST ISR: Sync cmd 0 arrived


09/29 12:19:40: LD 0 RdlongSize=22a #diff=16


09/29 12:19:40: LD 1 RdlongSize=22a #diff=16


09/29 12:19:41: LD 2 RdlongSize=22a #diff=16


09/29 12:19:41: LD 3 RdlongSize=23a #diff=6


09/29 12:19:41: LD 4 RdlongSize=22a #diff=16


09/29 12:19:41: LD 5 RdlongSize=23a #diff=6


09/29 12:19:41: LD 8 RdlongSize=23a #diff=6


09/29 12:19:41: LD a RdlongSize=22a #diff=16


09/29 12:20:10: LD 0 RdlongSize=22a #diff=16


09/29 12:20:10: LD 1 RdlongSize=22a #diff=16


09/29 12:20:10: LD 2 RdlongSize=22a #diff=16


09/29 12:20:10: LD 3 RdlongSize=23a #diff=6


09/29 12:20:10: LD 4 RdlongSize=22a #diff=16


09/29 12:20:10: LD 5 RdlongSize=23a #diff=6


09/29 12:20:10: LD 8 RdlongSize=23a #diff=6


09/29 12:20:10: LD a RdlongSize=22a #diff=16


09/29 12:20:24: LD 0 RdlongSize=22a #diff=16


09/29 12:20:24: LD 1 RdlongSize=22a #diff=16


09/29 12:20:24: LD 2 RdlongSize=22a #diff=16


09/29 12:20:24: LD 3 RdlongSize=23a #diff=6


09/29 12:20:24: LD 4 RdlongSize=22a #diff=16


09/29 12:20:24: LD 5 RdlongSize=23a #diff=6


09/29 12:20:24: LD 8 RdlongSize=23a #diff=6


09/29 12:20:24: LD a RdlongSize=22a #diff=16


���������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������









tty-3CY381S.log


TTY History for HA(0) -Bus 0x0a Device 0x06

������������������������������������

TTY History for HA(0) -Bus 0x0a Device 0x06

���������������������������������������9���





T0: LSI Logic MegaRAID i960 firmware loaded


T0: Firmware version 351S build on May 31 2005 at 21:05:31


T0: Board is type 1000/1960/1028/0518





T0: Can_flush=0


T0: DRAM SIZE=128 MB


T0: DRAM_ALT sig invalid from previous boot


T0: FLUSH_ON_SYSTEM_RESET=0





T0: (C) LSI Logic Corporation 2002


T0: MegaRAID Series 518 firmware version 351S


T0: WAIT FOR BIOS....


T0: BIOS UP!


T0: Enabling data cache


T0: EepromInit: Family=14, SN=94a5f2010000


T0: EepromInit: Board SN not programmed





T0: Environment data:


T0: VALIDATION=None





T0: MFC data:


T0:     Vendor/DeviceID=1000/1960


T0:     SubVendor/SubDevice=1028/0518


T0:     OEM=Dell, clusterDisable=0, flexRaidDisable=0


T0:     rebuildRate=30, stripeSize=128, flushTime=4


T0:     cachedIo=1, writeBack=1, readAhead=0


T0:     channelBase='0', smartMode=6, alarmDisable=0


T0:     fastInitDisable=0, coercion=128M, disablePredictiveFail=0


T0:     disableWebBios=1, disableCtrlM=0, writeThroughWhenBatteryBad=1


T0:     zcrConfig=Undefined, keepSafteFailedStatus=0, autoHotSpareRestore=0


T0:     variableChkConRate=0, enableNvramDiskMgmtChange=0


T0:     dirtyLedShowsDriveActivity=0, disableConsChkRestoration=0


T0:     biosContinueOnError=0, biosAutoConfig=Prompt


T0:     disableRandomDriveDeletion=0





T0: DISK_CACHE_ADDR=d0db4800


T0: MEM_END_ADDR=d7fffff0


T0: Found MPT LVD 1030(1/5/1) at f7ff0004/0, mapped to 8bff0000


T0: Found MPT LVD 1030(1/5/0) at f7fd0004/0, mapped to 8bfd0000


T0: Total LSI MPT Chips found 2 


T0: LSI_InitMPT : start_index 0 totalLSIMPTChips 2


T0: Set to Get f/w features chip 0 


T0: 	Verifying Image Signatures...VERIFIED


T0: 	Verifying image check sum... VERIFIED


T0: The FW version being loaded is MPTFW-01.03.35.00-IT


T0: NextImageHeaderOffset=9c70, ExtImageSize=818


T0: FW download complete... Expecting LSI FW to start excute and come to ready state


T3: MISM CHN_STATE_MPT_GET_FW_FEAT chip 1 


T3: PRESENT SCSI_ID = 0


T3: Changing scsiId to  7


T3: Check IOC FACTS chip 1


T3: MISM CHN_STATE_MPT_OPERATIONAL chip 1 


T3: MISM: Reply frame size 60 start addr d05258c0 


T3: fe reply free frames posted


T3: MISM CHN_STATE_MPT_INIT_BUS_RST chip 1 


T3: MPT_Poll: chip 0 CHN_STATE_MPT_WAIT


T3: MISM CHN_STATE_MPT_GET_FW_FEAT chip 0 


T3: PRESENT SCSI_ID = 0


T3: Changing scsiId to  7


T3: Check IOC FACTS chip 0


T3: MISM CHN_STATE_MPT_OPERATIONAL chip 0 


T3: MISM: Reply frame size 60 start addr d052b880 


T3: fe reply free frames posted


T3: MISM CHN_STATE_MPT_INIT_BUS_RST chip 0 


T4: MPT_Poll: chip 1 CHN_STATE_MPT_INIT_BUS_RST 


T6: MPT_Poll: chip 0 CHN_STATE_MPT_INIT_BUS_RST 


T8: DISM: Queued!


T8: MPT_ProcessIo Reply Fr 2 EVENT_NOTIFICATION


T8: MPI_EVENT_EVENT_CHANGE


T8: MPT_SetIocPageParameters: After Write CoalescingDepth=1, Timeout=0, Flags=1 


T8: MPT_ProcessIo Reply Fr 2 EVENT_NOTIFICATION


T8: MPI_EVENT_EVENT_CHANGE


T8: MPT_SetIocPageParameters: After Write CoalescingDepth=1, Timeout=0, Flags=1 


T8: DISM: Cluster peer is 0x6


T8: DISM: oper_chn_bmap is 1, operating_chn: 0


T11: DISM_ProcessPprState: DomainVal done on all disks


T14: DISM: Complete!!!





T14: Physical device info:


T14: ID  NVRState  Vendor    Product           Rev    6   7  56


T14: --  --------  --------  ----------------  ----  --  --  --


T14: 00  Online    SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 01  Online    SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 02  Online    SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 03  Online    FUJITSU   MAT3073NC         5704  01  3a  0d


T14: 04  Online    SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 05  HotSpare  FUJITSU   MAT3073NC         5704  01  3a  0d


T14: 06  Ready     DELL      PERC 4/DC         350O  00  00  00


T14: 08  Online    FUJITSU   MAT3073NC         5704  01  3a  0d


T14: 09  Online    FUJITSU   MAP3367NC         5605  01  3a  0f


T14: 0a  Online    SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 0f  Ready     DELL      PV22XS            E.17  40  00  00





T19: batteryinit: TransBBU pack present


T19: TBBU: TBBU h/w . Cache not dirty


T19: Verifying config struct at Addr e0001400


T19: NVRAM checksum OK - reading configuration


T20: DISK_CACHE_ADDR=d0db4800


T20: MEM_END_ADDR=d7fffff0


T20: Memory End d7fffff0


T20: Total memory available for disk cache: 724b3f0


T20: Total Number of Cache Lines 1824


T20: SS 128: mrs=2  lc=1824 ldc=1  ps=1 cm=ff ba=0 LDs: 0


T20: LD  0: L=1  SS=128  Size=10ef0000  NL=1824  Status=2  DT=255  BT=512


T20:        span 0: sBlk=00000000, nBlk=043bc000, dev=00-01


T20:        span 1: sBlk=00000000, nBlk=043bc000, dev=03-02


T20:        span 2: sBlk=00000000, nBlk=043bc000, dev=04-0a


T20:        span 3: sBlk=00000000, nBlk=043bc000, dev=09-08


T20: can_flush = 0


T20: No Reconst:Checking drive info


T20: MIGRATE: 40LD or 8ld new drive  ch 0 tgt 0


T20: REF drive found at ch 0 tgt 0


T20: Attempting to perform drive roaming


T20: NOT Flushing Cache


T20: RMW: NVRAM structure valid - checking for active RMWs


T20: Inside  SymRequestPoolInit


T20: Memory Pages USed=100


T20: Memory Pages USed=164


T20: RequestQ ADDR=d0210000


T20: exit   SymRequestPoolInit


T20: inside   SymMsgFifoInit


T20: InboundFreeQEnd=d0200000


T20: exit   SymMsgFifoInit


T20: inside   SymMsgFifoInit


T20: exit  SymMsgFifoInit


T20: Memory Pages USed=34a


T20: Memory Pages USed=34e


T20: Memory Pages USed=34f


T20: Memory Pages USed=374


T20: found 1 logical drives


T20: logDrv=0  Size=10ef0000


09/29 12:18:10: Time established at T45


09/29 12:18:10: BIOS CALL FOR DRV ROAMING : 55


09/29 12:18:10: drive roaming not done


09/29 12:18:40: HOST ISR: Sync cmd 0 arrived


09/29 12:19:40: LD 0 RdlongSize=22a #diff=16


09/29 12:19:40: LD 1 RdlongSize=22a #diff=16


09/29 12:19:41: LD 2 RdlongSize=22a #diff=16


09/29 12:19:41: LD 3 RdlongSize=23a #diff=6


09/29 12:19:41: LD 4 RdlongSize=22a #diff=16


09/29 12:19:41: LD 5 RdlongSize=23a #diff=6


09/29 12:19:41: LD 8 RdlongSize=23a #diff=6


09/29 12:19:41: LD a RdlongSize=22a #diff=16


09/29 12:20:10: LD 0 RdlongSize=22a #diff=16


09/29 12:20:10: LD 1 RdlongSize=22a #diff=16


09/29 12:20:10: LD 2 RdlongSize=22a #diff=16


09/29 12:20:10: LD 3 RdlongSize=23a #diff=6


09/29 12:20:10: LD 4 RdlongSize=22a #diff=16


09/29 12:20:10: LD 5 RdlongSize=23a #diff=6


09/29 12:20:10: LD 8 RdlongSize=23a #diff=6


09/29 12:20:10: LD a RdlongSize=22a #diff=16


09/29 12:20:24: LD 0 RdlongSize=22a #diff=16


09/29 12:20:24: LD 1 RdlongSize=22a #diff=16


09/29 12:20:24: LD 2 RdlongSize=22a #diff=16


09/29 12:20:24: LD 3 RdlongSize=23a #diff=6


09/29 12:20:24: LD 4 RdlongSize=22a #diff=16


09/29 12:20:24: LD 5 RdlongSize=23a #diff=6


09/29 12:20:24: LD 8 RdlongSize=23a #diff=6


09/29 12:20:24: LD a RdlongSize=22a #diff=16


���������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������









tty-3CY381S_BBU.log


TTY History for HA(0) -Bus 0x0a Device 0x06

������������������������������������

TTY History for HA(0) -Bus 0x0a Device 0x06

���������������������������������������“"��





T0: LSI Logic MegaRAID i960 firmware loaded


T0: Firmware version 351S build on May 31 2005 at 21:05:31


T0: Board is type 1000/1960/1028/0518





T0: Can_flush=0


T0: DRAM SIZE=128 MB


T0: DRAM_ALT sig invalid from previous boot


T0: FLUSH_ON_SYSTEM_RESET=0





T0: (C) LSI Logic Corporation 2002


T0: MegaRAID Series 518 firmware version 351S


T0: WAIT FOR BIOS....


T0: BIOS UP!


T0: Enabling data cache


T0: EepromInit: Family=14, SN=94a5f2010000


T0: EepromInit: Board SN not programmed





T0: Environment data:


T0: VALIDATION=None





T0: MFC data:


T0:     Vendor/DeviceID=1000/1960


T0:     SubVendor/SubDevice=1028/0518


T0:     OEM=Dell, clusterDisable=0, flexRaidDisable=0


T0:     rebuildRate=30, stripeSize=128, flushTime=4


T0:     cachedIo=1, writeBack=1, readAhead=0


T0:     channelBase='0', smartMode=6, alarmDisable=0


T0:     fastInitDisable=0, coercion=128M, disablePredictiveFail=0


T0:     disableWebBios=1, disableCtrlM=0, writeThroughWhenBatteryBad=1


T0:     zcrConfig=Undefined, keepSafteFailedStatus=0, autoHotSpareRestore=0


T0:     variableChkConRate=0, enableNvramDiskMgmtChange=0


T0:     dirtyLedShowsDriveActivity=0, disableConsChkRestoration=0


T0:     biosContinueOnError=0, biosAutoConfig=Prompt


T0:     disableRandomDriveDeletion=0





T0: DISK_CACHE_ADDR=d0db4800


T0: MEM_END_ADDR=d7fffff0


T0: Found MPT LVD 1030(1/5/1) at f7ff0004/0, mapped to 8bff0000


T0: Found MPT LVD 1030(1/5/0) at f7fd0004/0, mapped to 8bfd0000


T0: Total LSI MPT Chips found 2 


T0: LSI_InitMPT : start_index 0 totalLSIMPTChips 2


T0: Set to Get f/w features chip 0 


T0: 	Verifying Image Signatures...VERIFIED


T0: 	Verifying image check sum... VERIFIED


T0: The FW version being loaded is MPTFW-01.03.35.00-IT


T0: NextImageHeaderOffset=9c70, ExtImageSize=818


T0: FW download complete... Expecting LSI FW to start excute and come to ready state


T3: MISM CHN_STATE_MPT_GET_FW_FEAT chip 1 


T3: PRESENT SCSI_ID = 0


T3: Changing scsiId to  7


T3: Check IOC FACTS chip 1


T3: MISM CHN_STATE_MPT_OPERATIONAL chip 1 


T3: MISM: Reply frame size 60 start addr d05258c0 


T3: fe reply free frames posted


T3: MISM CHN_STATE_MPT_INIT_BUS_RST chip 1 


T3: MPT_Poll: chip 0 CHN_STATE_MPT_WAIT


T3: MISM CHN_STATE_MPT_GET_FW_FEAT chip 0 


T3: PRESENT SCSI_ID = 0


T3: Changing scsiId to  7


T3: Check IOC FACTS chip 0


T3: MISM CHN_STATE_MPT_OPERATIONAL chip 0 


T3: MISM: Reply frame size 60 start addr d052b880 


T3: fe reply free frames posted


T3: MISM CHN_STATE_MPT_INIT_BUS_RST chip 0 


T4: MPT_Poll: chip 1 CHN_STATE_MPT_INIT_BUS_RST 


T6: MPT_Poll: chip 0 CHN_STATE_MPT_INIT_BUS_RST 


T8: DISM: Queued!


T8: MPT_ProcessIo Reply Fr 2 EVENT_NOTIFICATION


T8: MPI_EVENT_EVENT_CHANGE


T8: MPT_SetIocPageParameters: After Write CoalescingDepth=1, Timeout=0, Flags=1 


T8: MPT_ProcessIo Reply Fr 2 EVENT_NOTIFICATION


T8: MPI_EVENT_EVENT_CHANGE


T8: MPT_SetIocPageParameters: After Write CoalescingDepth=1, Timeout=0, Flags=1 


T8: DISM: Cluster peer is 0x6


T8: DISM: oper_chn_bmap is 1, operating_chn: 0


T11: DISM_ProcessPprState: DomainVal done on all disks


T14: DISM: Complete!!!





T14: Physical device info:


T14: ID  NVRState  Vendor    Product           Rev    6   7  56


T14: --  --------  --------  ----------------  ----  --  --  --


T14: 00  Online    SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 01  Online    SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 02  Online    SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 03  Online    FUJITSU   MAT3073NC         5704  01  3a  0d


T14: 04  Online    SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 05  HotSpare  FUJITSU   MAT3073NC         5704  01  3a  0d


T14: 06  Ready     DELL      PERC 4/DC         350O  00  00  00


T14: 08  Online    FUJITSU   MAT3073NC         5704  01  3a  0d


T14: 09  Online    FUJITSU   MAP3367NC         5605  01  3a  0f


T14: 0a  Online    SEAGATE   ST336607LC        DS09  01  3e  0f


T14: 0f  Ready     DELL      PV22XS            E.17  40  00  00





T19: batteryinit: TransBBU pack present


T19: TBBU: TBBU h/w . Cache not dirty


T19: Verifying config struct at Addr e0001400


T19: NVRAM checksum OK - reading configuration


T20: DISK_CACHE_ADDR=d0db4800


T20: MEM_END_ADDR=d7fffff0


T20: Memory End d7fffff0


T20: Total memory available for disk cache: 724b3f0


T20: Total Number of Cache Lines 1824


T20: SS 128: mrs=2  lc=1824 ldc=1  ps=1 cm=ff ba=0 LDs: 0


T20: LD  0: L=1  SS=128  Size=10ef0000  NL=1824  Status=2  DT=255  BT=512


T20:        span 0: sBlk=00000000, nBlk=043bc000, dev=00-01


T20:        span 1: sBlk=00000000, nBlk=043bc000, dev=03-02


T20:        span 2: sBlk=00000000, nBlk=043bc000, dev=04-0a


T20:        span 3: sBlk=00000000, nBlk=043bc000, dev=09-08


T20: can_flush = 0


T20: No Reconst:Checking drive info


T20: MIGRATE: 40LD or 8ld new drive  ch 0 tgt 0


T20: REF drive found at ch 0 tgt 0


T20: Attempting to perform drive roaming


T20: NOT Flushing Cache


T20: RMW: NVRAM structure valid - checking for active RMWs


T20: Inside  SymRequestPoolInit


T20: Memory Pages USed=100


T20: Memory Pages USed=164


T20: RequestQ ADDR=d0210000


T20: exit   SymRequestPoolInit


T20: inside   SymMsgFifoInit


T20: InboundFreeQEnd=d0200000


T20: exit   SymMsgFifoInit


T20: inside   SymMsgFifoInit


T20: exit  SymMsgFifoInit


T20: Memory Pages USed=34a


T20: Memory Pages USed=34e


T20: Memory Pages USed=34f


T20: Memory Pages USed=374


T20: found 1 logical drives


T20: logDrv=0  Size=10ef0000


09/29 12:18:10: Time established at T45


09/29 12:18:10: BIOS CALL FOR DRV ROAMING : 55


09/29 12:18:10: drive roaming not done


09/29 12:18:40: HOST ISR: Sync cmd 0 arrived


09/29 12:19:40: LD 0 RdlongSize=22a #diff=16


09/29 12:19:40: LD 1 RdlongSize=22a #diff=16


09/29 12:19:41: LD 2 RdlongSize=22a #diff=16


09/29 12:19:41: LD 3 RdlongSize=23a #diff=6


09/29 12:19:41: LD 4 RdlongSize=22a #diff=16


09/29 12:19:41: LD 5 RdlongSize=23a #diff=6


09/29 12:19:41: LD 8 RdlongSize=23a #diff=6


09/29 12:19:41: LD a RdlongSize=22a #diff=16


09/29 12:20:10: LD 0 RdlongSize=22a #diff=16


09/29 12:20:10: LD 1 RdlongSize=22a #diff=16


09/29 12:20:10: LD 2 RdlongSize=22a #diff=16


09/29 12:20:10: LD 3 RdlongSize=23a #diff=6


09/29 12:20:10: LD 4 RdlongSize=22a #diff=16


09/29 12:20:10: LD 5 RdlongSize=23a #diff=6


09/29 12:20:10: LD 8 RdlongSize=23a #diff=6


09/29 12:20:10: LD a RdlongSize=22a #diff=16


09/29 12:20:24: LD 0 RdlongSize=22a #diff=16


09/29 12:20:24: LD 1 RdlongSize=22a #diff=16


09/29 12:20:24: LD 2 RdlongSize=22a #diff=16


09/29 12:20:24: LD 3 RdlongSize=23a #diff=6


09/29 12:20:24: LD 4 RdlongSize=22a #diff=16


09/29 12:20:24: LD 5 RdlongSize=23a #diff=6


09/29 12:20:24: LD 8 RdlongSize=23a #diff=6


09/29 12:20:24: LD a RdlongSize=22a #diff=16


09/29 12:38:17: MPT_ProcessIo Reply Fr 7c EVENT_NOTIFICATION


09/29 12:38:17: LD Check: FCM_StartScsiBusReset


09/29 12:38:17: StartScsiBusReset(chip 0, port 0):Send 0 msgs to LdWaitQ


09/29 12:38:17: REC: Chip 0/0 - StartScsiBusreset is not Called


09/29 12:38:17: CQM2: calling FCM_CompleteScsiBusReset chip 0 port 0


09/29 12:38:17: LD Check: FCM_CompleteScsiBusReset


09/29 12:38:17: MPT_ProcessIo Reply Fr 89 EVENT_ACK


09/29 12:38:24: TMM:Setting Peer id to 6


09/29 12:38:24: TMM:oper_chn_bmap is 0x1, operating_chn: 0x0


09/29 12:38:29: LD Unlocked... No Lock... Done


09/29 12:38:34: TMM:Setting Peer id to 6


09/29 12:38:34: TMM:oper_chn_bmap is 0x1, operating_chn: 0x0


09/29 12:41:23: MPT_ProcessIo Reply Fr be EVENT_NOTIFICATION


09/29 12:41:23: REC: Chip 0/0 - StartScsiBusreset is not Called


09/29 12:41:23: MPT_ProcessIo Reply Fr bf EVENT_ACK


09/29 12:41:28: TMM:Setting Peer id to 6


09/29 12:41:28: TMM:oper_chn_bmap is 0x1, operating_chn: 0x0


09/29 12:41:39: TMM:Setting Peer id to 6


09/29 12:41:39: TMM:oper_chn_bmap is 0x1, operating_chn: 0x0


09/29 12:44:49: LD Check: FCM_StartScsiBusReset


09/29 12:44:49: StartScsiBusReset(chip 0, port 0):Send 0 msgs to LdWaitQ


09/29 12:44:49: MPT_ProcessIo Reply Fr f6 EVENT_NOTIFICATION


09/29 12:44:49: REC: Chip 0/0 - StartScsiBusreset is not Called


09/29 12:44:49: CQM2: calling FCM_CompleteScsiBusReset chip 0 port 0


09/29 12:44:49: LD Check: FCM_CompleteScsiBusReset


09/29 12:44:49: MPT_ProcessIo Reply Fr f8 EVENT_ACK


09/29 12:44:53: LD Unlocked... No Lock... Done


09/29 12:44:54: TMM:Setting Peer id to 6


09/29 12:44:54: TMM:oper_chn_bmap is 0x1, operating_chn: 0x0


09/29 12:45:05: TMM:Setting Peer id to 6


09/29 12:45:05: TMM:oper_chn_bmap is 0x1, operating_chn: 0x0


09/29 12:46:33: MPT_ProcessIo Reply Fr 24 EVENT_NOTIFICATION


09/29 12:46:33: REC: Chip 0/0 - StartScsiBusreset is not Called


09/29 12:46:33: MPT_ProcessIo Reply Fr 25 EVENT_ACK


09/29 12:46:38: TMM:Setting Peer id to 6


09/29 12:46:38: TMM:oper_chn_bmap is 0x1, operating_chn: 0x0


09/29 12:46:49: TMM:Setting Peer id to 6


09/29 12:46:49: TMM:oper_chn_bmap is 0x1, operating_chn: 0x0


���������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������









_1235630830/Lsi_logs.txt


TTY History for HA(0) -Bus 0x01 Device 0x06

����������������������������������������€��





T0: LSI Logic MegaRAID i960 firmware loaded


T0: Firmware version 351X build on Jan 27 2006 at 12:04:37


T0: Board is type 1000/1960/1028/0518





T0: Can_flush=0


T0: DRAM SIZE=128 MB


T0: DRAM_ALT sig invalid from previous boot


T0: FLUSH_ON_SYSTEM_RESET=0





T0: (C) LSI Logic Corporation 2002


T0: MegaRAID Series 518 firmware version 351X


T0: WAIT FOR BIOS....


T0: BIOS UP!


T0: Enabling data cache


T0: EepromInit: Family=14, SN=21b96b010000


T0: EepromInit: Board SN not programmed





T0: Environment data:


T0: VALIDATION=None


T0: VERSIONS=CTLM_U827,BIOS_1.10,APP_351X,DBGR_N/A,BTBL_1.7.7DELL





T0: MFC data:


T0:     Vendor/DeviceID=1000/1960


T0:     SubVendor/SubDevice=1028/0518


T0:     OEM=Dell, clusterDisable=0, flexRaidDisable=0


T0:     rebuildRate=30, stripeSize=128, flushTime=4


T0:     cachedIo=1, writeBack=1, readAhead=0


T0:     channelBase='0', smartMode=6, alarmDisable=0


T0:     fastInitDisable=0, coercion=128M, disablePredictiveFail=0


T0:     disableWebBios=1, disableCtrlM=0, writeThroughWhenBatteryBad=1


T0:     zcrConfig=Undefined, keepSafteFailedStatus=0, autoHotSpareRestore=0


T0:     variableChkConRate=0, enableNvramDiskMgmtChange=0


T0:     dirtyLedShowsDriveActivity=0, disableConsChkRestoration=0


T0:     biosContinueOnError=0, biosAutoConfig=Prompt


T0:     disableRandomDriveDeletion=0





T0: DISK_CACHE_ADDR=d0db4800


T0: MEM_END_ADDR=d7fffff0


T0: Found MPT LVD 1030(1/5/1) at f7ff0004/0, mapped to 8bff0000


T0: Found MPT LVD 1030(1/5/0) at f7fd0004/0, mapped to 8bfd0000


T0: Total LSI MPT Chips found 2 


T0: LSI_InitMPT : start_index 0 totalLSIMPTChips 2


T0: Set to Get f/w features chip 0 


T0: 	Verifying Image Signatures...VERIFIED


T0: 	Verifying image check sum... VERIFIED


T0: The FW version being loaded is MPTFW-01.03.35.00-IT


T0: NextImageHeaderOffset=9c70, ExtImageSize=818


T0: FW download complete... Expecting LSI FW to start excute and come to ready state


T3: MISM CHN_STATE_MPT_GET_FW_FEAT chip 1 


T3: PRESENT SCSI_ID = 0


T3: Changing scsiId to  7


T3: Check IOC FACTS chip 1


T3: MISM CHN_STATE_MPT_OPERATIONAL chip 1 


T3: MISM: Reply frame size 60 start addr d05258c0 


T3: fe reply free frames posted


T3: MISM CHN_STATE_MPT_INIT_BUS_RST chip 1 


T3: MPT_Poll: chip 0 CHN_STATE_MPT_WAIT


T3: MISM CHN_STATE_MPT_GET_FW_FEAT chip 0 


T3: PRESENT SCSI_ID = 0


T3: Changing scsiId to  7


T3: Check IOC FACTS chip 0


T3: MISM CHN_STATE_MPT_OPERATIONAL chip 0 


T3: MISM: Reply frame size 60 start addr d052b880 


T3: fe reply free frames posted


T3: MISM CHN_STATE_MPT_INIT_BUS_RST chip 0 


T4: MPT_Poll: chip 1 CHN_STATE_MPT_INIT_BUS_RST 


T6: MPT_Poll: chip 0 CHN_STATE_MPT_INIT_BUS_RST 


T8: DISM: Queued!


T8: MPT_ProcessIo Reply Fr 2 EVENT_NOTIFICATION


T8: MPI_EVENT_EVENT_CHANGE


T8: MPT_SetIocPageParameters: After Write CoalescingDepth=1, Timeout=0, Flags=1 


T8: MPT_ProcessIo Reply Fr 2 EVENT_NOTIFICATION


T8: MPI_EVENT_EVENT_CHANGE


T8: MPT_SetIocPageParameters: After Write CoalescingDepth=1, Timeout=0, Flags=1 


T11: DISM_ProcessPprState: DomainVal done on all disks


T11: DISM: Complete!!!





T11: Physical device info:


T11: ID  NVRState  Vendor    Product           Rev    6   7  56


T11: --  --------  --------  ----------------  ----  --  --  --


T11: 00  Online    MAXTOR    ATLAS10K4_73SCA   DFX1  01  3e  0f


T11: 01  Online    SEAGATE   ST373307LC        DS09  01  3e  0f


T11: 06  Ready     PE/PV     2x2 SCSI BP       1.1   00  00  00


T11: 12  Online    MAXTOR    ATLAS10K4_146SCA  DFX1  01  3e  0f


T11: 13  Online    SEAGATE   ST3146807LC       DS09  01  3e  0f


T11: 14  Online    MAXTOR    ATLAS10K4_146SCA  DFX1  01  3e  0f


T11: 16  Ready     PE/PV     2x3 SCSI BP       1.1   00  00  00





T16: batteryinit: TransBBU pack present


T16: TBBU: TBBU h/w . Cache not dirty


T16: Verifying config struct at Addr e0001400


T16: NVRAM checksum OK - reading configuration


T16: DISK_CACHE_ADDR=d0db4800


T16: MEM_END_ADDR=d7fffff0


T16: Memory End d7fffff0


T16: Total memory available for disk cache: 724b3f0


T16: Total Number of Cache Lines 1824


T16: SS 128: mrs=3  lc=1824 ldc=2  ps=1 cm=ff ba=0 LDs: 0 1


T16: LD  0: L=1  SS=128  Size=887c000  NL=1824  Status=2  DT=255  BT=512


T16:        span 0: sBlk=00000000, nBlk=0887c000, dev=00-01


T16: LD  1: L=5  SS=128  Size=2227c000  NL=1824  Status=2  DT=255  BT=512


T16:        span 0: sBlk=00000000, nBlk=1113e000, dev=12-13-14


T16: can_flush = 0


T16: No Reconst:Checking drive info


T17: MIGRATE: 40LD or 8ld new drive  ch 0 tgt 0


T17: REF drive found at ch 0 tgt 0


T17: Attempting to perform drive roaming


T17: NOT Flushing Cache


T17: RMW: NVRAM structure valid - checking for active RMWs


T17: Inside  SymRequestPoolInit


T17: Memory Pages USed=100


T17: Memory Pages USed=164


T17: RequestQ ADDR=d0210000


T17: exit   SymRequestPoolInit


T17: inside   SymMsgFifoInit


T17: InboundFreeQEnd=d0200000


T17: exit   SymMsgFifoInit


T17: inside   SymMsgFifoInit


T17: exit  SymMsgFifoInit


T17: Memory Pages USed=34a


T17: Memory Pages USed=34e


T17: Memory Pages USed=34f


T17: Memory Pages USed=374


T17: found 2 logical drives


T17: logDrv=0  Size=887c000


T17: logDrv=1  Size=2227c000


04/25 10:36:10: Time established at T24


04/25 10:36:10: BIOS CALL FOR DRV ROAMING : 55


04/25 10:36:10: drive roaming not done


04/25 10:36:31: HOST ISR: Sync cmd 0 arrived


04/25 10:37:26: Rejecting Unknown FC DCMD 1f


04/25 10:37:26: Rejecting MISC opcode: unknown sub-opcode (0x26)


04/25 10:37:36: Rejecting Unknown FC DCMD 1f


04/25 10:37:36: Rejecting MISC opcode: unknown sub-opcode (0x26)


04/25 10:37:43: MPT_Rec: INQ Error - Negotiating LD[6] pRfm d00a17e0


04/25 10:37:43: MPT_Rec: INQ Error - Negotiating LD[16] pRfm d00a19a0


04/25 10:37:47: Rejecting Unknown FC DCMD 1f


04/25 10:37:47: Rejecting MISC opcode: unknown sub-opcode (0x26)


04/25 10:38:02: MPT_Rec: INQ Error - Negotiating LD[6] pRfm d00b0880


04/25 10:38:02: MPT_Rec: INQ Error - Negotiating LD[6] pRfm d00b13e0


04/25 10:38:02: MPT_Rec: INQ Error - Negotiating LD[16] pRfm d00bd2a0


04/25 10:38:02: MPT_Rec: INQ Error - Negotiating LD[16] pRfm d00b93a0


04/25 10:38:06: Rejecting Unknown FC DCMD 1f


04/25 10:38:06: Rejecting MISC opcode: unknown sub-opcode (0x26)


04/25 10:40:46: bginit started for 0


04/25 10:40:46: Actual cons started for ld 0 from stripe 19da8


04/25 10:40:46: bginit started for 1


04/25 10:40:46: Actual cons started for ld 1 from stripe 1c237


04/25 12:00:00: prCheckForBackgroundActivity: CC/BGI/Init active on ld=0


04/25 12:00:00: prDiskCheckOkToRun: PR NOT running due to background activity on adapter PD=00


04/25 12:00:00: prCheckForBackgroundActivity: CC/BGI/Init active on ld=0


04/25 12:00:00: prDiskCheckOkToRun: PR NOT running due to background activity on adapter PD=01


04/25 12:00:00: prCheckForBackgroundActivity: CC/BGI/Init active on ld=0


04/25 12:00:00: prDiskCheckOkToRun: PR NOT running due to background activity on adapter PD=12


04/25 12:00:00: prCheckForBackgroundActivity: CC/BGI/Init active on ld=0


04/25 12:00:00: prDiskCheckOkToRun: PR NOT running due to background activity on adapter PD=13


04/25 12:00:00: prCheckForBackgroundActivity: CC/BGI/Init active on ld=0


04/25 12:00:00: prDiskCheckOkToRun: PR NOT running due to background activity on adapter PD=14


04/25 12:00:00: Next PR scheduled to start at 04/25 16:00:00


04/25 12:25:06: REC:log MedErr on pd[0] #retries=0


04/25 12:25:06: LogSense: pd=00, cdb=28 00 03 b1 84 00 00 00 80 00 


04/25 12:25:06:           sense=f0 00 03 03 b1 84 6e 0a 1a 20 47 b4 11 0b 00 00 00 79 


04/25 12:25:06: REC: MedErr on LD[0] BadLba=3b1846e


04/25 12:25:07: REC:log MedErr on pd[1] #retries=0


04/25 12:25:07: LogSense: pd=01, cdb=28 00 03 b1 84 00 00 00 80 00 


04/25 12:25:07:           sense=f0 00 03 03 b1 84 6e 0a 00 00 00 00 11 00 e4 00 00 86 


04/25 12:25:07: REC: MedErr on LD[1] BadLba=3b1846e


04/25 12:25:07: Double media errors found! badLBA=3b1846e ld=0 pd1=0 pd2=1


04/25 12:25:09: REC:log MedErr on pd[0] #retries=0


04/25 12:25:09: LogSense: pd=00, cdb=28 00 03 b1 98 80 00 00 80 00 


04/25 12:25:09:           sense=f0 00 03 03 b1 98 ee 0a 18 c0 47 b5 11 0b 00 00 00 79 


04/25 12:25:09: REC: MedErr on LD[0] BadLba=3b198ee


04/25 12:25:10: REC:log MedErr on pd[1] #retries=0


04/25 12:25:10: LogSense: pd=01, cdb=28 00 03 b1 98 80 00 00 80 00 


04/25 12:25:10:           sense=f0 00 03 03 b1 98 ee 0a 00 00 00 00 11 00 e4 00 00 86 


04/25 12:25:10: REC: MedErr on LD[1] BadLba=3b198ee


04/25 12:25:10: Double media errors found! badLBA=3b198ee ld=0 pd1=0 pd2=1


04/25 12:25:11: REC:log MedErr on pd[0] #retries=0


04/25 12:25:11: LogSense: pd=00, cdb=28 00 03 b1 9b 00 00 00 80 00 


04/25 12:25:11:           sense=f0 00 03 03 b1 9b 4e 0a 18 20 47 b6 11 0b 00 00 00 79 


04/25 12:25:11: REC: MedErr on LD[0] BadLba=3b19b4e


04/25 12:25:11: REC:log MedErr on pd[1] #retries=0


04/25 12:25:11: LogSense: pd=01, cdb=28 00 03 b1 9b 00 00 00 80 00 


04/25 12:25:11:           sense=f0 00 03 03 b1 9b 4e 0a 00 00 00 00 11 00 e4 00 00 86 


04/25 12:25:11: REC: MedErr on LD[1] BadLba=3b19b4e


04/25 12:25:11: Double media errors found! badLBA=3b19b4e ld=0 pd1=0 pd2=1


04/25 12:26:53: REC:log MedErr on pd[0] #retries=0


04/25 12:26:53: LogSense: pd=00, cdb=28 00 03 bd 89 00 00 00 80 00 


04/25 12:26:53:           sense=f0 00 03 03 bd 89 65 0a 11 d4 48 ad 11 0b 00 00 00 79 


04/25 12:26:53: REC: MedErr on LD[0] BadLba=3bd8965


04/25 12:26:53: REC:log MedErr on pd[1] #retries=0


04/25 12:26:53: LogSense: pd=01, cdb=28 00 03 bd 89 00 00 00 80 00 


04/25 12:26:53:           sense=f0 00 03 03 bd 89 65 0a 00 00 00 00 11 00 e4 00 00 86 


04/25 12:26:53: REC: MedErr on LD[1] BadLba=3bd8965


04/25 12:26:53: Double media errors found! badLBA=3bd8965 ld=0 pd1=0 pd2=1


04/25 12:26:55: REC:log MedErr on pd[0] #retries=0


04/25 12:26:55: LogSense: pd=00, cdb=28 00 03 bd 93 80 00 00 80 00 


04/25 12:26:55:           sense=f0 00 03 03 bd 93 b9 0a 13 02 48 ae 11 0b 00 00 00 79 


04/25 12:26:55: REC: MedErr on LD[0] BadLba=3bd93b9


04/25 12:26:55: REC:log MedErr on pd[1] #retries=0


04/25 12:26:55: LogSense: pd=01, cdb=28 00 03 bd 93 80 00 00 80 00 


04/25 12:26:55:           sense=f0 00 03 03 bd 93 b9 0a 00 00 00 00 11 00 e4 00 00 86 


04/25 12:26:55: REC: MedErr on LD[1] BadLba=3bd93b9


04/25 12:26:55: Double media errors found! badLBA=3bd93b9 ld=0 pd1=0 pd2=1


04/25 12:27:10: REC:log MedErr on pd[0] #retries=0


04/25 12:27:10: LogSense: pd=00, cdb=28 00 03 bf 04 80 00 00 80 00 


04/25 12:27:10:           sense=f0 00 03 03 bf 04 f4 0a 12 df 48 cc 11 00 00 00 00 a5 


04/25 12:27:10: REC: MedErr on LD[0] BadLba=3bf04f4


04/25 12:27:10: REC:log MedErr on pd[1] #retries=0


04/25 12:27:10: LogSense: pd=01, cdb=28 00 03 bf 04 80 00 00 80 00 


04/25 12:27:10:           sense=f0 00 03 03 bf 04 f4 0a 00 00 00 00 11 00 e4 00 00 86 


04/25 12:27:10: REC: MedErr on LD[1] BadLba=3bf04f4


04/25 12:27:10: Double media errors found! badLBA=3bf04f4 ld=0 pd1=0 pd2=1


04/25 12:27:16: REC:log MedErr on pd[1] #retries=0


04/25 12:27:16: LogSense: pd=01, cdb=28 00 03 bf 83 00 00 00 80 00 


04/25 12:27:16:           sense=f0 00 03 03 bf 83 6f 0a 00 00 00 00 11 00 e4 00 00 86 


04/25 12:27:16: REC: MedErr on LD[1] BadLba=3bf836f


04/25 12:27:17: REC:log MedErr on pd[0] #retries=0


04/25 12:27:17: LogSense: pd=00, cdb=28 00 03 bf 83 00 00 00 80 00 


04/25 12:27:17:           sense=f0 00 03 03 bf 83 6f 0a 00 01 48 d6 11 0b 00 00 01 ab 


04/25 12:27:17: REC: MedErr on LD[0] BadLba=3bf836f


04/25 12:27:17: Double media errors found! badLBA=3bf836f ld=0 pd1=1 pd2=0


04/25 12:27:25: REC:log MedErr on pd[0] #retries=0


04/25 12:27:25: LogSense: pd=00, cdb=28 00 03 c0 59 00 00 00 80 00 


04/25 12:27:25:           sense=f0 00 03 03 c0 59 2f 0a 19 5f 48 e8 11 00 00 00 00 a5 


04/25 12:27:25: REC: MedErr on LD[0] BadLba=3c0592f


04/25 12:27:25: REC:log MedErr on pd[1] #retries=0


04/25 12:27:25: LogSense: pd=01, cdb=28 00 03 c0 59 00 00 00 80 00 


04/25 12:27:25:           sense=f0 00 03 03 c0 59 2f 0a 00 00 00 00 11 00 e4 00 00 86 


04/25 12:27:25: REC: MedErr on LD[1] BadLba=3c0592f


04/25 12:27:25: Double media errors found! badLBA=3c0592f ld=0 pd1=0 pd2=1


04/25 12:27:27: REC:log MedErr on pd[0] #retries=0


04/25 12:27:27: LogSense: pd=00, cdb=28 00 03 c0 70 00 00 00 80 00 


04/25 12:27:27:           sense=f0 00 03 03 c0 70 6f 0a 1a ff 48 e9 11 00 00 00 00 a5 


04/25 12:27:27: REC: MedErr on LD[0] BadLba=3c0706f


04/25 12:27:28: REC:log MedErr on pd[1] #retries=0


04/25 12:27:28: LogSense: pd=01, cdb=28 00 03 c0 70 00 00 00 80 00 


04/25 12:27:28:           sense=f0 00 03 03 c0 70 6f 0a 00 00 00 00 11 00 e4 00 00 86 


04/25 12:27:28: REC: MedErr on LD[1] BadLba=3c0706f


04/25 12:27:28: Double media errors found! badLBA=3c0706f ld=0 pd1=0 pd2=1


04/25 12:27:29: REC:log MedErr on pd[0] #retries=0


04/25 12:27:29: LogSense: pd=00, cdb=28 00 03 c0 72 80 00 00 80 00 


04/25 12:27:29:           sense=f0 00 03 03 c0 72 cf 0a 1a 5f 48 ea 11 00 00 00 00 a5 


04/25 12:27:29: REC: MedErr on LD[0] BadLba=3c072cf


04/25 12:27:30: REC:log MedErr on pd[1] #retries=0


04/25 12:27:30: LogSense: pd=01, cdb=28 00 03 c0 72 80 00 00 80 00 


04/25 12:27:30:           sense=f0 00 03 03 c0 72 cf 0a 00 00 00 00 11 00 e4 00 00 86 


04/25 12:27:30: REC: MedErr on LD[1] BadLba=3c072cf


04/25 12:27:30: Double media errors found! badLBA=3c072cf ld=0 pd1=0 pd2=1


04/25 12:27:32: REC:log MedErr on pd[0] #retries=0


04/25 12:27:32: LogSense: pd=00, cdb=28 00 03 c0 87 80 00 00 80 00 


04/25 12:27:32:           sense=f0 00 03 03 c0 87 de 0a 19 ce 48 eb 11 00 00 00 00 a5 


04/25 12:27:32: REC: MedErr on LD[0] BadLba=3c087de


04/25 12:27:32: REC:log MedErr on pd[1] #retries=0


04/25 12:27:32: LogSense: pd=01, cdb=28 00 03 c0 87 80 00 00 80 00 


04/25 12:27:32:           sense=f0 00 03 03 c0 87 de 0a 00 00 00 00 11 00 e4 00 00 86 


04/25 12:27:32: REC: MedErr on LD[1] BadLba=3c087de


04/25 12:27:33: Double media errors found! badLBA=3c087de ld=0 pd1=0 pd2=1


04/25 12:27:34: REC:log MedErr on pd[0] #retries=0


04/25 12:27:34: LogSense: pd=00, cdb=28 00 03 c0 8a 00 00 00 80 00 


04/25 12:27:34:           sense=f0 00 03 03 c0 8a 3e 0a 19 2e 48 ec 11 00 00 00 00 a5 


04/25 12:27:34: REC: MedErr on LD[0] BadLba=3c08a3e


04/25 12:27:35: REC:log MedErr on pd[1] #retries=0


04/25 12:27:35: LogSense: pd=01, cdb=28 00 03 c0 8a 00 00 00 80 00 


04/25 12:27:35:           sense=f0 00 03 03 c0 8a 3e 0a 00 00 00 00 11 00 e4 00 00 86 


04/25 12:27:35: REC: MedErr on LD[1] BadLba=3c08a3e


04/25 12:27:35: Double media errors found! badLBA=3c08a3e ld=0 pd1=0 pd2=1


04/25 12:27:37: REC:log MedErr on pd[0] #retries=0


04/25 12:27:37: LogSense: pd=00, cdb=28 00 03 c0 a3 80 00 00 80 00 


04/25 12:27:37:           sense=f0 00 03 03 c0 a3 de 0a 1a 2e 48 ee 11 00 00 00 00 a5 


04/25 12:27:37: REC: MedErr on LD[0] BadLba=3c0a3de


04/25 12:27:37: REC:log MedErr on pd[1] #retries=0


04/25 12:27:37: LogSense: pd=01, cdb=28 00 03 c0 a3 80 00 00 80 00 


04/25 12:27:37:           sense=f0 00 03 03 c0 a3 de 0a 00 00 00 00 11 00 e4 00 00 86 


04/25 12:27:37: REC: MedErr on LD[1] BadLba=3c0a3de


04/25 12:27:38: Double media errors found! badLBA=3c0a3de ld=0 pd1=0 pd2=1


04/25 12:27:39: REC:log MedErr on pd[0] #retries=0


04/25 12:27:39: LogSense: pd=00, cdb=28 00 03 c0 b8 00 00 00 80 00 


04/25 12:27:39:           sense=f0 00 03 03 c0 b8 1d 0a 18 cd 48 ef 11 00 00 00 00 a5 


04/25 12:27:39: REC: MedErr on LD[0] BadLba=3c0b81d


04/25 12:27:40: REC:log MedErr on pd[1] #retries=0


04/25 12:27:40: LogSense: pd=01, cdb=28 00 03 c0 b8 00 00 00 80 00 


04/25 12:27:40:           sense=f0 00 03 03 c0 b8 1d 0a 00 00 00 00 11 00 e4 00 00 86 


04/25 12:27:40: REC: MedErr on LD[1] BadLba=3c0b81d


04/25 12:27:40: REC:log MedErr on pd[0] #retries=4


04/25 12:27:40: LogSense: pd=00, cdb=28 00 03 c0 b8 1e 00 00 62 00 


04/25 12:27:40:           sense=f0 00 03 03 c0 b8 1e 0a 18 ce 48 ef 16 00 00 00 00 a6 


04/25 12:27:40: REC: MedErr on LD[0] BadLba=3c0b81e


04/25 12:27:42: REC:log MedErr on pd[1] #retries=4


04/25 12:27:42: LogSense: pd=01, cdb=28 00 03 c0 b8 1e 00 00 62 00 


04/25 12:27:42:           sense=f0 00 03 03 c0 b8 1e 0a 00 00 00 00 11 00 e4 00 00 86 


04/25 12:27:42: REC: MedErr on LD[1] BadLba=3c0b81e


04/25 12:27:42: Double media errors found! badLBA=3c0b81d ld=0 pd1=0 pd2=1


04/25 12:27:43: REC:log MedErr on pd[0] #retries=0


04/25 12:27:43: LogSense: pd=00, cdb=28 00 03 c0 ba 00 00 00 80 00 


04/25 12:27:43:           sense=f0 00 03 03 c0 ba 7e 0a 18 2e 48 f0 11 00 00 00 00 a5 


04/25 12:27:43: REC: MedErr on LD[0] BadLba=3c0ba7e


04/25 12:27:44: REC:log MedErr on pd[1] #retries=0


04/25 12:27:44: LogSense: pd=01, cdb=28 00 03 c0 ba 00 00 00 80 00 


04/25 12:27:44:           sense=f0 00 03 03 c0 ba 7e 0a 00 00 00 00 11 00 e4 00 00 86 


04/25 12:27:44: REC: MedErr on LD[1] BadLba=3c0ba7e


04/25 12:27:44: Double media errors found! badLBA=3c0ba7e ld=0 pd1=0 pd2=1


04/25 12:27:46: REC:log MedErr on pd[0] #retries=0


04/25 12:27:46: LogSense: pd=00, cdb=28 00 03 c0 d1 80 00 00 80 00 


04/25 12:27:46:           sense=f0 00 03 03 c0 d1 be 0a 19 ce 48 f1 11 00 00 00 00 a5 


04/25 12:27:46: REC: MedErr on LD[0] BadLba=3c0d1be


04/25 12:27:46: REC:log MedErr on pd[1] #retries=0


04/25 12:27:46: LogSense: pd=01, cdb=28 00 03 c0 d1 80 00 00 80 00 


04/25 12:27:46:           sense=f0 00 03 03 c0 d1 be 0a 00 00 00 00 11 00 e4 00 00 86 


04/25 12:27:46: REC: MedErr on LD[1] BadLba=3c0d1be


04/25 12:27:47: Double media errors found! badLBA=3c0d1be ld=0 pd1=0 pd2=1


04/25 12:27:50: REC:log MedErr on pd[0] #retries=0


04/25 12:27:50: LogSense: pd=00, cdb=28 00 03 c0 eb 00 00 00 80 00 


04/25 12:27:50:           sense=f0 00 03 03 c0 eb 53 0a 1a c3 48 f3 11 00 00 00 00 a5 


04/25 12:27:50: REC: MedErr on LD[0] BadLba=3c0eb53


04/25 12:27:51: REC:log MedErr on pd[1] #retries=0


04/25 12:27:51: LogSense: pd=01, cdb=28 00 03 c0 eb 00 00 00 80 00 


04/25 12:27:51:           sense=f0 00 03 03 c0 eb 53 0a 00 00 00 00 11 00 e4 00 00 86 


04/25 12:27:51: REC: MedErr on LD[1] BadLba=3c0eb53


04/25 12:27:51: Double media errors found! badLBA=3c0eb53 ld=0 pd1=0 pd2=1


04/25 12:27:53: REC:log MedErr on pd[0] #retries=0


04/25 12:27:53: LogSense: pd=00, cdb=28 00 03 c1 01 80 00 00 80 00 


04/25 12:27:53:           sense=f0 00 03 03 c1 01 f3 0a 18 c3 48 f5 11 00 00 00 00 a5 


04/25 12:27:53: REC: MedErr on LD[0] BadLba=3c101f3


04/25 12:27:53: REC:log MedErr on pd[1] #retries=0


04/25 12:27:53: LogSense: pd=01, cdb=28 00 03 c1 01 80 00 00 80 00 


04/25 12:27:53:           sense=f0 00 03 03 c1 01 f3 0a 00 00 00 00 11 00 e4 00 00 86 


04/25 12:27:53: REC: MedErr on LD[1] BadLba=3c101f3


04/25 12:27:54: Double media errors found! badLBA=3c101f3 ld=0 pd1=0 pd2=1


04/25 12:27:55: REC:log MedErr on pd[0] #retries=0


04/25 12:27:55: LogSense: pd=00, cdb=28 00 03 c1 04 00 00 00 80 00 


04/25 12:27:55:           sense=f0 00 03 03 c1 04 53 0a 18 23 48 f6 11 00 00 00 00 a5 


04/25 12:27:55: REC: MedErr on LD[0] BadLba=3c10453


04/25 12:27:55: REC:log MedErr on pd[1] #retries=0


04/25 12:27:55: LogSense: pd=01, cdb=28 00 03 c1 04 00 00 00 80 00 


04/25 12:27:55:           sense=f0 00 03 03 c1 04 53 0a 00 00 00 00 11 00 e4 00 00 86 


04/25 12:27:55: REC: MedErr on LD[1] BadLba=3c10453


04/25 12:27:56: Double media errors found! badLBA=3c10453 ld=0 pd1=0 pd2=1


04/25 12:28:01: REC:log MedErr on pd[0] #retries=0


04/25 12:28:01: LogSense: pd=00, cdb=28 00 03 c1 5d 00 00 00 80 00 


04/25 12:28:01:           sense=f0 00 03 03 c1 5d 6c 0a 03 0c 48 fd 11 00 00 00 00 a5 


04/25 12:28:01: REC: MedErr on LD[0] BadLba=3c15d6c


04/25 12:28:02: REC:log MedErr on pd[1] #retries=0


04/25 12:28:02: LogSense: pd=01, cdb=28 00 03 c1 5d 00 00 00 80 00 


04/25 12:28:02:           sense=f0 00 03 03 c1 5d 6c 0a 00 00 00 00 11 00 e4 00 00 86 


04/25 12:28:02: REC: MedErr on LD[1] BadLba=3c15d6c


04/25 12:28:02: Double media errors found! badLBA=3c15d6c ld=0 pd1=0 pd2=1


04/25 12:28:04: REC:log MedErr on pd[0] #retries=0


04/25 12:28:04: LogSense: pd=00, cdb=28 00 03 c1 71 80 00 00 80 00 


04/25 12:28:04:           sense=f0 00 03 03 c1 71 8d 0a 01 a2 48 fe 11 00 00 00 00 a5 


04/25 12:28:04: REC: MedErr on LD[0] BadLba=3c1718d


04/25 12:28:04: REC:log MedErr on pd[1] #retries=0


04/25 12:28:04: LogSense: pd=01, cdb=28 00 03 c1 71 80 00 00 80 00 


04/25 12:28:04:           sense=f0 00 03 03 c1 71 8d 0a 00 00 00 00 11 00 e4 00 00 86 


04/25 12:28:04: REC: MedErr on LD[1] BadLba=3c1718d


04/25 12:28:05: Double media errors found! badLBA=3c1718d ld=0 pd1=0 pd2=1


04/25 12:28:50: REC:log MedErr on pd[0] #retries=0


04/25 12:28:50: LogSense: pd=00, cdb=28 00 03 c6 4e 00 00 00 80 00 


04/25 12:28:50:           sense=f0 00 03 03 c6 4e 38 0a 1a a8 49 63 11 00 00 00 00 a5 


04/25 12:28:50: REC: MedErr on LD[0] BadLba=3c64e38


04/25 12:28:51: REC:log MedErr on pd[1] #retries=0


04/25 12:28:51: LogSense: pd=01, cdb=28 00 03 c6 4e 00 00 00 80 00 


04/25 12:28:51:           sense=f0 00 03 03 c6 4e 38 0a 00 00 00 00 11 00 e4 00 00 86 


04/25 12:28:51: REC: MedErr on LD[1] BadLba=3c64e38


04/25 12:28:51: Double media errors found! badLBA=3c64e38 ld=0 pd1=0 pd2=1


04/25 12:28:52: REC:log MedErr on pd[0] #retries=0


04/25 12:28:52: LogSense: pd=00, cdb=28 00 03 c6 50 80 00 00 80 00 


04/25 12:28:52:           sense=f0 00 03 03 c6 50 98 0a 1a 08 49 64 11 00 00 00 00 a5 


04/25 12:28:52: REC: MedErr on LD[0] BadLba=3c65098


04/25 12:28:53: REC:log MedErr on pd[1] #retries=0


04/25 12:28:53: LogSense: pd=01, cdb=28 00 03 c6 50 80 00 00 80 00 


04/25 12:28:53:           sense=f0 00 03 03 c6 50 98 0a 00 00 00 00 11 00 e4 00 00 86 


04/25 12:28:53: REC: MedErr on LD[1] BadLba=3c65098


04/25 12:28:53: Double media errors found! badLBA=3c65098 ld=0 pd1=0 pd2=1


04/25 12:28:55: REC:log MedErr on pd[0] #retries=0


04/25 12:28:55: LogSense: pd=00, cdb=28 00 03 c6 64 80 00 00 80 00 


04/25 12:28:55:           sense=f0 00 03 03 c6 64 d8 0a 18 a8 49 65 11 00 00 00 00 a5 


04/25 12:28:55: REC: MedErr on LD[0] BadLba=3c664d8


04/25 12:28:55: REC:log MedErr on pd[1] #retries=0


04/25 12:28:55: LogSense: pd=01, cdb=28 00 03 c6 64 80 00 00 80 00 


04/25 12:28:55:           sense=f0 00 03 03 c6 64 d8 0a 00 00 00 00 11 00 e4 00 00 86 


04/25 12:28:55: REC: MedErr on LD[1] BadLba=3c664d8


04/25 12:28:55: Double media errors found! badLBA=3c664d8 ld=0 pd1=0 pd2=1


04/25 15:24:28: Consistency check over 0


04/25 16:00:00: prCheckForBackgroundActivity: CC/BGI/Init active on ld=1


04/25 16:00:00: prDiskCheckOkToRun: PR NOT running due to background activity on adapter PD=00


04/25 16:00:00: prCheckForBackgroundActivity: CC/BGI/Init active on ld=1


04/25 16:00:00: prDiskCheckOkToRun: PR NOT running due to background activity on adapter PD=01


04/25 16:00:00: prCheckForBackgroundActivity: CC/BGI/Init active on ld=1


04/25 16:00:00: prDiskCheckOkToRun: PR NOT running due to background activity on adapter PD=12


04/25 16:00:00: prCheckForBackgroundActivity: CC/BGI/Init active on ld=1


04/25 16:00:00: prDiskCheckOkToRun: PR NOT running due to background activity on adapter PD=13


04/25 16:00:00: prCheckForBackgroundActivity: CC/BGI/Init active on ld=1


04/25 16:00:00: prDiskCheckOkToRun: PR NOT running due to background activity on adapter PD=14


04/25 16:00:00: Next PR scheduled to start at 04/25 20:00:00


04/25 17:32:02: battery monitoring: fc occurred and voltage < 1.4


04/25 18:58:14: REC:log MedErr on pd[0] #retries=0


04/25 18:58:14: LogSense: pd=00, cdb=28 00 03 b1 84 00 00 00 80 00 


04/25 18:58:14:           sense=f0 00 03 03 b1 84 6e 0a 1a 20 47 b4 11 0b 00 00 00 79 


04/25 18:58:14: REC: MedErr on LD[0] BadLba=3b1846e


04/25 18:58:15: REC:log MedErr on pd[1] #retries=0


04/25 18:58:15: LogSense: pd=01, cdb=28 00 03 b1 84 00 00 00 80 00 


04/25 18:58:15:           sense=f0 00 03 03 b1 84 6e 0a 00 00 00 00 11 00 e4 00 00 86 


04/25 18:58:15: REC: MedErr on LD[1] BadLba=3b1846e


04/25 18:58:15: Double media errors found! badLBA=3b1846e ld=0 pd1=0 pd2=1


04/25 18:58:15: LD 0 Check Consistency cmd FAILED - double errors


04/25 19:00:46: bginit started for 0


04/25 20:00:00: prCheckForBackgroundActivity: CC/BGI/Init active on ld=0


04/25 20:00:00: prDiskCheckOkToRun: PR NOT running due to background activity on adapter PD=00


04/25 20:00:00: prCheckForBackgroundActivity: CC/BGI/Init active on ld=0


04/25 20:00:00: prDiskCheckOkToRun: PR NOT running due to background activity on adapter PD=01


04/25 20:00:00: prCheckForBackgroundActivity: CC/BGI/Init active on ld=0


04/25 20:00:00: prDiskCheckOkToRun: PR NOT running due to background activity on adapter PD=12


04/25 20:00:00: prCheckForBackgroundActivity: CC/BGI/Init active on ld=0


04/25 20:00:00: prDiskCheckOkToRun: PR NOT running due to background activity on adapter PD=13


04/25 20:00:00: prCheckForBackgroundActivity: CC/BGI/Init active on ld=0


04/25 20:00:00: prDiskCheckOkToRun: PR NOT running due to background activity on adapter PD=14


04/25 20:00:00: Next PR scheduled to start at 04/26  0:00:00


04/25 20:57:43: Consistency check over 1


04/25 21:06:11: REC:log MedErr on pd[0] #retries=0


04/25 21:06:11: LogSense: pd=00, cdb=28 00 03 b1 84 00 00 00 80 00 


04/25 21:06:11:           sense=f0 00 03 03 b1 84 6e 0a 1a 20 47 b4 11 0b 00 00 00 79 


04/25 21:06:11: REC: MedErr on LD[0] BadLba=3b1846e


04/25 21:06:12: REC:log MedErr on pd[1] #retries=0


04/25 21:06:12: LogSense: pd=01, cdb=28 00 03 b1 84 00 00 00 80 00 


04/25 21:06:12:           sense=f0 00 03 03 b1 84 6e 0a 00 00 00 00 11 00 e4 00 00 86 


04/25 21:06:12: REC: MedErr on LD[1] BadLba=3b1846e


04/25 21:06:12: Double media errors found! badLBA=3b1846e ld=0 pd1=0 pd2=1


04/25 21:06:13: REC:log MedErr on pd[0] #retries=0


04/25 21:06:13: LogSense: pd=00, cdb=28 00 03 b1 98 80 00 00 80 00 


04/25 21:06:13:           sense=f0 00 03 03 b1 98 ee 0a 18 c0 47 b5 11 0b 00 00 00 79 


04/25 21:06:13: REC: MedErr on LD[0] BadLba=3b198ee


04/25 21:06:14: REC:log MedErr on pd[1] #retries=0


04/25 21:06:14: LogSense: pd=01, cdb=28 00 03 b1 98 80 00 00 80 00 


04/25 21:06:14:           sense=f0 00 03 03 b1 98 ee 0a 00 00 00 00 11 00 e4 00 00 86 


04/25 21:06:14: REC: MedErr on LD[1] BadLba=3b198ee


04/25 21:06:14: Double media errors found! badLBA=3b198ee ld=0 pd1=0 pd2=1


04/25 21:06:15: REC:log MedErr on pd[0] #retries=0


04/25 21:06:15: LogSense: pd=00, cdb=28 00 03 b1 9b 00 00 00 80 00 


04/25 21:06:15:           sense=f0 00 03 03 b1 9b 4e 0a 18 20 47 b6 11 0b 00 00 00 79 


04/25 21:06:15: REC: MedErr on LD[0] BadLba=3b19b4e


04/25 21:06:15: REC:log MedErr on pd[1] #retries=0


04/25 21:06:15: LogSense: pd=01, cdb=28 00 03 b1 9b 00 00 00 80 00 


04/25 21:06:15:           sense=f0 00 03 03 b1 9b 4e 0a 00 00 00 00 11 00 e4 00 00 86 


04/25 21:06:15: REC: MedErr on LD[1] BadLba=3b19b4e


04/25 21:06:15: Double media errors found! badLBA=3b19b4e ld=0 pd1=0 pd2=1


04/25 21:07:23: REC:log MedErr on pd[0] #retries=0


04/25 21:07:23: LogSense: pd=00, cdb=28 00 03 bd 89 00 00 00 80 00 


04/25 21:07:23:           sense=f0 00 03 03 bd 89 65 0a 11 d4 48 ad 11 0b 00 00 00 79 


04/25 21:07:23: REC: MedErr on LD[0] BadLba=3bd8965


04/25 21:07:24: REC:log MedErr on pd[1] #retries=0


04/25 21:07:24: LogSense: pd=01, cdb=28 00 03 bd 89 00 00 00 80 00 


04/25 21:07:24:           sense=f0 00 03 03 bd 89 65 0a 00 00 00 00 11 00 e4 00 00 86 


04/25 21:07:24: REC: MedErr on LD[1] BadLba=3bd8965


04/25 21:07:24: Double media errors found! badLBA=3bd8965 ld=0 pd1=0 pd2=1


04/25 21:07:25: REC:log MedErr on pd[0] #retries=0


04/25 21:07:25: LogSense: pd=00, cdb=28 00 03 bd 93 80 00 00 80 00 


04/25 21:07:25:           sense=f0 00 03 03 bd 93 b9 0a 13 02 48 ae 11 0b 00 00 00 79 


04/25 21:07:25: REC: MedErr on LD[0] BadLba=3bd93b9


04/25 21:07:26: REC:log MedErr on pd[1] #retries=0


04/25 21:07:26: LogSense: pd=01, cdb=28 00 03 bd 93 80 00 00 80 00 


04/25 21:07:26:           sense=f0 00 03 03 bd 93 b9 0a 00 00 00 00 11 00 e4 00 00 86 


04/25 21:07:26: REC: MedErr on LD[1] BadLba=3bd93b9


04/25 21:07:26: Double media errors found! badLBA=3bd93b9 ld=0 pd1=0 pd2=1


04/25 21:07:36: REC:log MedErr on pd[0] #retries=0


04/25 21:07:36: LogSense: pd=00, cdb=28 00 03 bf 04 80 00 00 80 00 


04/25 21:07:36:           sense=f0 00 03 03 bf 04 f4 0a 12 df 48 cc 11 00 00 00 00 a5 


04/25 21:07:36: REC: MedErr on LD[0] BadLba=3bf04f4


04/25 21:07:36: REC:log MedErr on pd[1] #retries=0


04/25 21:07:36: LogSense: pd=01, cdb=28 00 03 bf 04 80 00 00 80 00 


04/25 21:07:36:           sense=f0 00 03 03 bf 04 f4 0a 00 00 00 00 11 00 e4 00 00 86 


04/25 21:07:36: REC: MedErr on LD[1] BadLba=3bf04f4


04/25 21:07:36: Double media errors found! badLBA=3bf04f4 ld=0 pd1=0 pd2=1


04/25 21:07:42: REC:log MedErr on pd[1] #retries=0


04/25 21:07:42: LogSense: pd=01, cdb=28 00 03 bf 83 00 00 00 80 00 


04/25 21:07:42:           sense=f0 00 03 03 bf 83 6f 0a 00 00 00 00 11 00 e4 00 00 86 


04/25 21:07:42: REC: MedErr on LD[1] BadLba=3bf836f


04/25 21:07:43: REC:log MedErr on pd[0] #retries=0


04/25 21:07:43: LogSense: pd=00, cdb=28 00 03 bf 83 00 00 00 80 00 


04/25 21:07:43:           sense=f0 00 03 03 bf 83 6f 0a 00 01 48 d6 11 0b 00 00 01 ab 


04/25 21:07:43: REC: MedErr on LD[0] BadLba=3bf836f


04/25 21:07:43: Double media errors found! badLBA=3bf836f ld=0 pd1=1 pd2=0


04/25 21:07:50: REC:log MedErr on pd[0] #retries=0


04/25 21:07:50: LogSense: pd=00, cdb=28 00 03 c0 59 00 00 00 80 00 


04/25 21:07:50:           sense=f0 00 03 03 c0 59 2f 0a 19 5f 48 e8 11 00 00 00 00 a5 


04/25 21:07:50: REC: MedErr on LD[0] BadLba=3c0592f


04/25 21:07:50: REC:log MedErr on pd[1] #retries=0


04/25 21:07:50: LogSense: pd=01, cdb=28 00 03 c0 59 00 00 00 80 00 


04/25 21:07:50:           sense=f0 00 03 03 c0 59 2f 0a 00 00 00 00 11 00 e4 00 00 86 


04/25 21:07:50: REC: MedErr on LD[1] BadLba=3c0592f


04/25 21:07:50: Double media errors found! badLBA=3c0592f ld=0 pd1=0 pd2=1


04/25 21:07:52: REC:log MedErr on pd[0] #retries=0


04/25 21:07:52: LogSense: pd=00, cdb=28 00 03 c0 70 00 00 00 80 00 


04/25 21:07:52:           sense=f0 00 03 03 c0 70 6f 0a 1a ff 48 e9 11 00 00 00 00 a5 


04/25 21:07:52: REC: MedErr on LD[0] BadLba=3c0706f


04/25 21:07:53: REC:log MedErr on pd[1] #retries=0


04/25 21:07:53: LogSense: pd=01, cdb=28 00 03 c0 70 00 00 00 80 00 


04/25 21:07:53:           sense=f0 00 03 03 c0 70 6f 0a 00 00 00 00 11 00 e4 00 00 86 


04/25 21:07:53: REC: MedErr on LD[1] BadLba=3c0706f


04/25 21:07:53: Double media errors found! badLBA=3c0706f ld=0 pd1=0 pd2=1


04/25 21:07:54: REC:log MedErr on pd[0] #retries=0


04/25 21:07:54: LogSense: pd=00, cdb=28 00 03 c0 72 80 00 00 80 00 


04/25 21:07:54:           sense=f0 00 03 03 c0 72 cf 0a 1a 5f 48 ea 11 00 00 00 00 a5 


04/25 21:07:54: REC: MedErr on LD[0] BadLba=3c072cf


04/25 21:07:55: REC:log MedErr on pd[1] #retries=0


04/25 21:07:55: LogSense: pd=01, cdb=28 00 03 c0 72 80 00 00 80 00 


04/25 21:07:55:           sense=f0 00 03 03 c0 72 cf 0a 00 00 00 00 11 00 e4 00 00 86 


04/25 21:07:55: REC: MedErr on LD[1] BadLba=3c072cf


04/25 21:07:55: Double media errors found! badLBA=3c072cf ld=0 pd1=0 pd2=1


04/25 21:07:56: REC:log MedErr on pd[0] #retries=0


04/25 21:07:56: LogSense: pd=00, cdb=28 00 03 c0 87 80 00 00 80 00 


04/25 21:07:56:           sense=f0 00 03 03 c0 87 de 0a 19 ce 48 eb 11 00 00 00 00 a5 


04/25 21:07:56: REC: MedErr on LD[0] BadLba=3c087de


04/25 21:07:57: REC:log MedErr on pd[1] #retries=0


04/25 21:07:57: LogSense: pd=01, cdb=28 00 03 c0 87 80 00 00 80 00 


04/25 21:07:57:           sense=f0 00 03 03 c0 87 de 0a 00 00 00 00 11 00 e4 00 00 86 


04/25 21:07:57: REC: MedErr on LD[1] BadLba=3c087de


04/25 21:07:57: Double media errors found! badLBA=3c087de ld=0 pd1=0 pd2=1


04/25 21:07:59: REC:log MedErr on pd[0] #retries=0


04/25 21:07:59: LogSense: pd=00, cdb=28 00 03 c0 8a 00 00 00 80 00 


04/25 21:07:59:           sense=f0 00 03 03 c0 8a 3e 0a 19 2e 48 ec 11 00 00 00 00 a5 


04/25 21:07:59: REC: MedErr on LD[0] BadLba=3c08a3e


04/25 21:08:00: REC:log MedErr on pd[1] #retries=0


04/25 21:08:00: LogSense: pd=01, cdb=28 00 03 c0 8a 00 00 00 80 00 


04/25 21:08:00:           sense=f0 00 03 03 c0 8a 3e 0a 00 00 00 00 11 00 e4 00 00 86 


04/25 21:08:00: REC: MedErr on LD[1] BadLba=3c08a3e


04/25 21:08:00: Double media errors found! badLBA=3c08a3e ld=0 pd1=0 pd2=1


04/25 21:08:02: REC:log MedErr on pd[0] #retries=0


04/25 21:08:02: LogSense: pd=00, cdb=28 00 03 c0 a3 80 00 00 80 00 


04/25 21:08:02:           sense=f0 00 03 03 c0 a3 de 0a 1a 2e 48 ee 11 00 00 00 00 a6 


04/25 21:08:02: REC: MedErr on LD[0] BadLba=3c0a3de


04/25 21:08:02: REC:log MedErr on pd[1] #retries=0


04/25 21:08:02: LogSense: pd=01, cdb=28 00 03 c0 a3 80 00 00 80 00 


04/25 21:08:02:           sense=f0 00 03 03 c0 a3 de 0a 00 00 00 00 11 00 e4 00 00 86 


04/25 21:08:02: REC: MedErr on LD[1] BadLba=3c0a3de


04/25 21:08:03: Double media errors found! badLBA=3c0a3de ld=0 pd1=0 pd2=1


04/25 21:08:04: REC:log MedErr on pd[�€���€��ies=0


04/25 21:08:04: LogSense: pd=00, cdb=28 00 03 c0 b8 00 00 00 80 00 


04/25 21:08:04:           sense=f0 00 03 03 c0 b8 1d 0a 18 cd 48 ef 11 00 00 00 00 a5 


04/25 21:08:04: REC: MedErr on LD[0] BadLba=3c0b81d


04/25 21:08:05: REC:log MedErr on pd[1] #retries=0


04/25 21:08:05: LogSense: pd=01, cdb=28 00 03 c0 b8 00 00 00 80 00 


04/25 21:08:05:           sense=f0 00 03 03 c0 b8 1d 0a 00 00 00 00 11 00 e4 00 00 86 


04/25 21:08:05: REC: MedErr on LD[1] BadLba=3c0b81d


04/25 21:08:05: REC:log MedErr on pd[0] #retries=4


04/25 21:08:05: LogSense: pd=00, cdb=28 00 03 c0 b8 1e 00 00 62 00 


04/25 21:08:05:           sense=f0 00 03 03 c0 b8 1e 0a 18 ce 48 ef 16 00 00 00 00 a5 


04/25 21:08:05: REC: MedErr on LD[0] BadLba=3c0b81e


04/25 21:08:07: REC:log MedErr on pd[1] #retries=4


04/25 21:08:07: LogSense: pd=01, cdb=28 00 03 c0 b8 1e 00 00 62 00 


04/25 21:08:07:           sense=f0 00 03 03 c0 b8 1e 0a 00 00 00 00 11 00 e4 00 00 86 


04/25 21:08:07: REC: MedErr on LD[1] BadLba=3c0b81e


04/25 21:08:07: Double media errors found! badLBA=3c0b81d ld=0 pd1=0 pd2=1


04/25 21:08:09: REC:log MedErr on pd[0] #retries=0


04/25 21:08:09: LogSense: pd=00, cdb=28 00 03 c0 ba 00 00 00 80 00 


04/25 21:08:09:           sense=f0 00 03 03 c0 ba 7e 0a 18 2e 48 f0 11 00 00 00 00 a5 


04/25 21:08:09: REC: MedErr on LD[0] BadLba=3c0ba7e


04/25 21:08:10: REC:log MedErr on pd[1] #retries=0


04/25 21:08:10: LogSense: pd=01, cdb=28 00 03 c0 ba 00 00 00 80 00 


04/25 21:08:10:           sense=f0 00 03 03 c0 ba 7e 0a 00 00 00 00 11 00 e4 00 00 86 


04/25 21:08:10: REC: MedErr on LD[1] BadLba=3c0ba7e


04/25 21:08:10: Double media errors found! badLBA=3c0ba7e ld=0 pd1=0 pd2=1


04/25 21:08:11: REC:log MedErr on pd[0] #retries=0


04/25 21:08:11: LogSense: pd=00, cdb=28 00 03 c0 d1 80 00 00 80 00 


04/25 21:08:11:           sense=f0 00 03 03 c0 d1 be 0a 19 ce 48 f1 11 00 00 00 00 a6 


04/25 21:08:11: REC: MedErr on LD[0] BadLba=3c0d1be


04/25 21:08:12: REC:log MedErr on pd[1] #retries=0


04/25 21:08:12: LogSense: pd=01, cdb=28 00 03 c0 d1 80 00 00 80 00 


04/25 21:08:12:           sense=f0 00 03 03 c0 d1 be 0a 00 00 00 00 11 00 e4 00 00 86 


04/25 21:08:12: REC: MedErr on LD[1] BadLba=3c0d1be


04/25 21:08:12: Double media errors found! badLBA=3c0d1be ld=0 pd1=0 pd2=1


04/25 21:08:14: REC:log MedErr on pd[0] #retries=0


04/25 21:08:14: LogSense: pd=00, cdb=28 00 03 c0 eb 00 00 00 80 00 


04/25 21:08:14:           sense=f0 00 03 03 c0 eb 53 0a 1a c3 48 f3 11 00 00 00 00 a5 


04/25 21:08:14: REC: MedErr on LD[0] BadLba=3c0eb53


04/25 21:08:15: REC:log MedErr on pd[1] #retries=0


04/25 21:08:15: LogSense: pd=01, cdb=28 00 03 c0 eb 00 00 00 80 00 


04/25 21:08:15:           sense=f0 00 03 03 c0 eb 53 0a 00 00 00 00 11 00 e4 00 00 86 


04/25 21:08:15: REC: MedErr on LD[1] BadLba=3c0eb53


04/25 21:08:15: Double media errors found! badLBA=3c0eb53 ld=0 pd1=0 pd2=1


04/25 21:08:16: REC:log MedErr on pd[0] #retries=0


04/25 21:08:16: LogSense: pd=00, cdb=28 00 03 c1 01 80 00 00 80 00 


04/25 21:08:16:           sense=f0 00 03 03 c1 01 f3 0a 18 c3 48 f5 11 00 00 00 00 a5 


04/25 21:08:16: REC: MedErr on LD[0] BadLba=3c101f3


04/25 21:08:17: REC:log MedErr on pd[1] #retries=0


04/25 21:08:17: LogSense: pd=01, cdb=28 00 03 c1 01 80 00 00 80 00 


04/25 21:08:17:           sense=f0 00 03 03 c1 01 f3 0a 00 00 00 00 11 00 e4 00 00 86 


04/25 21:08:17: REC: MedErr on LD[1] BadLba=3c101f3


04/25 21:08:17: Double media errors found! badLBA=3c101f3 ld=0 pd1=0 pd2=1


04/25 21:08:19: REC:log MedErr on pd[0] #retries=0


04/25 21:08:19: LogSense: pd=00, cdb=28 00 03 c1 04 00 00 00 80 00 


04/25 21:08:19:           sense=f0 00 03 03 c1 04 53 0a 18 23 48 f6 11 00 00 00 00 a5 


04/25 21:08:19: REC: MedErr on LD[0] BadLba=3c10453


04/25 21:08:20: REC:log MedErr on pd[1] #retries=0


04/25 21:08:20: LogSense: pd=01, cdb=28 00 03 c1 04 00 00 00 80 00 


04/25 21:08:20:           sense=f0 00 03 03 c1 04 53 0a 00 00 00 00 11 00 e4 00 00 86 


04/25 21:08:20: REC: MedErr on LD[1] BadLba=3c10453


04/25 21:08:20: Double media errors found! badLBA=3c10453 ld=0 pd1=0 pd2=1


04/25 21:08:23: REC:log MedErr on pd[0] #retries=0


04/25 21:08:23: LogSense: pd=00, cdb=28 00 03 c1 5d 00 00 00 80 00 


04/25 21:08:23:           sense=f0 00 03 03 c1 5d 6c 0a 03 0c 48 fd 11 00 00 00 00 a5 


04/25 21:08:23: REC: MedErr on LD[0] BadLba=3c15d6c


04/25 21:08:24: REC:log MedErr on pd[1] #retries=0


04/25 21:08:24: LogSense: pd=01, cdb=28 00 03 c1 5d 00 00 00 80 00 


04/25 21:08:24:           sense=f0 00 03 03 c1 5d 6c 0a 00 00 00 00 11 00 e4 00 00 86 


04/25 21:08:24: REC: MedErr on LD[1] BadLba=3c15d6c


04/25 21:08:24: Double media errors found! badLBA=3c15d6c ld=0 pd1=0 pd2=1


04/25 21:08:26: REC:log MedErr on pd[0] #retries=0


04/25 21:08:26: LogSense: pd=00, cdb=28 00 03 c1 71 80 00 00 80 00 


04/25 21:08:26:           sense=f0 00 03 03 c1 71 8d 0a 01 a2 48 fe 11 00 00 00 00 a5 


04/25 21:08:26: REC: MedErr on LD[0] BadLba=3c1718d


04/25 21:08:26: REC:log MedErr on pd[1] #retries=0


04/25 21:08:26: LogSense: pd=01, cdb=28 00 03 c1 71 80 00 00 80 00 


04/25 21:08:26:           sense=f0 00 03 03 c1 71 8d 0a 00 00 00 00 11 00 e4 00 00 86 


04/25 21:08:26: REC: MedErr on LD[1] BadLba=3c1718d


04/25 21:08:27: Double media errors found! badLBA=3c1718d ld=0 pd1=0 pd2=1


04/25 21:08:56: REC:log MedErr on pd[0] #retries=0


04/25 21:08:56: LogSense: pd=00, cdb=28 00 03 c6 4e 00 00 00 80 00 


04/25 21:08:56:           sense=f0 00 03 03 c6 4e 38 0a 1a a8 49 63 11 00 00 00 00 a5 


04/25 21:08:56: REC: MedErr on LD[0] BadLba=3c64e38


04/25 21:08:57: REC:log MedErr on pd[1] #retries=0


04/25 21:08:57: LogSense: pd=01, cdb=28 00 03 c6 4e 00 00 00 80 00 


04/25 21:08:57:           sense=f0 00 03 03 c6 4e 38 0a 00 00 00 00 11 00 e4 00 00 86 


04/25 21:08:57: REC: MedErr on LD[1] BadLba=3c64e38


04/25 21:08:57: Double media errors found! badLBA=3c64e38 ld=0 pd1=0 pd2=1


04/25 21:08:59: REC:log MedErr on pd[0] #retries=0


04/25 21:08:59: LogSense: pd=00, cdb=28 00 03 c6 50 80 00 00 80 00 


04/25 21:08:59:           sense=f0 00 03 03 c6 50 98 0a 1a 08 49 64 11 00 00 00 00 a5 


04/25 21:08:59: REC: MedErr on LD[0] BadLba=3c65098


04/25 21:08:59: REC:log MedErr on pd[1] #retries=0


04/25 21:08:59: LogSense: pd=01, cdb=28 00 03 c6 50 80 00 00 80 00 


04/25 21:08:59:           sense=f0 00 03 03 c6 50 98 0a 00 00 00 00 11 00 e4 00 00 86 


04/25 21:08:59: REC: MedErr on LD[1] BadLba=3c65098


04/25 21:09:00: Double media errors found! badLBA=3c65098 ld=0 pd1=0 pd2=1


04/25 21:09:01: REC:log MedErr on pd[0] #retries=0


04/25 21:09:01: LogSense: pd=00, cdb=28 00 03 c6 64 80 00 00 80 00 


04/25 21:09:01:           sense=f0 00 03 03 c6 64 d8 0a 18 a8 49 65 11 00 00 00 00 a5 


04/25 21:09:01: REC: MedErr on LD[0] BadLba=3c664d8


04/25 21:09:02: REC:log MedErr on pd[1] #retries=0


04/25 21:09:02: LogSense: pd=01, cdb=28 00 03 c6 64 80 00 00 80 00 


04/25 21:09:02:           sense=f0 00 03 03 c6 64 d8 0a 00 00 00 00 11 00 e4 00 00 86 


04/25 21:09:02: REC: MedErr on LD[1] BadLba=3c664d8


04/25 21:09:02: Double media errors found! badLBA=3c664d8 ld=0 pd1=0 pd2=1


04/25 23:04:46: Consistency check over 0


04/26  0:00:00: prDiskStart: starting Patrol Read on PD=00


04/26  0:00:00: prDiskStart: starting Patrol Read on PD=01


04/26  0:00:00: prDiskStart: starting Patrol Read on PD=12


04/26  0:00:00: prDiskStart: starting Patrol Read on PD=13


04/26  0:00:00: prDiskStart: starting Patrol Read on PD=14


04/26  0:00:00: Next PR scheduled to start at 04/26  4:00:00


04/26  1:07:04: REC:log MedErr on pd[0] #retries=0


04/26  1:07:04: LogSense: pd=00, cdb=2f 00 03 b1 41 0f 00 ff ff 00 


04/26  1:07:04:           sense=f0 00 03 03 b1 84 6e 0a 1a 20 47 b4 11 0b 00 00 00 79 


04/26  1:07:04: REC: MedErr on LD[0] BadLba=3b1846e


04/26  1:07:05: prCallback: Medium Error on pd=00, StartLba=3b1410f, ErrLba=3b1846e


04/26  1:07:05: prRecQueue: starting pd=00 recovery - blocking host commands


04/26  1:07:05: prRecGo: Ready to attempt recovery errLBA=3b1846e on pd=00


04/26  1:07:05: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/26  1:07:05: prRecGo: dataErr found on ld 0 span 0 arm 0


04/26  1:07:05: prRecGo: data NOT in cache; cacheLn=ffffffff, row=76308, stripe=76308, refBlk=6e, type=1


04/26  1:07:05: prRecGo: RAID1 - read from peer pd=01


04/26  1:07:05: REC:log MedErr on pd[1] #retries=0


04/26  1:07:05: LogSense: pd=01, cdb=2f 00 03 b1 41 0f 00 ff ff 00 


04/26  1:07:05:           sense=f0 00 03 03 b1 84 6e 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  1:07:05: REC: MedErr on LD[1] BadLba=3b1846e


04/26  1:07:05: prCallback: Medium Error on pd=01, StartLba=3b1410f, ErrLba=3b1846e


04/26  1:07:05: prRecQueue: adding pd=01 to recovery wait queue


04/26  1:07:07: REC:log MedErr on pd[1] #retries=0


04/26  1:07:07: LogSense: pd=01, cdb=28 00 03 b1 84 6e 00 00 01 00 


04/26  1:07:07:           sense=f0 00 03 03 b1 84 6e 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  1:07:07: REC: MedErr on LD[1] BadLba=3b1846e


04/26  1:07:07: prRecCallback: Error, cdb=28, pd=01, err=2


04/26  1:07:07: PatrolReadStart: Log MedErr pd=00 errLBA=3b1846e in NVRAM_EVENT_LOG


04/26  1:07:07: prRecGo: Ready to attempt recovery errLBA=3b1846e on pd=01


04/26  1:07:07: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/26  1:07:07: prRecGo: dataErr found on ld 0 span 0 arm 1


04/26  1:07:07: prRecGo: data NOT in cache; cacheLn=ffffffff, row=76308, stripe=76308, refBlk=6e, type=1


04/26  1:07:07: prRecGo: RAID1 - read from peer pd=00


04/26  1:07:08: REC:log MedErr on pd[0] #retries=0


04/26  1:07:08: LogSense: pd=00, cdb=28 00 03 b1 84 6e 00 00 01 00 


04/26  1:07:08:           sense=f0 00 03 03 b1 84 6e 0a 1a 20 47 b4 11 0b 00 00 00 79 


04/26  1:07:08: REC: MedErr on LD[0] BadLba=3b1846e


04/26  1:07:08: prRecCallback: Error, cdb=28, pd=00, err=2


04/26  1:07:08: PatrolReadStart: Log MedErr pd=01 errLBA=3b1846e in NVRAM_EVENT_LOG


04/26  1:07:12: REC:log MedErr on pd[0] #retries=0


04/26  1:07:12: LogSense: pd=00, cdb=2f 00 03 b1 84 6f 00 ff ff 00 


04/26  1:07:12:           sense=f0 00 03 03 b1 98 ee 0a 18 c0 47 b5 11 0b 00 00 00 79 


04/26  1:07:12: REC: MedErr on LD[0] BadLba=3b198ee


04/26  1:07:12: prCallback: Medium Error on pd=00, StartLba=3b1846f, ErrLba=3b198ee


04/26  1:07:12: prRecQueue: starting pd=00 recovery - blocking host commands


04/26  1:07:12: prRecGo: Ready to attempt recovery errLBA=3b198ee on pd=00


04/26  1:07:12: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/26  1:07:12: prRecGo: dataErr found on ld 0 span 0 arm 0


04/26  1:07:12: prRecGo: data NOT in cache; cacheLn=ffffffff, row=76331, stripe=76331, refBlk=6e, type=1


04/26  1:07:12: prRecGo: RAID1 - read from peer pd=01


04/26  1:07:13: REC:log MedErr on pd[1] #retries=0


04/26  1:07:13: LogSense: pd=01, cdb=2f 00 03 b1 84 6f 00 ff ff 00 


04/26  1:07:13:           sense=f0 00 03 03 b1 98 ee 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  1:07:13: REC: MedErr on LD[1] BadLba=3b198ee


04/26  1:07:13: prCallback: Medium Error on pd=01, StartLba=3b1846f, ErrLba=3b198ee


04/26  1:07:13: prRecQueue: adding pd=01 to recovery wait queue


04/26  1:07:14: REC:log MedErr on pd[1] #retries=0


04/26  1:07:14: LogSense: pd=01, cdb=28 00 03 b1 98 ee 00 00 01 00 


04/26  1:07:14:           sense=f0 00 03 03 b1 98 ee 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  1:07:14: REC: MedErr on LD[1] BadLba=3b198ee


04/26  1:07:14: prRecCallback: Error, cdb=28, pd=01, err=2


04/26  1:07:14: PatrolReadStart: Log MedErr pd=00 errLBA=3b198ee in NVRAM_EVENT_LOG


04/26  1:07:15: prRecGo: Ready to attempt recovery errLBA=3b198ee on pd=01


04/26  1:07:15: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/26  1:07:15: prRecGo: dataErr found on ld 0 span 0 arm 1


04/26  1:07:15: prRecGo: data NOT in cache; cacheLn=ffffffff, row=76331, stripe=76331, refBlk=6e, type=1


04/26  1:07:15: prRecGo: RAID1 - read from peer pd=00


04/26  1:07:15: REC:log MedErr on pd[0] #retries=0


04/26  1:07:15: LogSense: pd=00, cdb=28 00 03 b1 98 ee 00 00 01 00 


04/26  1:07:15:           sense=f0 00 03 03 b1 98 ee 0a 18 c0 47 b5 11 0b 00 00 00 79 


04/26  1:07:15: REC: MedErr on LD[0] BadLba=3b198ee


04/26  1:07:15: prRecCallback: Error, cdb=28, pd=00, err=2


04/26  1:07:15: PatrolReadStart: Log MedErr pd=01 errLBA=3b198ee in NVRAM_EVENT_LOG


04/26  1:07:19: REC:log MedErr on pd[0] #retries=0


04/26  1:07:19: LogSense: pd=00, cdb=2f 00 03 b1 98 ef 00 ff ff 00 


04/26  1:07:19:           sense=f0 00 03 03 b1 9b 4e 0a 18 20 47 b6 11 0b 00 00 00 79 


04/26  1:07:19: REC: MedErr on LD[0] BadLba=3b19b4e


04/26  1:07:19: prCallback: Medium Error on pd=00, StartLba=3b198ef, ErrLba=3b19b4e


04/26  1:07:19: prRecQueue: starting pd=00 recovery - blocking host commands


04/26  1:07:19: prRecGo: Ready to attempt recovery errLBA=3b19b4e on pd=00


04/26  1:07:19: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/26  1:07:19: prRecGo: dataErr found on ld 0 span 0 arm 0


04/26  1:07:19: prRecGo: data NOT in cache; cacheLn=ffffffff, row=76336, stripe=76336, refBlk=4e, type=1


04/26  1:07:20: prRecGo: RAID1 - read from peer pd=01


04/26  1:07:21: REC:log MedErr on pd[1] #retries=0


04/26  1:07:21: LogSense: pd=01, cdb=28 00 03 b1 9b 4e 00 00 01 00 


04/26  1:07:21:           sense=f0 00 03 03 b1 9b 4e 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  1:07:21: REC: MedErr on LD[1] BadLba=3b19b4e


04/26  1:07:21: prRecCallback: Error, cdb=28, pd=01, err=2


04/26  1:07:21: PatrolReadStart: Log MedErr pd=00 errLBA=3b19b4e in NVRAM_EVENT_LOG


04/26  1:07:22: REC:log MedErr on pd[1] #retries=0


04/26  1:07:22: LogSense: pd=01, cdb=2f 00 03 b1 98 ef 00 08 00 00 


04/26  1:07:22:           sense=f0 00 03 03 b1 9b 4e 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  1:07:22: REC: MedErr on LD[1] BadLba=3b19b4e


04/26  1:07:22: prCallback: Medium Error on pd=01, StartLba=3b198ef, ErrLba=3b19b4e


04/26  1:07:22: prRecQueue: starting pd=01 recovery - blocking host commands


04/26  1:07:22: prRecGo: Ready to attempt recovery errLBA=3b19b4e on pd=01


04/26  1:07:23: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/26  1:07:23: prRecGo: dataErr found on ld 0 span 0 arm 1


04/26  1:07:23: prRecGo: data NOT in cache; cacheLn=ffffffff, row=76336, stripe=76336, refBlk=4e, type=1


04/26  1:07:23: prRecGo: RAID1 - read from peer pd=00


04/26  1:07:23: REC:log MedErr on pd[0] #retries=0


04/26  1:07:23: LogSense: pd=00, cdb=28 00 03 b1 9b 4e 00 00 01 00 


04/26  1:07:23:           sense=f0 00 03 03 b1 9b 4e 0a 18 20 47 b6 11 0b 00 00 00 79 


04/26  1:07:23: REC: MedErr on LD[0] BadLba=3b19b4e


04/26  1:07:23: prRecCallback: Error, cdb=28, pd=00, err=2


04/26  1:07:23: PatrolReadStart: Log MedErr pd=01 errLBA=3b19b4e in NVRAM_EVENT_LOG


04/26  1:08:10: REC:log MedErr on pd[0] #retries=0


04/26  1:08:10: LogSense: pd=00, cdb=2f 00 03 bc 9b 44 00 ff ff 00 


04/26  1:08:10:           sense=f0 00 03 03 bd 89 65 0a 11 d4 48 ad 11 0b 00 00 00 79 


04/26  1:08:10: REC: MedErr on LD[0] BadLba=3bd8965


04/26  1:08:10: prCallback: Medium Error on pd=00, StartLba=3bc9b44, ErrLba=3bd8965


04/26  1:08:10: prRecQueue: starting pd=00 recovery - blocking host commands


04/26  1:08:10: prRecGo: Ready to attempt recovery errLBA=3bd8965 on pd=00


04/26  1:08:10: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/26  1:08:10: prRecGo: dataErr found on ld 0 span 0 arm 0


04/26  1:08:10: prRecGo: data NOT in cache; cacheLn=ffffffff, row=77b12, stripe=77b12, refBlk=65, type=1


04/26  1:08:10: prRecGo: RAID1 - read from peer pd=01


04/26  1:08:11: REC:log MedErr on pd[1] #retries=0


04/26  1:08:11: LogSense: pd=01, cdb=28 00 03 bd 89 65 00 00 01 00 


04/26  1:08:11:           sense=f0 00 03 03 bd 89 65 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  1:08:11: REC: MedErr on LD[1] BadLba=3bd8965


04/26  1:08:12: prRecCallback: Error, cdb=28, pd=01, err=2


04/26  1:08:12: PatrolReadStart: Log MedErr pd=00 errLBA=3bd8965 in NVRAM_EVENT_LOG


04/26  1:08:16: REC:log MedErr on pd[0] #retries=0


04/26  1:08:16: LogSense: pd=00, cdb=2f 00 03 bd 89 66 00 ff ff 00 


04/26  1:08:16:           sense=f0 00 03 03 bd 93 b9 0a 13 02 48 ae 11 0b 00 00 00 79 


04/26  1:08:16: REC: MedErr on LD[0] BadLba=3bd93b9


04/26  1:08:16: prCallback: Medium Error on pd=00, StartLba=3bd8966, ErrLba=3bd93b9


04/26  1:08:16: prRecQueue: starting pd=00 recovery - blocking host commands


04/26  1:08:16: prRecGo: Ready to attempt recovery errLBA=3bd93b9 on pd=00


04/26  1:08:16: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/26  1:08:16: prRecGo: dataErr found on ld 0 span 0 arm 0


04/26  1:08:16: prRecGo: data NOT in cache; cacheLn=ffffffff, row=77b27, stripe=77b27, refBlk=39, type=1


04/26  1:08:17: prRecGo: RAID1 - read from peer pd=01


04/26  1:08:17: REC:log MedErr on pd[1] #retries=0


04/26  1:08:17: LogSense: pd=01, cdb=2f 00 03 bc 9b 84 00 ff ff 00 


04/26  1:08:17:           sense=f0 00 03 03 bd 89 65 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  1:08:17: REC: MedErr on LD[1] BadLba=3bd8965


04/26  1:08:18: prCallback: Medium Error on pd=01, StartLba=3bc9b84, ErrLba=3bd8965


04/26  1:08:18: prRecQueue: adding pd=01 to recovery wait queue


04/26  1:08:19: REC:log MedErr on pd[1] #retries=0


04/26  1:08:19: LogSense: pd=01, cdb=28 00 03 bd 93 b9 00 00 01 00 


04/26  1:08:19:           sense=f0 00 03 03 bd 93 b9 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  1:08:19: REC: MedErr on LD[1] BadLba=3bd93b9


04/26  1:08:19: prRecCallback: Error, cdb=28, pd=01, err=2


04/26  1:08:19: PatrolReadStart: Log MedErr pd=00 errLBA=3bd93b9 in NVRAM_EVENT_LOG


04/26  1:08:19: prRecGo: Ready to attempt recovery errLBA=3bd8965 on pd=01


04/26  1:08:19: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/26  1:08:19: prRecGo: dataErr found on ld 0 span 0 arm 1


04/26  1:08:19: prRecGo: data NOT in cache; cacheLn=ffffffff, row=77b12, stripe=77b12, refBlk=65, type=1


04/26  1:08:19: prRecGo: RAID1 - read from peer pd=00


04/26  1:08:20: REC:log MedErr on pd[0] #retries=0


04/26  1:08:20: LogSense: pd=00, cdb=28 00 03 bd 89 65 00 00 01 00 


04/26  1:08:20:           sense=f0 00 03 03 bd 89 65 0a 11 d4 48 ad 11 0b 00 00 00 79 


04/26  1:08:20: REC: MedErr on LD[0] BadLba=3bd8965


04/26  1:08:20: prRecCallback: Error, cdb=28, pd=00, err=2


04/26  1:08:20: PatrolReadStart: Log MedErr pd=01 errLBA=3bd8965 in NVRAM_EVENT_LOG


04/26  1:08:25: REC:log MedErr on pd[1] #retries=0


04/26  1:08:25: LogSense: pd=01, cdb=2f 00 03 bd 89 66 00 ff ff 00 


04/26  1:08:25:           sense=f0 00 03 03 bd 93 b9 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  1:08:25: REC: MedErr on LD[1] BadLba=3bd93b9


04/26  1:08:25: prCallback: Medium Error on pd=01, StartLba=3bd8966, ErrLba=3bd93b9


04/26  1:08:25: prRecQueue: starting pd=01 recovery - blocking host commands


04/26  1:08:25: prRecGo: Ready to attempt recovery errLBA=3bd93b9 on pd=01


04/26  1:08:25: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/26  1:08:25: prRecGo: dataErr found on ld 0 span 0 arm 1


04/26  1:08:25: prRecGo: data NOT in cache; cacheLn=ffffffff, row=77b27, stripe=77b27, refBlk=39, type=1


04/26  1:08:25: prRecGo: RAID1 - read from peer pd=00


04/26  1:08:26: REC:log MedErr on pd[0] #retries=0


04/26  1:08:26: LogSense: pd=00, cdb=28 00 03 bd 93 b9 00 00 01 00 


04/26  1:08:26:           sense=f0 00 03 03 bd 93 b9 0a 13 02 48 ae 11 0b 00 00 00 79 


04/26  1:08:26: REC: MedErr on LD[0] BadLba=3bd93b9


04/26  1:08:26: prRecCallback: Error, cdb=28, pd=00, err=2


04/26  1:08:26: PatrolReadStart: Log MedErr pd=01 errLBA=3bd93b9 in NVRAM_EVENT_LOG


04/26  1:08:28: REC:log MedErr on pd[0] #retries=0


04/26  1:08:28: LogSense: pd=00, cdb=2f 00 03 be 93 b9 00 ff ff 00 


04/26  1:08:28:           sense=f0 00 03 03 bf 04 f4 0a 12 df 48 cc 11 00 00 00 00 a5 


04/26  1:08:28: REC: MedErr on LD[0] BadLba=3bf04f4


04/26  1:08:28: prCallback: Medium Error on pd=00, StartLba=3be93b9, ErrLba=3bf04f4


04/26  1:08:28: prRecQueue: starting pd=00 recovery - blocking host commands


04/26  1:08:28: prRecGo: Ready to attempt recovery errLBA=3bf04f4 on pd=00


04/26  1:08:28: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/26  1:08:28: prRecGo: dataErr found on ld 0 span 0 arm 0


04/26  1:08:28: prRecGo: data NOT in cache; cacheLn=ffffffff, row=77e09, stripe=77e09, refBlk=74, type=1


04/26  1:08:28: prRecGo: RAID1 - read from peer pd=01


04/26  1:08:30: REC:log MedErr on pd[1] #retries=0


04/26  1:08:30: LogSense: pd=01, cdb=28 00 03 bf 04 f4 00 00 01 00 


04/26  1:08:30:           sense=f0 00 03 03 bf 04 f4 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  1:08:30: REC: MedErr on LD[1] BadLba=3bf04f4


04/26  1:08:30: prRecCallback: Error, cdb=28, pd=01, err=2


04/26  1:08:30: PatrolReadStart: Log MedErr pd=00 errLBA=3bf04f4 in NVRAM_EVENT_LOG


04/26  1:08:37: REC:log MedErr on pd[0] #retries=0


04/26  1:08:37: LogSense: pd=00, cdb=2f 00 03 bf 04 f5 00 ff ff 00 


04/26  1:08:37:           sense=f0 00 03 03 bf 83 6f 0a 00 01 48 d6 11 0b 00 00 01 ab 


04/26  1:08:37: REC: MedErr on LD[0] BadLba=3bf836f


04/26  1:08:37: prCallback: Medium Error on pd=00, StartLba=3bf04f5, ErrLba=3bf836f


04/26  1:08:37: prRecQueue: starting pd=00 recovery - blocking host commands


04/26  1:08:37: prRecGo: Ready to attempt recovery errLBA=3bf836f on pd=00


04/26  1:08:37: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/26  1:08:37: prRecGo: dataErr found on ld 0 span 0 arm 0


04/26  1:08:37: prRecGo: data NOT in cache; cacheLn=ffffffff, row=77f06, stripe=77f06, refBlk=6f, type=1


04/26  1:08:37: prRecGo: RAID1 - read from peer pd=01


04/26  1:08:38: REC:log MedErr on pd[1] #retries=0


04/26  1:08:38: LogSense: pd=01, cdb=28 00 03 bf 83 6f 00 00 01 00 


04/26  1:08:38:           sense=f0 00 03 03 bf 83 6f 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  1:08:38: REC: MedErr on LD[1] BadLba=3bf836f


04/26  1:08:38: prRecCallback: Error, cdb=28, pd=01, err=2


04/26  1:08:38: PatrolReadStart: Log MedErr pd=00 errLBA=3bf836f in NVRAM_EVENT_LOG


04/26  1:08:43: REC:log MedErr on pd[0] #retries=0


04/26  1:08:43: LogSense: pd=00, cdb=2f 00 03 bf 83 70 00 ff ff 00 


04/26  1:08:43:           sense=f0 00 03 03 c0 59 2f 0a 19 5f 48 e8 11 00 00 00 00 a5 


04/26  1:08:43: REC: MedErr on LD[0] BadLba=3c0592f


04/26  1:08:43: REC:log MedErr on pd[1] #retries=0


04/26  1:08:43: LogSense: pd=01, cdb=2f 00 03 be a3 b9 00 ff ff 00 


04/26  1:08:43:           sense=f0 00 03 03 bf 04 f4 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  1:08:43: REC: MedErr on LD[1] BadLba=3bf04f4


04/26  1:08:43: prCallback: Medium Error on pd=00, StartLba=3bf8370, ErrLba=3c0592f


04/26  1:08:43: prRecQueue: starting pd=00 recovery - blocking host commands


04/26  1:08:43: prCallback: Medium Error on pd=01, StartLba=3bea3b9, ErrLba=3bf04f4


04/26  1:08:43: prRecQueue: adding pd=01 to recovery wait queue


04/26  1:08:43: prRecGo: Ready to attempt recovery errLBA=3c0592f on pd=00


04/26  1:08:43: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/26  1:08:43: prRecGo: dataErr found on ld 0 span 0 arm 0


04/26  1:08:43: prRecGo: data NOT in cache; cacheLn=ffffffff, row=780b2, stripe=780b2, refBlk=2f, type=1


04/26  1:08:43: prRecGo: RAID1 - read from peer pd=01


04/26  1:08:45: REC:log MedErr on pd[1] #retries=0


04/26  1:08:45: LogSense: pd=01, cdb=28 00 03 c0 59 2f 00 00 01 00 


04/26  1:08:45:           sense=f0 00 03 03 c0 59 2f 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  1:08:45: REC: MedErr on LD[1] BadLba=3c0592f


04/26  1:08:45: prRecCallback: Error, cdb=28, pd=01, err=2


04/26  1:08:45: PatrolReadStart: Log MedErr pd=00 errLBA=3c0592f in NVRAM_EVENT_LOG


04/26  1:08:45: prRecGo: Ready to attempt recovery errLBA=3bf04f4 on pd=01


04/26  1:08:45: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/26  1:08:45: prRecGo: dataErr found on ld 0 span 0 arm 1


04/26  1:08:45: prRecGo: data NOT in cache; cacheLn=ffffffff, row=77e09, stripe=77e09, refBlk=74, type=1


04/26  1:08:45: prRecGo: RAID1 - read from peer pd=00


04/26  1:08:46: REC:log MedErr on pd[0] #retries=0


04/26  1:08:46: LogSense: pd=00, cdb=28 00 03 bf 04 f4 00 00 01 00 


04/26  1:08:46:           sense=f0 00 03 03 bf 04 f4 0a 12 df 48 cc 11 00 00 00 00 a5 


04/26  1:08:46: REC: MedErr on LD[0] BadLba=3bf04f4


04/26  1:08:46: prRecCallback: Error, cdb=28, pd=00, err=2


04/26  1:08:46: PatrolReadStart: Log MedErr pd=01 errLBA=3bf04f4 in NVRAM_EVENT_LOG


04/26  1:08:51: REC:log MedErr on pd[0] #retries=0


04/26  1:08:51: LogSense: pd=00, cdb=2f 00 03 c0 59 30 00 ff ff 00 


04/26  1:08:51:           sense=f0 00 03 03 c0 70 6f 0a 1a ff 48 e9 11 00 00 00 00 a5 


04/26  1:08:51: REC: MedErr on LD[0] BadLba=3c0706f


04/26  1:08:51: prCallback: Medium Error on pd=00, StartLba=3c05930, ErrLba=3c0706f


04/26  1:08:51: prRecQueue: starting pd=00 recovery - blocking host commands


04/26  1:08:51: prRecGo: Ready to attempt recovery errLBA=3c0706f on pd=00


04/26  1:08:51: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/26  1:08:51: prRecGo: dataErr found on ld 0 span 0 arm 0


04/26  1:08:51: prRecGo: data NOT in cache; cacheLn=ffffffff, row=780e0, stripe=780e0, refBlk=6f, type=1


04/26  1:08:51: prRecGo: RAID1 - read from peer pd=01


04/26  1:08:51: REC:log MedErr on pd[1] #retries=0


04/26  1:08:51: LogSense: pd=01, cdb=2f 00 03 bf 04 f5 00 ff ff 00 


04/26  1:08:51:           sense=f0 00 03 03 bf 83 6f 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  1:08:51: REC: MedErr on LD[1] BadLba=3bf836f


04/26  1:08:51: prCallback: Medium Error on pd=01, StartLba=3bf04f5, ErrLba=3bf836f


04/26  1:08:51: prRecQueue: adding pd=01 to recovery wait queue


04/26  1:08:53: REC:log MedErr on pd[1] #retries=0


04/26  1:08:53: LogSense: pd=01, cdb=28 00 03 c0 70 6f 00 00 01 00 


04/26  1:08:53:           sense=f0 00 03 03 c0 70 6f 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  1:08:53: REC: MedErr on LD[1] BadLba=3c0706f


04/26  1:08:53: prRecCallback: Error, cdb=28, pd=01, err=2


04/26  1:08:53: PatrolReadStart: Log MedErr pd=00 errLBA=3c0706f in NVRAM_EVENT_LOG


04/26  1:08:53: prRecGo: Ready to attempt recovery errLBA=3bf836f on pd=01


04/26  1:08:53: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/26  1:08:53: prRecGo: dataErr found on ld 0 span 0 arm 1


04/26  1:08:53: prRecGo: data NOT in cache; cacheLn=ffffffff, row=77f06, stripe=77f06, refBlk=6f, type=1


04/26  1:08:53: prRecGo: RAID1 - read from peer pd=00


04/26  1:08:56: REC:log MedErr on pd[0] #retries=0


04/26  1:08:56: LogSense: pd=00, cdb=28 00 03 bf 83 6f 00 00 01 00 


04/26  1:08:56:           sense=f0 00 03 03 bf 83 6f 0a 00 01 48 d6 11 0b 00 00 01 ab 


04/26  1:08:56: REC: MedErr on LD[0] BadLba=3bf836f


04/26  1:08:56: prRecCallback: Error, cdb=28, pd=00, err=2


04/26  1:08:56: PatrolReadStart: Log MedErr pd=01 errLBA=3bf836f in NVRAM_EVENT_LOG


04/26  1:08:58: REC:log MedErr on pd[0] #retries=0


04/26  1:08:58: LogSense: pd=00, cdb=2f 00 03 c0 70 70 00 ff ff 00 


04/26  1:08:58:           sense=f0 00 03 03 c0 72 cf 0a 1a 5f 48 ea 11 00 00 00 00 a5 


04/26  1:08:58: REC: MedErr on LD[0] BadLba=3c072cf


04/26  1:08:58: prCallback: Medium Error on pd=00, StartLba=3c07070, ErrLba=3c072cf


04/26  1:08:58: prRecQueue: starting pd=00 recovery - blocking host commands


04/26  1:08:58: prRecGo: Ready to attempt recovery errLBA=3c072cf on pd=00


04/26  1:08:58: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/26  1:08:58: prRecGo: dataErr found on ld 0 span 0 arm 0


04/26  1:08:58: prRecGo: data NOT in cache; cacheLn=ffffffff, row=780e5, stripe=780e5, refBlk=4f, type=1


04/26  1:08:58: prRecGo: RAID1 - read from peer pd=01


04/26  1:09:00: REC:log MedErr on pd[1] #retries=0


04/26  1:09:00: LogSense: pd=01, cdb=28 00 03 c0 72 cf 00 00 01 00 


04/26  1:09:00:           sense=f0 00 03 03 c0 72 cf 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  1:09:00: REC: MedErr on LD[1] BadLba=3c072cf


04/26  1:09:00: prRecCallback: Error, cdb=28, pd=01, err=2


04/26  1:09:00: PatrolReadStart: Log MedErr pd=00 errLBA=3c072cf in NVRAM_EVENT_LOG


04/26  1:09:05: REC:log MedErr on pd[1] #retries=0


04/26  1:09:05: LogSense: pd=01, cdb=2f 00 03 bf 8b 70 00 ff ff 00 


04/26  1:09:05:           sense=f0 00 03 03 c0 59 2f 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  1:09:05: REC: MedErr on LD[1] BadLba=3c0592f


04/26  1:09:05: prCallback: Medium Error on pd=01, StartLba=3bf8b70, ErrLba=3c0592f


04/26  1:09:05: prRecQueue: starting pd=01 recovery - blocking host commands


04/26  1:09:06: prRecGo: Ready to attempt recovery errLBA=3c0592f on pd=01


04/26  1:09:06: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/26  1:09:06: prRecGo: dataErr found on ld 0 span 0 arm 1


04/26  1:09:06: prRecGo: data NOT in cache; cacheLn=ffffffff, row=780b2, stripe=780b2, refBlk=2f, type=1


04/26  1:09:06: prRecGo: RAID1 - read from peer pd=00


04/26  1:09:06: REC:log MedErr on pd[0] #retries=0


04/26  1:09:06: LogSense: pd=00, cdb=2f 00 03 c0 72 d0 00 ff ff 00 


04/26  1:09:06:           sense=f0 00 03 03 c0 87 de 0a 19 ce 48 eb 11 00 00 00 00 a5 


04/26  1:09:06: REC: MedErr on LD[0] BadLba=3c087de


04/26  1:09:06: prCallback: Medium Error on pd=00, StartLba=3c072d0, ErrLba=3c087de


04/26  1:09:06: prRecQueue: adding pd=00 to recovery wait queue


04/26  1:09:07: REC:log MedErr on pd[0] #retries=0


04/26  1:09:07: LogSense: pd=00, cdb=28 00 03 c0 59 2f 00 00 01 00 


04/26  1:09:07:           sense=f0 00 03 03 c0 59 2f 0a 19 5f 48 e8 11 00 00 00 00 a5 


04/26  1:09:07: REC: MedErr on LD[0] BadLba=3c0592f


04/26  1:09:07: prRecCallback: Error, cdb=28, pd=00, err=2


04/26  1:09:07: PatrolReadStart: Log MedErr pd=01 errLBA=3c0592f in NVRAM_EVENT_LOG


04/26  1:09:07: prRecGo: Ready to attempt recovery errLBA=3c087de on pd=00


04/26  1:09:07: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/26  1:09:07: prRecGo: dataErr found on ld 0 span 0 arm 0


04/26  1:09:07: prRecGo: data NOT in cache; cacheLn=ffffffff, row=7810f, stripe=7810f, refBlk=5e, type=1


04/26  1:09:07: prRecGo: RAID1 - read from peer pd=01


04/26  1:09:09: REC:log MedErr on pd[1] #retries=0


04/26  1:09:09: LogSense: pd=01, cdb=28 00 03 c0 87 de 00 00 01 00 


04/26  1:09:09:           sense=f0 00 03 03 c0 87 de 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  1:09:09: REC: MedErr on LD[1] BadLba=3c087de


04/26  1:09:09: prRecCallback: Error, cdb=28, pd=01, err=2


04/26  1:09:09: PatrolReadStart: Log MedErr pd=00 errLBA=3c087de in NVRAM_EVENT_LOG


04/26  1:09:12: REC:log MedErr on pd[1] #retries=0


04/26  1:09:12: LogSense: pd=01, cdb=2f 00 03 c0 59 30 00 ff ff 00 


04/26  1:09:12:           sense=f0 00 03 03 c0 70 6f 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  1:09:12: REC: MedErr on LD[1] BadLba=3c0706f


04/26  1:09:12: prCallback: Medium Error on pd=01, StartLba=3c05930, ErrLba=3c0706f


04/26  1:09:12: prRecQueue: starting pd=01 recovery - blocking host commands


04/26  1:09:12: prRecGo: Ready to attempt recovery errLBA=3c0706f on pd=01


04/26  1:09:12: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/26  1:09:12: prRecGo: dataErr found on ld 0 span 0 arm 1


04/26  1:09:12: prRecGo: data NOT in cache; cacheLn=ffffffff, row=780e0, stripe=780e0, refBlk=6f, type=1


04/26  1:09:12: prRecGo: RAID1 - read from peer pd=00


04/26  1:09:15: REC:log MedErr on pd[0] #retries=0


04/26  1:09:15: LogSense: pd=00, cdb=28 00 03 c0 70 6f 00 00 01 00 


04/26  1:09:15:           sense=f0 00 03 03 c0 70 6f 0a 1a ff 48 e9 11 00 00 00 00 a5 


04/26  1:09:15: REC: MedErr on LD[0] BadLba=3c0706f


04/26  1:09:15: REC:log MedErr on pd[0] #retries=0


04/26  1:09:15: LogSense: pd=00, cdb=2f 00 03 c0 87 df 00 08 00 00 


04/26  1:09:15:           sense=f0 00 03 03 c0 8a 3e 0a 19 2e 48 ec 11 00 00 00 00 a5 


04/26  1:09:15: REC: MedErr on LD[0] BadLba=3c08a3e


04/26  1:09:15: prRecCallback: Error, cdb=28, pd=00, err=2


04/26  1:09:15: PatrolReadStart: Log MedErr pd=01 errLBA=3c0706f in NVRAM_EVENT_LOG


04/26  1:09:15: prCallback: Medium Error on pd=00, StartLba=3c087df, ErrLba=3c08a3e


04/26  1:09:15: prRecQueue: starting pd=00 recovery - blocking host commands


04/26  1:09:15: prRecGo: Ready to attempt recovery errLBA=3c08a3e on pd=00


04/26  1:09:15: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/26  1:09:15: prRecGo: dataErr found on ld 0 span 0 arm 0


04/26  1:09:15: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78114, stripe=78114, refBlk=3e, type=1


04/26  1:09:15: prRecGo: RAID1 - read from peer pd=01


04/26  1:09:17: REC:log MedErr on pd[1] #retries=0


04/26  1:09:17: LogSense: pd=01, cdb=28 00 03 c0 8a 3e 00 00 01 00 


04/26  1:09:17:           sense=f0 00 03 03 c0 8a 3e 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  1:09:17: REC: MedErr on LD[1] BadLba=3c08a3e


04/26  1:09:17: prRecCallback: Error, cdb=28, pd=01, err=2


04/26  1:09:17: PatrolReadStart: Log MedErr pd=00 errLBA=3c08a3e in NVRAM_EVENT_LOG


04/26  1:09:20: REC:log MedErr on pd[1] #retries=0


04/26  1:09:20: LogSense: pd=01, cdb=2f 00 03 c0 70 70 00 ff ff 00 


04/26  1:09:20:           sense=f0 00 03 03 c0 72 cf 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  1:09:20: REC: MedErr on LD[1] BadLba=3c072cf


04/26  1:09:20: prCallback: Medium Error on pd=01, StartLba=3�����€��ErrLba=3c072cf


04/26  1:09:20: prRecQueue: starting pd=01 recovery - blocking host commands


04/26  1:09:20: prRecGo: Ready to attempt recovery errLBA=3c072cf on pd=01


04/26  1:09:20: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/26  1:09:20: prRecGo: dataErr found on ld 0 span 0 arm 1


04/26  1:09:20: prRecGo: data NOT in cache; cacheLn=ffffffff, row=780e5, stripe=780e5, refBlk=4f, type=1


04/26  1:09:20: prRecGo: RAID1 - read from peer pd=00


04/26  1:09:22: REC:log MedErr on pd[0] #retries=0


04/26  1:09:22: LogSense: pd=00, cdb=28 00 03 c0 72 cf 00 00 01 00 


04/26  1:09:22:           sense=f0 00 03 03 c0 72 cf 0a 1a 5f 48 ea 11 00 00 00 00 a5 


04/26  1:09:22: REC: MedErr on LD[0] BadLba=3c072cf


04/26  1:09:22: prRecCallback: Error, cdb=28, pd=00, err=2


04/26  1:09:22: PatrolReadStart: Log MedErr pd=01 errLBA=3c072cf in NVRAM_EVENT_LOG


04/26  1:09:27: REC:log MedErr on pd[0] #retries=0


04/26  1:09:27: LogSense: pd=00, cdb=2f 00 03 c0 92 3f 00 ff ff 00 


04/26  1:09:27:           sense=f0 00 03 03 c0 a3 de 0a 1a 2e 48 ee 11 00 00 00 00 a5 


04/26  1:09:27: REC: MedErr on LD[0] BadLba=3c0a3de


04/26  1:09:27: prCallback: Medium Error on pd=00, StartLba=3c0923f, ErrLba=3c0a3de


04/26  1:09:27: prRecQueue: starting pd=00 recovery - blocking host commands


04/26  1:09:27: prRecGo: Ready to attempt recovery errLBA=3c0a3de on pd=00


04/26  1:09:27: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/26  1:09:27: prRecGo: dataErr found on ld 0 span 0 arm 0


04/26  1:09:27: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78147, stripe=78147, refBlk=5e, type=1


04/26  1:09:27: prRecGo: RAID1 - read from peer pd=01


04/26  1:09:27: REC:log MedErr on pd[1] #retries=0


04/26  1:09:27: LogSense: pd=01, cdb=2f 00 03 c0 72 d0 00 ff ff 00 


04/26  1:09:27:           sense=f0 00 03 03 c0 87 de 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  1:09:27: REC: MedErr on LD[1] BadLba=3c087de


04/26  1:09:27: prCallback: Medium Error on pd=01, StartLba=3c072d0, ErrLba=3c087de


04/26  1:09:27: prRecQueue: adding pd=01 to recovery wait queue


04/26  1:09:29: REC:log MedErr on pd[1] #retries=0


04/26  1:09:29: LogSense: pd=01, cdb=28 00 03 c0 a3 de 00 00 01 00 


04/26  1:09:29:           sense=f0 00 03 03 c0 a3 de 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  1:09:29: REC: MedErr on LD[1] BadLba=3c0a3de


04/26  1:09:29: prRecCallback: Error, cdb=28, pd=01, err=2


04/26  1:09:29: PatrolReadStart: Log MedErr pd=00 errLBA=3c0a3de in NVRAM_EVENT_LOG


04/26  1:09:29: prRecGo: Ready to attempt recovery errLBA=3c087de on pd=01


04/26  1:09:29: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/26  1:09:29: prRecGo: dataErr found on ld 0 span 0 arm 1


04/26  1:09:29: prRecGo: data NOT in cache; cacheLn=ffffffff, row=7810f, stripe=7810f, refBlk=5e, type=1


04/26  1:09:29: prRecGo: RAID1 - read from peer pd=00


04/26  1:09:31: REC:log MedErr on pd[0] #retries=0


04/26  1:09:31: LogSense: pd=00, cdb=28 00 03 c0 87 de 00 00 01 00 


04/26  1:09:31:           sense=f0 00 03 03 c0 87 de 0a 19 ce 48 eb 11 00 00 00 00 a5 


04/26  1:09:31: REC: MedErr on LD[0] BadLba=3c087de


04/26  1:09:31: prRecCallback: Error, cdb=28, pd=00, err=2


04/26  1:09:31: PatrolReadStart: Log MedErr pd=01 errLBA=3c087de in NVRAM_EVENT_LOG


04/26  1:09:35: REC:log MedErr on pd[0] #retries=0


04/26  1:09:35: LogSense: pd=00, cdb=2f 00 03 c0 a3 df 00 ff ff 00 


04/26  1:09:35:           sense=f0 00 03 03 c0 b8 1d 0a 18 cd 48 ef 11 00 00 00 00 a5 


04/26  1:09:35: REC: MedErr on LD[0] BadLba=3c0b81d


04/26  1:09:35: prCallback: Medium Error on pd=00, StartLba=3c0a3df, ErrLba=3c0b81d


04/26  1:09:35: prRecQueue: starting pd=00 recovery - blocking host commands


04/26  1:09:35: prRecGo: Ready to attempt recovery errLBA=3c0b81d on pd=00


04/26  1:09:35: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/26  1:09:35: prRecGo: dataErr found on ld 0 span 0 arm 0


04/26  1:09:35: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78170, stripe=78170, refBlk=1d, type=1


04/26  1:09:35: prRecGo: RAID1 - read from peer pd=01


04/26  1:09:36: REC:log MedErr on pd[1] #retries=0


04/26  1:09:36: LogSense: pd=01, cdb=2f 00 03 c0 87 df 00 ff ff 00 


04/26  1:09:36:           sense=f0 00 03 03 c0 8a 3e 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  1:09:36: REC: MedErr on LD[1] BadLba=3c08a3e


04/26  1:09:36: prCallback: Medium Error on pd=01, StartLba=3c087df, ErrLba=3c08a3e


04/26  1:09:36: prRecQueue: adding pd=01 to recovery wait queue


04/26  1:09:38: REC:log MedErr on pd[1] #retries=0


04/26  1:09:38: LogSense: pd=01, cdb=28 00 03 c0 b8 1d 00 00 01 00 


04/26  1:09:38:           sense=f0 00 03 03 c0 b8 1d 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  1:09:38: REC: MedErr on LD[1] BadLba=3c0b81d


04/26  1:09:38: prRecCallback: Error, cdb=28, pd=01, err=2


04/26  1:09:38: PatrolReadStart: Log MedErr pd=00 errLBA=3c0b81d in NVRAM_EVENT_LOG


04/26  1:09:38: prRecGo: Ready to attempt recovery errLBA=3c08a3e on pd=01


04/26  1:09:38: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/26  1:09:38: prRecGo: dataErr found on ld 0 span 0 arm 1


04/26  1:09:38: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78114, stripe=78114, refBlk=3e, type=1


04/26  1:09:38: prRecGo: RAID1 - read from peer pd=00


04/26  1:09:39: REC:log MedErr on pd[0] #retries=0


04/26  1:09:39: LogSense: pd=00, cdb=28 00 03 c0 8a 3e 00 00 01 00 


04/26  1:09:39:           sense=f0 00 03 03 c0 8a 3e 0a 19 2e 48 ec 11 00 00 00 00 a5 


04/26  1:09:39: REC: MedErr on LD[0] BadLba=3c08a3e


04/26  1:09:40: prRecCallback: Error, cdb=28, pd=00, err=2


04/26  1:09:40: PatrolReadStart: Log MedErr pd=01 errLBA=3c08a3e in NVRAM_EVENT_LOG


04/26  1:09:44: REC:log MedErr on pd[0] #retries=0


04/26  1:09:44: LogSense: pd=00, cdb=2f 00 03 c0 b8 1e 00 ff ff 00 


04/26  1:09:44:           sense=f0 00 03 03 c0 b8 1e 0a 18 ce 48 ef 11 00 00 00 00 a6 


04/26  1:09:44: REC: MedErr on LD[0] BadLba=3c0b81e


04/26  1:09:44: prCallback: Medium Error on pd=00, StartLba=3c0b81e, ErrLba=3c0b81e


04/26  1:09:44: prRecQueue: starting pd=00 recovery - blocking host commands


04/26  1:09:44: prRecGo: Ready to attempt recovery errLBA=3c0b81e on pd=00


04/26  1:09:44: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/26  1:09:44: prRecGo: dataErr found on ld 0 span 0 arm 0


04/26  1:09:44: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78170, stripe=78170, refBlk=1e, type=1


04/26  1:09:44: prRecGo: RAID1 - read from peer pd=01


04/26  1:09:45: REC:log MedErr on pd[1] #retries=0


04/26  1:09:45: LogSense: pd=01, cdb=2f 00 03 c0 8a 3f 00 ff ff 00 


04/26  1:09:45:           sense=f0 00 03 03 c0 a3 de 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  1:09:45: REC: MedErr on LD[1] BadLba=3c0a3de


04/26  1:09:45: prCallback: Medium Error on pd=01, StartLba=3c08a3f, ErrLba=3c0a3de


04/26  1:09:45: prRecQueue: adding pd=01 to recovery wait queue


04/26  1:09:47: REC:log MedErr on pd[1] #retries=0


04/26  1:09:47: LogSense: pd=01, cdb=28 00 03 c0 b8 1e 00 00 01 00 


04/26  1:09:47:           sense=f0 00 03 03 c0 b8 1e 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  1:09:47: REC: MedErr on LD[1] BadLba=3c0b81e


04/26  1:09:47: prRecCallback: Error, cdb=28, pd=01, err=2


04/26  1:09:47: PatrolReadStart: Log MedErr pd=00 errLBA=3c0b81e in NVRAM_EVENT_LOG


04/26  1:09:47: prRecGo: Ready to attempt recovery errLBA=3c0a3de on pd=01


04/26  1:09:47: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/26  1:09:47: prRecGo: dataErr found on ld 0 span 0 arm 1


04/26  1:09:47: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78147, stripe=78147, refBlk=5e, type=1


04/26  1:09:47: prRecGo: RAID1 - read from peer pd=00


04/26  1:09:49: REC:log MedErr on pd[0] #retries=0


04/26  1:09:49: LogSense: pd=00, cdb=28 00 03 c0 a3 de 00 00 01 00 


04/26  1:09:49:           sense=f0 00 03 03 c0 a3 de 0a 1a 2e 48 ee 11 00 00 00 00 a5 


04/26  1:09:49: REC: MedErr on LD[0] BadLba=3c0a3de


04/26  1:09:49: prRecCallback: Error, cdb=28, pd=00, err=2


04/26  1:09:49: PatrolReadStart: Log MedErr pd=01 errLBA=3c0a3de in NVRAM_EVENT_LOG


04/26  1:09:52: REC:log MedErr on pd[0] #retries=0


04/26  1:09:52: LogSense: pd=00, cdb=2f 00 03 c0 b8 1f 00 ff ff 00 


04/26  1:09:52:           sense=f0 00 03 03 c0 ba 7e 0a 18 2e 48 f0 11 00 00 00 00 a5 


04/26  1:09:52: REC: MedErr on LD[0] BadLba=3c0ba7e


04/26  1:09:52: prCallback: Medium Error on pd=00, StartLba=3c0b81f, ErrLba=3c0ba7e


04/26  1:09:52: prRecQueue: starting pd=00 recovery - blocking host commands


04/26  1:09:52: prRecGo: Ready to attempt recovery errLBA=3c0ba7e on pd=00


04/26  1:09:52: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/26  1:09:52: prRecGo: dataErr found on ld 0 span 0 arm 0


04/26  1:09:52: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78174, stripe=78174, refBlk=7e, type=1


04/26  1:09:52: prRecGo: RAID1 - read from peer pd=01


04/26  1:09:54: REC:log MedErr on pd[1] #retries=0


04/26  1:09:54: LogSense: pd=01, cdb=28 00 03 c0 ba 7e 00 00 01 00 


04/26  1:09:54:           sense=f0 00 03 03 c0 ba 7e 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  1:09:54: REC: MedErr on LD[1] BadLba=3c0ba7e


04/26  1:09:54: prRecCallback: Error, cdb=28, pd=01, err=2


04/26  1:09:54: PatrolReadStart: Log MedErr pd=00 errLBA=3c0ba7e in NVRAM_EVENT_LOG


04/26  1:09:59: REC:log MedErr on pd[0] #retries=0


04/26  1:09:59: LogSense: pd=00, cdb=2f 00 03 c0 ba 7f 00 ff ff 00 


04/26  1:09:59:           sense=f0 00 03 03 c0 d1 be 0a 19 ce 48 f1 11 00 00 00 00 a5 


04/26  1:09:59: REC: MedErr on LD[0] BadLba=3c0d1be


04/26  1:09:59: prCallback: Medium Error on pd=00, StartLba=3c0ba7f, ErrLba=3c0d1be


04/26  1:09:59: prRecQueue: starting pd=00 recovery - blocking host commands


04/26  1:09:59: prRecGo: Ready to attempt recovery errLBA=3c0d1be on pd=00


04/26  1:09:59: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/26  1:09:59: prRecGo: dataErr found on ld 0 span 0 arm 0


04/26  1:09:59: prRecGo: data NOT in cache; cacheLn=ffffffff, row=781a3, stripe=781a3, refBlk=3e, type=1


04/26  1:09:59: prRecGo: RAID1 - read from peer pd=01


04/26  1:09:59: REC:log MedErr on pd[1] #retries=0


04/26  1:09:59: LogSense: pd=01, cdb=2f 00 03 c0 ab df 00 ff ff 00 


04/26  1:09:59:           sense=f0 00 03 03 c0 b8 1d 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  1:09:59: REC: MedErr on LD[1] BadLba=3c0b81d


04/26  1:09:59: prCallback: Medium Error on pd=01, StartLba=3c0abdf, ErrLba=3c0b81d


04/26  1:09:59: prRecQueue: adding pd=01 to recovery wait queue


04/26  1:10:01: REC:log MedErr on pd[1] #retries=0


04/26  1:10:01: LogSense: pd=01, cdb=28 00 03 c0 d1 be 00 00 01 00 


04/26  1:10:01:           sense=f0 00 03 03 c0 d1 be 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  1:10:01: REC: MedErr on LD[1] BadLba=3c0d1be


04/26  1:10:01: prRecCallback: Error, cdb=28, pd=01, err=2


04/26  1:10:01: PatrolReadStart: Log MedErr pd=00 errLBA=3c0d1be in NVRAM_EVENT_LOG


04/26  1:10:01: prRecGo: Ready to attempt recovery errLBA=3c0b81d on pd=01


04/26  1:10:01: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/26  1:10:01: prRecGo: dataErr found on ld 0 span 0 arm 1


04/26  1:10:01: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78170, stripe=78170, refBlk=1d, type=1


04/26  1:10:01: prRecGo: RAID1 - read from peer pd=00


04/26  1:10:03: REC:log MedErr on pd[0] #retries=0


04/26  1:10:03: LogSense: pd=00, cdb=28 00 03 c0 b8 1d 00 00 01 00 


04/26  1:10:03:           sense=f0 00 03 03 c0 b8 1d 0a 18 cd 48 ef 11 00 00 00 00 a5 


04/26  1:10:03: REC: MedErr on LD[0] BadLba=3c0b81d


04/26  1:10:03: prRecCallback: Error, cdb=28, pd=00, err=2


04/26  1:10:03: PatrolReadStart: Log MedErr pd=01 errLBA=3c0b81d in NVRAM_EVENT_LOG


04/26  1:10:06: REC:log MedErr on pd[0] #retries=0


04/26  1:10:06: LogSense: pd=00, cdb=2f 00 03 c0 d1 bf 00 ff ff 00 


04/26  1:10:06:           sense=f0 00 03 03 c0 eb 53 0a 1a c3 48 f3 11 00 00 00 00 a5 


04/26  1:10:06: REC: MedErr on LD[0] BadLba=3c0eb53


04/26  1:10:06: prCallback: Medium Error on pd=00, StartLba=3c0d1bf, ErrLba=3c0eb53


04/26  1:10:06: prRecQueue: starting pd=00 recovery - blocking host commands


04/26  1:10:06: prRecGo: Ready to attempt recovery errLBA=3c0eb53 on pd=00


04/26  1:10:06: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/26  1:10:06: prRecGo: dataErr found on ld 0 span 0 arm 0


04/26  1:10:06: prRecGo: data NOT in cache; cacheLn=ffffffff, row=781d6, stripe=781d6, refBlk=53, type=1


04/26  1:10:06: prRecGo: RAID1 - read from peer pd=01


04/26  1:10:08: REC:log MedErr on pd[1] #retries=0


04/26  1:10:08: LogSense: pd=01, cdb=28 00 03 c0 eb 53 00 00 01 00 


04/26  1:10:08:           sense=f0 00 03 03 c0 eb 53 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  1:10:08: REC: MedErr on LD[1] BadLba=3c0eb53


04/26  1:10:08: prRecCallback: Error, cdb=28, pd=01, err=2


04/26  1:10:08: PatrolReadStart: Log MedErr pd=00 errLBA=3c0eb53 in NVRAM_EVENT_LOG


04/26  1:10:10: REC:log MedErr on pd[1] #retries=0


04/26  1:10:10: LogSense: pd=01, cdb=2f 00 03 c0 b8 1e 00 00 40 00 


04/26  1:10:10:           sense=f0 00 03 03 c0 b8 1e 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  1:10:10: REC: MedErr on LD[1] BadLba=3c0b81e


04/26  1:10:10: prCallback: Medium Error on pd=01, StartLba=3c0b81e, ErrLba=3c0b81e


04/26  1:10:10: prRecQueue: starting pd=01 recovery - blocking host commands


04/26  1:10:10: prRecGo: Ready to attempt recovery errLBA=3c0b81e on pd=01


04/26  1:10:10: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/26  1:10:10: prRecGo: dataErr found on ld 0 span 0 arm 1


04/26  1:10:10: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78170, stripe=78170, refBlk=1e, type=1


04/26  1:10:10: prRecGo: RAID1 - read from peer pd=00


04/26  1:10:11: REC:log MedErr on pd[0] #retries=0


04/26  1:10:11: LogSense: pd=00, cdb=28 00 03 c0 b8 1e 00 00 01 00 


04/26  1:10:11:           sense=f0 00 03 03 c0 b8 1e 0a 18 ce 48 ef 16 00 00 00 00 a5 


04/26  1:10:11: REC: MedErr on LD[0] BadLba=3c0b81e


04/26  1:10:11: prRecCallback: Error, cdb=28, pd=00, err=2


04/26  1:10:11: PatrolReadStart: Log MedErr pd=01 errLBA=3c0b81e in NVRAM_EVENT_LOG


04/26  1:10:14: REC:log MedErr on pd[0] #retries=0


04/26  1:10:14: LogSense: pd=00, cdb=2f 00 03 c0 eb 54 00 ff ff 00 


04/26  1:10:14:           sense=f0 00 03 03 c1 01 f3 0a 18 c3 48 f5 11 00 00 00 00 a5 


04/26  1:10:14: REC: MedErr on LD[0] BadLba=3c101f3


04/26  1:10:14: prCallback: Medium Error on pd=00, StartLba=3c0eb54, ErrLba=3c101f3


04/26  1:10:14: prRecQueue: starting pd=00 recovery - blocking host commands


04/26  1:10:14: prRecGo: Ready to attempt recovery errLBA=3c101f3 on pd=00


04/26  1:10:14: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/26  1:10:14: prRecGo: dataErr found on ld 0 span 0 arm 0


04/26  1:10:14: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78203, stripe=78203, refBlk=73, type=1


04/26  1:10:14: prRecGo: RAID1 - read from peer pd=01


04/26  1:10:16: REC:log MedErr on pd[1] #retries=0


04/26  1:10:16: LogSense: pd=01, cdb=28 00 03 c1 01 f3 00 00 01 00 


04/26  1:10:16:           sense=f0 00 03 03 c1 01 f3 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  1:10:16: REC: MedErr on LD[1] BadLba=3c101f3


04/26  1:10:16: prRecCallback: Error, cdb=28, pd=01, err=2


04/26  1:10:16: PatrolReadStart: Log MedErr pd=00 errLBA=3c101f3 in NVRAM_EVENT_LOG


04/26  1:10:18: REC:log MedErr on pd[1] #retries=0


04/26  1:10:18: LogSense: pd=01, cdb=2f 00 03 c0 b8 1f 00 08 00 00 


04/26  1:10:18:           sense=f0 00 03 03 c0 ba 7e 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  1:10:18: REC: MedErr on LD[1] BadLba=3c0ba7e


04/26  1:10:18: prCallback: Medium Error on pd=01, StartLba=3c0b81f, ErrLba=3c0ba7e


04/26  1:10:18: prRecQueue: starting pd=01 recovery - blocking host commands


04/26  1:10:18: prRecGo: Ready to attempt recovery errLBA=3c0ba7e on pd=01


04/26  1:10:18: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/26  1:10:18: prRecGo: dataErr found on ld 0 span 0 arm 1


04/26  1:10:18: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78174, stripe=78174, refBlk=7e, type=1


04/26  1:10:18: prRecGo: RAID1 - read from peer pd=00


04/26  1:10:19: REC:log MedErr on pd[0] #retries=0


04/26  1:10:19: LogSense: pd=00, cdb=28 00 03 c0 ba 7e 00 00 01 00 


04/26  1:10:19:           sense=f0 00 03 03 c0 ba 7e 0a 18 2e 48 f0 11 00 00 00 00 a5 


04/26  1:10:19: REC: MedErr on LD[0] BadLba=3c0ba7e


04/26  1:10:19: prRecCallback: Error, cdb=28, pd=00, err=2


04/26  1:10:19: PatrolReadStart: Log MedErr pd=01 errLBA=3c0ba7e in NVRAM_EVENT_LOG


04/26  1:10:21: REC:log MedErr on pd[0] #retries=0


04/26  1:10:21: LogSense: pd=00, cdb=2f 00 03 c1 01 f4 00 ff ff 00 


04/26  1:10:21:           sense=f0 00 03 03 c1 04 53 0a 18 23 48 f6 11 00 00 00 00 a5 


04/26  1:10:21: REC: MedErr on LD[0] BadLba=3c10453


04/26  1:10:21: prCallback: Medium Error on pd=00, StartLba=3c101f4, ErrLba=3c10453


04/26  1:10:21: prRecQueue: starting pd=00 recovery - blocking host commands


04/26  1:10:21: prRecGo: Ready to attempt recovery errLBA=3c10453 on pd=00


04/26  1:10:21: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/26  1:10:21: prRecGo: dataErr found on ld 0 span 0 arm 0


04/26  1:10:21: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78208, stripe=78208, refBlk=53, type=1


04/26  1:10:21: prRecGo: RAID1 - read from peer pd=01


04/26  1:10:23: REC:log MedErr on pd[1] #retries=0


04/26  1:10:23: LogSense: pd=01, cdb=28 00 03 c1 04 53 00 00 01 00 


04/26  1:10:23:           sense=f0 00 03 03 c1 04 53 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  1:10:23: REC: MedErr on LD[1] BadLba=3c10453


04/26  1:10:23: prRecCallback: Error, cdb=28, pd=01, err=2


04/26  1:10:23: PatrolReadStart: Log MedErr pd=00 errLBA=3c10453 in NVRAM_EVENT_LOG


04/26  1:10:28: REC:log MedErr on pd[0] #retries=0


04/26  1:10:28: LogSense: pd=00, cdb=2f 00 03 c1 04 54 00 ff ff 00 


04/26  1:10:28:           sense=f0 00 03 03 c1 5d 6c 0a 03 0c 48 fd 11 00 00 00 00 a5 


04/26  1:10:28: REC: MedErr on LD[0] BadLba=3c15d6c


04/26  1:10:28: REC:log MedErr on pd[1] #retries=0


04/26  1:10:28: LogSense: pd=01, cdb=2f 00 03 c0 ba bf 00 ff ff 00 


04/26  1:10:28:           sense=f0 00 03 03 c0 d1 be 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  1:10:28: REC: MedErr on LD[1] BadLba=3c0d1be


04/26  1:10:28: prCallback: Medium Error on pd=00, StartLba=3c10454, ErrLba=3c15d6c


04/26  1:10:28: prRecQueue: starting pd=00 recovery - blocking host commands


04/26  1:10:28: prCallback: Medium Error on pd=01, StartLba=3c0babf, ErrLba=3c0d1be


04/26  1:10:28: prRecQueue: adding pd=01 to recovery wait queue


04/26  1:10:28: prRecGo: Ready to attempt recovery errLBA=3c15d6c on pd=00


04/26  1:10:28: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/26  1:10:28: prRecGo: dataErr found on ld 0 span 0 arm 0


04/26  1:10:28: prRecGo: data NOT in cache; cacheLn=ffffffff, row=782ba, stripe=782ba, refBlk=6c, type=1


04/26  1:10:29: prRecGo: RAID1 - read from peer pd=01


04/26  1:10:30: REC:log MedErr on pd[1] #retries=0


04/26  1:10:30: LogSense: pd=01, cdb=28 00 03 c1 5d 6c 00 00 01 00 


04/26  1:10:30:           sense=f0 00 03 03 c1 5d 6c 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  1:10:30: REC: MedErr on LD[1] BadLba=3c15d6c


04/26  1:10:30: prRecCallback: Error, cdb=28, pd=01, err=2


04/26  1:10:30: PatrolReadStart: Log MedErr pd=00 errLBA=3c15d6c in NVRAM_EVENT_LOG


04/26  1:10:30: prRecGo: Ready to attempt recovery errLBA=3c0d1be on pd=01


04/26  1:10:30: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/26  1:10:30: prRecGo: dataErr found on ld 0 span 0 arm 1


04/26  1:10:30: prRecGo: data NOT in cache; cacheLn=ffffffff, row=781a3, stripe=781a3, refBlk=3e, type=1


04/26  1:10:30: prRecGo: RAID1 - read from peer pd=00


04/26  1:10:32: REC:log MedErr on pd[0] #retries=0


04/26  1:10:32: LogSense: pd=00, cdb=28 00 03 c0 d1 be 00 00 01 00 


04/26  1:10:32:           sense=f0 00 03 03 c0 d1 be 0a 19 ce 48 f1 11 00 00 00 00 a5 


04/26  1:10:32: REC: MedErr on LD[0] BadLba=3c0d1be


04/26  1:10:32: prRecCallback: Error, cdb=28, pd=00, err=2


04/26  1:10:32: PatrolReadStart: Log MedErr pd=01 errLBA=3c0d1be in NVRAM_EVENT_LOG


04/26  1:10:35: REC:log MedErr on pd[0] #retries=0


04/26  1:10:35: LogSense: pd=00, cdb=2f 00 03 c1 5d 6d 00 ff ff 00 


04/26  1:10:35:           sense=f0 00 03 03 c1 71 8d 0a 01 a2 48 fe 11 00 00 00 00 a5 


04/26  1:10:35: REC: MedErr on LD[0] BadLba=3c1718d


04/26  1:10:35: prCallback: Medium Error on pd=00, StartLba=3c15d6d, ErrLba=3c1718d


04/26  1:10:35: prRecQueue: starting pd=00 recovery - blocking host commands


04/26  1:10:35: prRecGo: Ready to attempt recovery errLBA=3c1718d on pd=00


04/26  1:10:35: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/26  1:10:35: prRecGo: dataErr found on ld 0 span 0 arm 0


04/26  1:10:35: prRecGo: data NOT in cache; cacheLn=ffffffff, row=782e3, stripe=782e3, refBlk=d, type=1


04/26  1:10:35: prRecGo: RAID1 - read from peer pd=01


04/26  1:10:37: REC:log MedErr on pd[1] #retries=0


04/26  1:10:37: LogSense: pd=01, cdb=28 00 03 c1 71 8d 00 00 01 00 


04/26  1:10:37:           sense=f0 00 03 03 c1 71 8d 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  1:10:37: REC: MedErr on LD[1] BadLba=3c1718d


04/26  1:10:37: prRecCallback: Error, cdb=28, pd=01, err=2


04/26  1:10:37: PatrolReadStart: Log MedErr pd=00 errLBA=3c1718d in NVRAM_EVENT_LOG


04/26  1:10:42: REC:log MedErr on pd[1] #retries=0


04/26  1:10:42: LogSense: pd=01, cdb=2f 00 03 c0 d9 bf 00 ff ff 00 


04/26  1:10:42:           sense=f0 00 03 03 c0 eb 53 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  1:10:42: REC: MedErr on LD[1] BadLba=3c0eb53


04/26  1:10:42: prCallback: Medium Error on pd=01, StartLba=3c0d9bf, ErrLba=3c0eb53


04/26  1:10:42: prRecQueue: starting pd=01 recovery - blocking host commands


04/26  1:10:42: prRecGo: Ready to attempt recovery errLBA=3c0eb53 on pd=01


04/26  1:10:42: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/26  1:10:42: prRecGo: dataErr found on ld 0 span 0 arm 1


04/26  1:10:42: prRecGo: data NOT in cache; cacheLn=ffffffff, row=781d6, stripe=781d6, refBlk=53, type=1


04/26  1:10:42: prRecGo: RAID1 - read from peer pd=00


04/26  1:10:44: REC:log MedErr on pd[0] #retries=0


04/26  1:10:44: LogSense: pd=00, cdb=28 00 03 c0 eb 53 00 00 01 00 


04/26  1:10:44:           sense=f0 00 03 03 c0 eb 53 0a 1a c3 48 f3 11 00 00 00 00 a5 


04/26  1:10:44: REC: MedErr on LD[0] BadLba=3c0eb53


04/26  1:10:44: prRecCallback: Error, cdb=28, pd=00, err=2


04/26  1:10:44: PatrolReadStart: Log MedErr pd=01 errLBA=3c0eb53 in NVRAM_EVENT_LOG


04/26  1:10:49: REC:log MedErr on pd[1] #retries=0


04/26  1:10:49: LogSense: pd=01, cdb=2f 00 03 c0 eb 54 00 ff ff 00 


04/26  1:10:49:           sense=f0 00 03 03 c1 01 f3 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  1:10:49: REC: MedErr on LD[1] BadLba=3c101f3


04/26  1:10:49: prCallback: Medium Error on pd=01, StartLba=3c0eb54, ErrLba=3c101f3


04/26  1:10:49: prRecQueue: starting pd=01 recovery - blocking host commands


04/26  1:10:49: prRecGo: Ready to attempt recovery errLBA=3c101f3 on pd=01


04/26  1:10:49: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/26  1:10:49: prRecGo: dataErr found on ld 0 span 0 arm 1


04/26  1:10:49: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78203, stripe=78203, refBlk=73, type=1


04/26  1:10:49: prRecGo: RAID1 - read from peer pd=00


04/26  1:10:51: REC:log MedErr on pd[0] #retries=0


04/26  1:10:51: LogSense: pd=00, cdb=28 00 03 c1 01 f3 00 00 01 00 


04/26  1:10:51:           sense=f0 00 03 03 c1 01 f3 0a 18 c3 48 f5 11 00 00 00 00 a5 


04/26  1:10:51: REC: MedErr on LD[0] BadLba=3c101f3


04/26  1:10:51: prRecCallback: Error, cdb=28, pd=00, err=2


04/26  1:10:51: PatrolReadStart: Log MedErr pd=01 errLBA=3c101f3 in NVRAM_EVENT_LOG


04/26  1:10:56: REC:log MedErr on pd[1] #retries=0


04/26  1:10:56: LogSense: pd=01, cdb=2f 00 03 c1 01 f4 00 ff ff 00 


04/26  1:10:56:           sense=f0 00 03 03 c1 04 53 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  1:10:56: REC: MedErr on LD[1] BadLba=3c10453


04/26  1:10:56: prCallback: Medium Error on pd=01, StartLba=3c101f4, ErrLba=3c10453


04/26  1:10:56: prRecQueue: starting pd=01 recovery - blocking host commands


04/26  1:10:56: prRecGo: Ready to attempt recovery errLBA=3c10453 on pd=01


04/26  1:10:56: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/26  1:10:56: prRecGo: dataErr found on ld 0 span 0 arm 1


04/26  1:10:56: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78208, stripe=78208, refBlk=53, type=1


04/26  1:10:56: prRecGo: RAID1 - read from peer pd=00


04/26  1:10:58: REC:log MedErr on pd[0] #retries=0


04/26  1:10:58: LogSense: pd=00, cdb=28 00 03 c1 04 53 00 00 01 00 


04/26  1:10:58:           sense=f0 00 03 03 c1 04 53 0a 18 23 48 f6 11 00 00 00 00 a5 


04/26  1:10:58: REC: MedErr on LD[0] BadLba=3c10453


04/26  1:10:58: prRecCallback: Error, cdb=28, pd=00, err=2


04/26  1:10:58: PatrolReadStart: Log MedErr pd=01 errLBA=3c10453 in NVRAM_EVENT_LOG


04/26  1:11:03: REC:log MedErr on pd[1] #retries=0


04/26  1:11:03: LogSense: pd=01, cdb=2f 00 03 c1 04 54 00 ff ff 00 


04/26  1:11:03:           sense=f0 00 03 03 c1 5d 6c 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  1:11:03: REC: MedErr on LD[1] BadLba=3c15d6c


04/26  1:11:03: prCallback: Medium Error on pd=01, StartLba=3c10454, ErrLba=3c15d6c


04/26  1:11:03: prRecQueue: starting pd=01 recovery - blocking host commands


04/26  1:11:04: prRecGo: Ready to attempt recovery errLBA=3c15d6c on pd=01


04/26  1:11:04: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/26  1:11:04: prRecGo: dataErr found on ld 0 span 0 arm 1


04/26  1:11:04: prRecGo: data NOT in cache; cacheLn=ffffffff, row=782ba, stripe=782ba, refBlk=6c, type=1


04/26  1:11:04: prRecGo: RAID1 - read from peer pd=00


04/26  1:11:04: REC:log MedErr on pd[0] #retries=0


04/26  1:11:04: LogSense: pd=00, cdb=2f 00 03 c5 72 8a 00 ff ff 00 


04/26  1:11:04:           sense=f0 00 03 03 c6 4e 38 0a 1a a8 49 63 11 00 00 00 00 a5 


04/26  1:11:04: REC: MedErr on LD[0] BadLba=3c64e38


04/26  1:11:04: prCallback: Medium Error on pd=00, StartLba=3c5728a, ErrLba=3c64e38


04/26  1:11:04: prRecQueue: adding pd=00 to recovery wait queue


04/26  1:11:05: REC:log MedErr on pd[0] #retries=0


04/26  1:11:05: LogSense: pd=00, cdb=28 00 03 c1 5d 6c 00 00 01 00 


04/26  1:11:05:           sense=f0 00 03 03 c1 5d 6c 0a 03 0c 48 fd 11 00 00 00 00 a5 


04/26  1:11:05: REC: MedErr on LD[0] BadLba=3c15d6c


04/26  1:11:06: prRecCallback: Error, cdb=28, pd=00, err=2


04/26  1:11:06: PatrolReadStart: Log MedErr pd=01 errLBA=3c15d6c in NVRAM_EVENT_LOG


04/26  1:11:06: prRecGo: Ready to attempt recovery errLBA=3c64e38 on pd=00


04/26  1:11:06: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/26  1:11:06: prRecGo: dataErr found on ld 0 span 0 arm 0


04/26  1:11:06: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78c9c, stripe=78c9c, refBlk=38, type=1


04/26  1:11:06: prRecGo: RAID1 - read from peer pd=01


04/26  1:11:07: REC:log MedErr on pd[1] #retries=0


04/26  1:11:07: LogSense: pd=01, cdb=28 00 03 c6 4e 38 00 00 01 00 


04/26  1:11:07:           sense=f0 00 03 03 c6 4e 38 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  1:11:07: REC: MedErr on LD[1] BadLba=3c64e38


04/26  1:11:07: prRecCallback: Error, cdb=28, pd=01, err=2


04/26  1:11:07: PatrolReadStart: Log MedErr pd=00 errLBA=3c64e38 in NVRAM_EVENT_LOG


04/26  1:11:11: REC:log MedErr on pd[1] #retries=0


04/26  1:11:11: LogSense: pd=01, cdb=2f 00 03 c1 5d 6d 00 ff ff 00 


04/26  1:11:11:           sense=f0 00 03 03 c1 71 8d 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  1:11:11: REC: MedErr on LD[1] BadLba=3c1718d


04/26  1:11:11: prCallback: Medium Error on pd=01, StartLba=3c15d6d, ErrLba=3c1718d


04/26  1:11:11: prRecQueue: starting pd=01 recovery - blocking host commands


04/26  1:11:11: prRecGo: Ready to attempt recovery errLBA=3c1718d on pd=01


04/26  1:11:11: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/26  1:11:11: prRecGo: dataErr found on ld 0 span 0 arm 1


04/26  1:11:11: prRecGo: data NOT in cache; cacheLn=ffffffff, row=782e3, stripe=782e3, refBlk=d, type=1


04/26  1:11:11: prRecGo: RAID1 - read from peer pd=00


04/26  1:11:12: REC:log MedErr on pd[0] #retries=0


04/26  1:11:12: LogSense: pd=00, cdb=2f 00 03 c6 4e 39 00 ff ff 00 


04/26  1:11:12:           sense=f0 00 03 03 c6 50 98 0a 1a 08 49 64 11 00 00 00 00 a5 


04/26  1:11:12: REC: MedErr on LD[0] BadLba=3c65098


04/26  1:11:12: prCallback: Medium Error on pd=00, StartLba=3c64e39, ErrLba=3c65098


04/26  1:11:12: prRecQueue: adding pd=00 to recovery wait queue


04/26  1:11:13: REC:log MedErr on pd[0] #retries=0


04/26  1:11:13: LogSense: pd=00, cdb=28 00 03 c1 71 8d 00 00 01 00 


04/26  1:11:13:           sense=f0 00 03 03 c1 71 8d 0a 01 a2 48 fe 11 00 00 00 00 a5 


04/26  1:11:13: REC: MedErr on LD[0] BadLba=3c1718d


04/26  1:11:13: prRecCallback: Error, cdb=28, pd=00, err=2


04/26  1:11:13: PatrolReadStart: Log MedErr pd=01 errLBA=3c1718d in NVRAM_EVENT_LOG


04/26  1:11:13: prRecGo: Ready to attempt recovery errLBA=3c65098 on pd=00


04/26  1:11:13: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/26  1:11:13: prRecGo: dataErr found on ld 0 span 0 arm 0


04/26  1:11:13: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78ca1, stripe=78ca1, refBlk=18, type=1


04/26  1:11:13: prRecGo: RAID1 - read from peer pd=01


04/26  1:11:15: REC:log MedErr on pd[1] #retries=0


04/26  1:11:15: LogSense: pd=01, cdb=28 00 03 c6 50 98 00 00 01 00 


04/26  1:11:15:           sense=f0 00 03 03 c6 50 98 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  1:11:15: REC: MedErr on LD[1] BadLba=3c65098


04/26  1:11:15: prRecCallback: Error, cdb=28, pd=01, err=2


04/26  1:11:15: PatrolReadStart: Log MedErr pd=00 errLBA=3c65098 in NVRAM_EVENT_LOG


04/26  1:11:20: REC:log MedErr on pd[0] #retries=0


04/26  1:11:20: LogSense: pd=00, cdb=2f 00 03 c6 50 99 00 ff ff 00 


04/26  1:11:20:           sense=f0 00 03 03 c6 64 d8 0a 18 a8 49 65 11 00 00 00 00 a5 


04/26  1:11:20: REC: MedErr on LD[0] BadLba=3c664d8


04/26  1:11:20: prCallback: Medium Error on pd=00, StartLba=3c65099, ErrLba=3c664d8


04/26  1:11:20: prRecQueue: starting pd=00 recovery - blocking host commands


04/26  1:11:20: prRecGo: Ready to attempt recovery errLBA=3c664d8 on pd=00


04/26  1:11:20: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/26  1:11:20: prRecGo: dataErr found on ld 0 span 0 arm 0


04/26  1:11:20: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78cc9, stripe=78cc9, refBlk=58, type=1


04/26  1:11:20: prRecGo: RAID1 - read from peer pd=01


04/26  1:11:22: REC:log MedErr on pd[1] #retries=0


04/26  1:11:22: LogSense: pd=01, cdb=28 00 03 c6 64 d8 00 00 01 00 


04/26  1:11:22:           sense=f0 00 03 03 c6 64 d8 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  1:11:22: REC: MedErr on LD[1] BadLba=3c664d8


04/26  1:11:22: prRecCallback: Error, cdb=28, pd=01, err=2


04/26  1:11:22: PatrolReadStart: Log MedErr pd=00 errLBA=3c664d8 in NVRAM_EVENT_LOG


04/26  1:11:40: REC:log MedErr on pd[1] #retries=0


04/26  1:11:40: LogSense: pd=01, cdb=2f 00 03 c5 79 8a 00 ff ff 00 


04/26  1:11:40:           sense=f0 00 03 03 c6 4e 38 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  1:11:40: REC: MedErr on LD[1] BadLba=3c64e38


04/26  1:11:40: prCallback: Medium Error on pd=01, StartLba=3c5798a, ErrLba=3c64e38


04/26  1:11:40: prRecQueue: starting pd=01 recovery - blocking host commands


04/26  1:11:40: prRecGo: Ready to attempt recovery errLBA=3c64e38 on pd=01


04/26  1:11:40: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/26  1:11:40: prRecGo: dataErr found on ld 0 span 0 arm 1


04/26  1:11:40: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78c9c, stripe=78c9c, refBlk=38, type=1


04/26  1:11:40: prRecGo: RAID1 - read from peer pd=00


04/26  1:11:41: REC:log MedErr on pd[0] #retries=0


04/26  1:11:41: LogSense: pd=00, cdb=28 00 03 c6 4e 38 00 00 01 00 


04/26  1:11:41:           sense=f0 00 03 03 c6 4e 38 0a 1a a8 49 63 11 00 00 00 00 a5 


04/26  1:11:41: REC: MedErr on LD[0] BadLba=3c64e38


04/26  1:11:41: prRecCallback: Error, cdb=28, pd=00, err=2


04/26  1:11:41: PatrolReadStart: Log MedErr pd=01 errLBA=3c64e38 in NVRAM_EVENT_LOG


04/26  1:11:46: REC:log MedErr on pd[1] #retries=0


04/26  1:11:46: LogSense: pd=01, cdb=2f 00 03 c6 4e 39 00 ff ff 00 


04/26  1:11:46:           sense=f0 00 03 03 c6 50 98 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  1:11:46: REC: MedErr on LD[1] BadLba=3c65098


04/26  1:11:46: prCallback: Medium Error on pd=01, StartLba=3c64e39, ErrLba=3c65098


04/26  1:11:46: prRecQueue: starting pd=01 recovery - blocking host commands


04/26  1:11:46: prRecGo: Ready to attempt recovery errLBA=3c65098 on pd=01


04/26  1:11:46: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/26  1:11:47: prRecGo: dataErr found on ld 0 span 0 arm 1


04/26  1:11:47: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78ca1, stripe=78ca1, refBlk=18, type=1


04/26  1:11:47: prRecGo: RAID1 - read from peer pd=00


04/26  1:11:48: REC:log MedErr on pd[0] #retries=0


04/26  1:11:48: LogSense: pd=00, cdb=28 00 03 c6 50 98 00 00 01 00 


04/26  1:11:48:           sense=f0 00 03 03 c6 50 98 0a 1a 08 49 64 11 00 00 00 00 a5 


04/26 �€���€��: REC: MedErr on LD[0] BadLba=3c65098


04/26  1:11:48: prRecCallback: Error, cdb=28, pd=00, err=2


04/26  1:11:48: PatrolReadStart: Log MedErr pd=01 errLBA=3c65098 in NVRAM_EVENT_LOG


04/26  1:11:53: REC:log MedErr on pd[1] #retries=0


04/26  1:11:53: LogSense: pd=01, cdb=2f 00 03 c6 50 99 00 ff ff 00 


04/26  1:11:53:           sense=f0 00 03 03 c6 64 d8 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  1:11:53: REC: MedErr on LD[1] BadLba=3c664d8


04/26  1:11:53: prCallback: Medium Error on pd=01, StartLba=3c65099, ErrLba=3c664d8


04/26  1:11:53: prRecQueue: starting pd=01 recovery - blocking host commands


04/26  1:11:53: prRecGo: Ready to attempt recovery errLBA=3c664d8 on pd=01


04/26  1:11:53: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/26  1:11:53: prRecGo: dataErr found on ld 0 span 0 arm 1


04/26  1:11:53: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78cc9, stripe=78cc9, refBlk=58, type=1


04/26  1:11:53: prRecGo: RAID1 - read from peer pd=00


04/26  1:11:55: REC:log MedErr on pd[0] #retries=0


04/26  1:11:55: LogSense: pd=00, cdb=28 00 03 c6 64 d8 00 00 01 00 


04/26  1:11:55:           sense=f0 00 03 03 c6 64 d8 0a 18 a8 49 65 11 00 00 00 00 a5 


04/26  1:11:55: REC: MedErr on LD[0] BadLba=3c664d8


04/26  1:11:55: prRecCallback: Error, cdb=28, pd=00, err=2


04/26  1:11:55: PatrolReadStart: Log MedErr pd=01 errLBA=3c664d8 in NVRAM_EVENT_LOG


04/26  2:37:51: prCallback: PR completed for pd=00


04/26  2:37:51: prDiskDone: finishing PR on PD=00


04/26  2:38:15: prCallback: PR completed for pd=01


04/26  2:38:15: prDiskDone: finishing PR on PD=01


04/26  4:51:44: prCallback: PR completed for pd=14


04/26  4:51:44: prDiskDone: finishing PR on PD=14


04/26  4:51:44: prCallback: PR completed for pd=12


04/26  4:51:44: prDiskDone: finishing PR on PD=12


04/26  4:51:44: prCallback: PR completed for pd=13


04/26  4:51:44: prDiskDone: finishing PR on PD=13


04/26  4:51:44: PR cycle complete


04/26  4:51:45: prDiskStart: starting Patrol Read on PD=00


04/26  4:51:45: prDiskStart: starting Patrol Read on PD=01


04/26  4:51:45: prDiskStart: starting Patrol Read on PD=12


04/26  4:51:45: prDiskStart: starting Patrol Read on PD=13


04/26  4:51:45: prDiskStart: starting Patrol Read on PD=14


04/26  4:51:45: Next PR scheduled to start at 04/26  8:00:00


04/26  5:59:02: REC:log MedErr on pd[1] #retries=0


04/26  5:59:02: LogSense: pd=01, cdb=2f 00 03 b1 31 cf 00 ff ff 00 


04/26  5:59:02:           sense=f0 00 03 03 b1 84 6e 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  5:59:02: REC: MedErr on LD[1] BadLba=3b1846e


04/26  5:59:02: prCallback: Medium Error on pd=01, StartLba=3b131cf, ErrLba=3b1846e


04/26  5:59:02: prRecQueue: starting pd=01 recovery - blocking host commands


04/26  5:59:02: prRecGo: Ready to attempt recovery errLBA=3b1846e on pd=01


04/26  5:59:02: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/26  5:59:02: prRecGo: dataErr found on ld 0 span 0 arm 1


04/26  5:59:02: prRecGo: data NOT in cache; cacheLn=ffffffff, row=76308, stripe=76308, refBlk=6e, type=1


04/26  5:59:02: prRecGo: RAID1 - read from peer pd=00


04/26  5:59:03: REC:log MedErr on pd[0] #retries=0


04/26  5:59:03: LogSense: pd=00, cdb=28 00 03 b1 84 6e 00 00 01 00 


04/26  5:59:03:           sense=f0 00 03 03 b1 84 6e 0a 1a 20 47 b4 11 0b 00 00 00 79 


04/26  5:59:03: REC: MedErr on LD[0] BadLba=3b1846e


04/26  5:59:03: prRecCallback: Error, cdb=28, pd=00, err=2


04/26  5:59:03: PatrolReadStart: Log MedErr pd=01 errLBA=3b1846e in NVRAM_EVENT_LOG


04/26  5:59:12: REC:log MedErr on pd[1] #retries=0


04/26  5:59:12: LogSense: pd=01, cdb=2f 00 03 b1 84 af 00 ff ff 00 


04/26  5:59:12:           sense=f0 00 03 03 b1 98 ee 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  5:59:12: REC: MedErr on LD[1] BadLba=3b198ee


04/26  5:59:12: prCallback: Medium Error on pd=01, StartLba=3b184af, ErrLba=3b198ee


04/26  5:59:12: prRecQueue: starting pd=01 recovery - blocking host commands


04/26  5:59:12: prRecGo: Ready to attempt recovery errLBA=3b198ee on pd=01


04/26  5:59:12: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/26  5:59:12: prRecGo: dataErr found on ld 0 span 0 arm 1


04/26  5:59:12: prRecGo: data NOT in cache; cacheLn=ffffffff, row=76331, stripe=76331, refBlk=6e, type=1


04/26  5:59:12: prRecGo: RAID1 - read from peer pd=00


04/26  5:59:13: REC:log MedErr on pd[0] #retries=0


04/26  5:59:13: LogSense: pd=00, cdb=28 00 03 b1 98 ee 00 00 01 00 


04/26  5:59:13:           sense=f0 00 03 03 b1 98 ee 0a 18 c0 47 b5 11 0b 00 00 00 79 


04/26  5:59:13: REC: MedErr on LD[0] BadLba=3b198ee


04/26  5:59:13: prRecCallback: Error, cdb=28, pd=00, err=2


04/26  5:59:13: PatrolReadStart: Log MedErr pd=01 errLBA=3b198ee in NVRAM_EVENT_LOG


04/26  5:59:18: REC:log MedErr on pd[1] #retries=0


04/26  5:59:18: LogSense: pd=01, cdb=2f 00 03 b1 98 ef 00 ff ff 00 


04/26  5:59:18:           sense=f0 00 03 03 b1 9b 4e 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  5:59:18: REC: MedErr on LD[1] BadLba=3b19b4e


04/26  5:59:18: prCallback: Medium Error on pd=01, StartLba=3b198ef, ErrLba=3b19b4e


04/26  5:59:18: prRecQueue: starting pd=01 recovery - blocking host commands


04/26  5:59:18: prRecGo: Ready to attempt recovery errLBA=3b19b4e on pd=01


04/26  5:59:18: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/26  5:59:18: prRecGo: dataErr found on ld 0 span 0 arm 1


04/26  5:59:18: prRecGo: data NOT in cache; cacheLn=ffffffff, row=76336, stripe=76336, refBlk=4e, type=1


04/26  5:59:18: prRecGo: RAID1 - read from peer pd=00


04/26  5:59:19: REC:log MedErr on pd[0] #retries=0


04/26  5:59:19: LogSense: pd=00, cdb=28 00 03 b1 9b 4e 00 00 01 00 


04/26  5:59:19:           sense=f0 00 03 03 b1 9b 4e 0a 18 20 47 b6 11 0b 00 00 00 79 


04/26  5:59:19: REC: MedErr on LD[0] BadLba=3b19b4e


04/26  5:59:19: prRecCallback: Error, cdb=28, pd=00, err=2


04/26  5:59:19: PatrolReadStart: Log MedErr pd=01 errLBA=3b19b4e in NVRAM_EVENT_LOG


04/26  5:59:54: REC:log MedErr on pd[0] #retries=0


04/26  5:59:54: LogSense: pd=00, cdb=2f 00 03 b0 a1 d0 00 ff ff 00 


04/26  5:59:54:           sense=f0 00 03 03 b1 84 6e 0a 1a 20 47 b4 11 0b 00 00 00 79 


04/26  5:59:54: REC: MedErr on LD[0] BadLba=3b1846e


04/26  5:59:54: prCallback: Medium Error on pd=00, StartLba=3b0a1d0, ErrLba=3b1846e


04/26  5:59:54: prRecQueue: starting pd=00 recovery - blocking host commands


04/26  5:59:54: prRecGo: Ready to attempt recovery errLBA=3b1846e on pd=00


04/26  5:59:54: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/26  5:59:54: prRecGo: dataErr found on ld 0 span 0 arm 0


04/26  5:59:54: prRecGo: data NOT in cache; cacheLn=ffffffff, row=76308, stripe=76308, refBlk=6e, type=1


04/26  5:59:54: prRecGo: RAID1 - read from peer pd=01


04/26  5:59:55: REC:log MedErr on pd[1] #retries=0


04/26  5:59:55: LogSense: pd=01, cdb=28 00 03 b1 84 6e 00 00 01 00 


04/26  5:59:55:           sense=f0 00 03 03 b1 84 6e 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  5:59:55: REC: MedErr on LD[1] BadLba=3b1846e


04/26  5:59:55: prRecCallback: Error, cdb=28, pd=01, err=2


04/26  5:59:56: PatrolReadStart: Log MedErr pd=00 errLBA=3b1846e in NVRAM_EVENT_LOG


04/26  6:00:01: REC:log MedErr on pd[0] #retries=0


04/26  6:00:01: LogSense: pd=00, cdb=2f 00 03 b1 84 6f 00 ff ff 00 


04/26  6:00:01:           sense=f0 00 03 03 b1 98 ee 0a 18 c0 47 b5 11 0b 00 00 00 79 


04/26  6:00:01: REC: MedErr on LD[0] BadLba=3b198ee


04/26  6:00:01: prCallback: Medium Error on pd=00, StartLba=3b1846f, ErrLba=3b198ee


04/26  6:00:01: prRecQueue: starting pd=00 recovery - blocking host commands


04/26  6:00:01: prRecGo: Ready to attempt recovery errLBA=3b198ee on pd=00


04/26  6:00:01: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/26  6:00:01: prRecGo: dataErr found on ld 0 span 0 arm 0


04/26  6:00:01: prRecGo: data NOT in cache; cacheLn=ffffffff, row=76331, stripe=76331, refBlk=6e, type=1


04/26  6:00:01: prRecGo: RAID1 - read from peer pd=01


04/26  6:00:03: REC:log MedErr on pd[1] #retries=0


04/26  6:00:03: LogSense: pd=01, cdb=28 00 03 b1 98 ee 00 00 01 00 


04/26  6:00:03:           sense=f0 00 03 03 b1 98 ee 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  6:00:03: REC: MedErr on LD[1] BadLba=3b198ee


04/26  6:00:03: prRecCallback: Error, cdb=28, pd=01, err=2


04/26  6:00:03: PatrolReadStart: Log MedErr pd=00 errLBA=3b198ee in NVRAM_EVENT_LOG


04/26  6:00:07: REC:log MedErr on pd[0] #retries=0


04/26  6:00:07: LogSense: pd=00, cdb=2f 00 03 b1 98 ef 00 ff ff 00 


04/26  6:00:07:           sense=f0 00 03 03 b1 9b 4e 0a 18 20 47 b6 11 0b 00 00 00 79 


04/26  6:00:07: REC: MedErr on LD[0] BadLba=3b19b4e


04/26  6:00:07: prCallback: Medium Error on pd=00, StartLba=3b198ef, ErrLba=3b19b4e


04/26  6:00:07: prRecQueue: starting pd=00 recovery - blocking host commands


04/26  6:00:07: prRecGo: Ready to attempt recovery errLBA=3b19b4e on pd=00


04/26  6:00:07: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/26  6:00:07: prRecGo: dataErr found on ld 0 span 0 arm 0


04/26  6:00:07: prRecGo: data NOT in cache; cacheLn=ffffffff, row=76336, stripe=76336, refBlk=4e, type=1


04/26  6:00:08: prRecGo: RAID1 - read from peer pd=01


04/26  6:00:09: REC:log MedErr on pd[1] #retries=0


04/26  6:00:09: LogSense: pd=01, cdb=28 00 03 b1 9b 4e 00 00 01 00 


04/26  6:00:09:           sense=f0 00 03 03 b1 9b 4e 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  6:00:09: REC: MedErr on LD[1] BadLba=3b19b4e


04/26  6:00:09: prRecCallback: Error, cdb=28, pd=01, err=2


04/26  6:00:09: PatrolReadStart: Log MedErr pd=00 errLBA=3b19b4e in NVRAM_EVENT_LOG


04/26  6:00:18: REC:log MedErr on pd[1] #retries=0


04/26  6:00:18: LogSense: pd=01, cdb=2f 00 03 bc ab 44 00 ff ff 00 


04/26  6:00:18:           sense=f0 00 03 03 bd 89 65 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  6:00:18: REC: MedErr on LD[1] BadLba=3bd8965


04/26  6:00:19: prCallback: Medium Error on pd=01, StartLba=3bcab44, ErrLba=3bd8965


04/26  6:00:19: prRecQueue: starting pd=01 recovery - blocking host commands


04/26  6:00:19: prRecGo: Ready to attempt recovery errLBA=3bd8965 on pd=01


04/26  6:00:19: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/26  6:00:19: prRecGo: dataErr found on ld 0 span 0 arm 1


04/26  6:00:19: prRecGo: data NOT in cache; cacheLn=ffffffff, row=77b12, stripe=77b12, refBlk=65, type=1


04/26  6:00:19: prRecGo: RAID1 - read from peer pd=00


04/26  6:00:19: REC:log MedErr on pd[0] #retries=0


04/26  6:00:19: LogSense: pd=00, cdb=28 00 03 bd 89 65 00 00 01 00 


04/26  6:00:19:           sense=f0 00 03 03 bd 89 65 0a 11 d4 48 ad 11 0b 00 00 00 79 


04/26  6:00:19: REC: MedErr on LD[0] BadLba=3bd8965


04/26  6:00:20: prRecCallback: Error, cdb=28, pd=00, err=2


04/26  6:00:20: PatrolReadStart: Log MedErr pd=01 errLBA=3bd8965 in NVRAM_EVENT_LOG


04/26  6:00:25: REC:log MedErr on pd[1] #retries=0


04/26  6:00:25: LogSense: pd=01, cdb=2f 00 03 bd 89 66 00 ff ff 00 


04/26  6:00:25:           sense=f0 00 03 03 bd 93 b9 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  6:00:25: REC: MedErr on LD[1] BadLba=3bd93b9


04/26  6:00:25: prCallback: Medium Error on pd=01, StartLba=3bd8966, ErrLba=3bd93b9


04/26  6:00:25: prRecQueue: starting pd=01 recovery - blocking host commands


04/26  6:00:25: prRecGo: Ready to attempt recovery errLBA=3bd93b9 on pd=01


04/26  6:00:25: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/26  6:00:25: prRecGo: dataErr found on ld 0 span 0 arm 1


04/26  6:00:25: prRecGo: data NOT in cache; cacheLn=ffffffff, row=77b27, stripe=77b27, refBlk=39, type=1


04/26  6:00:25: prRecGo: RAID1 - read from peer pd=00


04/26  6:00:26: REC:log MedErr on pd[0] #retries=0


04/26  6:00:26: LogSense: pd=00, cdb=28 00 03 bd 93 b9 00 00 01 00 


04/26  6:00:26:           sense=f0 00 03 03 bd 93 b9 0a 13 02 48 ae 11 0b 00 00 00 79 


04/26  6:00:26: REC: MedErr on LD[0] BadLba=3bd93b9


04/26  6:00:26: prRecCallback: Error, cdb=28, pd=00, err=2


04/26  6:00:26: PatrolReadStart: Log MedErr pd=01 errLBA=3bd93b9 in NVRAM_EVENT_LOG


04/26  6:00:35: REC:log MedErr on pd[1] #retries=0


04/26  6:00:35: LogSense: pd=01, cdb=2f 00 03 be 93 b9 00 ff ff 00 


04/26  6:00:35:           sense=f0 00 03 03 bf 04 f4 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  6:00:35: REC: MedErr on LD[1] BadLba=3bf04f4


04/26  6:00:35: prCallback: Medium Error on pd=01, StartLba=3be93b9, ErrLba=3bf04f4


04/26  6:00:35: prRecQueue: starting pd=01 recovery - blocking host commands


04/26  6:00:35: prRecGo: Ready to attempt recovery errLBA=3bf04f4 on pd=01


04/26  6:00:35: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/26  6:00:35: prRecGo: dataErr found on ld 0 span 0 arm 1


04/26  6:00:35: prRecGo: data NOT in cache; cacheLn=ffffffff, row=77e09, stripe=77e09, refBlk=74, type=1


04/26  6:00:35: prRecGo: RAID1 - read from peer pd=00


04/26  6:00:37: REC:log MedErr on pd[0] #retries=0


04/26  6:00:37: LogSense: pd=00, cdb=28 00 03 bf 04 f4 00 00 01 00 


04/26  6:00:37:           sense=f0 00 03 03 bf 04 f4 0a 12 df 48 cc 11 00 00 00 00 a5 


04/26  6:00:37: REC: MedErr on LD[0] BadLba=3bf04f4


04/26  6:00:37: prRecCallback: Error, cdb=28, pd=00, err=2


04/26  6:00:37: PatrolReadStart: Log MedErr pd=01 errLBA=3bf04f4 in NVRAM_EVENT_LOG


04/26  6:00:42: REC:log MedErr on pd[1] #retries=0


04/26  6:00:42: LogSense: pd=01, cdb=2f 00 03 bf 04 f5 00 ff ff 00 


04/26  6:00:42:           sense=f0 00 03 03 bf 83 6f 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  6:00:42: REC: MedErr on LD[1] BadLba=3bf836f


04/26  6:00:42: prCallback: Medium Error on pd=01, StartLba=3bf04f5, ErrLba=3bf836f


04/26  6:00:42: prRecQueue: starting pd=01 recovery - blocking host commands


04/26  6:00:42: prRecGo: Ready to attempt recovery errLBA=3bf836f on pd=01


04/26  6:00:42: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/26  6:00:42: prRecGo: dataErr found on ld 0 span 0 arm 1


04/26  6:00:42: prRecGo: data NOT in cache; cacheLn=ffffffff, row=77f06, stripe=77f06, refBlk=6f, type=1


04/26  6:00:42: prRecGo: RAID1 - read from peer pd=00


04/26  6:00:45: REC:log MedErr on pd[0] #retries=0


04/26  6:00:45: LogSense: pd=00, cdb=28 00 03 bf 83 6f 00 00 01 00 


04/26  6:00:45:           sense=f0 00 03 03 bf 83 6f 0a 00 01 48 d6 11 0b 00 00 01 ab 


04/26  6:00:45: REC: MedErr on LD[0] BadLba=3bf836f


04/26  6:00:45: prRecCallback: Error, cdb=28, pd=00, err=2


04/26  6:00:45: PatrolReadStart: Log MedErr pd=01 errLBA=3bf836f in NVRAM_EVENT_LOG


04/26  6:00:50: REC:log MedErr on pd[1] #retries=0


04/26  6:00:50: LogSense: pd=01, cdb=2f 00 03 bf 83 70 00 ff ff 00 


04/26  6:00:50:           sense=f0 00 03 03 c0 59 2f 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  6:00:50: REC: MedErr on LD[1] BadLba=3c0592f


04/26  6:00:50: prCallback: Medium Error on pd=01, StartLba=3bf8370, ErrLba=3c0592f


04/26  6:00:51: prRecQueue: starting pd=01 recovery - blocking host commands


04/26  6:00:51: prRecGo: Ready to attempt recovery errLBA=3c0592f on pd=01


04/26  6:00:51: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/26  6:00:51: prRecGo: dataErr found on ld 0 span 0 arm 1


04/26  6:00:51: prRecGo: data NOT in cache; cacheLn=ffffffff, row=780b2, stripe=780b2, refBlk=2f, type=1


04/26  6:00:51: prRecGo: RAID1 - read from peer pd=00


04/26  6:00:52: REC:log MedErr on pd[0] #retries=0


04/26  6:00:52: LogSense: pd=00, cdb=28 00 03 c0 59 2f 00 00 01 00 


04/26  6:00:52:           sense=f0 00 03 03 c0 59 2f 0a 19 5f 48 e8 11 00 00 00 00 a5 


04/26  6:00:52: REC: MedErr on LD[0] BadLba=3c0592f


04/26  6:00:52: prRecCallback: Error, cdb=28, pd=00, err=2


04/26  6:00:52: PatrolReadStart: Log MedErr pd=01 errLBA=3c0592f in NVRAM_EVENT_LOG


04/26  6:00:57: REC:log MedErr on pd[1] #retries=0


04/26  6:00:57: LogSense: pd=01, cdb=2f 00 03 c0 59 30 00 ff ff 00 


04/26  6:00:57:           sense=f0 00 03 03 c0 70 6f 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  6:00:57: REC: MedErr on LD[1] BadLba=3c0706f


04/26  6:00:57: prCallback: Medium Error on pd=01, StartLba=3c05930, ErrLba=3c0706f


04/26  6:00:57: prRecQueue: starting pd=01 recovery - blocking host commands


04/26  6:00:57: prRecGo: Ready to attempt recovery errLBA=3c0706f on pd=01


04/26  6:00:57: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/26  6:00:57: prRecGo: dataErr found on ld 0 span 0 arm 1


04/26  6:00:57: prRecGo: data NOT in cache; cacheLn=ffffffff, row=780e0, stripe=780e0, refBlk=6f, type=1


04/26  6:00:57: prRecGo: RAID1 - read from peer pd=00


04/26  6:00:59: REC:log MedErr on pd[0] #retries=0


04/26  6:00:59: LogSense: pd=00, cdb=28 00 03 c0 70 6f 00 00 01 00 


04/26  6:00:59:           sense=f0 00 03 03 c0 70 6f 0a 1a ff 48 e9 11 00 00 00 00 a5 


04/26  6:00:59: REC: MedErr on LD[0] BadLba=3c0706f


04/26  6:00:59: prRecCallback: Error, cdb=28, pd=00, err=2


04/26  6:00:59: PatrolReadStart: Log MedErr pd=01 errLBA=3c0706f in NVRAM_EVENT_LOG


04/26  6:01:04: REC:log MedErr on pd[1] #retries=0


04/26  6:01:04: LogSense: pd=01, cdb=2f 00 03 c0 70 70 00 ff ff 00 


04/26  6:01:04:           sense=f0 00 03 03 c0 72 cf 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  6:01:04: REC: MedErr on LD[1] BadLba=3c072cf


04/26  6:01:04: prCallback: Medium Error on pd=01, StartLba=3c07070, ErrLba=3c072cf


04/26  6:01:04: prRecQueue: starting pd=01 recovery - blocking host commands


04/26  6:01:04: prRecGo: Ready to attempt recovery errLBA=3c072cf on pd=01


04/26  6:01:04: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/26  6:01:04: prRecGo: dataErr found on ld 0 span 0 arm 1


04/26  6:01:04: prRecGo: data NOT in cache; cacheLn=ffffffff, row=780e5, stripe=780e5, refBlk=4f, type=1


04/26  6:01:04: prRecGo: RAID1 - read from peer pd=00


04/26  6:01:06: REC:log MedErr on pd[0] #retries=0


04/26  6:01:06: LogSense: pd=00, cdb=28 00 03 c0 72 cf 00 00 01 00 


04/26  6:01:06:           sense=f0 00 03 03 c0 72 cf 0a 1a 5f 48 ea 11 00 00 00 00 a5 


04/26  6:01:06: REC: MedErr on LD[0] BadLba=3c072cf


04/26  6:01:06: prRecCallback: Error, cdb=28, pd=00, err=2


04/26  6:01:06: PatrolReadStart: Log MedErr pd=01 errLBA=3c072cf in NVRAM_EVENT_LOG


04/26  6:01:11: REC:log MedErr on pd[1] #retries=0


04/26  6:01:11: LogSense: pd=01, cdb=2f 00 03 c0 72 d0 00 ff ff 00 


04/26  6:01:11:           sense=f0 00 03 03 c0 87 de 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  6:01:11: REC: MedErr on LD[1] BadLba=3c087de


04/26  6:01:11: prCallback: Medium Error on pd=01, StartLba=3c072d0, ErrLba=3c087de


04/26  6:01:11: prRecQueue: starting pd=01 recovery - blocking host commands


04/26  6:01:11: prRecGo: Ready to attempt recovery errLBA=3c087de on pd=01


04/26  6:01:11: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/26  6:01:11: prRecGo: dataErr found on ld 0 span 0 arm 1


04/26  6:01:11: prRecGo: data NOT in cache; cacheLn=ffffffff, row=7810f, stripe=7810f, refBlk=5e, type=1


04/26  6:01:11: prRecGo: RAID1 - read from peer pd=00


04/26  6:01:13: REC:log MedErr on pd[0] #retries=0


04/26  6:01:13: LogSense: pd=00, cdb=28 00 03 c0 87 de 00 00 01 00 


04/26  6:01:13:           sense=f0 00 03 03 c0 87 de 0a 19 ce 48 eb 11 00 00 00 00 a5 


04/26  6:01:13: REC: MedErr on LD[0] BadLba=3c087de


04/26  6:01:13: prRecCallback: Error, cdb=28, pd=00, err=2


04/26  6:01:13: PatrolReadStart: Log MedErr pd=01 errLBA=3c087de in NVRAM_EVENT_LOG


04/26  6:01:15: REC:log MedErr on pd[0] #retries=0


04/26  6:01:15: LogSense: pd=00, cdb=2f 00 03 bc ab c4 00 ff ff 00 


04/26  6:01:15:           sense=f0 00 03 03 bd 89 65 0a 11 d4 48 ad 11 0b 00 00 00 79 


04/26  6:01:15: REC: MedErr on LD[0] BadLba=3bd8965


04/26  6:01:15: prCallback: Medium Error on pd=00, StartLba=3bcabc4, ErrLba=3bd8965


04/26  6:01:15: prRecQueue: starting pd=00 recovery - blocking host commands


04/26  6:01:15: prRecGo: Ready to attempt recovery errLBA=3bd8965 on pd=00


04/26  6:01:15: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/26  6:01:15: prRecGo: dataErr found on ld 0 span 0 arm 0


04/26  6:01:15: prRecGo: data NOT in cache; cacheLn=ffffffff, row=77b12, stripe=77b12, refBlk=65, type=1


04/26  6:01:15: prRecGo: RAID1 - read from peer pd=01


04/26  6:01:16: REC:log MedErr on pd[1] #retries=0


04/26  6:01:16: LogSense: pd=01, cdb=28 00 03 bd 89 65 00 00 01 00 


04/26  6:01:16:           sense=f0 00 03 03 bd 89 65 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  6:01:16: REC: MedErr on LD[1] BadLba=3bd8965


04/26  6:01:16: prRecCallback: Error, cdb=28, pd=01, err=2


04/26  6:01:16: PatrolReadStart: Log MedErr pd=00 errLBA=3bd8965 in NVRAM_EVENT_LOG


04/26  6:01:18: REC:log MedErr on pd[1] #retries=0


04/26  6:01:18: LogSense: pd=01, cdb=2f 00 03 c0 87 df 00 ff ff 00 


04/26  6:01:18:           sense=f0 00 03 03 c0 8a 3e 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  6:01:18: REC: MedErr on LD[1] BadLba=3c08a3e


04/26  6:01:18: prCallback: Medium Error on pd=01, StartLba=3c087df, ErrLba=3c08a3e


04/26  6:01:18: prRecQueue: starting pd=01 recovery - blocking host commands


04/26  6:01:18: prRecGo: Ready to attempt recovery errLBA=3c08a3e on pd=01


04/26  6:01:18: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/26  6:01:18: prRecGo: dataErr found on ld 0 span 0 arm 1


04/26  6:01:18: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78114, stripe=78114, refBlk=3e, type=1


04/26  6:01:18: prRecGo: RAID1 - read from peer pd=00


04/26  6:01:20: REC:log MedErr on pd[0] #retries=0


04/26  6:01:20: LogSense: pd=00, cdb=28 00 03 c0 8a 3e 00 00 01 00 


04/26  6:01:20:           sense=f0 00 03 03 c0 8a 3e 0a 19 2e 48 ec 11 00 00 00 00 a5 


04/26  6:01:20: REC: MedErr on LD[0] BadLba=3c08a3e


04/26  6:01:20: prRecCallback: Error, cdb=28, pd=00, err=2


04/26  6:01:20: PatrolReadStart: Log MedErr pd=01 errLBA=3c08a3e in NVRAM_EVENT_LOG


04/26  6:01:24: REC:log MedErr on pd[0] #retries=0


04/26  6:01:24: LogSense: pd=00, cdb=2f 00 03 bd 91 66 00 ff ff 00 


04/26  6:01:24:           sense=f0 00 03 03 bd 93 b9 0a 13 02 48 ae 11 0b 00 00 00 79 


04/26  6:01:24: REC: MedErr on LD[0] BadLba=3bd93b9


04/26  6:01:24: prCallback: Medium Error on pd=00, StartLba=3bd9166, ErrLba=3bd93b9


04/26  6:01:24: prRecQueue: starting pd=00 recovery - blocking host commands


04/26  6:01:24: prRecGo: Ready to attempt recovery errLBA=3bd93b9 on pd=00


04/26  6:01:24: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/26  6:01:24: prRecGo: dataErr found on ld 0 span 0 arm 0


04/26  6:01:24: prRecGo: data NOT in cache; cacheLn=ffffffff, row=77b27, stripe=77b27, refBlk=39, type=1


04/26  6:01:25: prRecGo: RAID1 - read from peer pd=01


04/26  6:01:25: REC:log MedErr on pd[1] #retries=0


04/26  6:01:25: LogSense: pd=01, cdb=2f 00 03 c0 8a 3f 00 ff ff 00 


04/26  6:01:25:           sense=f0 00 03 03 c0 a3 de 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  6:01:25: REC: MedErr on LD[1] BadLba=3c0a3de


04/26  6:01:25: prCallback: Medium Error on pd=01, StartLba=3c08a3f, ErrLba=3c0a3de


04/26  6:01:25: prRecQueue: adding pd=01 to recovery wait queue


04/26  6:01:27: REC:log MedErr on pd[1] #retries=0


04/26  6:01:27: LogSense: pd=01, cdb=28 00 03 bd 93 b9 00 00 01 00 


04/26  6:01:27:           sense=f0 00 03 03 bd 93 b9 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  6:01:27: REC: MedErr on LD[1] BadLba=3bd93b9


04/26  6:01:27: prRecCallback: Error, cdb=28, pd=01, err=2


04/26  6:01:27: PatrolReadStart: Log MedErr pd=00 errLBA=3bd93b9 in NVRAM_EVENT_LOG


04/26  6:01:27: prRecGo: Ready to attempt recovery errLBA=3c0a3de on pd=01


04/26  6:01:27: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/26  6:01:27: prRecGo: dataErr found on ld 0 span 0 arm 1


04/26  6:01:27: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78147, stripe=78147, refBlk=5e, type=1


04/26  6:01:27: prRecGo: RAID1 - read from peer pd=00


04/26  6:01:28: REC:log MedErr on pd[0] #retries=0


04/26  6:01:28: LogSense: pd=00, cdb=28 00 03 c0 a3 de 00 00 01 00 


04/26  6:01:28:           sense=f0 00 03 03 c0 a3 de 0a 1a 2e 48 ee 11 00 00 00 00 a5 


04/26  6:01:28: REC: MedErr on LD[0] BadLba=3c0a3de


04/26  6:01:28: prRecCallback: Error, cdb=28, pd=00, err=2


04/26  6:01:28: PatrolReadStart: Log MedErr pd=01 errLBA=3c0a3de in NVRAM_EVENT_LOG


04/26  6:01:33: REC:log MedErr on pd[1] #retries=0


04/26  6:01:33: LogSense: pd=01, cdb=2f 00 03 c0 a3 df 00 ff ff 00 


04/26  6:01:33:           sense=f0 00 03 03 c0 b8 1d 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  6:01:33: REC: MedErr on LD[1] BadLba=3c0b81d


04/26  6:01:33: prCallback: Medium Error on pd=01, StartLba=3c0a3df, ErrLba=3c0b81d


04/26  6:01:33: prRecQueue: starting pd=01 recovery - blocking host commands


04/26  6:01:33: prRecGo: Ready to attempt recovery errLBA=3c0b81d on pd=01


04/26  6:01:33: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/26  6:01:33: prRecGo: dataErr found on ld 0 span 0 arm 1


04/26  6:01:33: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78170, stripe=78170, refBlk=1d, type=1


04/26  6:01:33: prRecGo: RAID1 - read from peer pd=00


04/26  6:01:35: REC:log MedErr on pd[0] #retries=0


04/26  6:01:35: LogSense: pd=00, cdb=28 00 03 c0 b8 1d 00 00 01 00 


04/26  6:01:35:           sense=f0 00 03 03 c0 b8 1d 0a 18 cd 48 ef 11 00 00 00 00 a5 


04/26  6:01:35: REC: MedErr on LD[0] BadLba=3c0b81d


04/26  6:01:35: prRecCallback: Error, cdb=28, pd=00, err=2


04/26  6:01:35: PatrolReadStart: Log MedErr pd=01 errLBA=3c0b81d in NVRAM_EVENT_LOG


04/26  6:01:40: REC:log MedErr on pd[0] #retries=0


04/26  6:01:40: LogSense: pd=00, cdb=2f 00 03 be 9b b9 00 ff ff 00 


04/26  6:01:40:           sense=f0 00 03 03 bf 04 f4 0a 12 df 48 cc 11 00 00 00 00 a5 


04/26  6:01:40: REC: MedErr on LD[0] BadLba=3bf04f4


04/26  6:01:40: prCallback: Medium Error on pd=00, StartLba=3be9bb9, ErrLba=3bf04f4


04/26  6:01:40: prRecQueue: starting pd=00 recovery - blocking host commands


04/26  6:01:40: prRecGo: Ready to attempt recovery errLBA=3bf04f4 on pd=00


04/26  6:01:40: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/26  6:01:40: prRecGo: dataErr found on ld 0 span 0 arm 0


04/26  6:01:40: prRecGo: data NOT in cache; cacheLn=ffffffff, row=77e09, stripe=77e09, refBlk=74, type=1


04/26  6:01:40: prRecGo: RAID1 - read from peer pd=01


04/26  6:01:40: REC:log MedErr on pd[1] #retries=0


04/26  6:01:40: LogSense: pd=01, cdb=2f 00 03 c0 b8 1e 00 ff ff 00 


04/26  6:01:40:           sense=f0 00 03 03 c0 b8 1e 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  6:01:40: REC: MedErr on LD[1] BadLba=3c0b81e


04/26  6:01:40: prCallback: Medium Error on pd=01, StartLba=3c0b81e, ErrLba=3c0b81e


04/26  6:01:40: prRecQueue: adding pd=01 to recovery wait queue


04/26  6:01:42: REC:log MedErr on pd[1] #retries=0


04/26  6:01:42: LogSense: pd=01, cdb=28 00 03 bf 04 f4 00 00 01 00 


04/26  6:01:42:           sense=f0 00 03 03 bf 04 f4 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  6:01:42: REC: MedErr on LD[1] BadLba=3bf04f4


04/26  6:01:42: prRecCallback: Error, cdb=28, pd=01, err=2


04/26  6:01:42: PatrolReadStart: Log MedErr pd=00 errLBA=3bf04f4 in NVRAM_EVENT_LOG


04/26  6:01:42: prRecGo: Ready to attempt recovery errLBA=3c0b81e on pd=01


04/26  6:01:42: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/26  6:01:42: prRecGo: dataErr found on ld 0 span 0 arm 1


04/26  6:01:42: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78170, stripe=78170, refBlk=1e, type=1


04/26  6:01:42: prRecGo: RAID1 - read from peer pd=00


04/26  6:01:45: REC:log MedErr on pd[0] #retries=0


04/26  6:01:45: LogSense: pd=00, cdb=28 00 03 c0 b8 1e 00 00 01 00 


04/26  6:01:45:           sense=f0 00 03 03 c0 b8 1e 0a 18 ce 48 ef 11 00 00 00 01 c6 


04/26  6:01:45: REC: MedErr on LD[0] BadLba=3c0b81e


04/26  6:01:45: prRecCallback: Error, cdb=28, pd=00, err=2


04/26  6:01:45: PatrolReadStart: Log MedErr pd=01 errLBA=3c0b81e in NVRAM_EVENT_LOG


04/26  6:01:49: REC:log MedErr on pd[0] #retries=0


04/26  6:01:49: LogSense: pd=00, cdb=2f 00 03 bf 04 f5 00 ff ff 00 


04/26  6:01:49:           sense=f0 00 03 03 bf 83 6f 0a 00 01 48 d6 11 0b 00 00 01 ab 


04/26  6:01:49: REC: MedErr on LD[0] BadLba=3bf836f


04/26  6:01:49: prCallback: Medium Error on pd=00, StartLba=3bf04f5, ErrLba=3bf836f


04/26  6:01:49: prRecQueue: starting pd=00 recovery - blocking host commands


04/26  6:01:49: prRecGo: Ready to attempt recovery errLBA=3bf836f on pd=00


04/26  6:01:49: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/26  6:01:49: prRecGo: dataErr found on ld 0 span 0 arm 0


04/26  6:01:49: prRecGo: data NOT in cache; cacheLn=ffffffff, row=77f06, stripe=77f06, refBlk=6f, type=1


04/26  6:01:49: prRecGo: RAID1 - read from peer pd=01


04/26  6:01:50: REC:log MedErr on pd[1] #retries=0


04/26  6:01:50: LogSense: pd=01, cdb=2f 00 03 c0 b8 1f 00 ff ff 00 


04/26  6:01:50:           sense=f0 00 03 03 c0 ba 7e 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  6:01:50: REC: MedErr on LD[1] BadLba=3c0ba7e


04/26  6:01:50: prCallback: Medium Error on pd=01, StartLba=3c0b81f, ErrLba=3c0ba7e


04/26  6:01:50: prRecQueue: adding pd=01 to recovery wait queue


04/26  6:01:52: REC:log MedErr on pd[1] #retries=0


04/26  6:01:52: LogSense: pd=01, cdb=28 00 03 bf 83 6f 00 00 01 00 


04/26  6:01:52:           sense=f0 00 03 03 bf 83 6f 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  6:01:52: REC: MedErr on LD[1] BadLba=3bf836f


04/26  6:01:52: prRecCallback: Error, cdb=28, pd=01, err=2


04/26  6:01:52: PatrolReadStart: Log MedErr pd=00 errLBA=3bf836f in NVRAM_EVENT_LOG


04/26  6:01:52: prRecGo: Ready to attempt recovery errLBA=3c0ba7e on pd=01


04/26  6:01:52: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/26  6:01:52: prRecGo: dataErr found on ld 0 span 0 arm 1


04/26  6:01:52: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78174, stripe=78174, refBlk=7e, type=1


04/26  6:01:52: prRecGo: RAID1 - read from peer pd=00


04/26  6:01:53: REC:log MedErr on pd[0] #retries=0


04/26  6:01:53: LogSense: pd=00, cdb=28 00 03 c0 ba 7e 00 00 01 00 


04/26  6:01:53:           sense=f0 00 03 03 c0 ba 7e 0a 18 2e 48 f0 11 00 00 00 00 a5 


04/26  6:01:53: REC: MedErr on LD[0] BadLba=3c0ba7e


04/26  6:01:53: prRecCallback: Error, cdb=28, pd=00, err=2


04/26  6:01:53: PatrolReadStart: Log MedErr pd=01 errLBA=3c0ba7e in NVRAM_EVENT_LOG


04/26  6:01:57: REC:log MedErr on pd[0] #retries=0


04/26  6:01:57: LogSense: pd=00, cdb=2f 00 03 bf 83 70 00 ff ff 00 


04/26  6:01:57:           sense=f0 00 03 03 c0 59 2f 0a 19 5f 48 e8 11 00 00 00 00 a5 


04/26  6:01:57: REC: MedErr on LD[0] BadLba=3c0592f


04/26  6:01:57: prCallback: Medium Error on pd=00, StartLba=3bf8370, ErrLba=3c0592f


04/26  6:01:57: prRecQueue: starting pd=00 recovery - blocking host commands


04/26  6:01:57: prRecGo: Ready to attempt recovery errLBA=3c0592f on pd=00


04/26  6:01:57: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/26  6:01:57: prRecGo: dataErr found on ld 0 span 0 arm 0


04/26  6:01:57: prRecGo: data NOT in cache; cacheLn=ffffffff, row=780b2, stripe=780b2, refBlk=2f, type=1


04/26  6:01:57: prRecGo: RAID1 - read from peer pd=01


04/26  6:01:58: REC:log MedErr on pd[1] #retries=0


04/26  6:01:58: LogSense: pd=01, cdb=2f 00 03 c0 ba 7f 00 ff ff 00 


04/26  6:01:58:           sense=f0 00 03 03 c0 d1 be 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  6:01:58: REC: MedErr on LD[1] BadLba=3c0d1be


04/26  6:01:58: prCallback: Medium Error on pd=01, StartLba=3c0ba7f, ErrLba=3c0d1be


04/26  6:01:58: prRecQueue: adding pd=01 to recovery wait queue


04/26  6:02:00: REC:log MedErr on pd[1] #retries=0


04/26  6:02:00: LogSense: pd=01, cdb=28 00 03 c0 59 2f 00 00 01 00 


04/26  6:02:00:           sense=f0 00 03 03 c0 59 2f 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  6:02:00: REC: MedErr on LD[1] BadLba=3c0592f


04/26  6:02:00: prRecCallback: Error, cdb=28, pd=01, err=2


04/26  6:02:00: PatrolReadStart: Log MedErr pd=00 errLBA=3c0592f in NVRAM_EVENT_LOG


04/26  6:02:00: prRecGo: Ready to attempt recovery errLBA=3c0d1be on pd=01


04/26  6:02:00: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/26  6:02:00: prRecGo: dataErr found on ld 0 span 0 arm 1


04/26  6:02:00: prRecGo: data NOT in cache; cacheLn=ffffffff, row=781a3, stripe=781a3, refBlk=3e, type=1


04/26  6:02:00: prRecGo: RAID1 - read from peer pd=00


04/26  6:02:01: REC:log MedErr on pd[0] #retries=0


04/26  6:02:01: LogSense: pd=00, cdb=28 00 03 c0 d1 be 00 00 01 00 


04/26  6:02:01:           sense=f0 00 03 03 c0 d1 be 0a 19 ce 48 f1 11 00 00 00 00 a5 


04/26  6:02:01: REC: MedErr on LD[0] BadLba=3c0d1be


04/26  6:02:01: prRecCallback: Error, cdb=28, pd=00, err=2


04/26  6:02:01: PatrolReadStart: Log MedErr pd=01 errLBA=3c0d1be in NVRAM_EVENT_LOG


04/26  6:02:06: REC:log MedErr on pd[0] #retries=0


04/26  6:02:06: LogSense: pd=00, cdb=2f 00 03 c0 59 30 00 ff ff 00 


04/26  6:02:06:           sense=f0 00 03 03 c0 70 6f 0a 1a ff 48 e9 11 00 00 00 00 a5 


04/26  6:02:06: REC: MedErr on LD[0] BadLba=3c0706f


04/26  6:02:06: prCallback: Medium Error on pd=00, StartLba=3c05930, ErrLba=3c0706f


04/26  6:02:06: prRecQueue: starting pd=00 recovery - blocking host commands


04/26  6:02:06: prRecGo: Ready to attempt recovery errLBA=3c0706f on pd=00


04/26  6:02:06: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/26  6:02:06: prRecGo: dataErr found on ld 0 span 0 arm 0


04/26  6:02:06: prRecGo: data NOT in cache; cacheLn=ffffffff, row=780e0, stripe=780e0, refBlk=6f, ty�����€��/26  6:02:06: prRecGo: RAID1 - read from peer pd=01


04/26  6:02:06: REC:log MedErr on pd[1] #retries=0


04/26  6:02:06: LogSense: pd=01, cdb=2f 00 03 c0 d1 bf 00 ff ff 00 


04/26  6:02:06:           sense=f0 00 03 03 c0 eb 53 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  6:02:06: REC: MedErr on LD[1] BadLba=3c0eb53


04/26  6:02:06: prCallback: Medium Error on pd=01, StartLba=3c0d1bf, ErrLba=3c0eb53


04/26  6:02:06: prRecQueue: adding pd=01 to recovery wait queue


04/26  6:02:08: REC:log MedErr on pd[1] #retries=0


04/26  6:02:08: LogSense: pd=01, cdb=28 00 03 c0 70 6f 00 00 01 00 


04/26  6:02:08:           sense=f0 00 03 03 c0 70 6f 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  6:02:08: REC: MedErr on LD[1] BadLba=3c0706f


04/26  6:02:08: prRecCallback: Error, cdb=28, pd=01, err=2


04/26  6:02:08: PatrolReadStart: Log MedErr pd=00 errLBA=3c0706f in NVRAM_EVENT_LOG


04/26  6:02:08: prRecGo: Ready to attempt recovery errLBA=3c0eb53 on pd=01


04/26  6:02:08: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/26  6:02:08: prRecGo: dataErr found on ld 0 span 0 arm 1


04/26  6:02:08: prRecGo: data NOT in cache; cacheLn=ffffffff, row=781d6, stripe=781d6, refBlk=53, type=1


04/26  6:02:08: prRecGo: RAID1 - read from peer pd=00


04/26  6:02:10: REC:log MedErr on pd[0] #retries=0


04/26  6:02:10: LogSense: pd=00, cdb=28 00 03 c0 eb 53 00 00 01 00 


04/26  6:02:10:           sense=f0 00 03 03 c0 eb 53 0a 1a c3 48 f3 11 00 00 00 00 a5 


04/26  6:02:10: REC: MedErr on LD[0] BadLba=3c0eb53


04/26  6:02:10: prRecCallback: Error, cdb=28, pd=00, err=2


04/26  6:02:10: PatrolReadStart: Log MedErr pd=01 errLBA=3c0eb53 in NVRAM_EVENT_LOG


04/26  6:02:13: REC:log MedErr on pd[0] #retries=0


04/26  6:02:13: LogSense: pd=00, cdb=2f 00 03 c0 70 70 00 ff ff 00 


04/26  6:02:13:           sense=f0 00 03 03 c0 72 cf 0a 1a 5f 48 ea 11 00 00 00 00 a5 


04/26  6:02:13: REC: MedErr on LD[0] BadLba=3c072cf


04/26  6:02:13: prCallback: Medium Error on pd=00, StartLba=3c07070, ErrLba=3c072cf


04/26  6:02:13: prRecQueue: starting pd=00 recovery - blocking host commands


04/26  6:02:13: prRecGo: Ready to attempt recovery errLBA=3c072cf on pd=00


04/26  6:02:13: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/26  6:02:13: prRecGo: dataErr found on ld 0 span 0 arm 0


04/26  6:02:13: prRecGo: data NOT in cache; cacheLn=ffffffff, row=780e5, stripe=780e5, refBlk=4f, type=1


04/26  6:02:13: prRecGo: RAID1 - read from peer pd=01


04/26  6:02:15: REC:log MedErr on pd[1] #retries=0


04/26  6:02:15: LogSense: pd=01, cdb=28 00 03 c0 72 cf 00 00 01 00 


04/26  6:02:15:           sense=f0 00 03 03 c0 72 cf 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  6:02:15: REC: MedErr on LD[1] BadLba=3c072cf


04/26  6:02:15: prRecCallback: Error, cdb=28, pd=01, err=2


04/26  6:02:15: PatrolReadStart: Log MedErr pd=00 errLBA=3c072cf in NVRAM_EVENT_LOG


04/26  6:02:20: REC:log MedErr on pd[1] #retries=0


04/26  6:02:20: LogSense: pd=01, cdb=2f 00 03 c0 f3 54 00 ff ff 00 


04/26  6:02:20:           sense=f0 00 03 03 c1 01 f3 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  6:02:20: REC: MedErr on LD[1] BadLba=3c101f3


04/26  6:02:20: prCallback: Medium Error on pd=01, StartLba=3c0f354, ErrLba=3c101f3


04/26  6:02:20: prRecQueue: starting pd=01 recovery - blocking host commands


04/26  6:02:20: prRecGo: Ready to attempt recovery errLBA=3c101f3 on pd=01


04/26  6:02:20: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/26  6:02:20: prRecGo: dataErr found on ld 0 span 0 arm 1


04/26  6:02:20: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78203, stripe=78203, refBlk=73, type=1


04/26  6:02:20: prRecGo: RAID1 - read from peer pd=00


04/26  6:02:21: REC:log MedErr on pd[0] #retries=0


04/26  6:02:21: LogSense: pd=00, cdb=2f 00 03 c0 72 d0 00 ff ff 00 


04/26  6:02:21:           sense=f0 00 03 03 c0 87 de 0a 19 ce 48 eb 11 00 00 00 00 a5 


04/26  6:02:21: REC: MedErr on LD[0] BadLba=3c087de


04/26  6:02:21: prCallback: Medium Error on pd=00, StartLba=3c072d0, ErrLba=3c087de


04/26  6:02:21: prRecQueue: adding pd=00 to recovery wait queue


04/26  6:02:22: REC:log MedErr on pd[0] #retries=0


04/26  6:02:22: LogSense: pd=00, cdb=28 00 03 c1 01 f3 00 00 01 00 


04/26  6:02:22:           sense=f0 00 03 03 c1 01 f3 0a 18 c3 48 f5 11 00 00 00 00 a5 


04/26  6:02:22: REC: MedErr on LD[0] BadLba=3c101f3


04/26  6:02:22: prRecCallback: Error, cdb=28, pd=00, err=2


04/26  6:02:22: PatrolReadStart: Log MedErr pd=01 errLBA=3c101f3 in NVRAM_EVENT_LOG


04/26  6:02:22: prRecGo: Ready to attempt recovery errLBA=3c087de on pd=00


04/26  6:02:22: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/26  6:02:22: prRecGo: dataErr found on ld 0 span 0 arm 0


04/26  6:02:22: prRecGo: data NOT in cache; cacheLn=ffffffff, row=7810f, stripe=7810f, refBlk=5e, type=1


04/26  6:02:22: prRecGo: RAID1 - read from peer pd=01


04/26  6:02:24: REC:log MedErr on pd[1] #retries=0


04/26  6:02:24: LogSense: pd=01, cdb=28 00 03 c0 87 de 00 00 01 00 


04/26  6:02:24:           sense=f0 00 03 03 c0 87 de 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  6:02:24: REC: MedErr on LD[1] BadLba=3c087de


04/26  6:02:24: prRecCallback: Error, cdb=28, pd=01, err=2


04/26  6:02:24: PatrolReadStart: Log MedErr pd=00 errLBA=3c087de in NVRAM_EVENT_LOG


04/26  6:02:27: REC:log MedErr on pd[1] #retries=0


04/26  6:02:27: LogSense: pd=01, cdb=2f 00 03 c1 01 f4 00 ff ff 00 


04/26  6:02:27:           sense=f0 00 03 03 c1 04 53 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  6:02:27: REC: MedErr on LD[1] BadLba=3c10453


04/26  6:02:27: prCallback: Medium Error on pd=01, StartLba=3c101f4, ErrLba=3c10453


04/26  6:02:27: prRecQueue: starting pd=01 recovery - blocking host commands


04/26  6:02:27: prRecGo: Ready to attempt recovery errLBA=3c10453 on pd=01


04/26  6:02:27: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/26  6:02:27: prRecGo: dataErr found on ld 0 span 0 arm 1


04/26  6:02:27: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78208, stripe=78208, refBlk=53, type=1


04/26  6:02:27: prRecGo: RAID1 - read from peer pd=00


04/26  6:02:29: REC:log MedErr on pd[0] #retries=0


04/26  6:02:29: LogSense: pd=00, cdb=28 00 03 c1 04 53 00 00 01 00 


04/26  6:02:29:           sense=f0 00 03 03 c1 04 53 0a 18 23 48 f6 11 00 00 00 00 a5 


04/26  6:02:29: REC: MedErr on LD[0] BadLba=3c10453


04/26  6:02:29: prRecCallback: Error, cdb=28, pd=00, err=2


04/26  6:02:29: PatrolReadStart: Log MedErr pd=01 errLBA=3c10453 in NVRAM_EVENT_LOG


04/26  6:02:30: REC:log MedErr on pd[0] #retries=0


04/26  6:02:30: LogSense: pd=00, cdb=2f 00 03 c0 87 df 00 08 00 00 


04/26  6:02:30:           sense=f0 00 03 03 c0 8a 3e 0a 19 2e 48 ec 11 00 00 00 00 a5 


04/26  6:02:30: REC: MedErr on LD[0] BadLba=3c08a3e


04/26  6:02:30: prCallback: Medium Error on pd=00, StartLba=3c087df, ErrLba=3c08a3e


04/26  6:02:30: prRecQueue: starting pd=00 recovery - blocking host commands


04/26  6:02:30: prRecGo: Ready to attempt recovery errLBA=3c08a3e on pd=00


04/26  6:02:30: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/26  6:02:30: prRecGo: dataErr found on ld 0 span 0 arm 0


04/26  6:02:30: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78114, stripe=78114, refBlk=3e, type=1


04/26  6:02:30: prRecGo: RAID1 - read from peer pd=01


04/26  6:02:32: REC:log MedErr on pd[1] #retries=0


04/26  6:02:32: LogSense: pd=01, cdb=28 00 03 c0 8a 3e 00 00 01 00 


04/26  6:02:32:           sense=f0 00 03 03 c0 8a 3e 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  6:02:32: REC: MedErr on LD[1] BadLba=3c08a3e


04/26  6:02:32: prRecCallback: Error, cdb=28, pd=01, err=2


04/26  6:02:32: PatrolReadStart: Log MedErr pd=00 errLBA=3c08a3e in NVRAM_EVENT_LOG


04/26  6:02:34: REC:log MedErr on pd[1] #retries=0


04/26  6:02:34: LogSense: pd=01, cdb=2f 00 03 c1 04 54 00 ff ff 00 


04/26  6:02:34:           sense=f0 00 03 03 c1 5d 6c 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  6:02:34: REC: MedErr on LD[1] BadLba=3c15d6c


04/26  6:02:34: prCallback: Medium Error on pd=01, StartLba=3c10454, ErrLba=3c15d6c


04/26  6:02:34: prRecQueue: starting pd=01 recovery - blocking host commands


04/26  6:02:34: prRecGo: Ready to attempt recovery errLBA=3c15d6c on pd=01


04/26  6:02:35: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/26  6:02:35: prRecGo: dataErr found on ld 0 span 0 arm 1


04/26  6:02:35: prRecGo: data NOT in cache; cacheLn=ffffffff, row=782ba, stripe=782ba, refBlk=6c, type=1


04/26  6:02:35: prRecGo: RAID1 - read from peer pd=00


04/26  6:02:36: REC:log MedErr on pd[0] #retries=0


04/26  6:02:36: LogSense: pd=00, cdb=28 00 03 c1 5d 6c 00 00 01 00 


04/26  6:02:36:           sense=f0 00 03 03 c1 5d 6c 0a 03 0c 48 fd 11 00 00 00 00 a5 


04/26  6:02:36: REC: MedErr on LD[0] BadLba=3c15d6c


04/26  6:02:36: prRecCallback: Error, cdb=28, pd=00, err=2


04/26  6:02:36: PatrolReadStart: Log MedErr pd=01 errLBA=3c15d6c in NVRAM_EVENT_LOG


04/26  6:02:41: REC:log MedErr on pd[0] #retries=0


04/26  6:02:41: LogSense: pd=00, cdb=2f 00 03 c0 92 3f 00 ff ff 00 


04/26  6:02:41:           sense=f0 00 03 03 c0 a3 de 0a 1a 2e 48 ee 11 00 00 00 00 a5 


04/26  6:02:41: REC: MedErr on LD[0] BadLba=3c0a3de


04/26  6:02:41: prCallback: Medium Error on pd=00, StartLba=3c0923f, ErrLba=3c0a3de


04/26  6:02:41: prRecQueue: starting pd=00 recovery - blocking host commands


04/26  6:02:41: prRecGo: Ready to attempt recovery errLBA=3c0a3de on pd=00


04/26  6:02:41: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/26  6:02:41: prRecGo: dataErr found on ld 0 span 0 arm 0


04/26  6:02:41: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78147, stripe=78147, refBlk=5e, type=1


04/26  6:02:41: prRecGo: RAID1 - read from peer pd=01


04/26  6:02:41: REC:log MedErr on pd[1] #retries=0


04/26  6:02:41: LogSense: pd=01, cdb=2f 00 03 c1 5d 6d 00 ff ff 00 


04/26  6:02:41:           sense=f0 00 03 03 c1 71 8d 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  6:02:41: REC: MedErr on LD[1] BadLba=3c1718d


04/26  6:02:41: prCallback: Medium Error on pd=01, StartLba=3c15d6d, ErrLba=3c1718d


04/26  6:02:41: prRecQueue: adding pd=01 to recovery wait queue


04/26  6:02:43: REC:log MedErr on pd[1] #retries=0


04/26  6:02:43: LogSense: pd=01, cdb=28 00 03 c0 a3 de 00 00 01 00 


04/26  6:02:43:           sense=f0 00 03 03 c0 a3 de 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  6:02:43: REC: MedErr on LD[1] BadLba=3c0a3de


04/26  6:02:43: prRecCallback: Error, cdb=28, pd=01, err=2


04/26  6:02:43: PatrolReadStart: Log MedErr pd=00 errLBA=3c0a3de in NVRAM_EVENT_LOG


04/26  6:02:43: prRecGo: Ready to attempt recovery errLBA=3c1718d on pd=01


04/26  6:02:43: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/26  6:02:43: prRecGo: dataErr found on ld 0 span 0 arm 1


04/26  6:02:43: prRecGo: data NOT in cache; cacheLn=ffffffff, row=782e3, stripe=782e3, refBlk=d, type=1


04/26  6:02:43: prRecGo: RAID1 - read from peer pd=00


04/26  6:02:45: REC:log MedErr on pd[0] #retries=0


04/26  6:02:45: LogSense: pd=00, cdb=28 00 03 c1 71 8d 00 00 01 00 


04/26  6:02:45:           sense=f0 00 03 03 c1 71 8d 0a 01 a2 48 fe 11 00 00 00 00 a5 


04/26  6:02:45: REC: MedErr on LD[0] BadLba=3c1718d


04/26  6:02:45: prRecCallback: Error, cdb=28, pd=00, err=2


04/26  6:02:45: PatrolReadStart: Log MedErr pd=01 errLBA=3c1718d in NVRAM_EVENT_LOG


04/26  6:02:49: REC:log MedErr on pd[0] #retries=0


04/26  6:02:49: LogSense: pd=00, cdb=2f 00 03 c0 a3 df 00 ff ff 00 


04/26  6:02:49:           sense=f0 00 03 03 c0 b8 1d 0a 18 cd 48 ef 11 00 00 00 00 a5 


04/26  6:02:49: REC: MedErr on LD[0] BadLba=3c0b81d


04/26  6:02:49: prCallback: Medium Error on pd=00, StartLba=3c0a3df, ErrLba=3c0b81d


04/26  6:02:49: prRecQueue: starting pd=00 recovery - blocking host commands


04/26  6:02:49: prRecGo: Ready to attempt recovery errLBA=3c0b81d on pd=00


04/26  6:02:49: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/26  6:02:49: prRecGo: dataErr found on ld 0 span 0 arm 0


04/26  6:02:49: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78170, stripe=78170, refBlk=1d, type=1


04/26  6:02:49: prRecGo: RAID1 - read from peer pd=01


04/26  6:02:51: REC:log MedErr on pd[1] #retries=0


04/26  6:02:51: LogSense: pd=01, cdb=28 00 03 c0 b8 1d 00 00 01 00 


04/26  6:02:51:           sense=f0 00 03 03 c0 b8 1d 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  6:02:51: REC: MedErr on LD[1] BadLba=3c0b81d


04/26  6:02:51: prRecCallback: Error, cdb=28, pd=01, err=2


04/26  6:02:51: PatrolReadStart: Log MedErr pd=00 errLBA=3c0b81d in NVRAM_EVENT_LOG


04/26  6:02:57: REC:log MedErr on pd[0] #retries=0


04/26  6:02:57: LogSense: pd=00, cdb=2f 00 03 c0 b8 1e 00 ff ff 00 


04/26  6:02:57:           sense=f0 00 03 03 c0 b8 1e 0a 18 ce 48 ef 11 00 00 00 00 a6 


04/26  6:02:57: REC: MedErr on LD[0] BadLba=3c0b81e


04/26  6:02:57: prCallback: Medium Error on pd=00, StartLba=3c0b81e, ErrLba=3c0b81e


04/26  6:02:57: prRecQueue: starting pd=00 recovery - blocking host commands


04/26  6:02:57: prRecGo: Ready to attempt recovery errLBA=3c0b81e on pd=00


04/26  6:02:57: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/26  6:02:57: prRecGo: dataErr found on ld 0 span 0 arm 0


04/26  6:02:57: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78170, stripe=78170, refBlk=1e, type=1


04/26  6:02:57: prRecGo: RAID1 - read from peer pd=01


04/26  6:02:59: REC:log MedErr on pd[1] #retries=0


04/26  6:02:59: LogSense: pd=01, cdb=28 00 03 c0 b8 1e 00 00 01 00 


04/26  6:02:59:           sense=f0 00 03 03 c0 b8 1e 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  6:02:59: REC: MedErr on LD[1] BadLba=3c0b81e


04/26  6:02:59: prRecCallback: Error, cdb=28, pd=01, err=2


04/26  6:02:59: PatrolReadStart: Log MedErr pd=00 errLBA=3c0b81e in NVRAM_EVENT_LOG


04/26  6:03:04: REC:log MedErr on pd[0] #retries=0


04/26  6:03:04: LogSense: pd=00, cdb=2f 00 03 c0 b8 1f 00 ff ff 00 


04/26  6:03:04:           sense=f0 00 03 03 c0 ba 7e 0a 18 2e 48 f0 11 00 00 00 00 a5 


04/26  6:03:04: REC: MedErr on LD[0] BadLba=3c0ba7e


04/26  6:03:04: prCallback: Medium Error on pd=00, StartLba=3c0b81f, ErrLba=3c0ba7e


04/26  6:03:04: prRecQueue: starting pd=00 recovery - blocking host commands


04/26  6:03:04: prRecGo: Ready to attempt recovery errLBA=3c0ba7e on pd=00


04/26  6:03:04: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/26  6:03:04: prRecGo: dataErr found on ld 0 span 0 arm 0


04/26  6:03:04: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78174, stripe=78174, refBlk=7e, type=1


04/26  6:03:04: prRecGo: RAID1 - read from peer pd=01


04/26  6:03:06: REC:log MedErr on pd[1] #retries=0


04/26  6:03:06: LogSense: pd=01, cdb=28 00 03 c0 ba 7e 00 00 01 00 


04/26  6:03:06:           sense=f0 00 03 03 c0 ba 7e 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  6:03:06: REC: MedErr on LD[1] BadLba=3c0ba7e


04/26  6:03:06: prRecCallback: Error, cdb=28, pd=01, err=2


04/26  6:03:06: PatrolReadStart: Log MedErr pd=00 errLBA=3c0ba7e in NVRAM_EVENT_LOG


04/26  6:03:11: REC:log MedErr on pd[0] #retries=0


04/26  6:03:11: LogSense: pd=00, cdb=2f 00 03 c0 ba 7f 00 ff ff 00 


04/26  6:03:11:           sense=f0 00 03 03 c0 d1 be 0a 19 ce 48 f1 11 00 00 00 00 a5 


04/26  6:03:11: REC: MedErr on LD[0] BadLba=3c0d1be


04/26  6:03:11: prCallback: Medium Error on pd=00, StartLba=3c0ba7f, ErrLba=3c0d1be


04/26  6:03:11: prRecQueue: starting pd=00 recovery - blocking host commands


04/26  6:03:11: prRecGo: Ready to attempt recovery errLBA=3c0d1be on pd=00


04/26  6:03:11: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/26  6:03:11: prRecGo: dataErr found on ld 0 span 0 arm 0


04/26  6:03:11: prRecGo: data NOT in cache; cacheLn=ffffffff, row=781a3, stripe=781a3, refBlk=3e, type=1


04/26  6:03:11: prRecGo: RAID1 - read from peer pd=01


04/26  6:03:12: REC:log MedErr on pd[1] #retries=0


04/26  6:03:12: LogSense: pd=01, cdb=2f 00 03 c5 72 8a 00 ff ff 00 


04/26  6:03:12:           sense=f0 00 03 03 c6 4e 38 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  6:03:12: REC: MedErr on LD[1] BadLba=3c64e38


04/26  6:03:12: prCallback: Medium Error on pd=01, StartLba=3c5728a, ErrLba=3c64e38


04/26  6:03:12: prRecQueue: adding pd=01 to recovery wait queue


04/26  6:03:14: REC:log MedErr on pd[1] #retries=0


04/26  6:03:14: LogSense: pd=01, cdb=28 00 03 c0 d1 be 00 00 01 00 


04/26  6:03:14:           sense=f0 00 03 03 c0 d1 be 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  6:03:14: REC: MedErr on LD[1] BadLba=3c0d1be


04/26  6:03:14: prRecCallback: Error, cdb=28, pd=01, err=2


04/26  6:03:14: PatrolReadStart: Log MedErr pd=00 errLBA=3c0d1be in NVRAM_EVENT_LOG


04/26  6:03:14: prRecGo: Ready to attempt recovery errLBA=3c64e38 on pd=01


04/26  6:03:14: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/26  6:03:14: prRecGo: dataErr found on ld 0 span 0 arm 1


04/26  6:03:14: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78c9c, stripe=78c9c, refBlk=38, type=1


04/26  6:03:14: prRecGo: RAID1 - read from peer pd=00


04/26  6:03:15: REC:log MedErr on pd[0] #retries=0


04/26  6:03:15: LogSense: pd=00, cdb=28 00 03 c6 4e 38 00 00 01 00 


04/26  6:03:15:           sense=f0 00 03 03 c6 4e 38 0a 1a a8 49 63 11 00 00 00 00 a5 


04/26  6:03:15: REC: MedErr on LD[0] BadLba=3c64e38


04/26  6:03:15: prRecCallback: Error, cdb=28, pd=00, err=2


04/26  6:03:15: PatrolReadStart: Log MedErr pd=01 errLBA=3c64e38 in NVRAM_EVENT_LOG


04/26  6:03:19: REC:log MedErr on pd[0] #retries=0


04/26  6:03:19: LogSense: pd=00, cdb=2f 00 03 c0 d1 bf 00 ff ff 00 


04/26  6:03:19:           sense=f0 00 03 03 c0 eb 53 0a 1a c3 48 f3 11 00 00 00 00 a5 


04/26  6:03:19: REC: MedErr on LD[0] BadLba=3c0eb53


04/26  6:03:19: prCallback: Medium Error on pd=00, StartLba=3c0d1bf, ErrLba=3c0eb53


04/26  6:03:19: prRecQueue: starting pd=00 recovery - blocking host commands


04/26  6:03:19: prRecGo: Ready to attempt recovery errLBA=3c0eb53 on pd=00


04/26  6:03:19: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/26  6:03:19: prRecGo: dataErr found on ld 0 span 0 arm 0


04/26  6:03:19: prRecGo: data NOT in cache; cacheLn=ffffffff, row=781d6, stripe=781d6, refBlk=53, type=1


04/26  6:03:19: prRecGo: RAID1 - read from peer pd=01


04/26  6:03:20: REC:log MedErr on pd[1] #retries=0


04/26  6:03:20: LogSense: pd=01, cdb=2f 00 03 c6 4e 39 00 ff ff 00 


04/26  6:03:20:           sense=f0 00 03 03 c6 50 98 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  6:03:20: REC: MedErr on LD[1] BadLba=3c65098


04/26  6:03:20: prCallback: Medium Error on pd=01, StartLba=3c64e39, ErrLba=3c65098


04/26  6:03:20: prRecQueue: adding pd=01 to recovery wait queue


04/26  6:03:22: REC:log MedErr on pd[1] #retries=0


04/26  6:03:22: LogSense: pd=01, cdb=28 00 03 c0 eb 53 00 00 01 00 


04/26  6:03:22:           sense=f0 00 03 03 c0 eb 53 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  6:03:22: REC: MedErr on LD[1] BadLba=3c0eb53


04/26  6:03:22: prRecCallback: Error, cdb=28, pd=01, err=2


04/26  6:03:22: PatrolReadStart: Log MedErr pd=00 errLBA=3c0eb53 in NVRAM_EVENT_LOG


04/26  6:03:22: prRecGo: Ready to attempt recovery errLBA=3c65098 on pd=01


04/26  6:03:22: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/26  6:03:22: prRecGo: dataErr found on ld 0 span 0 arm 1


04/26  6:03:22: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78ca1, stripe=78ca1, refBlk=18, type=1


04/26  6:03:22: prRecGo: RAID1 - read from peer pd=00


04/26  6:03:23: REC:log MedErr on pd[0] #retries=0


04/26  6:03:23: LogSense: pd=00, cdb=28 00 03 c6 50 98 00 00 01 00 


04/26  6:03:23:           sense=f0 00 03 03 c6 50 98 0a 1a 08 49 64 11 00 00 00 00 a5 


04/26  6:03:23: REC: MedErr on LD[0] BadLba=3c65098


04/26  6:03:24: prRecCallback: Error, cdb=28, pd=00, err=2


04/26  6:03:24: PatrolReadStart: Log MedErr pd=01 errLBA=3c65098 in NVRAM_EVENT_LOG


04/26  6:03:28: REC:log MedErr on pd[0] #retries=0


04/26  6:03:28: LogSense: pd=00, cdb=2f 00 03 c0 eb 54 00 ff ff 00 


04/26  6:03:28:           sense=f0 00 03 03 c1 01 f3 0a 18 c3 48 f5 11 00 00 00 00 a5 


04/26  6:03:28: REC: MedErr on LD[0] BadLba=3c101f3


04/26  6:03:28: prCallback: Medium Error on pd=00, StartLba=3c0eb54, ErrLba=3c101f3


04/26  6:03:28: prRecQueue: starting pd=00 recovery - blocking host commands


04/26  6:03:28: prRecGo: Ready to attempt recovery errLBA=3c101f3 on pd=00


04/26  6:03:28: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/26  6:03:28: prRecGo: dataErr found on ld 0 span 0 arm 0


04/26  6:03:28: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78203, stripe=78203, refBlk=73, type=1


04/26  6:03:28: prRecGo: RAID1 - read from peer pd=01


04/26  6:03:29: REC:log MedErr on pd[1] #retries=0


04/26  6:03:29: LogSense: pd=01, cdb=2f 00 03 c6 50 99 00 ff ff 00 


04/26  6:03:29:           sense=f0 00 03 03 c6 64 d8 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  6:03:29: REC: MedErr on LD[1] BadLba=3c664d8


04/26  6:03:29: prCallback: Medium Error on pd=01, StartLba=3c65099, ErrLba=3c664d8


04/26  6:03:29: prRecQueue: adding pd=01 to recovery wait queue


04/26  6:03:31: REC:log MedErr on pd[1] #retries=0


04/26  6:03:31: LogSense: pd=01, cdb=28 00 03 c1 01 f3 00 00 01 00 


04/26  6:03:31:           sense=f0 00 03 03 c1 01 f3 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  6:03:31: REC: MedErr on LD[1] BadLba=3c101f3


04/26  6:03:31: prRecCallback: Error, cdb=28, pd=01, err=2


04/26  6:03:31: PatrolReadStart: Log MedErr pd=00 errLBA=3c101f3 in NVRAM_EVENT_LOG


04/26  6:03:31: prRecGo: Ready to attempt recovery errLBA=3c664d8 on pd=01


04/26  6:03:31: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/26  6:03:31: prRecGo: dataErr found on ld 0 span 0 arm 1


04/26  6:03:31: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78cc9, stripe=78cc9, refBlk=58, type=1


04/26  6:03:31: prRecGo: RAID1 - read from peer pd=00


04/26  6:03:33: REC:log MedErr on pd[0] #retries=0


04/26  6:03:33: LogSense: pd=00, cdb=28 00 03 c6 64 d8 00 00 01 00 


04/26  6:03:33:           sense=f0 00 03 03 c6 64 d8 0a 18 a8 49 65 11 00 00 00 00 a5 


04/26  6:03:33: REC: MedErr on LD[0] BadLba=3c664d8


04/26  6:03:33: prRecCallback: Error, cdb=28, pd=00, err=2


04/26  6:03:33: PatrolReadStart: Log MedErr pd=01 errLBA=3c664d8 in NVRAM_EVENT_LOG


04/26  6:03:36: REC:log MedErr on pd[0] #retries=0


04/26  6:03:36: LogSense: pd=00, cdb=2f 00 03 c1 01 f4 00 ff ff 00 


04/26  6:03:36:           sense=f0 00 03 03 c1 04 53 0a 18 23 48 f6 11 00 00 00 00 a5 


04/26  6:03:36: REC: MedErr on LD[0] BadLba=3c10453


04/26  6:03:36: prCallback: Medium Error on pd=00, StartLba=3c101f4, ErrLba=3c10453


04/26  6:03:36: prRecQueue: starting pd=00 recovery - blocking host commands


04/26  6:03:36: prRecGo: Ready to attempt recovery errLBA=3c10453 on pd=00


04/26  6:03:36: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/26  6:03:36: prRecGo: dataErr found on ld 0 span 0 arm 0


04/26  6:03:36: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78208, stripe=78208, refBlk=53, type=1


04/26  6:03:36: prRecGo: RAID1 - read from peer pd=01


04/26  6:03:38: REC:log MedErr on pd[1] #retries=0


04/26  6:03:38: LogSense: pd=01, cdb=28 00 03 c1 04 53 00 00 01 00 


04/26  6:03:38:           sense=f0 00 03 03 c1 04 53 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  6:03:38: REC: MedErr on LD[1] BadLba=3c10453


04/26  6:03:38: prRecCallback: Error, cdb=28, pd=01, err=2


04/26  6:03:38: PatrolReadStart: Log MedErr pd=00 errLBA=3c10453 in NVRAM_EVENT_LOG


04/26  6:03:43: REC:log MedErr on pd[0] #retries=0


04/26  6:03:43: LogSense: pd=00, cdb=2f 00 03 c1 04 54 00 ff ff 00 


04/26  6:03:43:           sense=f0 00 03 03 c1 5d 6c 0a 03 0c 48 fd 11 00 00 00 00 a5 


04/26  6:03:43: REC: MedErr on LD[0] BadLba=3c15d6c


04/26  6:03:43: prCallback: Medium Error on pd=00, StartLba=3c10454, ErrLba=3c15d6c


04/26  6:03:43: prRecQueue: starting pd=00 recovery - blocking host commands


04/26  6:03:43: prRecGo: Ready to attempt recovery errLBA=3c15d6c on pd=00


04/26  6:03:43: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/26  6:03:43: prRecGo: dataErr found on ld 0 span 0 arm 0


04/26  6:03:43: prRecGo: data NOT in cache; cacheLn=ffffffff, row=782ba, stripe=782ba, refBlk=6c, type=1


04/26  6:03:43: prRecGo: RAID1 - read from peer pd=01


04/26  6:03:45: REC:log MedErr on pd[1] #retries=0


04/26  6:03:45: LogSense: pd=01, cdb=28 00 03 c1 5d 6c 00 00 01 00 


04/26  6:03:45:           sense=f0 00 03 03 c1 5d 6c 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  6:03:45: REC: MedErr on LD[1] BadLba=3c15d6c


04/26  6:03:45: prRecCallback: Error, cdb=28, pd=01, err=2


04/26  6:03:45: PatrolReadStart: Log MedErr pd=00 errLBA=3c15d6c in NVRAM_EVENT_LOG


04/26  6:03:50: REC:log MedErr on pd[0] #retries=0


04/26  6:03:50: LogSense: pd=00, cdb=2f 00 03 c1 5d 6d 00 ff ff 00 


04/26  6:03:50:           sense=f0 00 03 03 c1 71 8d 0a 01 a2 48 fe 11 00 00 00 00 a5 


04/26  6:03:50: REC: MedErr on LD[0] BadLba=3c1718d


04/26  6:03:50: prCallback: Medium Error on pd=00, StartLba=3c15d6d, ErrLba=3c1718d


04/26  6:03:50: prRecQueue: starting pd=00 recovery - blocking host commands


04/26  6:03:50: prRecGo: Ready to attempt recovery errLBA=3c1718d on pd=00


04/26  6:03:50: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/26  6:03:50: prRecGo: dataErr found on ld 0 span 0 arm 0


04/26  6:03:50: prRecGo: data NOT in cache; cacheLn=ffffffff, row=782e3, stripe=782e3, refBlk=d, type=1


04/26  6:03:50: prRecGo: RAID1 - read from peer pd=01


04/26  6:03:52: REC:log MedErr on pd[1] #retries=0


04/26  6:03:52: LogSense: pd=01, cdb=28 00 03 c1 71 8d 00 00 01 00 


04/26  6:03:52:           sense=f0 00 03 03 c1 71 8d 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  6:03:52: REC: MedErr on LD[1] BadLba=3c1718d


04/26  6:03:52: prRecCallback: Error, cdb=28, pd=01, err=2


04/26  6:03:52: PatrolReadStart: Log MedErr pd=00 errLBA=3c1718d in NVRAM_EVENT_LOG


04/26  6:04:14: REC:log MedErr on pd[0] #retries=0


04/26  6:04:14: LogSense: pd=00, cdb=2f 00 03 c5 71 8a 00 ff ff 00 


04/26  6:04:14:           sense=f0 00 03 03 c6 4e 38 0a 1a a8 49 63 11 00 00 00 00 a5 


04/26  6:04:14: REC: MedErr on LD[0] BadLba=3c64e38


04/26  6:04:14: prCallback: Medium Error on pd=00, StartLba=3c5718a, ErrLba=3c64e38


04/26  6:04:14: prRecQueue: starting pd=00 recovery - blocking host commands


04/26  6:04:14: prRecGo: Ready to attempt recovery errLBA=3c64e38 on pd=00


04/26  6:04:14: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/26  6:04:14: prRecGo: dataErr found on ld 0 span 0 arm 0


04/26  6:04:15: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78c9c, stripe=78c9c, refBlk=38, type=1


04/26  6:04:15: prRecGo: RAID1 - read from peer pd=01


04/26  6:04:16: REC:log MedErr on pd[1] #retries=0


04/26  6:04:16: LogSense: pd=01, cdb=28 00 03 c6 4e 38 00 00 01 00 


04/26  6:04:16:           sense=f0 00 03 03 c6 4e 38 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  6:04:16: REC: MedErr on LD[1] BadLba=3c64e38


04/26  6:04:16: prRecCallback: Error, cdb=28, pd=01, err=2


04/26  6:04:16: PatrolReadStart: Log MedErr pd=00 errLBA=3c64e38 in NVRAM_EVENT_LOG


04/26  6:04:21: REC:log MedErr on pd[0] #retries=0


04/26  6:04:21: LogSense: pd=00, cdb=2f 00 03 c6 4e 39 00 ff ff 00 


04/26  6:04:21:           sense=f0 00 03 03 c6 50 98 0a 1a 08 49 64 11 00 00 00 00 a5 


04/26  6:04:21: REC: MedErr on LD[0] BadLba=3c65098


04/26  6:04:21: prCallback: Medium Error on pd=00, StartLba=3c64e39, ErrLba=3c65098


04/26  6:04:21: prRecQueue: starting pd=00 recovery - blocking host commands


04/26  6:04:21: prRecGo: Ready to attempt recovery errLBA=3c65098 on pd=00


04/26  6:04:21: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/26  6:04:21: prRecGo: dataErr found on ld 0 span 0 arm 0


04/26  6:04:21: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78ca1, stripe=78ca1, refBlk=18, type=1


04/26  6:04:21: prRecGo: RAID1 - read from peer pd=01


04/26  6:04:23: REC:log MedErr on pd[1] #retries=0


04/26  6:04:23: LogSense: pd=01, cdb=28 00 03 c6 50 98 00 00 01 00 


04/26  6:04:23:           sense=f0 00 03 03 c6 50 98 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  6:04:23: REC: MedErr on LD[1] BadLba=3c65098


04/26  6:04:23: prRecCallback: Error, cdb=28, pd=01, err=2


04/26  6:04:23: PatrolReadStart: Log MedErr pd=00 errLBA=3c65098 in NVRAM_EVENT_LOG


04/26  6:04:28: REC:log MedErr on pd[0] #retries=0


04/26  6:04:28: LogSense: pd=00, cdb=2f 00 03 c6 50 99 00 ff ff 00 


04/26  6:04:28:           sense=f0 00 03 03 c6 64 d8 0a 18 a8 49 65 11 00 00 00 00 a5 


04/26  6:04:28: REC: MedErr on LD[0] BadLba=3c664d8


04/26  6:04:28: prCallback: Medium Error on pd=00, StartLba=3c65099, ErrLba=3c664d8


04/26  6:04:28: prRecQueue: starting pd=00 recovery - blocking host commands


04/26  6:04:28: prRecGo: Ready to attempt recovery errLBA=3c664d8 on pd=00


04/26  6:04:28: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/26  6:04:28: prRecGo: dataErr found on ld 0 span 0 arm 0


04/26  6:04:28: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78cc9, stripe=78cc9, refBlk=58, type=1


04/26  6:04:28: prRecGo: RAID1 - read from peer pd=01


04/26  6:04:30: REC:log MedErr on pd[1] #retries=0


04/26  6:04:30: LogSense: pd=01, cdb=28 00 03 c6 64 d8 00 00 01 00 


04/26  6:04:30:           sense=f0 00 03 03 c6 64 d8 0a 00 00 00 00 11 00 e4 00 00 86 


04/26  6:04:30: REC: MedErr on LD[1] BadLba=3c664d8


04/26  6:04:30: prRecCallback: Error, cdb=28, pd=01, err=2


04/26  6:04:30: PatrolReadStart: Log MedErr pd=00 errLBA=3c664d8 in NVRAM_EVENT_LOG


04/26  7:30:03: prCallback: PR completed for pd=01


04/26  7:30:03: prDiskDone: finishing PR on PD=01


04/26  7:31:10: prCallback: PR completed for pd=00


04/26  7:31:10: prDiskDone: finishing PR on PD=00


04/26  8:21:45: prCheckForBackgroundActivity: CC/BGI/Init active on ld=1


04/26  8:21:45: prDiskCheckOkToRun: PR NOT running due to background activity on adapter PD=14


04/26  8:21:45: prCallback: PR being stopped for pd=14 - state changed


04/26  8:21:45: prDiskDone: finishing PR on PD=14


04/26  8:21:45: prCheckForBackgroundActivity: CC/BGI/Init active on ld=1


04/26  8:21:45: prDiskCheckOkToRun: PR NOT running due to background activity on adapter PD=12


04/26  8:21:45: prCallback: PR being stopped for pd=12 - state changed


04/26  8:21:45: prDiskDone: finishing PR on PD=12


04/26  8:21:45: prCheckForBackgroundActivity: CC/BGI/Init active on ld=1


04/26  8:21:45: prDiskCheckOkToRun: PR NOT running due to background activity on adapter PD=13


04/26  8:21:45: prCallback: PR being stopped for pd=13 - state changed


04/26  8:21:45: prDiskDone: finishing PR on PD=13


04/26  8:21:45: PR cycle complete


04/26  8:21:46: prCheckForBackgroundActivity: CC/BGI/Init active on ld=1


04/26  8:21:46: prDiskCheckOkToRun: PR NOT running due to background activity on adapter PD=00


04/26  8:21:46: prCheckForBackgroundActivity: CC/BGI/Init active on ld=1


04/26  8:21:46: prDiskCheckOkToRun: PR NOT running due to background activity on adapter PD=01


04/26  8:21:46: prCheckForBackgroundActivity: CC/BGI/Init active on ld=1


04/26  8:21:46: prDiskCheckOkToRun: PR NOT running due to background activity on adapter PD=12


04/26  8:21:46: prCheckForBackgroundActivity: CC/BGI/Init active on ld=1


04/26  8:21:46: prDiskCheckOkToRun: PR NOT running due to background activity on adapter PD=13


04/26  8:21:46: prCheckForBackgroundActivity: CC/BGI/Init active on ld=1


04/26  8:21:46: prDiskCheckOkToRun: PR NOT running due to background activity on adapter PD=14


04/26  8:21:46: Next PR scheduled to start at 04/26 12:00:00


04/26 10:42:57: encl: Device removed from encl 06, slot 1


04/26 10:42:57: fail_reason=f5, channel=0, target=1


04/26 10:42:57: UpdateDrvInfo: notify config to slave


04/26 10:42:57: UpdateDrvInfo: notify config to slave


04/26 10:45:17: encl: Device inserted in encl 06, slot 1


04/26 10:45:21: enclMonitorCallback: Encl Index 1 


04/26 10:45:21: enclPprCallback: Encl Index 1 Pd 1 


04/26 10:45:21: TUR:ReqSense C=0,T=1, status=2, SK=2, ASC=4, ASCQ=1


04/26 10:45:21: RAID SSU: status 1 - Retrying... 


04/26 10:45:23: TUR:ReqSense C=0,T=1, status=2, SK=2, ASC=4, ASCQ=1


04/26 10:45:23: RAID SSU: status 1 - Retrying... 


04/26 10:45:25: TUR:ReqSense C=0,T=1, status=2, SK=2, ASC=4, ASCQ=1


04/26 10:45:25: RAID SSU: status 1 - Retrying... 


04/26 10:45:27: TUR:ReqSense C=0,T=1, status=2, SK=2, ASC=4, ASCQ=1


04/26 10:45:27: RAID SSU: status 1 - Retrying... 


04/26 10:45:28: LogSense: pd=01, cdb=25 00 00 00 00 00 00 00 00 00 


04/26 10:45:28:           sense=70 00 02 00 00 00 00 0a 00 00 00 00 04 01 02 00 00 00 


04/26 10:45:28: LogSense: pd=01, cdb=25 00 00 00 00 00 00 00 00 00 


04/26 10:45:28:           sense=70 00 02 00 00 00 00 0a 00 00 00 00 04 01 02 00 00 00 


04/26 10:45:29: TUR:ReqSense C=0,T=1, status=2, SK=2, ASC=4, ASCQ=1


04/26 10:45:29: LogSense: pd=01, cdb=25 00 00 00 00 00 00 00 00 00 


04/26 10:45:29:           sense=70 00 02 00 00 00 00 0a 00 00 00 00 04 01 02 00 00 00 


04/26 10:45:29: RAID SSU: status 1 - Retrying... 


04/26 10:45:29: LogSense: pd=01, cdb=25 00 00 00 00 00 00 00 00 00 


04/26 10:45:29:           sense=70 00 02 00 00 00 00 0a 00 00 00 00 04 01 02 00 00 00 


04/26 10:45:29: LogSense: pd=01, cdb=25 00 00 00 00 00 00 00 00 00 


04/26 10:45:29:           sense=70 00 02 00 00 00 00 0a 00�€���€��0 04 01 02 00 00 00 


04/26 10:45:29: LogSense: pd=01, cdb=25 00 00 00 00 00 00 00 00 00 


04/26 10:45:29:           sense=70 00 02 00 00 00 00 0a 00 00 00 00 04 01 02 00 00 00 


04/26 10:45:29: readCapacityCallback:rdcap status 2


04/26 10:45:31: LD 1 RdlongSize=236 #diff=a


04/26 10:45:31: Negotiation on insertion: INQUIRY fired to pd=01


04/26 10:45:31: Issuing ReadCap for 01


04/26 10:45:31: Ch:tg-0:1, scsi_siz-0x88bb939, drv_siz-0x88b9800


04/26 10:45:31: post_scsi_dcmd_status - auto rebuild for id 0x1


04/26 10:45:31: rbld to fire: pd=01


04/26 10:45:31: rbld about to fire: physId=01


04/26 10:45:31: UpdateDrvInfo: notify config to slave


04/26 12:00:00: prCheckForBackgroundActivity: Rebuild active on ld=0


04/26 12:00:00: prDiskCheckOkToRun: PR NOT running due to background activity on adapter PD=00


04/26 12:00:00: prCheckForBackgroundActivity: Rebuild active on ld=0


04/26 12:00:00: prDiskCheckOkToRun: PR NOT running due to background activity on adapter PD=12


04/26 12:00:00: prCheckForBackgroundActivity: Rebuild active on ld=0


04/26 12:00:00: prDiskCheckOkToRun: PR NOT running due to background activity on adapter PD=13


04/26 12:00:00: prCheckForBackgroundActivity: Rebuild active on ld=0


04/26 12:00:00: prDiskCheckOkToRun: PR NOT running due to background activity on adapter PD=14


04/26 12:00:00: Next PR scheduled to start at 04/26 16:00:00


04/26 12:30:08: REC:log MedErr on pd[0] #retries=0


04/26 12:30:08: LogSense: pd=00, cdb=28 00 03 b1 84 00 00 00 80 00 


04/26 12:30:08:           sense=f0 00 03 03 b1 84 6e 0a 1a 20 47 b4 11 00 00 00 00 a5 


04/26 12:30:08: REC: MedErr on LD[0] BadLba=3b1846e


04/26 12:30:08: BBMProcessReadError: REBUILD, pd=00, pdErrLba=3b1846e - puncture source/target drives


04/26 12:30:08: bbmMarkBadBlock: pd=00, pdLBA=3b1846e


04/26 12:30:08: bbmMarkBadBlock: pd=01, pdLBA=3b1846e


04/26 12:30:08: BBMNvramAddEntry: ld=0, pd=0, ldErrLba=3b1846e, pdErrLba=3b1846e


04/26 12:30:08: BBMNvramAddEntry: ld=0, pd=1, ldErrLba=3b1846e, pdErrLba=3b1846e


04/26 12:30:09: REC:log MedErr on pd[0] #retries=0


04/26 12:30:09: LogSense: pd=00, cdb=28 00 03 b1 98 80 00 00 80 00 


04/26 12:30:09:           sense=f0 00 03 03 b1 98 ee 0a 18 c0 47 b5 11 0b 00 00 00 79 


04/26 12:30:09: REC: MedErr on LD[0] BadLba=3b198ee


04/26 12:30:10: BBMProcessReadError: REBUILD, pd=00, pdErrLba=3b198ee - puncture source/target drives


04/26 12:30:10: bbmMarkBadBlock: pd=00, pdLBA=3b198ee


04/26 12:30:10: bbmMarkBadBlock: pd=01, pdLBA=3b198ee


04/26 12:30:10: BBMNvramAddEntry: ld=0, pd=0, ldErrLba=3b198ee, pdErrLba=3b198ee


04/26 12:30:10: BBMNvramAddEntry: ld=0, pd=1, ldErrLba=3b198ee, pdErrLba=3b198ee


04/26 12:30:11: REC:log MedErr on pd[0] #retries=0


04/26 12:30:11: LogSense: pd=00, cdb=28 00 03 b1 9b 00 00 00 80 00 


04/26 12:30:11:           sense=f0 00 03 03 b1 9b 4e 0a 18 20 47 b6 11 00 00 00 00 a5 


04/26 12:30:11: REC: MedErr on LD[0] BadLba=3b19b4e


04/26 12:30:11: BBMProcessReadError: REBUILD, pd=00, pdErrLba=3b19b4e - puncture source/target drives


04/26 12:30:11: bbmMarkBadBlock: pd=00, pdLBA=3b19b4e


04/26 12:30:11: bbmMarkBadBlock: pd=01, pdLBA=3b19b4e


04/26 12:30:11: BBMNvramAddEntry: ld=0, pd=0, ldErrLba=3b19b4e, pdErrLba=3b19b4e


04/26 12:30:11: BBMNvramAddEntry: ld=0, pd=1, ldErrLba=3b19b4e, pdErrLba=3b19b4e


04/26 12:31:31: REC:log MedErr on pd[0] #retries=0


04/26 12:31:31: LogSense: pd=00, cdb=28 00 03 bd 89 00 00 00 80 00 


04/26 12:31:31:           sense=f0 00 03 03 bd 89 65 0a 11 d4 48 ad 11 0b 00 00 00 79 


04/26 12:31:31: REC: MedErr on LD[0] BadLba=3bd8965


04/26 12:31:31: BBMProcessReadError: REBUILD, pd=00, pdErrLba=3bd8965 - puncture source/target drives


04/26 12:31:31: bbmMarkBadBlock: pd=00, pdLBA=3bd8965


04/26 12:31:31: bbmMarkBadBlock: pd=01, pdLBA=3bd8965


04/26 12:31:31: BBMNvramAddEntry: ld=0, pd=0, ldErrLba=3bd8965, pdErrLba=3bd8965


04/26 12:31:31: BBMNvramAddEntry: ld=0, pd=1, ldErrLba=3bd8965, pdErrLba=3bd8965


04/26 12:31:32: REC:log MedErr on pd[0] #retries=0


04/26 12:31:32: LogSense: pd=00, cdb=28 00 03 bd 93 80 00 00 80 00 


04/26 12:31:32:           sense=f0 00 03 03 bd 93 b9 0a 13 02 48 ae 11 0b 00 00 00 79 


04/26 12:31:32: REC: MedErr on LD[0] BadLba=3bd93b9


04/26 12:31:32: BBMProcessReadError: REBUILD, pd=00, pdErrLba=3bd93b9 - puncture source/target drives


04/26 12:31:32: bbmMarkBadBlock: pd=00, pdLBA=3bd93b9


04/26 12:31:32: bbmMarkBadBlock: pd=01, pdLBA=3bd93b9


04/26 12:31:32: BBMNvramAddEntry: ld=0, pd=0, ldErrLba=3bd93b9, pdErrLba=3bd93b9


04/26 12:31:32: BBMNvramAddEntry: ld=0, pd=1, ldErrLba=3bd93b9, pdErrLba=3bd93b9


04/26 12:31:43: REC:log MedErr on pd[0] #retries=0


04/26 12:31:43: LogSense: pd=00, cdb=28 00 03 bf 04 80 00 00 80 00 


04/26 12:31:43:           sense=f0 00 03 03 bf 04 f4 0a 12 df 48 cc 11 00 00 00 00 a5 


04/26 12:31:43: REC: MedErr on LD[0] BadLba=3bf04f4


04/26 12:31:43: BBMProcessReadError: REBUILD, pd=00, pdErrLba=3bf04f4 - puncture source/target drives


04/26 12:31:43: bbmMarkBadBlock: pd=00, pdLBA=3bf04f4


04/26 12:31:43: bbmMarkBadBlock: pd=01, pdLBA=3bf04f4


04/26 12:31:43: BBMNvramAddEntry: ld=0, pd=0, ldErrLba=3bf04f4, pdErrLba=3bf04f4


04/26 12:31:43: BBMNvramAddEntry: ld=0, pd=1, ldErrLba=3bf04f4, pdErrLba=3bf04f4


04/26 12:31:49: REC:log MedErr on pd[0] #retries=0


04/26 12:31:49: LogSense: pd=00, cdb=28 00 03 bf 83 00 00 00 80 00 


04/26 12:31:49:           sense=f0 00 03 03 bf 83 6f 0a 00 01 48 d6 11 0b 00 00 01 ab 


04/26 12:31:49: REC: MedErr on LD[0] BadLba=3bf836f


04/26 12:31:49: BBMProcessReadError: REBUILD, pd=00, pdErrLba=3bf836f - puncture source/target drives


04/26 12:31:50: bbmMarkBadBlock: pd=00, pdLBA=3bf836f


04/26 12:31:50: bbmMarkBadBlock: pd=01, pdLBA=3bf836f


04/26 12:31:50: BBMNvramAddEntry: ld=0, pd=0, ldErrLba=3bf836f, pdErrLba=3bf836f


04/26 12:31:50: BBMNvramAddEntry: ld=0, pd=1, ldErrLba=3bf836f, pdErrLba=3bf836f


04/26 12:31:57: REC:log MedErr on pd[0] #retries=0


04/26 12:31:57: LogSense: pd=00, cdb=28 00 03 c0 59 00 00 00 80 00 


04/26 12:31:57:           sense=f0 00 03 03 c0 59 2f 0a 19 5f 48 e8 11 00 00 00 00 a5 


04/26 12:31:57: REC: MedErr on LD[0] BadLba=3c0592f


04/26 12:31:57: BBMProcessReadError: REBUILD, pd=00, pdErrLba=3c0592f - puncture source/target drives


04/26 12:31:57: bbmMarkBadBlock: pd=00, pdLBA=3c0592f


04/26 12:31:57: bbmMarkBadBlock: pd=01, pdLBA=3c0592f


04/26 12:31:57: BBMNvramAddEntry: ld=0, pd=0, ldErrLba=3c0592f, pdErrLba=3c0592f


04/26 12:31:57: BBMNvramAddEntry: ld=0, pd=1, ldErrLba=3c0592f, pdErrLba=3c0592f


04/26 12:31:59: REC:log MedErr on pd[0] #retries=0


04/26 12:31:59: LogSense: pd=00, cdb=28 00 03 c0 70 00 00 00 80 00 


04/26 12:31:59:           sense=f0 00 03 03 c0 70 6f 0a 1a ff 48 e9 11 00 00 00 00 a5 


04/26 12:31:59: REC: MedErr on LD[0] BadLba=3c0706f


04/26 12:31:59: BBMProcessReadError: REBUILD, pd=00, pdErrLba=3c0706f - puncture source/target drives


04/26 12:31:59: bbmMarkBadBlock: pd=00, pdLBA=3c0706f


04/26 12:31:59: bbmMarkBadBlock: pd=01, pdLBA=3c0706f


04/26 12:31:59: BBMNvramAddEntry: ld=0, pd=0, ldErrLba=3c0706f, pdErrLba=3c0706f


04/26 12:31:59: BBMNvramAddEntry: ld=0, pd=1, ldErrLba=3c0706f, pdErrLba=3c0706f


04/26 12:32:00: REC:log MedErr on pd[0] #retries=0


04/26 12:32:00: LogSense: pd=00, cdb=28 00 03 c0 72 80 00 00 80 00 


04/26 12:32:00:           sense=f0 00 03 03 c0 72 cf 0a 1a 5f 48 ea 11 00 00 00 00 a5 


04/26 12:32:00: REC: MedErr on LD[0] BadLba=3c072cf


04/26 12:32:00: BBMProcessReadError: REBUILD, pd=00, pdErrLba=3c072cf - puncture source/target drives


04/26 12:32:00: bbmMarkBadBlock: pd=00, pdLBA=3c072cf


04/26 12:32:00: bbmMarkBadBlock: pd=01, pdLBA=3c072cf


04/26 12:32:00: BBMNvramAddEntry: ld=0, pd=0, ldErrLba=3c072cf, pdErrLba=3c072cf


04/26 12:32:00: BBMNvramAddEntry: ld=0, pd=1, ldErrLba=3c072cf, pdErrLba=3c072cf


04/26 12:32:02: REC:log MedErr on pd[0] #retries=0


04/26 12:32:02: LogSense: pd=00, cdb=28 00 03 c0 87 80 00 00 80 00 


04/26 12:32:02:           sense=f0 00 03 03 c0 87 de 0a 19 ce 48 eb 11 00 00 00 00 a6 


04/26 12:32:02: REC: MedErr on LD[0] BadLba=3c087de


04/26 12:32:02: BBMProcessReadError: REBUILD, pd=00, pdErrLba=3c087de - puncture source/target drives


04/26 12:32:02: bbmMarkBadBlock: pd=00, pdLBA=3c087de


04/26 12:32:02: bbmMarkBadBlock: pd=01, pdLBA=3c087de


04/26 12:32:02: BBMNvramAddEntry: ld=0, pd=0, ldErrLba=3c087de, pdErrLba=3c087de


04/26 12:32:02: BBMNvramAddEntry: ld=0, pd=1, ldErrLba=3c087de, pdErrLba=3c087de


04/26 12:32:04: REC:log MedErr on pd[0] #retries=0


04/26 12:32:04: LogSense: pd=00, cdb=28 00 03 c0 8a 00 00 00 80 00 


04/26 12:32:04:           sense=f0 00 03 03 c0 8a 3e 0a 19 2e 48 ec 11 00 00 00 00 a5 


04/26 12:32:04: REC: MedErr on LD[0] BadLba=3c08a3e


04/26 12:32:04: BBMProcessReadError: REBUILD, pd=00, pdErrLba=3c08a3e - puncture source/target drives


04/26 12:32:04: bbmMarkBadBlock: pd=00, pdLBA=3c08a3e


04/26 12:32:04: bbmMarkBadBlock: pd=01, pdLBA=3c08a3e


04/26 12:32:04: BBMNvramAddEntry: ld=0, pd=0, ldErrLba=3c08a3e, pdErrLba=3c08a3e


04/26 12:32:04: BBMNvramAddEntry: ld=0, pd=1, ldErrLba=3c08a3e, pdErrLba=3c08a3e


04/26 12:32:06: REC:log MedErr on pd[0] #retries=0


04/26 12:32:06: LogSense: pd=00, cdb=28 00 03 c0 a3 80 00 00 80 00 


04/26 12:32:06:           sense=f0 00 03 03 c0 a3 de 0a 1a 2e 48 ee 11 00 00 00 00 a5 


04/26 12:32:06: REC: MedErr on LD[0] BadLba=3c0a3de


04/26 12:32:06: BBMProcessReadError: REBUILD, pd=00, pdErrLba=3c0a3de - puncture source/target drives


04/26 12:32:06: bbmMarkBadBlock: pd=00, pdLBA=3c0a3de


04/26 12:32:06: bbmMarkBadBlock: pd=01, pdLBA=3c0a3de


04/26 12:32:06: BBMNvramAddEntry: ld=0, pd=0, ldErrLba=3c0a3de, pdErrLba=3c0a3de


04/26 12:32:06: BBMNvramAddEntry: ld=0, pd=1, ldErrLba=3c0a3de, pdErrLba=3c0a3de


04/26 12:32:08: REC:log MedErr on pd[0] #retries=0


04/26 12:32:08: LogSense: pd=00, cdb=28 00 03 c0 b8 00 00 00 80 00 


04/26 12:32:08:           sense=f0 00 03 03 c0 b8 1d 0a 18 cd 48 ef 11 00 00 00 00 a5 


04/26 12:32:08: REC: MedErr on LD[0] BadLba=3c0b81d


04/26 12:32:08: BBMProcessReadError: REBUILD, pd=00, pdErrLba=3c0b81d - puncture source/target drives


04/26 12:32:08: bbmMarkBadBlock: pd=00, pdLBA=3c0b81d


04/26 12:32:08: bbmMarkBadBlock: pd=01, pdLBA=3c0b81d


04/26 12:32:08: BBMNvramAddEntry: ld=0, pd=0, ldErrLba=3c0b81d, pdErrLba=3c0b81d


04/26 12:32:08: BBMNvramAddEntry: ld=0, pd=1, ldErrLba=3c0b81d, pdErrLba=3c0b81d


04/26 12:32:09: REC:log MedErr on pd[0] #retries=4


04/26 12:32:09: LogSense: pd=00, cdb=28 00 03 c0 b8 1e 00 00 62 00 


04/26 12:32:09:           sense=f0 00 03 03 c0 b8 1e 0a 18 ce 48 ef 11 00 00 00 00 a5 


04/26 12:32:09: REC: MedErr on LD[0] BadLba=3c0b81e


04/26 12:32:09: BBMProcessReadError: REBUILD, pd=00, pdErrLba=3c0b81e - puncture source/target drives


04/26 12:32:09: bbmMarkBadBlock: pd=00, pdLBA=3c0b81e


04/26 12:32:09: bbmMarkBadBlock: pd=01, pdLBA=3c0b81e


04/26 12:32:09: BBMNvramAddEntry: ld=0, pd=0, ldErrLba=3c0b81e, pdErrLba=3c0b81e


04/26 12:32:09: BBMNvramAddEntry: ld=0, pd=1, ldErrLba=3c0b81e, pdErrLba=3c0b81e


04/26 12:32:10: REC:log MedErr on pd[0] #retries=0


04/26 12:32:10: LogSense: pd=00, cdb=28 00 03 c0 ba 00 00 00 80 00 


04/26 12:32:10:           sense=f0 00 03 03 c0 ba 7e 0a 18 2e 48 f0 11 00 00 00 00 a5 


04/26 12:32:10: REC: MedErr on LD[0] BadLba=3c0ba7e


04/26 12:32:10: BBMProcessReadError: REBUILD, pd=00, pdErrLba=3c0ba7e - puncture source/target drives


04/26 12:32:10: bbmMarkBadBlock: pd=00, pdLBA=3c0ba7e


04/26 12:32:10: bbmMarkBadBlock: pd=01, pdLBA=3c0ba7e


04/26 12:32:10: BBMNvramAddEntry: ld=0, pd=0, ldErrLba=3c0ba7e, pdErrLba=3c0ba7e


04/26 12:32:11: BBMNvramAddEntry: ld=0, pd=1, ldErrLba=3c0ba7e, pdErrLba=3c0ba7e


04/26 12:32:12: REC:log MedErr on pd[0] #retries=0


04/26 12:32:12: LogSense: pd=00, cdb=28 00 03 c0 d1 80 00 00 80 00 


04/26 12:32:12:           sense=f0 00 03 03 c0 d1 be 0a 19 ce 48 f1 11 00 00 00 00 a5 


04/26 12:32:12: REC: MedErr on LD[0] BadLba=3c0d1be


04/26 12:32:12: BBMProcessReadError: REBUILD, pd=00, pdErrLba=3c0d1be - puncture source/target drives


04/26 12:32:12: bbmMarkBadBlock: pd=00, pdLBA=3c0d1be


04/26 12:32:12: bbmMarkBadBlock: pd=01, pdLBA=3c0d1be


04/26 12:32:12: BBMNvramAddEntry: ld=0, pd=0, ldErrLba=3c0d1be, pdErrLba=3c0d1be


04/26 12:32:12: BBMNvramAddEntry: ld=0, pd=1, ldErrLba=3c0d1be, pdErrLba=3c0d1be


04/26 12:32:14: REC:log MedErr on pd[0] #retries=0


04/26 12:32:14: LogSense: pd=00, cdb=28 00 03 c0 eb 00 00 00 80 00 


04/26 12:32:14:           sense=f0 00 03 03 c0 eb 53 0a 1a c3 48 f3 11 00 00 00 00 a5 


04/26 12:32:14: REC: MedErr on LD[0] BadLba=3c0eb53


04/26 12:32:14: BBMProcessReadError: REBUILD, pd=00, pdErrLba=3c0eb53 - puncture source/target drives


04/26 12:32:14: bbmMarkBadBlock: pd=00, pdLBA=3c0eb53


04/26 12:32:14: bbmMarkBadBlock: pd=01, pdLBA=3c0eb53


04/26 12:32:14: BBMNvramAddEntry: ld=0, pd=0, ldErrLba=3c0eb53, pdErrLba=3c0eb53


04/26 12:32:14: BBMNvramAddEntry: ld=0, pd=1, ldErrLba=3c0eb53, pdErrLba=3c0eb53


04/26 12:32:16: REC:log MedErr on pd[0] #retries=0


04/26 12:32:16: LogSense: pd=00, cdb=28 00 03 c1 01 80 00 00 80 00 


04/26 12:32:16:           sense=f0 00 03 03 c1 01 f3 0a 18 c3 48 f5 11 00 00 00 00 a5 


04/26 12:32:16: REC: MedErr on LD[0] BadLba=3c101f3


04/26 12:32:16: BBMProcessReadError: REBUILD, pd=00, pdErrLba=3c101f3 - puncture source/target drives


04/26 12:32:16: bbmMarkBadBlock: pd=00, pdLBA=3c101f3


04/26 12:32:16: bbmMarkBadBlock: pd=01, pdLBA=3c101f3


04/26 12:32:16: BBMNvramAddEntry: ld=0, pd=0, ldErrLba=3c101f3, pdErrLba=3c101f3


04/26 12:32:16: BBMNvramAddEntry: ld=0, pd=1, ldErrLba=3c101f3, pdErrLba=3c101f3


04/26 12:32:18: REC:log MedErr on pd[0] #retries=0


04/26 12:32:18: LogSense: pd=00, cdb=28 00 03 c1 04 00 00 00 80 00 


04/26 12:32:18:           sense=f0 00 03 03 c1 04 53 0a 18 23 48 f6 11 00 00 00 00 a5 


04/26 12:32:18: REC: MedErr on LD[0] BadLba=3c10453


04/26 12:32:18: BBMProcessReadError: REBUILD, pd=00, pdErrLba=3c10453 - puncture source/target drives


04/26 12:32:18: bbmMarkBadBlock: pd=00, pdLBA=3c10453


04/26 12:32:18: bbmMarkBadBlock: pd=01, pdLBA=3c10453


04/26 12:32:18: BBMNvramAddEntry: ld=0, pd=0, ldErrLba=3c10453, pdErrLba=3c10453


04/26 12:32:18: BBMNvramAddEntry: ld=0, pd=1, ldErrLba=3c10453, pdErrLba=3c10453


04/26 12:32:22: REC:log MedErr on pd[0] #retries=0


04/26 12:32:22: LogSense: pd=00, cdb=28 00 03 c1 5d 00 00 00 80 00 


04/26 12:32:22:           sense=f0 00 03 03 c1 5d 6c 0a 03 0c 48 fd 11 00 00 00 00 a5 


04/26 12:32:22: REC: MedErr on LD[0] BadLba=3c15d6c


04/26 12:32:22: BBMProcessReadError: REBUILD, pd=00, pdErrLba=3c15d6c - puncture source/target drives


04/26 12:32:22: bbmMarkBadBlock: pd=00, pdLBA=3c15d6c


04/26 12:32:22: bbmMarkBadBlock: pd=01, pdLBA=3c15d6c


04/26 12:32:22: BBMNvramAddEntry: ld=0, pd=0, ldErrLba=3c15d6c, pdErrLba=3c15d6c


04/26 12:32:22: BBMNvramAddEntry: ld=0, pd=1, ldErrLba=3c15d6c, pdErrLba=3c15d6c


04/26 12:32:24: REC:log MedErr on pd[0] #retries=0


04/26 12:32:24: LogSense: pd=00, cdb=28 00 03 c1 71 80 00 00 80 00 


04/26 12:32:24:           sense=f0 00 03 03 c1 71 8d 0a 01 a2 48 fe 11 00 00 00 00 a5 


04/26 12:32:24: REC: MedErr on LD[0] BadLba=3c1718d


04/26 12:32:24: BBMProcessReadError: REBUILD, pd=00, pdErrLba=3c1718d - puncture source/target drives


04/26 12:32:24: bbmMarkBadBlock: pd=00, pdLBA=3c1718d


04/26 12:32:24: bbmMarkBadBlock: pd=01, pdLBA=3c1718d


04/26 12:32:24: BBMNvramAddEntry: ld=0, pd=0, ldErrLba=3c1718d, pdErrLba=3c1718d


04/26 12:32:24: BBMNvramAddEntry: ld=0, pd=1, ldErrLba=3c1718d, pdErrLba=3c1718d


04/26 12:32:58: REC:log MedErr on pd[0] #retries=0


04/26 12:32:58: LogSense: pd=00, cdb=28 00 03 c6 4e 00 00 00 80 00 


04/26 12:32:58:           sense=f0 00 03 03 c6 4e 38 0a 1a a8 49 63 11 00 00 00 00 a5 


04/26 12:32:58: REC: MedErr on LD[0] BadLba=3c64e38


04/26 12:32:58: BBMProcessReadError: REBUILD, pd=00, pdErrLba=3c64e38 - puncture source/target drives


04/26 12:32:58: bbmMarkBadBlock: pd=00, pdLBA=3c64e38


04/26 12:32:58: bbmMarkBadBlock: pd=01, pdLBA=3c64e38


04/26 12:32:58: BBMNvramAddEntry: ld=0, pd=0, ldErrLba=3c64e38, pdErrLba=3c64e38


04/26 12:32:58: BBMNvramAddEntry: ld=0, pd=1, ldErrLba=3c64e38, pdErrLba=3c64e38


04/26 12:32:59: REC:log MedErr on pd[0] #retries=0


04/26 12:32:59: LogSense: pd=00, cdb=28 00 03 c6 50 80 00 00 80 00 


04/26 12:32:59:           sense=f0 00 03 03 c6 50 98 0a 1a 08 49 64 11 00 00 00 00 a5 


04/26 12:32:59: REC: MedErr on LD[0] BadLba=3c65098


04/26 12:32:59: BBMProcessReadError: REBUILD, pd=00, pdErrLba=3c65098 - puncture source/target drives


04/26 12:32:59: bbmMarkBadBlock: pd=00, pdLBA=3c65098


04/26 12:32:59: bbmMarkBadBlock: pd=01, pdLBA=3c65098


04/26 12:32:59: BBMNvramAddEntry: ld=0, pd=0, ldErrLba=3c65098, pdErrLba=3c65098


04/26 12:32:59: BBMNvramAddEntry: ld=0, pd=1, ldErrLba=3c65098, pdErrLba=3c65098


04/26 12:33:01: REC:log MedErr on pd[0] #retries=0


04/26 12:33:01: LogSense: pd=00, cdb=28 00 03 c6 64 80 00 00 80 00 


04/26 12:33:01:           sense=f0 00 03 03 c6 64 d8 0a 18 a8 49 65 11 00 00 00 00 a5 


04/26 12:33:01: REC: MedErr on LD[0] BadLba=3c664d8


04/26 12:33:01: BBMProcessReadError: REBUILD, pd=00, pdErrLba=3c664d8 - puncture source/target drives


04/26 12:33:01: bbmMarkBadBlock: pd=00, pdLBA=3c664d8


04/26 12:33:01: bbmMarkBadBlock: pd=01, pdLBA=3c664d8


04/26 12:33:01: BBMNvramAddEntry: ld=0, pd=0, ldErrLba=3c664d8, pdErrLba=3c664d8


04/26 12:33:01: BBMNvramAddEntry: ld=0, pd=1, ldErrLba=3c664d8, pdErrLba=3c664d8


04/26 14:53:20: LD 0 made consistent


04/26 14:53:20: Rebuild complete on LD 0, Arm 1, Span 0


04/26 14:53:20: Rebuild complete for C0, T1


04/26 14:53:20: R 0 MADE ONLINE


04/26 14:53:20: UpdateDrvInfo: notify config to slave


04/26 16:00:00: prCheckForBackgroundActivity: CC/BGI/Init active on ld=1


04/26 16:00:00: prDiskCheckOkToRun: PR NOT running due to background activity on adapter PD=00


04/26 16:00:00: prCheckForBackgroundActivity: CC/BGI/Init active on ld=1


04/26 16:00:00: prDiskCheckOkToRun: PR NOT running due to background activity on adapter PD=01


04/26 16:00:00: prCheckForBackgroundActivity: CC/BGI/Init active on ld=1


04/26 16:00:00: prDiskCheckOkToRun: PR NOT running due to background activity on adapter PD=12


04/26 16:00:00: prCheckForBackgroundActivity: CC/BGI/Init active on ld=1


04/26 16:00:00: prDiskCheckOkToRun: PR NOT running due to background activity on adapter PD=13


04/26 16:00:00: prCheckForBackgroundActivity: CC/BGI/Init active on ld=1


04/26 16:00:00: prDiskCheckOkToRun: PR NOT running due to background activity on adapter PD=14


04/26 16:00:00: Next PR scheduled to start at 04/26 20:00:00


04/26 17:48:05: Consistency check over 1


04/26 18:20:16: REC:log MedErr on pd[0] #retries=0


04/26 18:20:16: LogSense: pd=00, cdb=28 00 03 b1 84 00 00 00 80 00 


04/26 18:20:16:           sense=f0 00 03 03 b1 84 6e 0a 1a 20 47 b4 11 0b 00 00 00 79 


04/26 18:20:16: REC: MedErr on LD[0] BadLba=3b1846e


04/26 18:20:17: REC:log MedErr on pd[1] #retries=0


04/26 18:20:17: LogSense: pd=01, cdb=28 00 03 b1 84 00 00 00 80 00 


04/26 18:20:17:           sense=f0 00 03 03 b1 84 6e 0a 00 00 00 00 11 00 e4 00 00 8e 


04/26 18:20:17: REC: MedErr on LD[1] BadLba=3b1846e


04/26 18:20:18: Double media errors found! badLBA=3b1846e ld=0 pd1=0 pd2=1


04/26 18:20:18: LD 0 Check Consistency cmd FAILED - double errors


04/26 18:20:46: bginit started for 0


04/26 19:46:37: REC:log MedErr on pd[0] #retries=0


04/26 19:46:37: LogSense: pd=00, cdb=28 00 03 b1 84 00 00 00 80 00 


04/26 19:46:37:           sense=f0 00 03 03 b1 84 6e 0a 1a 20 47 b4 11 0b 00 00 00 79 


04/26 19:46:37: REC: MedErr on LD[0] BadLba=3b1846e


04/26 19:46:38: REC:log MedErr on pd[1] #retries=0


04/26 19:46:38: LogSense: pd=01, cdb=28 00 03 b1 84 00 00 00 80 00 


04/26 19:46:38:           sense=f0 00 03 03 b1 84 6e 0a 00 00 00 00 11 00 e4 00 00 8e 


04/26 19:46:38: REC: MedErr on LD[1] BadLba=3b1846e


04/26 19:46:38: Double media errors found! badLBA=3b1846e ld=0 pd1=0 pd2=1


04/26 19:46:39: REC:log MedErr on pd[0] #retries=0


04/26 19:46:39: LogSense: pd=00, cdb=28 00 03 b1 98 80 00 00 80 00 


04/26 19:46:39:           sense=f0 00 03 03 b1 98 ee 0a 18 c0 47 b5 11 0b 00 00 00 79 


04/26 19:46:39: REC: MedErr on LD[0] BadLba=3b198ee


04/26 19:46:41: REC:log MedErr on pd[1] #retries=0


04/26 19:46:41: LogSense: pd=01, cdb=28 00 03 b1 98 80 00 00 80 00 


04/26 19:46:41:           sense=f0 00 03 03 b1 98 ee 0a 00 00 00 00 11 00 e4 00 00 8e 


04/26 19:46:41: REC: MedErr on LD[1] BadLba=3b198ee


04/26 19:46:41: Double media errors found! badLBA=3b198ee ld=0 pd1=0 pd2=1


04/26 19:46:42: REC:log MedErr on pd[0] #retries=0


04/26 19:46:42: LogSense: pd=00, cdb=28 00 03 b1 9b 00 00 00 80 00 


04/26 19:46:42:           sense=f0 00 03 03 b1 9b 4e 0a 18 20 47 b6 11 0b 00 00 00 79 


04/26 19:46:42: REC: MedErr on LD[0] BadLba=3b19b4e


04/26 19:46:43: REC:log MedErr on pd[1] #retries=0


04/26 19:46:43: LogSense: pd=01, cdb=28 00 03 b1 9b 00 00 00 80 00 


04/26 19:46:43:           sense=f0 00 03 03 b1 9b 4e 0a 00 00 00 00 11 00 e4 00 00 8e 


04/26 19:46:43: REC: MedErr on LD[1] BadLba=3b19b4e


04/26 19:46:43: Double media errors found! badLBA=3b19b4e ld=0 pd1=0 pd2=1


04/26 19:47:51: REC:log MedErr on pd[0] #retries=0


04/26 19:47:51: LogSense: pd=00, cdb=28 00 03 bd 89 00 00 00 80 00 


04/26 19:47:51:           sense=f0 00 03 03 bd 89 65 0a 11 d4 48 ad 11 0b 00 00 00 79 


04/26 19:47:51: REC: MedErr on LD[0] BadLba=3bd8965


04/26 19:47:53: REC:log MedErr on pd[1] #retries=0


04/26 19:47:53: LogSense: pd=01, cdb=28 00 03 bd 89 00 00 00 80 00 


04/26 19:47:53:           sense=f0 00 03 03 bd 89 65 0a 00 00 00 00 11 00 e4 00 00 8e 


04/26 19:47:53: REC: MedErr on LD[1] BadLba=3bd8965


04/26 19:47:53: Double media errors found! badLBA=3bd8965 ld=0 pd1=0 pd2=1


04/26 19:47:54: REC:log MedErr on pd[0] #retries=0


04/26 19:47:54: LogSense: pd=00, cdb=28 00 03 bd 93 80 00 00 80 00 


04/26 19:47:54:           sense=f0 00 03 03 bd 93 b9 0a 13 02 48 ae 11 0b 00 00 00 79 


04/26 19:47:54: REC: MedErr on LD[0] BadLba=3bd93b9


04/26 19:47:55: REC:log MedErr on pd[1] #retries=0


04/26 19:47:55: LogSense: pd=01, cdb=28 00 03 bd 93 80 00 00 80 00 


04/26 19:47:55:           sense=f0 00 03 03 bd 93 b9 0a 00 00 00 00 11 00 e4 00 00 8e 


04/26 19:47:55: REC: MedErr on LD[1] BadLba=3bd93b9


04/26 19:47:56: Double media errors found! badLBA=3bd93b9 ld=0 pd1=0 pd2=1


04/26 19:48:05: REC:log MedErr on pd[0] #retries=0


04/26 19:48:05: LogSense: pd=00, cdb=28 00 03 bf 04 80 00 00 80 00 


04/26 19:48:05:           sense=f0 00 03 03 bf 04 f4 0a 12 df 48 cc 11 00 00 00 00 a5 


04/26 19:48:05: REC: MedErr on LD[0] BadLba=3bf04f4


04/26 19:48:06: REC:log MedErr on pd[1] #retries=0


04/26 19:48:06: LogSense: pd=01, cdb=28 00 03 bf 04 80 00 00 80 00 


04/26 19:48:06:           sense=f0 00 03 03 bf 04 f4 0a 00 00 00 00 11 00 e4 00 00 8e 


04/26 19:48:06: REC: MedErr on LD[1] BadLba=3bf04f4


04/26 19:48:07: Double media errors found! badLBA=3bf04f4 ld=0 pd1=0 pd2=1


04/26 19:48:12: REC:log MedErr on pd[1] #retries=0


04/26 19:48:12: LogSense: pd=01, cdb=28 00 03 bf 83 00 00 00 80 00 


04/26 19:48:12:           sense=f0 00 03 03 bf 83 6f 0a 00 00 00 00 11 00 e4 00 00 8e 


04/26 19:48:12: REC: MedErr on LD[1] BadLba=3bf836f


04/26 19:48:13: REC:log MedErr on pd[0] #retries=0


04/26 19:48:13: LogSense: pd=00, cdb=28 00 03 bf 83 00 00 00 80 00 


04/26 19:48:13:           sense=f0 00 03 03 bf 83 6f 0a 00 01 48 d6 11 0b 00 00 01 ab 


04/26 19:48:13: REC: MedErr on LD[0] BadLba=3bf836f


04/26 19:48:13: Double media errors found! badLBA=3bf836f ld=0 pd1=1 pd2=0


04/26 19:48:20: REC:log MedErr on pd[0] #retries=0


04/26 19:48:20: LogSense: pd=00, cdb=28 00 03 c0 59 00 00 00 80 00 


04/26 19:48:20:           sense=f0 00 03 03 c0 59 2f 0a 19 5f 48 e8 11 00 00 00 00 a5 


04/26 19:48:20: REC: MedErr on LD[0] BadLba=3c0592f


04/26 19:48:21: REC:log MedErr on pd[1] #retries=0


04/26 19:48:21: LogSense: pd=01, cdb=28 00 03 c0 59 00 00 00 80 00 


04/26 19:48:21:           sense=f0 00 03 03 c0 59 2f 0a 00 00 00 00 11 00 e4 00 00 8e 


04/26 19:48:21: REC: MedErr on LD[1] BadLba=3c0592f


04/26 19:48:21: Double media errors found! badLBA=3c0592f ld=0 pd1=0 pd2=1


04/26 19:48:23: REC:log MedErr on pd[0] #retries=0


04/26 19:48:23: LogSense: pd=00, cdb=28 00 03 c0 70 00 00 00 80 00 


04/26 19:48:23:           sense=f0 00 03 03 c0 70 6f 0a 1a ff 48 e9 11 00 00 00 00 a5 


04/26 19:48:23: REC: MedErr on LD[0] BadLba=3c0706f


04/26 19:48:24: REC:log MedErr on pd[1] #retries=0


04/26 19:48:24: LogSense: pd=01, cdb=28 00 03 c0 70 00 00 00 80 00 


04/26 19:48:24:           sense=f0 00 03 03 c0 70 6f 0a 00 00 00 00 11 00 e4 00 00 8e 


04/26 19:48:24: REC: MedErr on LD[1] BadLba=3c0706f


04/26 19:48:24: Double media errors found! badLBA=3c0706f ld=0 pd1=0 pd2=1


04/26 19:48:25: REC:log MedErr on pd[0] #retries=0


04/26 19:48:25: LogSense: pd=00, cdb=28 00 03 c0 72 80 00 00 80 00 


04/26 19:48:25:           sense=f0 00 03 03 c0 72 cf 0a 1a 5f 48 ea 11 00 00 00 00 a5 


04/26 19:48:25: REC: MedErr on LD[0] BadLba=3c072cf


04/26 19:48:26: REC:log MedErr on pd[1] #retries=0


04/26 19:48:26: LogSense: pd=01, cdb=28 00 03 c0 72 80 00 00 80 00 


04/26 19:48:26:           sense=f0 00 03 03 c0 72 cf 0a 00 00 00 00 11 00 e4 00 00 8e 


04/26 19:48:26: REC: MedErr on LD[1] BadLba=3c072cf


04/26 19:48:27: Double media errors found! badLBA=3c072cf ld=0 pd1=0 pd2=1


04/26 19:48:29: REC:log MedErr on pd[0] #retries=0


04/26 19:48:29: LogSense: pd=00, cdb=28 00 03 c0 87 80 00 00 80 00 


04/26 19:48:29:           sense=f0 00 03 03 c0 87 de 0a 19 ce 48 eb 11 00 00 00 00 a5 


04/26 19:48:29: REC: MedErr on LD[0] BadLba=3c087de


04/26 19:48:30: REC:log MedErr on pd[1] #retries=0


04/26 19:48:30: LogSense: pd=01, cdb=28 00 03 c0 87 80 00 00 80 00 


04/26 19:48:30:           sense=f0 00 03 03 c0 87 de 0a 00 00 00 00 11 00 e4 00 00 8e 


04/26 19:48:30: REC: MedErr on LD[1] BadLba=3c087de


04/26 19:48:30: Double media errors found! badLBA=3c087de ld=0 pd1=0 pd2=1


04/26 19:48:32: REC:log MedErr on pd[0] #retries=0


04/26 19:48:32: LogSense: pd=00, cdb=28 00 03 c0 8a 00 00 00 80 00 


04/26 19:48:32:           sense=f0 00 03 03 c0 8a 3e 0a 19 2e 48 ec 11 00 00 00 00 a5 


04/26 19:48:32: REC: MedErr on LD[0] BadLba=3c08a3e


04/26 19:48:33: REC:log MedErr on pd[1] #retries=0


04/26 19:48:33: LogSense: pd=01, cdb=28 00 03 c0 8a 00 00 00 80 00 


04/26 19:48:33:           sense=f0 00 03 03 c0 8a 3e 0a 00 00 00 00 11 00 e4 00 00 8e 


04/26 19:48:33: REC: MedErr on LD[1] BadLba=3c08a3e


04/26 19:48:33: Double media errors found! badLBA=3c08a3e ld=0 pd1=0 pd2=1


04/26 19:48:36: REC:log MedErr on pd[1] #retries=0


04/26 19:48:36: LogSense: pd=01, cdb=28 00 03 c0 a3 80 00 00 80 00 


04/26 19:48:36:           sense=f0 00 03 03 c0 a3 de 0a 00 00 00 00 11 00 e4 00 00 8e 


04/26 19:48:36: REC: MedErr on LD[1] BadLba=3c0a3de


04/26 19:48:36: REC:log MedErr on pd[0] #retries=0


04/26 19:48:36: LogSense: pd=00, cdb=28 00 03 c0 a3 80 00 00 80 00 


04/26 19:48:36:           sense=f0 00 03 03 c0 a3 de 0a 1a 2e 48 ee 16 00 00 00 01 c6 


04/26 19:48:36: REC: MedErr on LD[0] BadLba=3c0a3de


04/26 19:48:36: Double media errors found! badLBA=3c0a3de ld=0 pd1=1 pd2=0


04/26 19:48:39: REC:log MedErr on pd[0] #retries=0


04/26 19:48:39: LogSense: pd=00, cdb=28 00 03 c0 b8 00 00 00 80 00 


04/26 19:48:39:           sense=f0 00 03 03 c0 b8 1d 0a 18 cd 48 ef 11 00 00 00 00 a5 


04/26 19:48:39: REC: MedErr on LD[0] BadLba=3c0b81d


04/26 19:48:41: REC:log MedErr on pd[1] #retries=0


04/26 19:48:41: LogSense: pd=01, cdb=28 00 03 c0 b8 00 00 00 80 00 


04/26 19:48:41:           sense=f0 00 03 03 c0 b8 1d 0a 00 00 00 00 11 00 e4 00 00 8e 


04/26 19:48:41: REC: MedErr on LD[1] BadLba=3c0b81d


04/26 19:48:41: REC:log MedErr on pd[0] #retries=4


04/26 19:48:41: LogSense: pd=00, cdb=28 00 03 c0 b8 1e 00 00 62 00 


04/26 19:48:41:           sense=f0 00 03 03 c0 b8 1e 0a 18 ce 48 ef 11 00 00 00 00 a5 


04/26 19:48:41: REC: MedErr on LD[0] BadLba=3c0b81e


04/26 19:48:43: REC:log MedErr on pd[1] #retries=4


04/26 19:48:43: LogSense: pd=01, cdb=28 00 03 c0 b8 1e 00 00 62 00 


04/26 19:48:43:           sense=f0 00 03 03 c0 b8 1e 0a 00 00 00 00 11 00 e4 00 00 8e 


04/26 19:48:43: REC: MedErr on LD[1] BadLba=3c0b81e


04/26 19:48:43: Double media errors found! badLBA=3c0b81d ld=0 pd1=0 pd2=1


04/26 19:48:45: REC:log MedErr on pd[0] #retries=0


04/26 19:48:45: LogSense: pd=00, cdb=28 00 03 c0 ba 00 00 00 80 00 


04/26 19:48:45:           sense=f0 00 03 03 c0 ba 7e 0a 18 2e 48 f0 11 00 00 00 00 a5 


04/26 19:48:45: REC: MedErr on LD[0] BadLba=3c0ba7e


04/26 19:48:46: REC:log MedErr on pd[1] #retries=0


04/26 19:48:46: LogSense: pd=01, cdb=28 00 03 c0 ba 00 00 00 80 00 


04/26 19:48:46:           sense=f0 00 03 03 c0 ba 7e 0a 00 00 00 00 11 00 e4 00 00 8e 


04/26 19:48:46: REC: MedErr on LD[1] BadLba=3c0ba7e


04/26 19:48:46: Double media errors found! badLBA=3c0ba7e ld=0 pd1=0 pd2=1


04/26 19:48:47: REC:log MedErr on pd[0] #retries=0


04/26 19:48:47: LogSense: pd=00, cdb=28 00 03 c0 d1 80 00 00 80 00 


04/26 19:48:47:           sense=f0 00 03 03 c0 d1 be 0a 19 ce 48 f1 11 00 00 00 00 a6 


04/26 19:48:47: REC: MedErr on LD[0] BadLba=3c0d1be


04/26 19:48:49: REC:log MedErr on pd[1] #retries=0


04/26 19:48:49: LogSense: pd=01, cdb=28 00 03 c0 d1 80 00 00 80 00 


04/26 19:48:49:           sense=f0 00 03 03 c0 d1 be 0a 00 00 00 00 11 00 e4 00 00 8e 


04/26 19:48:49: REC: MedErr on LD[1] BadLba=3c0d1be


04/26 19:48:49: Double media errors found! badLBA=3c0d1be ld=0 pd1=0 pd2=1


04/26 19:48:50: REC:log MedErr on pd[0] #retries=0


04/26 19:48:50: LogSense: pd=00, cdb=28 00 03 c0 eb 00 00 00 80 00 


04/26 19:48:50:           sense=f0 00 03 03 c0 eb 53 0a 1a c3 48 f3 11 00 00 00 00 a5 


04/26 19:48:50: REC: MedErr on LD[0] BadLba=3c0eb53


04/26 19:48:52: REC:log MedErr on pd[1] #retries=0


04/26 19:48:52: LogSense: pd=01, cdb=28 00 03 c0 eb 00 00 00 80 00 


04/26 19:48:52:           sense=f0 00 03 03 c0 eb 53 0a 00 00 00 00 11 00 e4 00 00 8e 


04/26 19:48:52: REC: MedErr on LD[1] BadLba=3c0eb53


04/26 19:48:52: Double media errors found! badLBA=3c0eb53 ld=0 pd1=0 pd2=1


04/26 19:48:53: REC:log MedErr on pd[0] #retries=0


04/26 19:48:53: LogSense: pd=00, cdb=28 00 03 c1 01 80 00 00 80 00 


04/26 19:48:53:           sense=f0 00 03 03 c1 01 f3 0a 18 c3 48 f5 11 00 00 00 00 a5 


04/26 19:48:53: REC: MedErr on LD[0] BadLba=3c101f3


04/26 19:48:55: REC:log MedErr on pd[1] #retries=0


04/26 19:48:55: LogSense: pd=01, cdb=28 00 03 c1 01 80 00 00 80 00 


04/26 19:48:55:           sense=f0 00 03 03 c1 01 f3 0a 00 00 00 00 11 00 e4 00 00 8e 


04/26 19:48:55: REC: MedErr on LD[1] BadLba=3c101f3


04/26 19:48:55: Double media errors found! badLBA=3c101f3 ld=0 pd1=0 pd2=1


04/26 19:48:56: REC:log MedErr on pd[0] #retries=0


04/26 19:48:56: LogSense: pd=00, cdb=28 00 03 c1 04 00 00 00 80 00 


04/26 19:48:56:           sense=f0 00 03 03 c1 04 53 0a 18 23 48 f6 11 00 00 00 00 a5 


04/26 19:48:56: REC: MedErr on LD[0] BadLba=3c10453


04/26 19:48:57: REC:log MedErr on pd[1] #retries=0


04/26 19:48:57: LogSense: pd=01, cdb=28 00 03 c1 04 00 00 00 80 00 


04/26 19:48:57:           sense=f0 00 03 03 c1 04 53 0a 00 00 00 00 11 00 e4 00 00 8e 


04/26 19:48:57: REC: MedErr on LD[1] BadLba=3c10453


04/26 19:48:57: Double media errors found! badLBA=3c10453 ld=0 pd1=0 pd2=1


04/26 19:49:01: REC:log MedErr on pd[0] #retries=0


04/26 19:49:01: LogSense: pd=00, cdb=28 00 03 c1 5d 00 00 00 80 00 


04/26 19:49:01:           sense=f0 00 03 03 c1 5d 6c 0a 03 0c 48 fd 11 00 00 00 00 a5 


04/26 19:49:01: REC: MedErr on LD[0] BadLba=3c15d6c


04/26 19:49:02: REC:log MedErr on pd[1] #retries=0


04/26 19:49:02: LogSense: pd=01, cdb=28 00 03 c1 5d 00 00 00 80 00 


04/26 19:49:02:           sense=f0 00 03 03 c1 5d 6c 0a 00 00 00 00 11 00 e4 00 00 8e 


04/26 19:49:02: REC: MedErr on LD[1] BadLba=3c15d6c


04/26 19:49:03: Double media errors found! badLBA=3c15d6c ld=0 pd1=0 pd2=1


04/26 19:49:04: REC:log MedErr on pd[0] #retries=0


04/26 19:49:04: LogSense: pd=00, cdb=28 00 03 c1 71 80 00 00 80 00 


04/26 19:49:04:           sense=f0 00 03 03 c1 71 8d 0a 01 a2 48 fe 11 00 00 00 00 a6 


04/26 19:49:04: REC: MedErr on LD[0] BadLba=3c1718d


04/26 19:49:05: REC:log MedErr on pd[1] #retries=0


04/26 19:49:05: LogSense: pd=01, cdb=28 00 03 c1 71 80 00 00 80 00 


04/26 19:49:05:           sense=f0 00 03 03 c1 71 8d 0a 00 00 00 00 11 00 e4 00 00 8e 


04/26 19:49:05: REC: MedErr on LD[1] BadLba=3c1718d


04/26 19:49:06: Double media errors found! badLBA=3c1718d ld=0 pd1=0 pd2=1


04/26 19:49:35: REC:log MedErr on pd[0] #retries=0


04/26 19:49:35: LogSense: pd=00, cdb=28 00 03 c6 4e 00 00 00 80 00 


04/26 19:49:35:           sense=f0 00 03 03 c6 4e 38 0a 1a a8 49 63 11 00 00 00 00 a5 


04/26 19:49:35: REC: MedErr on LD[0] BadLba=3c64e38


04/26 19:49:36: REC:log MedErr on pd[1] #retries=0


04/26 19:49:36: LogSense: pd=01, cdb=28 00 03 c6 4e 00 00 00 80 00 


04/26 19:49:�����€��      sense=f0 00 03 03 c6 4e 38 0a 00 00 00 00 11 00 e4 00 00 8e 


04/26 19:49:36: REC: MedErr on LD[1] BadLba=3c64e38


04/26 19:49:36: Double media errors found! badLBA=3c64e38 ld=0 pd1=0 pd2=1


04/26 19:49:37: REC:log MedErr on pd[0] #retries=0


04/26 19:49:37: LogSense: pd=00, cdb=28 00 03 c6 50 80 00 00 80 00 


04/26 19:49:37:           sense=f0 00 03 03 c6 50 98 0a 1a 08 49 64 11 00 00 00 00 a5 


04/26 19:49:37: REC: MedErr on LD[0] BadLba=3c65098


04/26 19:49:38: REC:log MedErr on pd[1] #retries=0


04/26 19:49:38: LogSense: pd=01, cdb=28 00 03 c6 50 80 00 00 80 00 


04/26 19:49:38:           sense=f0 00 03 03 c6 50 98 0a 00 00 00 00 11 00 e4 00 00 8e 


04/26 19:49:38: REC: MedErr on LD[1] BadLba=3c65098


04/26 19:49:39: Double media errors found! badLBA=3c65098 ld=0 pd1=0 pd2=1


04/26 19:49:40: REC:log MedErr on pd[0] #retries=0


04/26 19:49:40: LogSense: pd=00, cdb=28 00 03 c6 64 80 00 00 80 00 


04/26 19:49:40:           sense=f0 00 03 03 c6 64 d8 0a 18 a8 49 65 11 00 00 00 00 a5 


04/26 19:49:40: REC: MedErr on LD[0] BadLba=3c664d8


04/26 19:49:41: REC:log MedErr on pd[1] #retries=0


04/26 19:49:41: LogSense: pd=01, cdb=28 00 03 c6 64 80 00 00 80 00 


04/26 19:49:41:           sense=f0 00 03 03 c6 64 d8 0a 00 00 00 00 11 00 e4 00 00 8e 


04/26 19:49:41: REC: MedErr on LD[1] BadLba=3c664d8


04/26 19:49:41: Double media errors found! badLBA=3c664d8 ld=0 pd1=0 pd2=1


04/26 20:00:00: prCheckForBackgroundActivity: CC/BGI/Init active on ld=0


04/26 20:00:00: prDiskCheckOkToRun: PR NOT running due to background activity on adapter PD=00


04/26 20:00:00: prCheckForBackgroundActivity: CC/BGI/Init active on ld=0


04/26 20:00:00: prDiskCheckOkToRun: PR NOT running due to background activity on adapter PD=01


04/26 20:00:00: prCheckForBackgroundActivity: CC/BGI/Init active on ld=0


04/26 20:00:00: prDiskCheckOkToRun: PR NOT running due to background activity on adapter PD=12


04/26 20:00:00: prCheckForBackgroundActivity: CC/BGI/Init active on ld=0


04/26 20:00:00: prDiskCheckOkToRun: PR NOT running due to background activity on adapter PD=13


04/26 20:00:00: prCheckForBackgroundActivity: CC/BGI/Init active on ld=0


04/26 20:00:00: prDiskCheckOkToRun: PR NOT running due to background activity on adapter PD=14


04/26 20:00:00: Next PR scheduled to start at 04/27  0:00:00


04/26 21:44:39: Consistency check over 0


04/27  0:00:00: prDiskStart: starting Patrol Read on PD=00


04/27  0:00:00: prDiskStart: starting Patrol Read on PD=01


04/27  0:00:00: prDiskStart: starting Patrol Read on PD=12


04/27  0:00:00: prDiskStart: starting Patrol Read on PD=13


04/27  0:00:00: prDiskStart: starting Patrol Read on PD=14


04/27  0:00:00: Next PR scheduled to start at 04/27  4:00:00


04/27  1:07:25: REC:log MedErr on pd[0] #retries=0


04/27  1:07:25: LogSense: pd=00, cdb=2f 00 03 b1 33 8f 00 ff ff 00 


04/27  1:07:25:           sense=f0 00 03 03 b1 84 6e 0a 1a 20 47 b4 11 0b 00 00 00 79 


04/27  1:07:25: REC: MedErr on LD[0] BadLba=3b1846e


04/27  1:07:25: prCallback: Medium Error on pd=00, StartLba=3b1338f, ErrLba=3b1846e


04/27  1:07:25: prRecQueue: starting pd=00 recovery - blocking host commands


04/27  1:07:25: prRecGo: Ready to attempt recovery errLBA=3b1846e on pd=00


04/27  1:07:25: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/27  1:07:25: prRecGo: dataErr found on ld 0 span 0 arm 0


04/27  1:07:25: prRecGo: data NOT in cache; cacheLn=ffffffff, row=76308, stripe=76308, refBlk=6e, type=1


04/27  1:07:25: prRecGo: RAID1 - read from peer pd=01


04/27  1:07:26: REC:log MedErr on pd[1] #retries=0


04/27  1:07:26: LogSense: pd=01, cdb=2f 00 03 b1 33 8f 00 ff ff 00 


04/27  1:07:26:           sense=f0 00 03 03 b1 84 6e 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  1:07:26: REC: MedErr on LD[1] BadLba=3b1846e


04/27  1:07:26: prCallback: Medium Error on pd=01, StartLba=3b1338f, ErrLba=3b1846e


04/27  1:07:26: prRecQueue: adding pd=01 to recovery wait queue


04/27  1:07:28: REC:log MedErr on pd[1] #retries=0


04/27  1:07:28: LogSense: pd=01, cdb=28 00 03 b1 84 6e 00 00 01 00 


04/27  1:07:28:           sense=f0 00 03 03 b1 84 6e 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  1:07:28: REC: MedErr on LD[1] BadLba=3b1846e


04/27  1:07:28: prRecCallback: Error, cdb=28, pd=01, err=2


04/27  1:07:28: PatrolReadStart: Log MedErr pd=00 errLBA=3b1846e in NVRAM_EVENT_LOG


04/27  1:07:28: prRecGo: Ready to attempt recovery errLBA=3b1846e on pd=01


04/27  1:07:28: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/27  1:07:28: prRecGo: dataErr found on ld 0 span 0 arm 1


04/27  1:07:28: prRecGo: data NOT in cache; cacheLn=ffffffff, row=76308, stripe=76308, refBlk=6e, type=1


04/27  1:07:28: prRecGo: RAID1 - read from peer pd=00


04/27  1:07:29: REC:log MedErr on pd[0] #retries=0


04/27  1:07:29: LogSense: pd=00, cdb=28 00 03 b1 84 6e 00 00 01 00 


04/27  1:07:29:           sense=f0 00 03 03 b1 84 6e 0a 1a 20 47 b4 11 0b 00 00 00 79 


04/27  1:07:29: REC: MedErr on LD[0] BadLba=3b1846e


04/27  1:07:29: prRecCallback: Error, cdb=28, pd=00, err=2


04/27  1:07:29: PatrolReadStart: Log MedErr pd=01 errLBA=3b1846e in NVRAM_EVENT_LOG


04/27  1:07:33: REC:log MedErr on pd[0] #retries=0


04/27  1:07:33: LogSense: pd=00, cdb=2f 00 03 b1 84 6f 00 ff ff 00 


04/27  1:07:33:           sense=f0 00 03 03 b1 98 ee 0a 18 c0 47 b5 11 0b 00 00 00 79 


04/27  1:07:33: REC: MedErr on LD[0] BadLba=3b198ee


04/27  1:07:33: prCallback: Medium Error on pd=00, StartLba=3b1846f, ErrLba=3b198ee


04/27  1:07:33: prRecQueue: starting pd=00 recovery - blocking host commands


04/27  1:07:33: prRecGo: Ready to attempt recovery errLBA=3b198ee on pd=00


04/27  1:07:33: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/27  1:07:33: prRecGo: dataErr found on ld 0 span 0 arm 0


04/27  1:07:33: prRecGo: data NOT in cache; cacheLn=ffffffff, row=76331, stripe=76331, refBlk=6e, type=1


04/27  1:07:33: prRecGo: RAID1 - read from peer pd=01


04/27  1:07:35: REC:log MedErr on pd[1] #retries=0


04/27  1:07:35: LogSense: pd=01, cdb=2f 00 03 b1 84 6f 00 ff ff 00 


04/27  1:07:35:           sense=f0 00 03 03 b1 98 ee 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  1:07:35: REC: MedErr on LD[1] BadLba=3b198ee


04/27  1:07:35: prCallback: Medium Error on pd=01, StartLba=3b1846f, ErrLba=3b198ee


04/27  1:07:35: prRecQueue: adding pd=01 to recovery wait queue


04/27  1:07:37: REC:log MedErr on pd[1] #retries=0


04/27  1:07:37: LogSense: pd=01, cdb=28 00 03 b1 98 ee 00 00 01 00 


04/27  1:07:37:           sense=f0 00 03 03 b1 98 ee 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  1:07:37: REC: MedErr on LD[1] BadLba=3b198ee


04/27  1:07:37: prRecCallback: Error, cdb=28, pd=01, err=2


04/27  1:07:37: PatrolReadStart: Log MedErr pd=00 errLBA=3b198ee in NVRAM_EVENT_LOG


04/27  1:07:37: prRecGo: Ready to attempt recovery errLBA=3b198ee on pd=01


04/27  1:07:37: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/27  1:07:37: prRecGo: dataErr found on ld 0 span 0 arm 1


04/27  1:07:37: prRecGo: data NOT in cache; cacheLn=ffffffff, row=76331, stripe=76331, refBlk=6e, type=1


04/27  1:07:37: prRecGo: RAID1 - read from peer pd=00


04/27  1:07:38: REC:log MedErr on pd[0] #retries=0


04/27  1:07:38: LogSense: pd=00, cdb=28 00 03 b1 98 ee 00 00 01 00 


04/27  1:07:38:           sense=f0 00 03 03 b1 98 ee 0a 18 c0 47 b5 11 0b 00 00 00 79 


04/27  1:07:38: REC: MedErr on LD[0] BadLba=3b198ee


04/27  1:07:38: prRecCallback: Error, cdb=28, pd=00, err=2


04/27  1:07:38: PatrolReadStart: Log MedErr pd=01 errLBA=3b198ee in NVRAM_EVENT_LOG


04/27  1:07:41: REC:log MedErr on pd[0] #retries=0


04/27  1:07:41: LogSense: pd=00, cdb=2f 00 03 b1 98 ef 00 ff ff 00 


04/27  1:07:41:           sense=f0 00 03 03 b1 9b 4e 0a 18 20 47 b6 11 0b 00 00 00 79 


04/27  1:07:41: REC: MedErr on LD[0] BadLba=3b19b4e


04/27  1:07:41: prCallback: Medium Error on pd=00, StartLba=3b198ef, ErrLba=3b19b4e


04/27  1:07:41: prRecQueue: starting pd=00 recovery - blocking host commands


04/27  1:07:41: prRecGo: Ready to attempt recovery errLBA=3b19b4e on pd=00


04/27  1:07:41: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/27  1:07:41: prRecGo: dataErr found on ld 0 span 0 arm 0


04/27  1:07:41: prRecGo: data NOT in cache; cacheLn=ffffffff, row=76336, stripe=76336, refBlk=4e, type=1


04/27  1:07:42: prRecGo: RAID1 - read from peer pd=01


04/27  1:07:44: REC:log MedErr on pd[1] #retries=0


04/27  1:07:44: LogSense: pd=01, cdb=28 00 03 b1 9b 4e 00 00 01 00 


04/27  1:07:44:           sense=f0 00 03 03 b1 9b 4e 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  1:07:44: REC: MedErr on LD[1] BadLba=3b19b4e


04/27  1:07:44: prRecCallback: Error, cdb=28, pd=01, err=2


04/27  1:07:44: PatrolReadStart: Log MedErr pd=00 errLBA=3b19b4e in NVRAM_EVENT_LOG


04/27  1:07:46: REC:log MedErr on pd[1] #retries=0


04/27  1:07:46: LogSense: pd=01, cdb=2f 00 03 b1 98 ef 00 08 00 00 


04/27  1:07:46:           sense=f0 00 03 03 b1 9b 4e 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  1:07:46: REC: MedErr on LD[1] BadLba=3b19b4e


04/27  1:07:46: prCallback: Medium Error on pd=01, StartLba=3b198ef, ErrLba=3b19b4e


04/27  1:07:46: prRecQueue: starting pd=01 recovery - blocking host commands


04/27  1:07:46: prRecGo: Ready to attempt recovery errLBA=3b19b4e on pd=01


04/27  1:07:46: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/27  1:07:46: prRecGo: dataErr found on ld 0 span 0 arm 1


04/27  1:07:46: prRecGo: data NOT in cache; cacheLn=ffffffff, row=76336, stripe=76336, refBlk=4e, type=1


04/27  1:07:46: prRecGo: RAID1 - read from peer pd=00


04/27  1:07:47: REC:log MedErr on pd[0] #retries=0


04/27  1:07:47: LogSense: pd=00, cdb=28 00 03 b1 9b 4e 00 00 01 00 


04/27  1:07:47:           sense=f0 00 03 03 b1 9b 4e 0a 18 20 47 b6 11 0b 00 00 00 79 


04/27  1:07:47: REC: MedErr on LD[0] BadLba=3b19b4e


04/27  1:07:47: prRecCallback: Error, cdb=28, pd=00, err=2


04/27  1:07:47: PatrolReadStart: Log MedErr pd=01 errLBA=3b19b4e in NVRAM_EVENT_LOG


04/27  1:08:33: REC:log MedErr on pd[0] #retries=0


04/27  1:08:33: LogSense: pd=00, cdb=2f 00 03 bc 9b 44 00 ff ff 00 


04/27  1:08:33:           sense=f0 00 03 03 bd 89 65 0a 11 d4 48 ad 11 0b 00 00 00 79 


04/27  1:08:33: REC: MedErr on LD[0] BadLba=3bd8965


04/27  1:08:33: prCallback: Medium Error on pd=00, StartLba=3bc9b44, ErrLba=3bd8965


04/27  1:08:33: prRecQueue: starting pd=00 recovery - blocking host commands


04/27  1:08:33: prRecGo: Ready to attempt recovery errLBA=3bd8965 on pd=00


04/27  1:08:33: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/27  1:08:33: prRecGo: dataErr found on ld 0 span 0 arm 0


04/27  1:08:33: prRecGo: data NOT in cache; cacheLn=ffffffff, row=77b12, stripe=77b12, refBlk=65, type=1


04/27  1:08:33: prRecGo: RAID1 - read from peer pd=01


04/27  1:08:35: REC:log MedErr on pd[1] #retries=0


04/27  1:08:35: LogSense: pd=01, cdb=28 00 03 bd 89 65 00 00 01 00 


04/27  1:08:35:           sense=f0 00 03 03 bd 89 65 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  1:08:35: REC: MedErr on LD[1] BadLba=3bd8965


04/27  1:08:35: prRecCallback: Error, cdb=28, pd=01, err=2


04/27  1:08:35: PatrolReadStart: Log MedErr pd=00 errLBA=3bd8965 in NVRAM_EVENT_LOG


04/27  1:08:39: REC:log MedErr on pd[0] #retries=0


04/27  1:08:39: LogSense: pd=00, cdb=2f 00 03 bd 89 66 00 ff ff 00 


04/27  1:08:39:           sense=f0 00 03 03 bd 93 b9 0a 13 02 48 ae 11 0b 00 00 00 79 


04/27  1:08:39: REC: MedErr on LD[0] BadLba=3bd93b9


04/27  1:08:39: prCallback: Medium Error on pd=00, StartLba=3bd8966, ErrLba=3bd93b9


04/27  1:08:39: prRecQueue: starting pd=00 recovery - blocking host commands


04/27  1:08:39: prRecGo: Ready to attempt recovery errLBA=3bd93b9 on pd=00


04/27  1:08:39: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/27  1:08:39: prRecGo: dataErr found on ld 0 span 0 arm 0


04/27  1:08:40: prRecGo: data NOT in cache; cacheLn=ffffffff, row=77b27, stripe=77b27, refBlk=39, type=1


04/27  1:08:40: prRecGo: RAID1 - read from peer pd=01


04/27  1:08:42: REC:log MedErr on pd[1] #retries=0


04/27  1:08:42: LogSense: pd=01, cdb=28 00 03 bd 93 b9 00 00 01 00 


04/27  1:08:42:           sense=f0 00 03 03 bd 93 b9 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  1:08:42: REC: MedErr on LD[1] BadLba=3bd93b9


04/27  1:08:42: prRecCallback: Error, cdb=28, pd=01, err=2


04/27  1:08:42: PatrolReadStart: Log MedErr pd=00 errLBA=3bd93b9 in NVRAM_EVENT_LOG


04/27  1:08:45: REC:log MedErr on pd[1] #retries=0


04/27  1:08:45: LogSense: pd=01, cdb=2f 00 03 bc a3 84 00 ff ff 00 


04/27  1:08:45:           sense=f0 00 03 03 bd 89 65 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  1:08:45: REC: MedErr on LD[1] BadLba=3bd8965


04/27  1:08:45: prCallback: Medium Error on pd=01, StartLba=3bca384, ErrLba=3bd8965


04/27  1:08:45: prRecQueue: starting pd=01 recovery - blocking host commands


04/27  1:08:45: prRecGo: Ready to attempt recovery errLBA=3bd8965 on pd=01


04/27  1:08:45: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/27  1:08:45: prRecGo: dataErr found on ld 0 span 0 arm 1


04/27  1:08:45: prRecGo: data NOT in cache; cacheLn=ffffffff, row=77b12, stripe=77b12, refBlk=65, type=1


04/27  1:08:45: prRecGo: RAID1 - read from peer pd=00


04/27  1:08:46: REC:log MedErr on pd[0] #retries=0


04/27  1:08:46: LogSense: pd=00, cdb=28 00 03 bd 89 65 00 00 01 00 


04/27  1:08:46:           sense=f0 00 03 03 bd 89 65 0a 11 d4 48 ad 11 0b 00 00 00 79 


04/27  1:08:46: REC: MedErr on LD[0] BadLba=3bd8965


04/27  1:08:46: prRecCallback: Error, cdb=28, pd=00, err=2


04/27  1:08:46: PatrolReadStart: Log MedErr pd=01 errLBA=3bd8965 in NVRAM_EVENT_LOG


04/27  1:08:52: REC:log MedErr on pd[1] #retries=0


04/27  1:08:52: LogSense: pd=01, cdb=2f 00 03 bd 89 66 00 ff ff 00 


04/27  1:08:52:           sense=f0 00 03 03 bd 93 b9 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  1:08:52: REC: MedErr on LD[1] BadLba=3bd93b9


04/27  1:08:52: prCallback: Medium Error on pd=01, StartLba=3bd8966, ErrLba=3bd93b9


04/27  1:08:52: prRecQueue: starting pd=01 recovery - blocking host commands


04/27  1:08:52: prRecGo: Ready to attempt recovery errLBA=3bd93b9 on pd=01


04/27  1:08:52: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/27  1:08:52: prRecGo: dataErr found on ld 0 span 0 arm 1


04/27  1:08:52: prRecGo: data NOT in cache; cacheLn=ffffffff, row=77b27, stripe=77b27, refBlk=39, type=1


04/27  1:08:52: prRecGo: RAID1 - read from peer pd=00


04/27  1:08:53: REC:log MedErr on pd[0] #retries=0


04/27  1:08:53: LogSense: pd=00, cdb=28 00 03 bd 93 b9 00 00 01 00 


04/27  1:08:53:           sense=f0 00 03 03 bd 93 b9 0a 13 02 48 ae 11 0b 00 00 00 79 


04/27  1:08:53: REC: MedErr on LD[0] BadLba=3bd93b9


04/27  1:08:53: prRecCallback: Error, cdb=28, pd=00, err=2


04/27  1:08:53: PatrolReadStart: Log MedErr pd=01 errLBA=3bd93b9 in NVRAM_EVENT_LOG


04/27  1:08:55: REC:log MedErr on pd[0] #retries=0


04/27  1:08:55: LogSense: pd=00, cdb=2f 00 03 be 9b b9 00 ff ff 00 


04/27  1:08:55:           sense=f0 00 03 03 bf 04 f4 0a 12 df 48 cc 11 00 00 00 00 a5 


04/27  1:08:55: REC: MedErr on LD[0] BadLba=3bf04f4


04/27  1:08:55: prCallback: Medium Error on pd=00, StartLba=3be9bb9, ErrLba=3bf04f4


04/27  1:08:55: prRecQueue: starting pd=00 recovery - blocking host commands


04/27  1:08:55: prRecGo: Ready to attempt recovery errLBA=3bf04f4 on pd=00


04/27  1:08:55: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/27  1:08:55: prRecGo: dataErr found on ld 0 span 0 arm 0


04/27  1:08:55: prRecGo: data NOT in cache; cacheLn=ffffffff, row=77e09, stripe=77e09, refBlk=74, type=1


04/27  1:08:55: prRecGo: RAID1 - read from peer pd=01


04/27  1:08:57: REC:log MedErr on pd[1] #retries=0


04/27  1:08:57: LogSense: pd=01, cdb=28 00 03 bf 04 f4 00 00 01 00 


04/27  1:08:57:           sense=f0 00 03 03 bf 04 f4 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  1:08:57: REC: MedErr on LD[1] BadLba=3bf04f4


04/27  1:08:57: prRecCallback: Error, cdb=28, pd=01, err=2


04/27  1:08:57: PatrolReadStart: Log MedErr pd=00 errLBA=3bf04f4 in NVRAM_EVENT_LOG


04/27  1:09:04: REC:log MedErr on pd[0] #retries=0


04/27  1:09:04: LogSense: pd=00, cdb=2f 00 03 bf 04 f5 00 ff ff 00 


04/27  1:09:04:           sense=f0 00 03 03 bf 83 6f 0a 00 01 48 d6 11 0b 00 00 01 ab 


04/27  1:09:04: REC: MedErr on LD[0] BadLba=3bf836f


04/27  1:09:04: prCallback: Medium Error on pd=00, StartLba=3bf04f5, ErrLba=3bf836f


04/27  1:09:04: prRecQueue: starting pd=00 recovery - blocking host commands


04/27  1:09:04: prRecGo: Ready to attempt recovery errLBA=3bf836f on pd=00


04/27  1:09:04: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/27  1:09:04: prRecGo: dataErr found on ld 0 span 0 arm 0


04/27  1:09:04: prRecGo: data NOT in cache; cacheLn=ffffffff, row=77f06, stripe=77f06, refBlk=6f, type=1


04/27  1:09:04: prRecGo: RAID1 - read from peer pd=01


04/27  1:09:06: REC:log MedErr on pd[1] #retries=0


04/27  1:09:06: LogSense: pd=01, cdb=28 00 03 bf 83 6f 00 00 01 00 


04/27  1:09:06:           sense=f0 00 03 03 bf 83 6f 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  1:09:06: REC: MedErr on LD[1] BadLba=3bf836f


04/27  1:09:06: prRecCallback: Error, cdb=28, pd=01, err=2


04/27  1:09:06: PatrolReadStart: Log MedErr pd=00 errLBA=3bf836f in NVRAM_EVENT_LOG


04/27  1:09:11: REC:log MedErr on pd[0] #retries=0


04/27  1:09:11: LogSense: pd=00, cdb=2f 00 03 bf 83 70 00 ff ff 00 


04/27  1:09:11:           sense=f0 00 03 03 c0 59 2f 0a 19 5f 48 e8 11 00 00 00 00 a5 


04/27  1:09:11: REC: MedErr on LD[0] BadLba=3c0592f


04/27  1:09:11: prCallback: Medium Error on pd=00, StartLba=3bf8370, ErrLba=3c0592f


04/27  1:09:11: prRecQueue: starting pd=00 recovery - blocking host commands


04/27  1:09:11: prRecGo: Ready to attempt recovery errLBA=3c0592f on pd=00


04/27  1:09:11: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/27  1:09:11: prRecGo: dataErr found on ld 0 span 0 arm 0


04/27  1:09:11: prRecGo: data NOT in cache; cacheLn=ffffffff, row=780b2, stripe=780b2, refBlk=2f, type=1


04/27  1:09:11: prRecGo: RAID1 - read from peer pd=01


04/27  1:09:12: REC:log MedErr on pd[1] #retries=0


04/27  1:09:12: LogSense: pd=01, cdb=2f 00 03 be 9b f9 00 ff ff 00 


04/27  1:09:12:           sense=f0 00 03 03 bf 04 f4 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  1:09:12: REC: MedErr on LD[1] BadLba=3bf04f4


04/27  1:09:12: prCallback: Medium Error on pd=01, StartLba=3be9bf9, ErrLba=3bf04f4


04/27  1:09:12: prRecQueue: adding pd=01 to recovery wait queue


04/27  1:09:14: REC:log MedErr on pd[1] #retries=0


04/27  1:09:14: LogSense: pd=01, cdb=28 00 03 c0 59 2f 00 00 01 00 


04/27  1:09:14:           sense=f0 00 03 03 c0 59 2f 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  1:09:14: REC: MedErr on LD[1] BadLba=3c0592f


04/27  1:09:14: prRecCallback: Error, cdb=28, pd=01, err=2


04/27  1:09:14: PatrolReadStart: Log MedErr pd=00 errLBA=3c0592f in NVRAM_EVENT_LOG


04/27  1:09:14: prRecGo: Ready to attempt recovery errLBA=3bf04f4 on pd=01


04/27  1:09:14: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/27  1:09:14: prRecGo: dataErr found on ld 0 span 0 arm 1


04/27  1:09:14: prRecGo: data NOT in cache; cacheLn=ffffffff, row=77e09, stripe=77e09, refBlk=74, type=1


04/27  1:09:14: prRecGo: RAID1 - read from peer pd=00


04/27  1:09:15: REC:log MedErr on pd[0] #retries=0


04/27  1:09:15: LogSense: pd=00, cdb=28 00 03 bf 04 f4 00 00 01 00 


04/27  1:09:15:           sense=f0 00 03 03 bf 04 f4 0a 12 df 48 cc 11 00 00 00 00 a5 


04/27  1:09:15: REC: MedErr on LD[0] BadLba=3bf04f4


04/27  1:09:15: prRecCallback: Error, cdb=28, pd=00, err=2


04/27  1:09:15: PatrolReadStart: Log MedErr pd=01 errLBA=3bf04f4 in NVRAM_EVENT_LOG


04/27  1:09:20: REC:log MedErr on pd[0] #retries=0


04/27  1:09:20: LogSense: pd=00, cdb=2f 00 03 c0 59 30 00 ff ff 00 


04/27  1:09:20:           sense=f0 00 03 03 c0 70 6f 0a 1a ff 48 e9 11 00 00 00 00 a5 


04/27  1:09:20: REC: MedErr on LD[0] BadLba=3c0706f


04/27  1:09:20: prCallback: Medium Error on pd=00, StartLba=3c05930, ErrLba=3c0706f


04/27  1:09:20: prRecQueue: starting pd=00 recovery - blocking host commands


04/27  1:09:20: prRecGo: Ready to attempt recovery errLBA=3c0706f on pd=00


04/27  1:09:20: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/27  1:09:20: prRecGo: dataErr found on ld 0 span 0 arm 0


04/27  1:09:20: prRecGo: data NOT in cache; cacheLn=ffffffff, row=780e0, stripe=780e0, refBlk=6f, type=1


04/27  1:09:20: prRecGo: RAID1 - read from peer pd=01


04/27  1:09:21: REC:log MedErr on pd[1] #retries=0


04/27  1:09:21: LogSense: pd=01, cdb=2f 00 03 bf 04 f5 00 ff ff 00 


04/27  1:09:21:           sense=f0 00 03 03 bf 83 6f 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  1:09:21: REC: MedErr on LD[1] BadLba=3bf836f


04/27  1:09:21: prCallback: Medium Error on pd=01, StartLba=3bf04f5, ErrLba=3bf836f


04/27  1:09:21: prRecQueue: adding pd=01 to recovery wait queue


04/27  1:09:23: REC:log MedErr on pd[1] #retries=0


04/27  1:09:23: LogSense: pd=01, cdb=28 00 03 c0 70 6f 00 00 01 00 


04/27  1:09:23:           sense=f0 00 03 03 c0 70 6f 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  1:09:23: REC: MedErr on LD[1] BadLba=3c0706f


04/27  1:09:23: prRecCallback: Error, cdb=28, pd=01, err=2


04/27  1:09:23: PatrolReadStart: Log MedErr pd=00 errLBA=3c0706f in NVRAM_EVENT_LOG


04/27  1:09:23: prRecGo: Ready to attempt recovery errLBA=3bf836f on pd=01


04/27  1:09:23: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/27  1:09:23: prRecGo: dataErr found on ld 0 span 0 arm 1


04/27  1:09:23: prRecGo: data NOT in cache; cacheLn=ffffffff, row=77f06, stripe=77f06, refBlk=6f, type=1


04/27  1:09:23: prRecGo: RAID1 - read from peer pd=00


04/27  1:09:26: REC:log MedErr on pd[0] #retries=0


04/27  1:09:26: LogSense: pd=00, cdb=28 00 03 bf 83 6f 00 00 01 00 


04/27  1:09:26:           sense=f0 00 03 03 bf 83 6f 0a 00 01 48 d6 11 0b 00 00 01 ab 


04/27  1:09:26: REC: MedErr on LD[0] BadLba=3bf836f


04/27  1:09:26: prRecCallback: Error, cdb=28, pd=00, err=2


04/27  1:09:26: PatrolReadStart: Log MedErr pd=01 errLBA=3bf836f in NVRAM_EVENT_LOG


04/27  1:09:28: REC:log MedErr on pd[0] #retries=0


04/27  1:09:28: LogSense: pd=00, cdb=2f 00 03 c0 70 70 00 ff ff 00 


04/27  1:09:28:           sense=f0 00 03 03 c0 72 cf 0a 1a 5f 48 ea 11 00 00 00 00 a5 


04/27  1:09:28: REC: MedErr on LD[0] BadLba=3c072cf


04/27  1:09:28: prCallback: Medium Error on pd=00, StartLba=3c07070, ErrLba=3c072cf


04/27  1:09:28: prRecQueue: starting pd=00 recovery - blocking host commands


04/27  1:09:28: prRecGo: Ready to attempt recovery errLBA=3c072cf on pd=00


04/27  1:09:28: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/27  1:09:28: prRecGo: dataErr found on ld 0 span 0 arm 0


04/27  1:09:28: prRecGo: data NOT in cache; cacheLn=ffffffff, row=780e5, stripe=780e5, refBlk=4f, type=1


04/27  1:09:28: prRecGo: RAID1 - read from peer pd=01


04/27  1:09:30: REC:log MedErr on pd[1] #retries=0


04/27  1:09:30: LogSense: pd=01, cdb=28 00 03 c0 72 cf 00 00 01 00 


04/27  1:09:30:           sense=f0 00 03 03 c0 72 cf 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  1:09:30: REC: MedErr on LD[1] BadLba=3c072cf


04/27  1:09:30: prRecCallback: Error, cdb=28, pd=01, err=2


04/27  1:09:30: PatrolReadStart: Log MedErr pd=00 errLBA=3c072cf in NVRAM_EVENT_LOG


04/27  1:09:36: REC:log MedErr on pd[0] #retries=0


04/27  1:09:36: LogSense: pd=00, cdb=2f 00 03 c0 72 d0 00 ff ff 00 


04/27  1:09:36:           sense=f0 00 03 03 c0 87 de 0a 19 ce 48 eb 11 00 00 00 00 a5 


04/27  1:09:36: REC: MedErr on LD[0] BadLba=3c087de


04/27  1:09:36: prCallback: Medium Error on pd=00, StartLba=3c072d0, ErrLba=3c087de


04/27  1:09:36: prRecQueue: starting pd=00 recovery - blocking host commands


04/27  1:09:36: prRecGo: Ready to attempt recovery errLBA=3c087de on pd=00


04/27  1:09:36: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/27  1:09:36: prRecGo: dataErr found on ld 0 span 0 arm 0


04/27  1:09:36: prRecGo: data NOT in cache; cacheLn=ffffffff, row=7810f, stripe=7810f, refBlk=5e, type=1


04/27  1:09:36: prRecGo: RAID1 - read from peer pd=01


04/27  1:09:36: REC:log MedErr on pd[1] #retries=0


04/27  1:09:36: LogSense: pd=01, cdb=2f 00 03 bf 8b 70 00 ff ff 00 


04/27  1:09:36:           sense=f0 00 03 03 c0 59 2f 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  1:09:36: REC: MedErr on LD[1] BadLba=3c0592f


04/27  1:09:36: prCallback: Medium Error on pd=01, StartLba=3bf8b70, ErrLba=3c0592f


04/27  1:09:36: prRecQueue: adding pd=01 to recovery wait queue


04/27  1:09:38: REC:log MedErr on pd[1] #retries=0


04/27  1:09:38: LogSense: pd=01, cdb=28 00 03 c0 87 de 00 00 01 00 


04/27  1:09:38:           sense=f0 00 03 03 c0 87 de 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  1:09:38: REC: MedErr on LD[1] BadLba=3c087de


04/27  1:09:38: prRecCallback: Error, cdb=28, pd=01, err=2


04/27  1:09:38: PatrolReadStart: Log MedErr pd=00 errLBA=3c087de in NVRAM_EVENT_LOG


04/27  1:09:38: prRecGo: Ready to attempt recovery errLBA=3c0592f on pd=01


04/27  1:09:39: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/27  1:09:39: prRecGo: dataErr found on ld 0 span 0 arm 1


04/27  1:09:39: prRecGo: data NOT in cache; cacheLn=ffffffff, row=780b2, stripe=780b2, refBlk=2f, type=1


04/27  1:09:39: prRecGo: RAID1 - read from peer pd=00


04/27  1:09:40: REC:log MedErr on pd[0] #retries=0


04/27  1:09:40: LogSense: pd=00, cdb=28 00 03 c0 59 2f 00 00 01 00 


04/27  1:09:40:           sense=f0 00 03 03 c0 59 2f 0a 19 5f 48 e8 11 00 00 00 00 a5 


04/27  1:09:40: REC: MedErr on LD[0] BadLba=3c0592f


04/27  1:09:40: prRecCallback: Error, cdb=28, pd=00, err=2


04/27  1:09:40: PatrolReadStart: Log MedErr pd=01 errLBA=3c0592f in NVRAM_EVENT_LOG


04/27  1:09:43: REC:log MedErr on pd[0] #retries=0


04/27  1:09:43: LogSense: pd=00, cdb=2f 00 03 c0 87 df 00 ff ff 00 


04/27  1:09:43:           sense=f0 00 03 03 c0 8a 3e 0a 19 2e 48 ec 11 00 00 00 00 a5 


04/27  1:09:43: REC: MedErr on LD[0] BadLba=3c08a3e


04/27  1:09:43: prCallback: Medium Error on pd=00, StartLba=3c087df, ErrLba=3c08a3e


04/27  1:09:43: prRecQueue: starting pd=00 recovery - blocking host commands


04/27  1:09:43: prRecGo: Ready to attempt recovery errLBA=3c08a3e on pd=00


04/27  1:09:43: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/27  1:09:43: prRecGo: dataErr found on ld 0 span 0 arm 0


04/27  1:09:43: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78114, stripe=78114, refBlk=3e, type=1


04/27  1:09:43: prRecGo: RAID1 - read from peer pd=01


04/27  1:09:45: REC:log MedErr on pd[1] #retries=0


04/27  1:09:45: LogSense: pd=01, cdb=28 00 03 c0 8a 3e 00 00 01 00 


04/27  1:09:45:           sense=f0 00 03 03 c0 8a 3e 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  1:09:45: REC: MedErr on LD[1] BadLba=3c08a3e


04/27  1:09:45: prRecCallback: Error, cdb=28, pd=01, err=2


04/27  1:09:45: PatrolReadStart: Log MedErr pd=00 errLBA=3c08a3e in NVRAM_EVENT_LOG


04/27  1:09:50: REC:log MedErr on pd[0] #retries=0


04/27  1:09:50: LogSense: pd=00, cdb=2f 00 03 c0 8a 3f 00 ff ff 00 


04/27  1:09:50:           sense=f0 00 03 03 c0 a3 de 0a 1a 2e 48 ee 11 00 00 00 00 a5 


04/27  1:09:50: REC: MedErr on LD[0] BadLba=3c0a3de


04/27  1:09:50: prCallback: Medium Error on pd=00, StartLba=3c08a3f, ErrLba=3c0a3de


04/27  1:09:50: prRecQueue: starting pd=00 recovery - blocking host commands


04/27  1:09:50: prRecGo: Ready to attempt recovery errLBA=3c0a3de on pd=00


04/27  1:09:50: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/27  1:09:50: prRecGo: dataErr found on ld 0 span 0 arm 0


04/27  1:09:50: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78147, stripe=78147, refBlk=5e, type=1


04/27  1:09:50: prRecGo: RAID1 - read from peer pd=01


04/27  1:09:51: REC:log MedErr on pd[1] #retries=0


04/27  1:09:51: LogSense: pd=01, cdb=2f 00 03 c0 61 30 00 ff ff 00 


04/27  1:09:51:           sense=f0 00 03 03 c0 70 6f 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  1:09:51: REC: MedErr on LD[1] BadLba=3c0706f


04/27  1:09:51: prCallback: Medium Error on pd=01, StartLba=3c06130, ErrLba=3c0706f


04/27  1:09:51: prRecQueue: adding pd=01 to recovery wait queue


04/27  1:09:53: REC:log MedErr on pd[1] #retries=0


04/27  1:09:53: LogSense: pd=01, cdb=28 00 03 c0 a3 de 00 00 01 00 


04/27  1:09:53:           sense=f0 00 03 03 c0 a3 de 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  1:09:53: REC: MedErr on LD[1] BadLba=3c0a3de


04/27  1:09:53: prRecCallback: Error, cdb=28, pd=01, err=2


04/27  1:09:53: PatrolReadStart: Log MedErr pd=00 errLBA=3c0a3de in NVRAM_EVENT_LOG


04/27  1:09:53: prRecGo: Ready to attempt recovery errLBA=3c0706f on pd=01


04/27  1:09:53: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/27  1:09:53: prRecGo: dataErr found on ld 0 span 0 arm 1


04/27  1:09:53: prRecGo: data NOT in cache; cacheLn=ffffffff, row=780e0, stripe=780e0, refBlk=6f, type=1


04/27  1:09:53: prRecGo: RAID1 - read from peer pd=00


04/27  1:09:55: REC:log MedErr on pd[0] #retries=0


04/27  1:09:55: LogSense: pd=00, cdb=28 00 03 c0 70 6f 00 00 01 00 


04/27  1:09:55:           sense=f0 00 03 03 c0 70 6f 0a 1a ff 48 e9 11 00 00 00 00 a5 


04/27  1:09:55: REC: MedErr on LD[0] BadLba=3c0706f


04/27  1:09:55: prRecCallback: Error, cdb=28, pd=00, err=2


04/27  1:09:55: PatrolReadStart: Log MedErr pd=01 errLBA=3c0706f in NVRAM_EVENT_LOG


04/27  1:09:59: REC:log MedErr on pd[0] #retries=0


04/27  1:09:59: LogSense: pd=00, cdb=2f 00 03 c0 a3 df 00 ff ff 00 


04/27  1:09:59:           sense=f0 00 03 03 c0 b8 1d 0a 18 cd 48 ef 11 00 00 00 00 a5 


04/27  1:09:59: REC: MedErr on LD[0] BadLba=3c0b81d


04/27  1:09:59: prCallback: Medium Error on pd=00, StartLba=3c0a3df, ErrLba=3c0b81d


04/27  1:09:59: prRecQueue: starting pd=00 recovery - blocking host commands


04/27  1:09:59: prRecGo: Ready to attempt recovery errLBA=3c0b81d on pd=00


04/27  1:09:59: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/27  1:09:59: prRecGo: dataErr found on ld 0 span 0 arm 0


04/27  1:09:59: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78170, stripe=78170, refBlk=1d, type=1


04/27  1:09:59: prRecGo: RAID1 - read from peer pd=01


04/27  1:10:01: REC:log MedErr on pd[1] #retries=0


04/27  1:10:01: LogSense: pd=01, cdb=2f 00 03 c0 70 70 00 ff ff 00 


04/27  1:10:01:           sense=f0 00 03 03 c0 72 cf 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  1:10:01: REC: MedErr on LD[1] BadLba=3c072cf


04/27  1:10:01: prCallback: Medium Error on pd=01, StartLba=3c07070, ErrLba=3c072cf


04/27  1:10:01: prRecQueue: adding pd=01 to recovery wait queue


04/27  1:10:03: REC:log MedErr on pd[1] #retries=0


04/27  1:10:03: LogSense: pd=01, cdb=28 00 03 c0 b8 1d 00 00 01 00 


04/27  1:10:03:           sense=f0 00 03 03 c0 b8 1d 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  1:10:03: REC: MedErr on LD[1] BadLba=3c0b81d


04/27  1:10:03: prRecCallback: Error, cdb=28, pd=01, err=2


04/27  1:10:03: PatrolReadStart: Log MedErr pd=00 errLBA=3c0b81d in NVRAM_EVENT_LOG


04/27  1:10:03: prRecGo: Ready to attempt recovery errLBA=3c072cf on pd=01


04/27  1:10:03: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/27  1:10:03: prRecGo: dataErr found on ld 0 span 0 arm 1


04/27  1:10:03: prRecGo: data NOT in cache; cacheLn=ffffffff, row=780e5, stripe=780e5, refBlk=4f, type=1


04/27  1:10:03: prRecGo: RAID1 - read from peer pd=00


04/27  1:10:05: REC:log MedErr on pd[0] #retries=0


04/27  1:10:05: LogSense: pd=00, cdb=28 00 03 c0 72 cf 00 00 01 00 


04/27  1:10:05:           sense=f0 00 03 03 c0 72 cf 0a 1a 5f 48 ea 11 00 00 00 00 a5 


04/27  1:10:05: REC: MedErr on LD[0] BadLba=3c072cf


04/27  1:10:05: prRecCallback: Error, cdb=28, pd=00, err=2


04/27  1:10:05: PatrolReadStart: Log MedErr pd=01 errLBA=3c072cf in NVRAM_EVENT_LOG


04/27  1:10:10: REC:log MedErr on pd[0] #retries=0


04/27  1:10:10: LogSense: pd=00, cdb=2f 00 03 c0 b8 1e 00 ff ff 00 


04/27  1:10:10:           sense=f0 00 03 03 c0 b8 1e 0a 18 ce 48 ef 11 00 00 00 01 c6 


04/27  1:10:10: REC: MedErr on LD[0] BadLba=3c0b81e


04/27  1:10:11: prCallback: Medium Error on pd=00, StartLba=3c0b81e, ErrLba=3c0b81e


04/27  1:10:11: prRecQueue: starting pd=00 recovery - blocking host commands


04/27  1:10:11: REC:log MedErr on pd[1] #retries=0


04/27  1:10:11: LogSense: pd=01, cdb=2f 00 03 c0 72 d0 00 ff ff 00 


04/27  1:10:11:           sense=f0 00 03 03 c0 87 de 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  1:10:11: REC: MedErr on LD[1] BadLba=3c087de


04/27  1:10:11: prCallback: Medium Error on pd=01, StartLba=3c072d0, ErrLba=3c087de


04/27  1:10:11: prRecQueue: adding pd=01 to recovery wait queue


04/27  1:10:11: prRecGo: Ready to attempt recovery errLBA=3c0b81e on pd=00


04/27  1:10:11: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/27  1:10:11: prRecGo: dataErr found on ld 0 span 0 arm 0


04/27  1:10:11: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78170, stripe=78170, refBlk=1e, type=1


04/27  1:10:11: prRecGo: RAID1 - read from peer pd=01


04/27  1:10:13: REC:log MedErr on pd[1] #retries=0


04/27  1:10:13: LogSense: pd=01, cdb=28 00 03 c0 b8 1e 00 00 01 00 


04/27  1:10:13:           sense=f0 00 03 03 c0 b8 1e 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  1:10:13: REC: MedErr on LD[1] BadLba=3c0b81e


04/27  1:10:14: prRecCallback: Error, cdb=28, pd=01, err=2


04/27  1:10�€���€��rolReadStart: Log MedErr pd=00 errLBA=3c0b81e in NVRAM_EVENT_LOG


04/27  1:10:14: prRecGo: Ready to attempt recovery errLBA=3c087de on pd=01


04/27  1:10:14: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/27  1:10:14: prRecGo: dataErr found on ld 0 span 0 arm 1


04/27  1:10:14: prRecGo: data NOT in cache; cacheLn=ffffffff, row=7810f, stripe=7810f, refBlk=5e, type=1


04/27  1:10:14: prRecGo: RAID1 - read from peer pd=00


04/27  1:10:15: REC:log MedErr on pd[0] #retries=0


04/27  1:10:15: LogSense: pd=00, cdb=28 00 03 c0 87 de 00 00 01 00 


04/27  1:10:15:           sense=f0 00 03 03 c0 87 de 0a 19 ce 48 eb 11 00 00 00 00 a5 


04/27  1:10:15: REC: MedErr on LD[0] BadLba=3c087de


04/27  1:10:15: prRecCallback: Error, cdb=28, pd=00, err=2


04/27  1:10:15: PatrolReadStart: Log MedErr pd=01 errLBA=3c087de in NVRAM_EVENT_LOG


04/27  1:10:19: REC:log MedErr on pd[0] #retries=0


04/27  1:10:19: LogSense: pd=00, cdb=2f 00 03 c0 b8 1f 00 ff ff 00 


04/27  1:10:19:           sense=f0 00 03 03 c0 ba 7e 0a 18 2e 48 f0 11 00 00 00 00 a5 


04/27  1:10:19: REC: MedErr on LD[0] BadLba=3c0ba7e


04/27  1:10:19: prCallback: Medium Error on pd=00, StartLba=3c0b81f, ErrLba=3c0ba7e


04/27  1:10:19: prRecQueue: starting pd=00 recovery - blocking host commands


04/27  1:10:19: prRecGo: Ready to attempt recovery errLBA=3c0ba7e on pd=00


04/27  1:10:19: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/27  1:10:19: prRecGo: dataErr found on ld 0 span 0 arm 0


04/27  1:10:19: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78174, stripe=78174, refBlk=7e, type=1


04/27  1:10:19: prRecGo: RAID1 - read from peer pd=01


04/27  1:10:21: REC:log MedErr on pd[1] #retries=0


04/27  1:10:21: LogSense: pd=01, cdb=2f 00 03 c0 87 df 00 ff ff 00 


04/27  1:10:21:           sense=f0 00 03 03 c0 8a 3e 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  1:10:21: REC: MedErr on LD[1] BadLba=3c08a3e


04/27  1:10:21: prCallback: Medium Error on pd=01, StartLba=3c087df, ErrLba=3c08a3e


04/27  1:10:21: prRecQueue: adding pd=01 to recovery wait queue


04/27  1:10:23: REC:log MedErr on pd[1] #retries=0


04/27  1:10:23: LogSense: pd=01, cdb=28 00 03 c0 ba 7e 00 00 01 00 


04/27  1:10:23:           sense=f0 00 03 03 c0 ba 7e 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  1:10:23: REC: MedErr on LD[1] BadLba=3c0ba7e


04/27  1:10:23: prRecCallback: Error, cdb=28, pd=01, err=2


04/27  1:10:23: PatrolReadStart: Log MedErr pd=00 errLBA=3c0ba7e in NVRAM_EVENT_LOG


04/27  1:10:23: prRecGo: Ready to attempt recovery errLBA=3c08a3e on pd=01


04/27  1:10:23: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/27  1:10:23: prRecGo: dataErr found on ld 0 span 0 arm 1


04/27  1:10:23: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78114, stripe=78114, refBlk=3e, type=1


04/27  1:10:23: prRecGo: RAID1 - read from peer pd=00


04/27  1:10:24: REC:log MedErr on pd[0] #retries=0


04/27  1:10:24: LogSense: pd=00, cdb=28 00 03 c0 8a 3e 00 00 01 00 


04/27  1:10:24:           sense=f0 00 03 03 c0 8a 3e 0a 19 2e 48 ec 11 00 00 00 00 a5 


04/27  1:10:24: REC: MedErr on LD[0] BadLba=3c08a3e


04/27  1:10:25: prRecCallback: Error, cdb=28, pd=00, err=2


04/27  1:10:25: PatrolReadStart: Log MedErr pd=01 errLBA=3c08a3e in NVRAM_EVENT_LOG


04/27  1:10:28: REC:log MedErr on pd[0] #retries=0


04/27  1:10:28: LogSense: pd=00, cdb=2f 00 03 c0 ba 7f 00 ff ff 00 


04/27  1:10:28:           sense=f0 00 03 03 c0 d1 be 0a 19 ce 48 f1 11 00 00 00 00 a5 


04/27  1:10:28: REC: MedErr on LD[0] BadLba=3c0d1be


04/27  1:10:28: prCallback: Medium Error on pd=00, StartLba=3c0ba7f, ErrLba=3c0d1be


04/27  1:10:28: prRecQueue: starting pd=00 recovery - blocking host commands


04/27  1:10:28: prRecGo: Ready to attempt recovery errLBA=3c0d1be on pd=00


04/27  1:10:28: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/27  1:10:28: prRecGo: dataErr found on ld 0 span 0 arm 0


04/27  1:10:28: prRecGo: data NOT in cache; cacheLn=ffffffff, row=781a3, stripe=781a3, refBlk=3e, type=1


04/27  1:10:28: prRecGo: RAID1 - read from peer pd=01


04/27  1:10:30: REC:log MedErr on pd[1] #retries=0


04/27  1:10:30: LogSense: pd=01, cdb=28 00 03 c0 d1 be 00 00 01 00 


04/27  1:10:30:           sense=f0 00 03 03 c0 d1 be 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  1:10:30: REC: MedErr on LD[1] BadLba=3c0d1be


04/27  1:10:30: prRecCallback: Error, cdb=28, pd=01, err=2


04/27  1:10:30: PatrolReadStart: Log MedErr pd=00 errLBA=3c0d1be in NVRAM_EVENT_LOG


04/27  1:10:35: REC:log MedErr on pd[0] #retries=0


04/27  1:10:35: LogSense: pd=00, cdb=2f 00 03 c0 d1 bf 00 ff ff 00 


04/27  1:10:35:           sense=f0 00 03 03 c0 eb 53 0a 1a c3 48 f3 11 00 00 00 00 a5 


04/27  1:10:35: REC: MedErr on LD[0] BadLba=3c0eb53


04/27  1:10:35: prCallback: Medium Error on pd=00, StartLba=3c0d1bf, ErrLba=3c0eb53


04/27  1:10:35: prRecQueue: starting pd=00 recovery - blocking host commands


04/27  1:10:35: prRecGo: Ready to attempt recovery errLBA=3c0eb53 on pd=00


04/27  1:10:35: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/27  1:10:35: prRecGo: dataErr found on ld 0 span 0 arm 0


04/27  1:10:35: prRecGo: data NOT in cache; cacheLn=ffffffff, row=781d6, stripe=781d6, refBlk=53, type=1


04/27  1:10:35: prRecGo: RAID1 - read from peer pd=01


04/27  1:10:36: REC:log MedErr on pd[1] #retries=0


04/27  1:10:36: LogSense: pd=01, cdb=2f 00 03 c0 92 3f 00 ff ff 00 


04/27  1:10:36:           sense=f0 00 03 03 c0 a3 de 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  1:10:36: REC: MedErr on LD[1] BadLba=3c0a3de


04/27  1:10:36: prCallback: Medium Error on pd=01, StartLba=3c0923f, ErrLba=3c0a3de


04/27  1:10:36: prRecQueue: adding pd=01 to recovery wait queue


04/27  1:10:38: REC:log MedErr on pd[1] #retries=0


04/27  1:10:38: LogSense: pd=01, cdb=28 00 03 c0 eb 53 00 00 01 00 


04/27  1:10:38:           sense=f0 00 03 03 c0 eb 53 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  1:10:38: REC: MedErr on LD[1] BadLba=3c0eb53


04/27  1:10:38: prRecCallback: Error, cdb=28, pd=01, err=2


04/27  1:10:38: PatrolReadStart: Log MedErr pd=00 errLBA=3c0eb53 in NVRAM_EVENT_LOG


04/27  1:10:38: prRecGo: Ready to attempt recovery errLBA=3c0a3de on pd=01


04/27  1:10:38: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/27  1:10:38: prRecGo: dataErr found on ld 0 span 0 arm 1


04/27  1:10:38: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78147, stripe=78147, refBlk=5e, type=1


04/27  1:10:39: prRecGo: RAID1 - read from peer pd=00


04/27  1:10:40: REC:log MedErr on pd[0] #retries=0


04/27  1:10:40: LogSense: pd=00, cdb=28 00 03 c0 a3 de 00 00 01 00 


04/27  1:10:40:           sense=f0 00 03 03 c0 a3 de 0a 1a 2e 48 ee 16 00 00 00 00 a5 


04/27  1:10:40: REC: MedErr on LD[0] BadLba=3c0a3de


04/27  1:10:40: prRecCallback: Error, cdb=28, pd=00, err=2


04/27  1:10:40: PatrolReadStart: Log MedErr pd=01 errLBA=3c0a3de in NVRAM_EVENT_LOG


04/27  1:10:44: REC:log MedErr on pd[0] #retries=0


04/27  1:10:44: LogSense: pd=00, cdb=2f 00 03 c0 eb 54 00 ff ff 00 


04/27  1:10:44:           sense=f0 00 03 03 c1 01 f3 0a 18 c3 48 f5 11 00 00 00 00 a5 


04/27  1:10:44: REC: MedErr on LD[0] BadLba=3c101f3


04/27  1:10:44: prCallback: Medium Error on pd=00, StartLba=3c0eb54, ErrLba=3c101f3


04/27  1:10:44: prRecQueue: starting pd=00 recovery - blocking host commands


04/27  1:10:44: prRecGo: Ready to attempt recovery errLBA=3c101f3 on pd=00


04/27  1:10:44: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/27  1:10:44: prRecGo: dataErr found on ld 0 span 0 arm 0


04/27  1:10:44: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78203, stripe=78203, refBlk=73, type=1


04/27  1:10:44: prRecGo: RAID1 - read from peer pd=01


04/27  1:10:46: REC:log MedErr on pd[1] #retries=0


04/27  1:10:46: LogSense: pd=01, cdb=2f 00 03 c0 a3 df 00 ff ff 00 


04/27  1:10:46:           sense=f0 00 03 03 c0 b8 1d 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  1:10:46: REC: MedErr on LD[1] BadLba=3c0b81d


04/27  1:10:46: prCallback: Medium Error on pd=01, StartLba=3c0a3df, ErrLba=3c0b81d


04/27  1:10:46: prRecQueue: adding pd=01 to recovery wait queue


04/27  1:10:48: REC:log MedErr on pd[1] #retries=0


04/27  1:10:48: LogSense: pd=01, cdb=28 00 03 c1 01 f3 00 00 01 00 


04/27  1:10:48:           sense=f0 00 03 03 c1 01 f3 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  1:10:48: REC: MedErr on LD[1] BadLba=3c101f3


04/27  1:10:48: prRecCallback: Error, cdb=28, pd=01, err=2


04/27  1:10:48: PatrolReadStart: Log MedErr pd=00 errLBA=3c101f3 in NVRAM_EVENT_LOG


04/27  1:10:48: prRecGo: Ready to attempt recovery errLBA=3c0b81d on pd=01


04/27  1:10:48: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/27  1:10:48: prRecGo: dataErr found on ld 0 span 0 arm 1


04/27  1:10:48: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78170, stripe=78170, refBlk=1d, type=1


04/27  1:10:48: prRecGo: RAID1 - read from peer pd=00


04/27  1:10:50: REC:log MedErr on pd[0] #retries=0


04/27  1:10:50: LogSense: pd=00, cdb=28 00 03 c0 b8 1d 00 00 01 00 


04/27  1:10:50:           sense=f0 00 03 03 c0 b8 1d 0a 18 cd 48 ef 11 00 00 00 00 a5 


04/27  1:10:50: REC: MedErr on LD[0] BadLba=3c0b81d


04/27  1:10:50: prRecCallback: Error, cdb=28, pd=00, err=2


04/27  1:10:50: PatrolReadStart: Log MedErr pd=01 errLBA=3c0b81d in NVRAM_EVENT_LOG


04/27  1:10:53: REC:log MedErr on pd[0] #retries=0


04/27  1:10:53: LogSense: pd=00, cdb=2f 00 03 c1 01 f4 00 ff ff 00 


04/27  1:10:53:           sense=f0 00 03 03 c1 04 53 0a 18 23 48 f6 11 00 00 00 00 a5 


04/27  1:10:53: REC: MedErr on LD[0] BadLba=3c10453


04/27  1:10:53: prCallback: Medium Error on pd=00, StartLba=3c101f4, ErrLba=3c10453


04/27  1:10:53: prRecQueue: starting pd=00 recovery - blocking host commands


04/27  1:10:53: prRecGo: Ready to attempt recovery errLBA=3c10453 on pd=00


04/27  1:10:53: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/27  1:10:53: prRecGo: dataErr found on ld 0 span 0 arm 0


04/27  1:10:53: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78208, stripe=78208, refBlk=53, type=1


04/27  1:10:53: prRecGo: RAID1 - read from peer pd=01


04/27  1:10:55: REC:log MedErr on pd[1] #retries=0


04/27  1:10:55: LogSense: pd=01, cdb=28 00 03 c1 04 53 00 00 01 00 


04/27  1:10:55:           sense=f0 00 03 03 c1 04 53 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  1:10:55: REC: MedErr on LD[1] BadLba=3c10453


04/27  1:10:55: prRecCallback: Error, cdb=28, pd=01, err=2


04/27  1:10:55: PatrolReadStart: Log MedErr pd=00 errLBA=3c10453 in NVRAM_EVENT_LOG


04/27  1:10:57: REC:log MedErr on pd[1] #retries=0


04/27  1:10:57: LogSense: pd=01, cdb=2f 00 03 c0 b8 1e 00 08 00 00 


04/27  1:10:57:           sense=f0 00 03 03 c0 b8 1e 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  1:10:57: REC: MedErr on LD[1] BadLba=3c0b81e


04/27  1:10:58: prCallback: Medium Error on pd=01, StartLba=3c0b81e, ErrLba=3c0b81e


04/27  1:10:58: prRecQueue: starting pd=01 recovery - blocking host commands


04/27  1:10:58: prRecGo: Ready to attempt recovery errLBA=3c0b81e on pd=01


04/27  1:10:58: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/27  1:10:58: prRecGo: dataErr found on ld 0 span 0 arm 1


04/27  1:10:58: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78170, stripe=78170, refBlk=1e, type=1


04/27  1:10:58: prRecGo: RAID1 - read from peer pd=00


04/27  1:10:58: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  1:10:58: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  1:10:58:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  1:10:59: REC:log MedErr on pd[0] #retries=0


04/27  1:10:59: LogSense: pd=00, cdb=28 00 03 c0 b8 1e 00 00 01 00 


04/27  1:10:59:           sense=f0 00 03 03 c0 b8 1e 0a 18 ce 48 ef 11 00 00 00 00 a5 


04/27  1:10:59: REC: MedErr on LD[0] BadLba=3c0b81e


04/27  1:10:59: prRecCallback: Error, cdb=28, pd=00, err=2


04/27  1:10:59: PatrolReadStart: Log MedErr pd=01 errLBA=3c0b81e in NVRAM_EVENT_LOG


04/27  1:11:04: REC:log MedErr on pd[0] #retries=0


04/27  1:11:04: LogSense: pd=00, cdb=2f 00 03 c1 0c 54 00 ff ff 00 


04/27  1:11:04:           sense=f0 00 03 03 c1 5d 6c 0a 03 0c 48 fd 11 00 00 00 00 a5 


04/27  1:11:04: REC: MedErr on LD[0] BadLba=3c15d6c


04/27  1:11:04: prCallback: Medium Error on pd=00, StartLba=3c10c54, ErrLba=3c15d6c


04/27  1:11:04: prRecQueue: starting pd=00 recovery - blocking host commands


04/27  1:11:04: prRecGo: Ready to attempt recovery errLBA=3c15d6c on pd=00


04/27  1:11:04: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/27  1:11:04: prRecGo: dataErr found on ld 0 span 0 arm 0


04/27  1:11:04: prRecGo: data NOT in cache; cacheLn=ffffffff, row=782ba, stripe=782ba, refBlk=6c, type=1


04/27  1:11:04: prRecGo: RAID1 - read from peer pd=01


04/27  1:11:05: REC:log MedErr on pd[1] #retries=0


04/27  1:11:05: LogSense: pd=01, cdb=2f 00 03 c0 b8 1f 00 ff ff 00 


04/27  1:11:05:           sense=f0 00 03 03 c0 ba 7e 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  1:11:05: REC: MedErr on LD[1] BadLba=3c0ba7e


04/27  1:11:05: prCallback: Medium Error on pd=01, StartLba=3c0b81f, ErrLba=3c0ba7e


04/27  1:11:05: prRecQueue: adding pd=01 to recovery wait queue


04/27  1:11:07: REC:log MedErr on pd[1] #retries=0


04/27  1:11:07: LogSense: pd=01, cdb=28 00 03 c1 5d 6c 00 00 01 00 


04/27  1:11:07:           sense=f0 00 03 03 c1 5d 6c 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  1:11:07: REC: MedErr on LD[1] BadLba=3c15d6c


04/27  1:11:07: prRecCallback: Error, cdb=28, pd=01, err=2


04/27  1:11:07: PatrolReadStart: Log MedErr pd=00 errLBA=3c15d6c in NVRAM_EVENT_LOG


04/27  1:11:07: prRecGo: Ready to attempt recovery errLBA=3c0ba7e on pd=01


04/27  1:11:07: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/27  1:11:07: prRecGo: dataErr found on ld 0 span 0 arm 1


04/27  1:11:07: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78174, stripe=78174, refBlk=7e, type=1


04/27  1:11:07: prRecGo: RAID1 - read from peer pd=00


04/27  1:11:08: REC:log MedErr on pd[0] #retries=0


04/27  1:11:08: LogSense: pd=00, cdb=28 00 03 c0 ba 7e 00 00 01 00 


04/27  1:11:08:           sense=f0 00 03 03 c0 ba 7e 0a 18 2e 48 f0 11 00 00 00 00 a5 


04/27  1:11:08: REC: MedErr on LD[0] BadLba=3c0ba7e


04/27  1:11:09: prRecCallback: Error, cdb=28, pd=00, err=2


04/27  1:11:09: PatrolReadStart: Log MedErr pd=01 errLBA=3c0ba7e in NVRAM_EVENT_LOG


04/27  1:11:12: REC:log MedErr on pd[0] #retries=0


04/27  1:11:12: LogSense: pd=00, cdb=2f 00 03 c1 5d 6d 00 ff ff 00 


04/27  1:11:12:           sense=f0 00 03 03 c1 71 8d 0a 01 a2 48 fe 11 00 00 00 00 a5 


04/27  1:11:12: REC: MedErr on LD[0] BadLba=3c1718d


04/27  1:11:12: prCallback: Medium Error on pd=00, StartLba=3c15d6d, ErrLba=3c1718d


04/27  1:11:12: prRecQueue: starting pd=00 recovery - blocking host commands


04/27  1:11:12: prRecGo: Ready to attempt recovery errLBA=3c1718d on pd=00


04/27  1:11:12: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/27  1:11:12: prRecGo: dataErr found on ld 0 span 0 arm 0


04/27  1:11:12: prRecGo: data NOT in cache; cacheLn=ffffffff, row=782e3, stripe=782e3, refBlk=d, type=1


04/27  1:11:12: prRecGo: RAID1 - read from peer pd=01


04/27  1:11:14: REC:log MedErr on pd[1] #retries=0


04/27  1:11:14: LogSense: pd=01, cdb=28 00 03 c1 71 8d 00 00 01 00 


04/27  1:11:14:           sense=f0 00 03 03 c1 71 8d 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  1:11:14: REC: MedErr on LD[1] BadLba=3c1718d


04/27  1:11:14: prRecCallback: Error, cdb=28, pd=01, err=2


04/27  1:11:14: PatrolReadStart: Log MedErr pd=00 errLBA=3c1718d in NVRAM_EVENT_LOG


04/27  1:11:20: REC:log MedErr on pd[1] #retries=0


04/27  1:11:20: LogSense: pd=01, cdb=2f 00 03 c0 c2 7f 00 ff ff 00 


04/27  1:11:20:           sense=f0 00 03 03 c0 d1 be 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  1:11:20: REC: MedErr on LD[1] BadLba=3c0d1be


04/27  1:11:20: prCallback: Medium Error on pd=01, StartLba=3c0c27f, ErrLba=3c0d1be


04/27  1:11:20: prRecQueue: starting pd=01 recovery - blocking host commands


04/27  1:11:20: prRecGo: Ready to attempt recovery errLBA=3c0d1be on pd=01


04/27  1:11:20: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/27  1:11:20: prRecGo: dataErr found on ld 0 span 0 arm 1


04/27  1:11:20: prRecGo: data NOT in cache; cacheLn=ffffffff, row=781a3, stripe=781a3, refBlk=3e, type=1


04/27  1:11:20: prRecGo: RAID1 - read from peer pd=00


04/27  1:11:21: REC:log MedErr on pd[0] #retries=0


04/27  1:11:21: LogSense: pd=00, cdb=28 00 03 c0 d1 be 00 00 01 00 


04/27  1:11:21:           sense=f0 00 03 03 c0 d1 be 0a 19 ce 48 f1 11 00 00 00 00 a5 


04/27  1:11:21: REC: MedErr on LD[0] BadLba=3c0d1be


04/27  1:11:21: prRecCallback: Error, cdb=28, pd=00, err=2


04/27  1:11:21: PatrolReadStart: Log MedErr pd=01 errLBA=3c0d1be in NVRAM_EVENT_LOG


04/27  1:11:27: REC:log MedErr on pd[1] #retries=0


04/27  1:11:27: LogSense: pd=01, cdb=2f 00 03 c0 d1 bf 00 ff ff 00 


04/27  1:11:27:           sense=f0 00 03 03 c0 eb 53 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  1:11:27: REC: MedErr on LD[1] BadLba=3c0eb53


04/27  1:11:27: prCallback: Medium Error on pd=01, StartLba=3c0d1bf, ErrLba=3c0eb53


04/27  1:11:27: prRecQueue: starting pd=01 recovery - blocking host commands


04/27  1:11:27: prRecGo: Ready to attempt recovery errLBA=3c0eb53 on pd=01


04/27  1:11:27: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/27  1:11:27: prRecGo: dataErr found on ld 0 span 0 arm 1


04/27  1:11:27: prRecGo: data NOT in cache; cacheLn=ffffffff, row=781d6, stripe=781d6, refBlk=53, type=1


04/27  1:11:27: prRecGo: RAID1 - read from peer pd=00


04/27  1:11:28: REC:log MedErr on pd[0] #retries=0


04/27  1:11:28: LogSense: pd=00, cdb=28 00 03 c0 eb 53 00 00 01 00 


04/27  1:11:28:           sense=f0 00 03 03 c0 eb 53 0a 1a c3 48 f3 11 00 00 00 00 a5 


04/27  1:11:28: REC: MedErr on LD[0] BadLba=3c0eb53


04/27  1:11:28: prRecCallback: Error, cdb=28, pd=00, err=2


04/27  1:11:28: PatrolReadStart: Log MedErr pd=01 errLBA=3c0eb53 in NVRAM_EVENT_LOG


04/27  1:11:34: REC:log MedErr on pd[1] #retries=0


04/27  1:11:34: LogSense: pd=01, cdb=2f 00 03 c0 eb 54 00 ff ff 00 


04/27  1:11:34:           sense=f0 00 03 03 c1 01 f3 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  1:11:34: REC: MedErr on LD[1] BadLba=3c101f3


04/27  1:11:34: prCallback: Medium Error on pd=01, StartLba=3c0eb54, ErrLba=3c101f3


04/27  1:11:34: prRecQueue: starting pd=01 recovery - blocking host commands


04/27  1:11:34: prRecGo: Ready to attempt recovery errLBA=3c101f3 on pd=01


04/27  1:11:34: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/27  1:11:34: prRecGo: dataErr found on ld 0 span 0 arm 1


04/27  1:11:34: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78203, stripe=78203, refBlk=73, type=1


04/27  1:11:34: prRecGo: RAID1 - read from peer pd=00


04/27  1:11:35: REC:log MedErr on pd[0] #retries=0


04/27  1:11:35: LogSense: pd=00, cdb=28 00 03 c1 01 f3 00 00 01 00 


04/27  1:11:35:           sense=f0 00 03 03 c1 01 f3 0a 18 c3 48 f5 11 00 00 00 00 a5 


04/27  1:11:35: REC: MedErr on LD[0] BadLba=3c101f3


04/27  1:11:35: prRecCallback: Error, cdb=28, pd=00, err=2


04/27  1:11:35: PatrolReadStart: Log MedErr pd=01 errLBA=3c101f3 in NVRAM_EVENT_LOG


04/27  1:11:40: REC:log MedErr on pd[0] #retries=0


04/27  1:11:40: LogSense: pd=00, cdb=2f 00 03 c5 71 ca 00 ff ff 00 


04/27  1:11:40:           sense=f0 00 03 03 c6 4e 38 0a 1a a8 49 63 11 00 00 00 00 a5 


04/27  1:11:40: REC: MedErr on LD[0] BadLba=3c64e38


04/27  1:11:40: prCallback: Medium Error on pd=00, StartLba=3c571ca, ErrLba=3c64e38


04/27  1:11:40: prRecQueue: starting pd=00 recovery - blocking host commands


04/27  1:11:40: prRecGo: Ready to attempt recovery errLBA=3c64e38 on pd=00


04/27  1:11:40: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/27  1:11:40: prRecGo: dataErr found on ld 0 span 0 arm 0


04/27  1:11:41: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78c9c, stripe=78c9c, refBlk=38, type=1


04/27  1:11:41: prRecGo: RAID1 - read from peer pd=01


04/27  1:11:41: REC:log MedErr on pd[1] #retries=0


04/27  1:11:41: LogSense: pd=01, cdb=2f 00 03 c1 01 f4 00 ff ff 00 


04/27  1:11:41:           sense=f0 00 03 03 c1 04 53 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  1:11:41: REC: MedErr on LD[1] BadLba=3c10453


04/27  1:11:41: prCallback: Medium Error on pd=01, StartLba=3c101f4, ErrLba=3c10453


04/27  1:11:41: prRecQueue: adding pd=01 to recovery wait queue


04/27  1:11:43: REC:log MedErr on pd[1] #retries=0


04/27  1:11:43: LogSense: pd=01, cdb=28 00 03 c6 4e 38 00 00 01 00 


04/27  1:11:43:           sense=f0 00 03 03 c6 4e 38 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  1:11:43: REC: MedErr on LD[1] BadLba=3c64e38


04/27  1:11:43: prRecCallback: Error, cdb=28, pd=01, err=2


04/27  1:11:43: PatrolReadStart: Log MedErr pd=00 errLBA=3c64e38 in NVRAM_EVENT_LOG


04/27  1:11:43: prRecGo: Ready to attempt recovery errLBA=3c10453 on pd=01


04/27  1:11:43: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/27  1:11:43: prRecGo: dataErr found on ld 0 span 0 arm 1


04/27  1:11:43: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78208, stripe=78208, refBlk=53, type=1


04/27  1:11:43: prRecGo: RAID1 - read from peer pd=00


04/27  1:11:44: REC:log MedErr on pd[0] #retries=0


04/27  1:11:44: LogSense: pd=00, cdb=28 00 03 c1 04 53 00 00 01 00 


04/27  1:11:44:           sense=f0 00 03 03 c1 04 53 0a 18 23 48 f6 11 00 00 00 00 a5 


04/27  1:11:44: REC: MedErr on LD[0] BadLba=3c10453


04/27  1:11:44: prRecCallback: Error, cdb=28, pd=00, err=2


04/27  1:11:44: PatrolReadStart: Log MedErr pd=01 errLBA=3c10453 in NVRAM_EVENT_LOG


04/27  1:11:48: REC:log MedErr on pd[0] #retries=0


04/27  1:11:48: LogSense: pd=00, cdb=2f 00 03 c6 4e 39 00 ff ff 00 


04/27  1:11:48:           sense=f0 00 03 03 c6 50 98 0a 1a 08 49 64 11 00 00 00 00 a5 


04/27  1:11:48: REC: MedErr on LD[0] BadLba=3c65098


04/27  1:11:48: prCallback: Medium Error on pd=00, StartLba=3c64e39, ErrLba=3c65098


04/27  1:11:48: prRecQueue: starting pd=00 recovery - blocking host commands


04/27  1:11:48: prRecGo: Ready to attempt recovery errLBA=3c65098 on pd=00


04/27  1:11:48: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/27  1:11:48: prRecGo: dataErr found on ld 0 span 0 arm 0


04/27  1:11:48: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78ca1, stripe=78ca1, refBlk=18, type=1


04/27  1:11:48: prRecGo: RAID1 - read from peer pd=01


04/27  1:11:50: REC:log MedErr on pd[1] #retries=0


04/27  1:11:50: LogSense: pd=01, cdb=2f 00 03 c1 04 54 00 ff ff 00 


04/27  1:11:50:           sense=f0 00 03 03 c1 5d 6c 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  1:11:50: REC: MedErr on LD[1] BadLba=3c15d6c


04/27  1:11:50: prCallback: Medium Error on pd=01, StartLba=3c10454, ErrLba=3c15d6c


04/27  1:11:50: prRecQueue: adding pd=01 to recovery wait queue


04/27  1:11:52: REC:log MedErr on pd[1] #retries=0


04/27  1:11:52: LogSense: pd=01, cdb=28 00 03 c6 50 98 00 00 01 00 


04/27  1:11:52:           sense=f0 00 03 03 c6 50 98 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  1:11:52: REC: MedErr on LD[1] BadLba=3c65098


04/27  1:11:52: prRecCallback: Error, cdb=28, pd=01, err=2


04/27  1:11:52: PatrolReadStart: Log MedErr pd=00 errLBA=3c65098 in NVRAM_EVENT_LOG


04/27  1:11:52: prRecGo: Ready to attempt recovery errLBA=3c15d6c on pd=01


04/27  1:11:52: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/27  1:11:52: prRecGo: dataErr found on ld 0 span 0 arm 1


04/27  1:11:52: prRecGo: data NOT in cache; cacheLn=ffffffff, row=782ba, stripe=782ba, refBlk=6c, type=1


04/27  1:11:52: prRecGo: RAID1 - read from peer pd=00


04/27  1:11:54: REC:log MedErr on pd[0] #retries=0


04/27  1:11:54: LogSense: pd=00, cdb=28 00 03 c1 5d 6c 00 00 01 00 


04/27  1:11:54:           sense=f0 00 03 03 c1 5d 6c 0a 03 0c 48 fd 11 00 00 00 00 a5 


04/27  1:11:54: REC: MedErr on LD[0] BadLba=3c15d6c


04/27  1:11:54: prRecCallback: Error, cdb=28, pd=00, err=2


04/27  1:11:54: PatrolReadStart: Log MedErr pd=01 errLBA=3c15d6c in NVRAM_EVENT_LOG


04/27  1:11:57: REC:log MedErr on pd[0] #retries=0


04/27  1:11:57: LogSense: pd=00, cdb=2f 00 03 c6 50 99 00 ff ff 00 


04/27  1:11:57:           sense=f0 00 03 03 c6 64 d8 0a 18 a8 49 65 11 00 00 00 00 a5 


04/27  1:11:57: REC: MedErr on LD[0] BadLba=3c664d8


04/27  1:11:57: prCallback: Medium Error on pd=00, StartLba=3c65099, ErrLba=3c664d8


04/27  1:11:57: prRecQueue: starting pd=00 recovery - blocking host commands


04/27  1:11:57: prRecGo: Ready to attempt recovery errLBA=3c664d8 on pd=00


04/27  1:11:57: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/27  1:11:57: prRecGo: dataErr found on ld 0 span 0 arm 0


04/27  1:11:57: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78cc9, stripe=78cc9, refBlk=58, type=1


04/27  1:11:57: prRecGo: RAID1 - read from peer pd=01


04/27  1:11:59: REC:log MedErr on pd[1] #retries=0


04/27  1:11:59: LogSense: pd=01, cdb=28 00 03 c6 64 d8 00 00 01 00 


04/27  1:11:59:           sense=f0 00 03 03 c6 64 d8 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  1:11:59: REC: MedErr on LD[1] BadLba=3c664d8


04/27  1:11:59: prRecCallback: Error, cdb=28, pd=01, err=2


04/27  1:11:59: PatrolReadStart: Log MedErr pd=00 errLBA=3c664d8 in NVRAM_EVENT_LOG


04/27  1:12:05: REC:log MedErr on pd[1] #retries=0


04/27  1:12:05: LogSense: pd=01, cdb=2f 00 03 c1 65 6d 00 ff ff 00 


04/27  1:12:05:           sense=f0 00 03 03 c1 71 8d 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  1:12:05: REC: MedErr on LD[1] BadLba=3c1718d


04/27  1:12:05: prCallback: Medium Error on pd=01, StartLba=3c1656d, ErrLba=3c1718d


04/27  1:12:05: prRecQueue: starting pd=01 recovery - blocking host commands


04/27  1:12:05: prRecGo: Ready to attempt recovery errLBA=3c1718d on pd=01


04/27  1:12:05: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/27  1:12:05: prRecGo: dataErr found on ld 0 span 0 arm 1


04/27  1:12:05: prRecGo: data NOT in cache; cacheLn=ffffffff, row=782e3, stripe=782e3, refBlk=d, type=1


04/27  1:12:05: prRecGo: RAID1 - read from peer pd=00


04/27  1:12:06: REC:log MedErr on pd[0] #retries=0


04/27  1:12:06: LogSense: pd=00, cdb=28 00 03 c1 71 8d 00 00 01 00 


04/27  1:12:06:           sense=f0 00 03 03 c1 71 8d 0a 01 a2 48 fe 11 00 00 00 00 a5 


04/27  1:12:06: REC: MedErr on LD[0] BadLba=3c1718d


04/27  1:12:06: prRecCallback: Error, cdb=28, pd=00, err=2


04/27  1:12:06: PatrolReadStart: Log MedErr pd=01 errLBA=3c1718d in NVRAM_EVENT_LOG


04/27  1:12:28: REC:log MedErr on pd[1] #retries=0


04/27  1:12:28: LogSense: pd=01, cdb=2f 00 03 c5 71 8a 00 ff ff 00 


04/27  1:12:28:           sense=f0 00 03 03 c6 4e 38 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  1:12:28: REC: MedErr on LD[1] BadLba=3c64e38


04/27  1:12:28: prCallback: Medium Error on pd=01, StartLba=3c5718a, ErrLba=3c64e38


04/27  1:12:28: prRecQueue: starting pd=01 recovery - blocking host commands


04/27  1:12:28: prRecGo: Ready to attempt recovery errLBA=3c64e38 on pd=01


04/27  1:12:28: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/27  1:12:28: prRecGo: dataErr found on ld 0 span 0 arm 1


04/27  1:12:28: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78c9c, stripe=78c9c, refBlk=38, type=1


04/27  1:12:28: prRecGo: RAID1 - read from peer pd=00


04/27  1:12:30: REC:log MedErr on pd[0] #retries=0


04/27  1:12:30: LogSense: pd=00, cdb=28 00 03 c6 4e 38 00 00 01 00 


04/27  1:12:30:           sense=f0 00 03 03 c6 4e 38 0a 1a a8 49 63 11 00 00 00 00 a5 


04/27  1:12:30: REC: MedErr on LD[0] BadLba=3c64e38


04/27  1:12:30: prRecCallback: Error, cdb=28, pd=00, err=2


04/27  1:12:30: PatrolReadStart: Log MedErr pd=01 errLBA=3c64e38 in NVRAM_EVENT_LOG


04/27  1:12:36: REC:log MedErr on pd[1] #retries=0


04/27  1:12:36: LogSense: pd=01, cdb=2f 00 03 c6 4e 39 00 ff ff 00 


04/27  1:12:36:           sense=f0 00 03 03 c6 50 98 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  1:12:36: REC: MedErr on LD[1] BadLba=3c65098


04/27  1:12:36: prCallback: Medium Error on pd=01, StartLba=3c64e39, ErrLba=3c65098


04/27  1:12:36: prRecQueue: starting pd=01 recovery - blocking host commands


04/27  1:12:36: prRecGo: Ready to attempt recovery errLBA=3c65098 on pd=01


04/27  1:12:36: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/27  1:12:36: prRecGo: dataErr found on ld 0 span 0 arm 1


04/27  1:12:36: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78ca1, stripe=78ca1, refBlk=18, type=1


04/27  1:12:36: prRecGo: RAID1 - read from peer pd=00


04/27  1:12:37: REC:log MedErr on pd[0] #retries=0


04/27  1:12:37: LogSense: pd=00, cdb=28 00 03 c6 50 98 00 00 01 00 


04/27  1:12:37:           sense=f0 00 03 03 c6 50 98 0a 1a 08 49 64 11 00 00 00 00 a5 


04/27  1:12:37: REC: MedErr on LD[0] BadLba=3c65098


04/27  1:12:37: prRecCallback: Error, cdb=28, pd=00, err=2


04/27  1:12:37: PatrolReadStart: Log MedErr pd=01 errLBA=3c65098 in NVRAM_EVENT_LOG


04/27  1:12:43: REC:log MedErr on pd[1] #retries=0


04/27  1:12:43: LogSense: pd=01, cdb=2f 00 03 c6 50 99 00 ff ff 00 


04/27  1:12:43:           sense=f0 00 03 03 c6 64 d8 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  1:12:43: REC: MedErr on LD[1] BadLba=3c664d8


04/27  1:12:43: prCallback: Medium Error on pd=01, StartLba=3c65099, ErrLba=3c664d8


04/27  1:12:43: prRecQueue: starting pd=01 recovery - blocking host commands


04/27  1:12:43: prRecGo: Ready to attempt recovery errLBA=3c664d8 on pd=01


04/27  1:12:43: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/27  1:12:43: prRecGo: dataErr found on ld 0 span 0 arm 1


04/27  1:12:43: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78cc9, stripe=78cc9, refBlk=58, type=1


04/27  1:12:43: prRecGo: RAID1 - read from peer pd=00


04/27  1:12:44: REC:log MedErr on pd[0] #retries=0


04/27  1:12:44: LogSense: pd=00, cdb=28 00 03 c6 64 d8 00 00 01 00 


04/27  1:12:44:           sense=f0 00 03 03 c6 64 d8 0a 18 a8 49 65 11 00 00 00 00 a5 


04/27  1:12:44: REC: MedErr on LD[0] BadLba=3c664d8


04/27  1:12:44: prRecCallback: Error, cdb=28, pd=00, err=2


04/27  1:12:44: PatrolReadStart: Log MedErr pd=01 errLBA=3c664d8 in NVRAM_EVENT_LOG


04/27  1:15:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  1:15:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  1:15:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  1:20:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  1:20:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  1:20:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  1:25:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  1:25:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  1:25:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  1:30:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  1:30:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  1:30:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  1:35:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  1:35:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  1:35:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  1:40:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  1:40:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  1:40:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  1:45:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  1:45:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  1:45:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  1:50:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  1:50:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  1:50:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  1:55:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  1:55:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  1:55:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  2:00:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  2:00:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  2:00:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  2:05:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  2:05:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  2:05:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  2:10:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  2:10:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  2:10:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  2:15:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  2:15:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  2:15:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  2:20:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  2:20:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  2:20:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  2:25:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  2:25:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  2:25:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  �����€�� LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  2:30:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  2:30:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  2:35:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  2:35:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  2:35:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  2:38:28: prCallback: PR completed for pd=00


04/27  2:38:28: prDiskDone: finishing PR on PD=00


04/27  2:40:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  2:40:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  2:40:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  2:41:56: prCallback: PR completed for pd=01


04/27  2:41:56: prDiskDone: finishing PR on PD=01


04/27  2:45:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  2:45:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  2:45:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  2:50:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  2:50:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  2:50:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  2:55:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  2:55:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  2:55:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  3:00:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  3:00:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  3:00:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  3:05:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  3:05:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  3:05:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  3:10:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  3:10:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  3:10:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  3:15:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  3:15:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  3:15:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  3:20:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  3:20:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  3:20:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  3:25:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  3:25:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  3:25:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  3:30:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  3:30:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  3:30:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  3:35:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  3:35:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  3:35:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  3:40:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  3:40:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  3:40:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  3:45:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  3:45:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  3:45:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  3:50:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  3:50:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  3:50:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  3:55:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  3:55:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  3:55:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  4:00:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  4:00:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  4:00:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  4:05:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  4:05:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  4:05:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  4:10:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  4:10:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  4:10:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  4:15:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  4:15:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  4:15:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  4:20:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  4:20:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  4:20:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  4:25:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  4:25:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  4:25:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  4:30:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  4:30:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  4:30:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  4:35:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  4:35:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  4:35:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  4:40:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  4:40:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  4:40:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  4:45:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  4:45:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  4:45:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  4:50:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  4:50:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  4:50:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  4:51:44: prCallback: PR completed for pd=14


04/27  4:51:44: prDiskDone: finishing PR on PD=14


04/27  4:51:44: prCallback: PR completed for pd=12


04/27  4:51:44: prDiskDone: finishing PR on PD=12


04/27  4:51:45: prCallback: PR completed for pd=13


04/27  4:51:45: prDiskDone: finishing PR on PD=13


04/27  4:51:45: PR cycle complete


04/27  4:51:46: prDiskStart: starting Patrol Read on PD=00


04/27  4:51:46: prDiskStart: starting Patrol Read on PD=01


04/27  4:51:46: prDiskStart: starting Patrol Read on PD=12


04/27  4:51:46: prDiskStart: starting Patrol Read on PD=13


04/27  4:51:46: prDiskStart: starting Patrol Read on PD=14


04/27  4:51:46: Next PR scheduled to start at 04/27  8:00:00


04/27  4:55:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  4:55:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  4:55:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  5:00:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  5:00:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  5:00:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  5:05:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  5:05:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  5:05:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  5:10:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  5:10:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  5:10:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  5:15:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  5:15:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  5:15:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  5:20:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  5:20:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  5:20:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  5:25:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  5:25:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  5:25:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  5:30:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  5:30:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  5:30:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  5:35:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  5:35:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  5:35:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  5:40:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  5:40:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  5:40:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  5:45:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  5:45:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  5:45:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  5:50:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  5:50:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  5:50:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  5:55:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  5:55:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  5:55:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  5:58:51: REC:log MedErr on pd[0] #retries=0


04/27  5:58:51: LogSense: pd=00, cdb=2f 00 03 b1 26 0f 00 ff ff 00 


04/27  5:58:51:           sense=f0 00 03 03 b1 84 6e 0a 1a 20 47 b4 11 0b 00 00 00 79 


04/27  5:58:51: REC: MedErr on LD[0] BadLba=3b1846e


04/27  5:58:51: prCallback: Medium Error on pd=00, StartLba=3b1260f, ErrLba=3b1846e


04/27  5:58:51: prRecQueue: starting pd=00 recovery - blocking host commands


04/27  5:58:51: prRecGo: Ready to attempt recovery errLBA=3b1846e on pd=00


04/27  5:58:51: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/27  5:58:51: prRecGo: dataErr found on ld 0 span 0 arm 0


04/27  5:58:51: prRecGo: data NOT in cache; cacheLn=ffffffff, row=76308, stripe=76308, refBlk=6e, type=1


04/27  5:58:51: prRecGo: RAID1 - read from peer pd=01


04/27  5:58:52: REC:log MedErr on pd[1] #retries=0


04/27  5:58:52: LogSense: pd=01, cdb=2f 00 03 b1 26 0f 00 ff ff 00 


04/27  5:58:52:           sense=f0 00 03 03 b1 84 6e 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  5:58:52: REC: MedErr on LD[1] BadLba=3b1846e


04/27  5:58:52: prCallback: Medium Error on pd=01, StartLba=3b1260f, ErrLba=3b1846e


04/27  5:58:52: prRecQueue: adding pd=01 to recovery wait queue


04/27  5:58:54: REC:log MedErr on pd[1] #retries=0


04/27  5:58:54: LogSense: pd=01, cdb=28 00 03 b1 84 6e 00 00 01 00 


04/27  5:58:54:           sense=f0 00 03 03 b1 84 6e 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  5:58:54: REC: MedErr on LD[1] BadLba=3b1846e


04/27  5:58:54: prRecCallback: Error, cdb=28, pd=01, err=2


04/27  5:58:54: PatrolReadStart: Log MedErr pd=00 errLBA=3b1846e in NVRAM_EVENT_LOG


04/27  5:58:54: prRecGo: Ready to attempt recovery errLBA=3b1846e on pd=01


04/27  5:58:54: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/27  5:58:54: prRecGo: dataErr found on ld 0 span 0 arm 1


04/27  5:58:54: prRecGo: data NOT in cache; cacheLn=ffffffff, row=76308, stripe=76308, refBlk=6e, type=1


04/27  5:58:54: prRecGo: RAID1 - read from peer pd=00


04/27  5:58:55: REC:log MedErr on pd[0] #retries=0


04/27  5:58:55: LogSense: pd=00, cdb=28 00 03 b1 84 6e 00 00 01 00 


04/27  5:58:55:           sense=f0 00 03 03 b1 84 6e 0a 1a 20 47 b4 11 0b 00 00 00 79 


04/27  5:58:55: REC: MedErr on LD[0] BadLba=3b1846e


04/27  5:58:55: prRecCallback: Error, cdb=28, pd=00, err=2


04/27  5:58:55: PatrolReadStart: Log MedErr pd=01 errLBA=3b1846e in NVRAM_EVENT_LOG


04/27  5:58:59: REC:log MedErr on pd[0] #retries=0


04/27  5:58:59: LogSense: pd=00, cdb=2f 00 03 b1 84 6f 00 ff ff 00 


04/27  5:58:59:           sense=f0 00 03 03 b1 98 ee 0a 18 c0 47 b5 11 0b 00 00 00 79 


04/27  5:58:59: REC: MedErr on LD[0] BadLba=3b198ee


04/27  5:58:59: prCallback: Medium Error on pd=00, StartLba=3b1846f, ErrLba=3b198ee


04/27  5:58:59: prRecQueue: starting pd=00 recovery - blocking host commands


04/27  5:58:59: prRecGo: Ready to attempt recovery errLBA=3b198ee on pd=00


04/27  5:58:59: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/27  5:58:59: prRecGo: dataErr found on ld 0 span 0 arm 0


04/27  5:58:59: prRecGo: data NOT in cache; cacheLn=ffffffff, row=76331, stripe=76331, refBlk=6e, type=1


04/27  5:58:59: prRecGo: RAID1 - read from peer pd=01


04/27  5:59:01: REC:log MedErr on pd[1] #retries=0


04/27  5:59:01: LogSense: pd=01, cdb=2f 00 03 b1 84 6f 00 ff ff 00 


04/27  5:59:01:           sense=f0 00 03 03 b1 98 ee 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  5:59:01: REC: MedErr on LD[1] BadLba=3b198ee


04/27  5:59:01: prCallback: Medium Error on pd=01, StartLba=3b1846f, ErrLba=3b198ee


04/27  5:59:01: prRecQueue: adding pd=01 to recovery wait queue


04/27  5:59:03: REC:log MedErr on pd[1] #retries=0


04/27  5:59:03: LogSense: pd=01, cdb=28 00 03 b1 98 ee 00 00 01 00 


04/27  5:59:03:           sense=f0 00 03 03 b1 98 ee 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  5:59:03: REC: MedErr on LD[1] BadLba=3b198ee


04/27  5:59:03: prRecCallback: Error, cdb=28, pd=01, err=2


04/27  5:59:03: PatrolReadStart: Log MedErr pd=00 errLBA=3b198ee in NVRAM_EVENT_LOG


04/27  5:59:03: prRecGo: Ready to attempt recovery errLBA=3b198ee on pd=01


04/27  5:59:03: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/27  5:59:03: prRecGo: dataErr found on ld 0 span 0 arm 1


04/27  5:59:03: prRecGo: data NOT in cache; cacheLn=ffffffff, row=76331, stripe=76331, refBlk=6e, type=1


04/27  5:59:03: prRecGo: RAID1 - read from peer pd=00


04/27  5:59:04: REC:log MedErr on pd[0] #retries=0


04/27  5:59:04: LogSense: pd=00, cdb=28 00 03 b1 98 ee 00 00 01 00 


04/27  5:59:04:           sense=f0 00 03 03 b1 98 ee 0a 18 c0 47 b5 11 0b 00 00 00 79 


04/27  5:59:04: REC: MedErr on LD[0] BadLba=3b198ee


04/27  5:59:04: prRecCallback: Error, cdb=28, pd=00, err=2


04/27  5:59:04: PatrolReadStart: Log MedErr pd=01 errLBA=3b198ee in NVRAM_EVENT_LOG


04/27  5:59:07: REC:log MedErr on pd[0] #retries=0


04/27  5:59:07: LogSense: pd=00, cdb=2f 00 03 b1 98 ef 00 ff ff 00 


04/27  5:59:07:           sense=f0 00 03 03 b1 9b 4e 0a 18 20 47 b6 11 0b 00 00 00 79 


04/27  5:59:07: REC: MedErr on LD[0] BadLba=3b19b4e


04/27  5:59:07: prCallback: Medium Error on pd=00, StartLba=3b198ef, ErrLba=3b19b4e


04/27  5:59:07: prRecQueue: starting pd=00 recovery - blocking host commands


04/27  5:59:07: prRecGo: Ready to attempt recovery errLBA=3b19b4e on pd=00


04/27  5:59:07: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/27  5:59:07: prRecGo: dataErr found on ld 0 span 0 arm 0


04/27  5:59:07: prRecGo: data NOT in cache; cacheLn=ffffffff, row=76336, stripe=76336, refBlk=4e, type=1


04/27  5:59:08: prRecGo: RAID1 - read from peer pd=01


04/27  5:59:10: REC:log MedErr on pd[1] #retries=0


04/27  5:59:10: LogSense: pd=01, cdb=28 00 03 b1 9b 4e 00 00 01 00 


04/27  5:59:10:           sense=f0 00 03 03 b1 9b 4e 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  5:59:10: REC: MedErr on LD[1] BadLba=3b19b4e


04/27  5:59:10: prRecCallback: Error, cdb=28, pd=01, err=2


04/27  5:59:10: PatrolReadStart: Log MedErr pd=00 errLBA=3b19b4e in NVRAM_EVENT_LOG


04/27  5:59:12: REC:log MedErr on pd[1] #retries=0


04/27  5:59:12: LogSense: pd=01, cdb=2f 00 03 b1 98 ef 00 08 00 00 


04/27  5:59:12:           sense=f0 00 03 03 b1 9b 4e 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  5:59:12: REC: MedErr on LD[1] BadLba=3b19b4e


04/27  5:59:12: prCallback: Medium Error on pd=01, StartLba=3b198ef, ErrLba=3b19b4e


04/27  5:59:12: prRecQueue: starting pd=01 recovery - blocking host commands


04/27  5:59:12: prRecGo: Ready to attempt recovery errLBA=3b19b4e on pd=01


04/27  5:59:12: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/27  5:59:12: prRecGo: dataErr found on ld 0 span 0 arm 1


04/27  5:59:12: prRecGo: data NOT in cache; cacheLn=ffffffff, row=76336, stripe=76336, refBlk=4e, type=1


04/27  5:59:12: prRecGo: RAID1 - read from peer pd=00


04/27  5:59:13: REC:log MedErr on pd[0] #retries=0


04/27  5:59:13: LogSense: pd=00, cdb=28 00 03 b1 9b 4e 00 00 01 00 


04/27  5:59:13:           sense=f0 00 03 03 b1 9b 4e 0a 18 20 47 b6 11 0b 00 00 00 79 


04/27  5:59:13: REC: MedErr on LD[0] BadLba=3b19b4e


04/27  5:59:13: prRecCallback: Error, cdb=28, pd=00, err=2


04/27  5:59:13: PatrolReadStart: Log MedErr pd=01 errLBA=3b19b4e in NVRAM_EVENT_LOG


04/27  6:00:03: REC:log MedErr on pd[0] #retries=0


04/27  6:00:03: LogSense: pd=00, cdb=2f 00 03 bc 9b 84 00 ff ff 00 


04/27  6:00:03:           sense=f0 00 03 03 bd 89 65 0a 11 d4 48 ad 11 0b 00 00 00 79 


04/27  6:00:03: REC: MedErr on LD[0] BadLba=3bd8965


04/27  6:00:03: prCallback: Medium Error on pd=00, StartLba=3bc9b84, ErrLba=3bd8965


04/27  6:00:03: prRecQueue: starting pd=00 recovery - blocking host commands


04/27  6:00:03: prRecGo: Ready to attempt recovery errLBA=3bd8965 on pd=00


04/27  6:00:03: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/27  6:00:03: prRecGo: dataErr found on ld 0 span 0 arm 0


04/27  6:00:03: prRecGo: data NOT in cache; cacheLn=ffffffff, row=77b12, stripe=77b12, refBlk=65, type=1


04/27  6:00:03: prRecGo: RAID1 - read from peer pd=01


04/27  6:00:05: REC:log MedErr on pd[1] #retries=0


04/27  6:00:05: LogSense: pd=01, cdb=28 00 03 bd 89 65 00 00 01 00 


04/27  6:00:05:           sense=f0 00 03 03 bd 89 65 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  6:00:05: REC: MedErr on LD[1] BadLba=3bd8965


04/27  6:00:05: prRecCallback: Error, cdb=28, pd=01, err=2


04/27  6:00:05: PatrolReadStart: Log MedErr pd=00 errLBA=3bd8965 in NVRAM_EVENT_LOG


04/27  6:00:09: REC:log MedErr on pd[0] #retries=0


04/27  6:00:09: LogSense: pd=00, cdb=2f 00 03 bd 89 66 00 ff ff 00 


04/27  6:00:09:           sense=f0 00 03 03 bd 93 b9 0a 13 02 48 ae 11 0b 00 00 00 79 


04/27  6:00:09: REC: MedErr on LD[0] BadLba=3bd93b9


04/27  6:00:09: prCallback: Medium Error on pd=00, StartLba=3bd8966, ErrLba=3bd93b9


04/27  6:00:09: prRecQueue: starting pd=00 recovery - blocking host commands


04/27  6:00:09: prRecGo: Ready to attempt recovery errLBA=3bd93b9 on pd=00


04/27  6:00:09: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/27  6:00:09: prRecGo: dataErr found on ld 0 span 0 arm 0


04/27  6:00:10: prRecGo: data NOT in cache; cacheLn=ffffffff, row=77b27, stripe=77b27, refBlk=39, type=1


04/27  6:00:10: prRecGo: RAID1 - read from peer pd=01


04/27  6:00:12: REC:log MedErr on pd[1] #retries=0


04/27  6:00:12: LogSense: pd=01, cdb=28 00 03 bd 93 b9 00 00 01 00 


04/27  6:00:12:           sense=f0 00 03 03 bd 93 b9 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  6:00:12: REC: MedErr on LD[1] BadLba=3bd93b9


04/27  6:00:12: prRecCallback: Error, cdb=28, pd=01, err=2


04/27  6:00:12: PatrolReadStart: Log MedErr pd=00 errLBA=3bd93b9 in NVRAM_EVENT_LOG


04/27  6:00:15: REC:log MedErr on pd[1] #retries=0


04/27  6:00:15: LogSense: pd=01, cdb=2f 00 03 bc a3 c4 00 ff ff 00 


04/27  6:00:15:           sense=f0 00 03 03 bd 89 65 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  6:00:15: REC: MedErr on LD[1] BadLba=3bd8965


04/27  6:00:15: prCallback: Medium Error on pd=01, StartLba=3bca3c4, ErrLba=3bd8965


04/27  6:00:15: prRecQueue: starting pd=01 recovery - blocking host commands


04/27  6:00:15: prRecGo: Ready to attempt recovery errLBA=3bd8965 on pd=01


04/27  6:00:15: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/27  6:00:15: prRecGo: dataErr found on ld 0 span 0 arm 1


04/27  6:00:15: prRecGo: data NOT in cache; cacheLn=ffffffff, row=77b12, stripe=77b12, refBlk=65, type=1


04/27  6:00:15: prRecGo: RAID1 - read from peer pd=00


04/27  6:00:16: REC:log MedErr on pd[0] #retries=0


04/27  6:00:16: LogSense: pd=00, cdb=28 00 03 bd 89 65 00 00 01 00 


04/27  6:00:16:           sense=f0 00 03 03 bd 89 65 0a 11 d4 48 ad 11 0b 00 00 00 79 


04/27  6:00:16: REC: MedErr on LD[0] BadLba=3bd8965


04/27  6:00:16: prRecCallback: Error, cdb=28, pd=00, err=2


04/27  6:00:16: PatrolReadStart: Log MedErr pd=01 errLBA=3bd8965 in NVRAM_EVENT_LOG


04/27  6:00:22: REC:log MedErr on pd[1] #retries=0


04/27  6:00:22: LogSense: pd=01, cdb=2f 00 03 bd 89 66 00 ff ff 00 


04/27  6:00:22:           sense=f0 00 03 03 bd 93 b9 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  6:00:22: REC: MedErr on LD[1] BadLba=3bd93b9


04/27  6:00:22: prCallback: Medium Error on pd=01, StartLba=3bd8966, ErrLba=3bd93b9


04/27  6:00:22: prRecQueue: starting pd=01 recovery - blocking host commands


04/27  6:00:22: prRecGo: Ready to attempt recovery errLBA=3bd93b9 on pd=01


04/27  6:00:22: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/27  6:00:22: prRecGo: dataErr found on ld 0 span 0 arm 1


04/27  6:00:22: prRecGo: data NOT in cache; cacheLn=ffffffff, row=77b27, stripe=77b27, refBlk=39, type=1


04/27  6:00:22: prRecGo: RAID1 - read from peer pd=00


04/27  6:00:23: REC:log MedErr on pd[0] #retries=0


04/27  6:00:23: LogSense: pd=00, cdb=28 00 03 bd 93 b9 00 00 01 00 


04/27  6:00:23:           sense=f0 00 03 03 bd 93 b9 0a 13 02 48 ae 11 0b 00 00 00 79 


04/27  6:00:23: REC: MedErr on LD[0] BadLba=3bd93b9


04/27  6:00:23: prRecCallback: Error, cdb=28, pd=00, err=2


04/27  6:00:23: PatrolReadStart: Log MedErr pd=01 errLBA=3bd93b9 in NVRAM_EVENT_LOG


04/27  6:00:25: REC:log MedErr on pd[0] #retries=0


04/27  6:00:25: LogSense: pd=00, cdb=2f 00 03 be 93 f9 00 ff ff 00 


04/27  6:00:25:           sense=f0 00 03 03 bf 04 f4 0a 12 df 48 cc 11 00 00 00 00 a5 


04/27  6:00:25: REC: MedErr on LD[0] BadLba=3bf04f4


04/27  6:00:25: prCallback: Medium Error on pd=00, StartLba=3be93f9, ErrLba=3bf04f4


04/27  6:00:25: prRecQueue: starting pd=00 recovery - blocking host commands


04/27  6:00:25: prRecGo: Ready to attempt recovery errLBA=3bf04f4 on pd=00


04/27  6:00:25: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/27  6:00:25: prRecGo: dataErr found on ld 0 span 0 arm 0


04/27  6:00:25: prRecGo: data NOT in cache; cacheLn=ffffffff, row=77e09, stripe=77e09, refBlk=74, type=1


04/27  6:00:25: prRecGo: RAID1 - read from peer pd=01


04/27  6:00:27: REC:log MedErr on pd[1] #retries=0


04/27  6:00:27: LogSense: pd=01, cdb=28 00 03 bf 04 f4 00 00 01 00 


04/27  6:00:27:           sense=f0 00 03 03 bf 04 f4 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  6:00:27: REC: MedErr on LD[1] BadLba=3bf04f4


04/27  6:00:27: prRecCallback: Error, cdb=28, pd=01, err=2


04/27  6:00:27: PatrolReadStart: Log MedErr pd=00 errLBA=3bf04f4 in NVRAM_EVENT_LOG


04/27  6:00:34: REC:log MedErr on pd[0] #retries=0


04/27  6:00:34: LogSense: pd=00, cdb=2f 00 03 bf 04 f5 00 ff ff 00 


04/27  6:00:34:           sense=f0 00 03 03 bf 83 6f 0a 00 01 48 d6 11 0b 00 00 01 ab 


04/27  6:00:34: REC: MedErr on LD[0] BadLba=3bf836f


04/27  6:00:34: prCallback: Medium Error on pd=00, StartLba=3bf04f5, ErrLba=3bf836f


04/27  6:00:34: prRecQueue: starting pd=00 recovery - blocking host commands


04/27  6:00:34: prRecGo: Ready to attempt recovery errLBA=3bf836f on pd=00


04/27  6:00:34: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/27  6:00:34: prRecGo: dataErr found on ld 0 span 0 arm 0


04/27  6:00:34: prRecGo: data NOT in cache; cacheLn=ffffffff, row=77f06, stripe=77f06, refBlk=6f, type=1


04/27  6:00:34: prRecGo: RAID1 - read from peer pd=01


04/27  6:00:36: REC:log MedErr on pd[1] #retries=0


04/27  6:00:36: LogSense: pd=01, cdb=28 00 03 bf 83 6f 00 00 01 00 


04/27  6:00:36:           sense=f0 00 03 03 bf 83 6f 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  6:00:36: REC: MedErr on LD[1] BadLba=3bf836f


04/27  6:00:36: prRecCallback: Error, cdb=28, pd=01, err=2


04/27  6:00:36: PatrolReadStart: Log MedErr pd=00 errLBA=3bf836f in NVRAM_EVENT_LOG


04/27  6:00:41: REC:log MedErr on pd[0] #retries=0


04/27  6:00:41: LogSense: pd=00, cdb=2f 00 03 bf 83 70 00 ff ff 00 


04/27  6:00:41:           sense=f0 00 03 03 c0 59 2f 0a 19 5f 48 e8 11 00 00 00 00 a5 


04/27  6:00:41: REC: MedErr on LD[0] BadLba=3c0592f


04/27  6:00:41: prCallback: Medium Error on pd=00, StartLba=3bf8370, ErrLba=3c0592f


04/27  6:00:41: prRecQueue: starting pd=00 recovery - blocking host commands


04/27  6:00:41: prRecGo: Ready to attempt recovery errLBA=3c0592f on pd=00


04/27  6:00:41: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/27  6:00:41: prRecGo: dataErr found on ld 0 span 0 arm 0


04/27  6:00:41: prRecGo: data NOT in cache; cacheLn=ffffffff, row=780b2, stripe=780b2, refBlk=2f, type=1


04/27  6:00:41: prRecGo: RAID1 - read from peer pd=01


04/27  6:00:42: REC:log MedErr on pd[1] #retries=0


04/27  6:00:42: LogSense: pd=01, cdb=2f 00 03 be 9b f9 00 ff ff 00 


04/27  6:00:42:           sense=f0 00 03 03 bf 04 f4 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  6:00:42: REC: MedErr on LD[1] BadLba=3bf04f4


04/27  6:00:42: prCallback: Medium Error on pd=01, StartLba=3be9bf9, ErrLba=3bf04f4


04/27  6:00:42: prRecQueue: adding pd=01 to recovery wait queue


04/27  6:00:44: REC:log MedErr on pd[1] #retries=0


04/27  6:00:44: LogSense: pd=01, cdb=28 00 03 c0 59 2f 00 00 01 00 


04/27  6:00:44:           sense=f0 00 03 03 c0 59 2f 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  6:00:44: REC: MedErr on LD[1] BadLba=3c0592f


04/27  6:00:44: prRecCallback: Error, cdb=28, pd=01, err=2


04/27  6:00:44: PatrolReadStart: Log MedErr pd=00 errLBA=3c0592f in NVRAM_EVENT_LOG


04/27  6:00:44: prRecGo: Ready to attempt recovery errLBA=3bf04f4 on pd=01


04/27  6:00:44: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/27  6:00:44: prRecGo: dataErr found on ld 0 span 0 arm 1


04/27  6:00:44: prRecGo: data NOT in cache; cacheLn=ffffffff, row=77e09, stripe=77e09, refBlk=74, type=1


04/27  6:00:44: prRecGo: RAID1 - read from peer pd=00


04/27  6:00:45: REC:log MedErr on pd[0] #retries=0


04/27  6:00:45: LogSense: pd=00, cdb=28 00 03 bf 04 f4 00 00 01 00 


04/27  6:00:45:           sense=f0 00 03 03 bf 04 f4 0a 12 df 48 cc 11 00 00 00 00 a5 


04/27  6:00:45: REC: MedErr on LD[0] BadLba=3bf04f4


04/27  6:00:45: prRecCallback: Error, cdb=28, pd=00, err=2


04/27  6:00:45: PatrolReadStart: Log MedErr pd=01 errLBA=3bf04f4 in NVRAM_EVENT_LOG


04/27  6:00:50: REC:log MedErr on pd[0] #retries=0


04/27  6:00:50: LogSense: pd=00, cdb=2f 00 03 c0 59 30 00 ff ff 00 


04/27  6:00:50:           sense=f0 00 03 03 c0 70 6f 0a 1a ff 48 e9 11 00 00 00 00 a5 


04/27  6:00:50: REC: MedErr on LD[0] BadLba=3c0706f


04/27  6:00:50: prCallback: Medium Error on pd=00, StartLba=3c05930, ErrLba=3c0706f


04/27  6:00:50: prRecQueue: starting pd=00 recovery - blocking host commands


04/27  6:00:50: prRecGo: Ready to attempt recovery errLBA=3c0706f on pd=00


04/27  6:00:50: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/27  6:00:50: prRecGo: dataErr found on ld 0 span 0 arm 0


04/27  6:00:50: prRecGo: data NOT in cache; cacheLn=ffffffff, row=780e0, stripe=780e0, refBlk=6f, type=1


04/27  6:00:50: prRecGo: RAID1 - read from peer pd=01


04/27  6:00:51: REC:log MedErr on pd[1] #retries=0


04/27  6:00:51: LogSense: pd=01, cdb=2f 00 03 bf 04 f5 00 ff ff 00 


04/27  6:00:51:           sense=f0 00 03 03 bf 83 6f 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  6:00:51: REC: MedErr on LD[1] BadLba=3bf836f


04/27  6:00:51: prCallback: Medium Error on pd=01, StartLba=3bf04f5, ErrLba=3bf836f


04/27  6:00:51: prRecQueue: adding pd=01 to recovery wait queue


04/27  6:00:53: REC:log MedErr on pd[1] #retries=0


04/27  6:00:53: LogSense: pd=01, cdb=28 00 03 c0 70 6f 00 00 01 00 


04/27  6:00:53:           sense=f0 00 03 03 c0 70 6f 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  6:00:53: REC: MedErr on LD[1] BadLba=3c0706f


04/27  6:00:53: prRecCallback: Error, cdb=28, pd=01, err=2


04/27  6:00:53: PatrolReadStart: Log MedErr pd=00 errLBA=3c0706f in NVRAM_EVENT_LOG


04/27  6:00:53: prRecGo: Ready to attempt recovery errLBA=3bf836f on pd=01


04/27  6:00:53: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/27  6:00:53: prRecGo: dataErr found on ld 0 span 0 arm 1


04/27  6:00:53: prRecGo: data NOT in cache; cacheLn=ffffffff, row=77f06, stripe=77f06, refBlk=6f, type=1


04/27  6:00:53: prRecGo: RAID1 - read from peer pd=00


04/27  6:00:56: REC:log MedErr on pd[0] #retries=0


04/27  6:00:56: LogSense: pd=00, cdb=28 00 03 bf 83 6f 00 00 01 00 


04/27  6:00:56:           sense=f0 00 03 03 bf 83 6f 0a 00 01 48 d6 11 0b 00 00 01 ab 


04/27  6:00:56: REC: MedErr on LD[0] BadLba=3bf836f


04/27  6:00:56: prRecCallback: Error, cdb=28, pd=00, err=2


04/27  6:00:56: PatrolReadStart: Log MedErr pd=01 errLBA=3bf836f in NVRAM_EVENT_LOG


04/27  6:00:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  6:00:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  6:00:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  6:00:58: REC:log MedErr on pd[0] #retries=0


04/27  6:00:58: LogSense: pd=00, cdb=2f 00 03 c0 70 70 00 08 00 00 


04/27  6:00:58:           sense=f0 00 03 03 c0 72 cf 0a 1a 5f 48 ea 11 00 00 00 00 a5 


04/27  6:00:58: REC: MedErr on LD[0] BadLba=3c072cf


04/27  6:00:58: prCallback: Medium Error on pd=00, StartLba=3c07070, ErrLba=3c072cf


04/27  6:00:58: prRecQueue: starting pd=00 recovery - blocking host commands


04/27  6:00:58: prRecGo: Ready to attempt recovery errLBA=3c072cf on pd=00


04/27  6:00:58: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/27  6:00:58: prRecGo: dataErr found on ld 0 span 0 arm 0


04/27  6:00:58: prRecGo: data NOT in cache; cacheLn=ffffffff, row=780e5, stripe=780e5, refBlk=4f, type=1


04/27  6:00:58: prRecGo: RAID1 - read from peer pd=01


04/27  6:01:00: REC:log MedErr on pd[1] #retries=0


04/27  6:01:00: LogSense: pd=01, cdb=28 00 03 c0 72 cf 00 00 01 00 


04/27  6:01:00:           sense=f0 00 03 03 c0 72 cf 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  6:01:00: REC: MedErr on LD[1] BadLba=3c072cf


04/27  6:01:00: prRecCallback: Error, cdb=28, pd=01, err=2


04/27  6:01:00: PatrolReadStart: Log MedErr pd=00 errLBA=3c072cf in NVRAM_EVENT_LOG


04/27  6:01:06: REC:log MedErr on pd[0] #retries=0


04/27  6:01:06: LogSense: pd=00, cdb=2f 00 03 c0 72 d0 00 ff ff 00 


04/27  6:01:06:           sense=f0 00 03 03 c0 87 de 0a 19 ce 48 eb 11 00 00 00 00 a5 


04/27  6:01:06: REC: MedErr on LD[0] BadLba=3c087de


04/27  6:01:06: prCallback: Medium Error on pd=00, StartLba=3c072d0, ErrLba=3c087de


04/27  6:01:06: prRecQueue: starting pd=00 recovery - blocking host commands


04/27  6:01:06: prRecGo: Ready to attempt recovery errLBA=3c087de on pd=00


04/27  6:01:06: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/27  6:01:06: prRecGo: dataErr found on ld 0 span 0 arm 0


04/27  6:01:06: prRecGo: data NOT in cache; cacheLn=ffffffff, row=7810f, stripe=7810f, refBlk=5e, type=1


04/27  6:01:06: prRecGo: RAID1 - read from peer pd=01


04/27  6:01:06: REC:log MedErr on pd[1] #retries=0


04/27  6:01:06: LogSense: pd=01, cdb=2f 00 03 bf 8b 70 00 ff ff 00 


04/27  6:01:06:           sense=f0 00 03 03 c0 59 2f 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  6:01:06: REC: MedErr on LD[1] BadLba=3c0592f


04/27  6:01:06: prCallback: Medium Error on pd=01, StartLba=3bf8b70, ErrLba=3c0592f


04/27  6:01:06: prRecQueue: adding pd=01 to recovery wait queue


04/27  6:01:08: REC:log MedErr on pd[1] #retries=0


04/27  6:01:08: LogSense: pd=01, cdb=28 00 03 c0 87 de 00 00 01 00 


04/27  6:01:08:           sense=f0 00 03 03 c0 87 de 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  6:01:08: REC: MedErr on LD[1] BadLba=3c087de


04/27  6:01:08: prRecCallback: Error, cdb=28, pd=01, err=2


04/27  6:01:08: PatrolReadStart: Log MedErr pd=00 errLBA=3c087de in NVRAM_EVENT_LOG


04/27  6:01:08: prRecGo: Ready to attempt recovery errLBA=3c0592f on pd=01


04/27  6:01:08: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/27  6:01:08: prRecGo: dataErr found on ld 0 span 0 arm 1


04/27  6:01:08: prRecGo: data NOT in cache; cacheLn=ffffffff, row=780b2, stripe=780b2, refBlk=2f, type=1


04/27  6:01:09: prRecGo: RAID1 - read from peer pd=00


04/27  6:01:10: REC:log MedErr on pd[0] #retries=0


04/27  6:01:10: LogSense: pd=00, cdb=28 00 03 c0 59 2f 00 00 01 00 


04/27  6:01:10:           sense=f0 00 03 03 c0 59 2f 0a 19 5f 48 e8 11 00 00 00 00 a5 


04/27  6:01:10: REC: MedErr on LD[0] BadLba=3c0592f


04/27  6:01:10: prRecCallback: Error, cdb=28, pd=00, err=2


04/27  6:01:10: PatrolReadStart: Log MedErr pd=01 errLBA=3c0592f in NVRAM_EVENT_LOG


04/27  6:01:13: REC:log MedErr on pd[0] #retries=0


04/27  6:01:13: LogSense: pd=00, cdb=2f 00 03 c0 87 df 00 ff ff 00 


04/27  6:01:13:           sense=f0 00 03 03�€���€��e 0a 19 2e 48 ec 11 00 00 00 00 a5 


04/27  6:01:13: REC: MedErr on LD[0] BadLba=3c08a3e


04/27  6:01:13: prCallback: Medium Error on pd=00, StartLba=3c087df, ErrLba=3c08a3e


04/27  6:01:13: prRecQueue: starting pd=00 recovery - blocking host commands


04/27  6:01:13: prRecGo: Ready to attempt recovery errLBA=3c08a3e on pd=00


04/27  6:01:13: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/27  6:01:13: prRecGo: dataErr found on ld 0 span 0 arm 0


04/27  6:01:13: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78114, stripe=78114, refBlk=3e, type=1


04/27  6:01:13: prRecGo: RAID1 - read from peer pd=01


04/27  6:01:15: REC:log MedErr on pd[1] #retries=0


04/27  6:01:15: LogSense: pd=01, cdb=28 00 03 c0 8a 3e 00 00 01 00 


04/27  6:01:15:           sense=f0 00 03 03 c0 8a 3e 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  6:01:15: REC: MedErr on LD[1] BadLba=3c08a3e


04/27  6:01:15: prRecCallback: Error, cdb=28, pd=01, err=2


04/27  6:01:15: PatrolReadStart: Log MedErr pd=00 errLBA=3c08a3e in NVRAM_EVENT_LOG


04/27  6:01:20: REC:log MedErr on pd[0] #retries=0


04/27  6:01:20: LogSense: pd=00, cdb=2f 00 03 c0 8a 3f 00 ff ff 00 


04/27  6:01:20:           sense=f0 00 03 03 c0 a3 de 0a 1a 2e 48 ee 11 00 00 00 00 a5 


04/27  6:01:20: REC: MedErr on LD[0] BadLba=3c0a3de


04/27  6:01:20: prCallback: Medium Error on pd=00, StartLba=3c08a3f, ErrLba=3c0a3de


04/27  6:01:20: prRecQueue: starting pd=00 recovery - blocking host commands


04/27  6:01:20: prRecGo: Ready to attempt recovery errLBA=3c0a3de on pd=00


04/27  6:01:20: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/27  6:01:20: prRecGo: dataErr found on ld 0 span 0 arm 0


04/27  6:01:20: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78147, stripe=78147, refBlk=5e, type=1


04/27  6:01:20: prRecGo: RAID1 - read from peer pd=01


04/27  6:01:21: REC:log MedErr on pd[1] #retries=0


04/27  6:01:21: LogSense: pd=01, cdb=2f 00 03 c0 61 30 00 ff ff 00 


04/27  6:01:21:           sense=f0 00 03 03 c0 70 6f 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  6:01:21: REC: MedErr on LD[1] BadLba=3c0706f


04/27  6:01:21: prCallback: Medium Error on pd=01, StartLba=3c06130, ErrLba=3c0706f


04/27  6:01:21: prRecQueue: adding pd=01 to recovery wait queue


04/27  6:01:23: REC:log MedErr on pd[1] #retries=0


04/27  6:01:23: LogSense: pd=01, cdb=28 00 03 c0 a3 de 00 00 01 00 


04/27  6:01:23:           sense=f0 00 03 03 c0 a3 de 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  6:01:23: REC: MedErr on LD[1] BadLba=3c0a3de


04/27  6:01:23: prRecCallback: Error, cdb=28, pd=01, err=2


04/27  6:01:23: PatrolReadStart: Log MedErr pd=00 errLBA=3c0a3de in NVRAM_EVENT_LOG


04/27  6:01:23: prRecGo: Ready to attempt recovery errLBA=3c0706f on pd=01


04/27  6:01:23: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/27  6:01:23: prRecGo: dataErr found on ld 0 span 0 arm 1


04/27  6:01:23: prRecGo: data NOT in cache; cacheLn=ffffffff, row=780e0, stripe=780e0, refBlk=6f, type=1


04/27  6:01:23: prRecGo: RAID1 - read from peer pd=00


04/27  6:01:25: REC:log MedErr on pd[0] #retries=0


04/27  6:01:25: LogSense: pd=00, cdb=28 00 03 c0 70 6f 00 00 01 00 


04/27  6:01:25:           sense=f0 00 03 03 c0 70 6f 0a 1a ff 48 e9 11 00 00 00 00 a5 


04/27  6:01:25: REC: MedErr on LD[0] BadLba=3c0706f


04/27  6:01:25: prRecCallback: Error, cdb=28, pd=00, err=2


04/27  6:01:25: PatrolReadStart: Log MedErr pd=01 errLBA=3c0706f in NVRAM_EVENT_LOG


04/27  6:01:29: REC:log MedErr on pd[0] #retries=0


04/27  6:01:29: LogSense: pd=00, cdb=2f 00 03 c0 a3 df 00 ff ff 00 


04/27  6:01:29:           sense=f0 00 03 03 c0 b8 1d 0a 18 cd 48 ef 11 00 00 00 00 a5 


04/27  6:01:29: REC: MedErr on LD[0] BadLba=3c0b81d


04/27  6:01:29: prCallback: Medium Error on pd=00, StartLba=3c0a3df, ErrLba=3c0b81d


04/27  6:01:29: prRecQueue: starting pd=00 recovery - blocking host commands


04/27  6:01:29: prRecGo: Ready to attempt recovery errLBA=3c0b81d on pd=00


04/27  6:01:29: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/27  6:01:29: prRecGo: dataErr found on ld 0 span 0 arm 0


04/27  6:01:29: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78170, stripe=78170, refBlk=1d, type=1


04/27  6:01:29: prRecGo: RAID1 - read from peer pd=01


04/27  6:01:31: REC:log MedErr on pd[1] #retries=0


04/27  6:01:31: LogSense: pd=01, cdb=2f 00 03 c0 70 70 00 ff ff 00 


04/27  6:01:31:           sense=f0 00 03 03 c0 72 cf 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  6:01:31: REC: MedErr on LD[1] BadLba=3c072cf


04/27  6:01:31: prCallback: Medium Error on pd=01, StartLba=3c07070, ErrLba=3c072cf


04/27  6:01:31: prRecQueue: adding pd=01 to recovery wait queue


04/27  6:01:33: REC:log MedErr on pd[1] #retries=0


04/27  6:01:33: LogSense: pd=01, cdb=28 00 03 c0 b8 1d 00 00 01 00 


04/27  6:01:33:           sense=f0 00 03 03 c0 b8 1d 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  6:01:33: REC: MedErr on LD[1] BadLba=3c0b81d


04/27  6:01:33: prRecCallback: Error, cdb=28, pd=01, err=2


04/27  6:01:33: PatrolReadStart: Log MedErr pd=00 errLBA=3c0b81d in NVRAM_EVENT_LOG


04/27  6:01:33: prRecGo: Ready to attempt recovery errLBA=3c072cf on pd=01


04/27  6:01:33: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/27  6:01:33: prRecGo: dataErr found on ld 0 span 0 arm 1


04/27  6:01:33: prRecGo: data NOT in cache; cacheLn=ffffffff, row=780e5, stripe=780e5, refBlk=4f, type=1


04/27  6:01:33: prRecGo: RAID1 - read from peer pd=00


04/27  6:01:34: REC:log MedErr on pd[0] #retries=0


04/27  6:01:34: LogSense: pd=00, cdb=28 00 03 c0 72 cf 00 00 01 00 


04/27  6:01:34:           sense=f0 00 03 03 c0 72 cf 0a 1a 5f 48 ea 11 00 00 00 00 a5 


04/27  6:01:34: REC: MedErr on LD[0] BadLba=3c072cf


04/27  6:01:34: prRecCallback: Error, cdb=28, pd=00, err=2


04/27  6:01:35: PatrolReadStart: Log MedErr pd=01 errLBA=3c072cf in NVRAM_EVENT_LOG


04/27  6:01:39: REC:log MedErr on pd[0] #retries=0


04/27  6:01:39: LogSense: pd=00, cdb=2f 00 03 c0 b8 1e 00 ff ff 00 


04/27  6:01:39:           sense=f0 00 03 03 c0 b8 1e 0a 18 ce 48 ef 11 00 00 00 00 a5 


04/27  6:01:39: REC: MedErr on LD[0] BadLba=3c0b81e


04/27  6:01:39: prCallback: Medium Error on pd=00, StartLba=3c0b81e, ErrLba=3c0b81e


04/27  6:01:39: prRecQueue: starting pd=00 recovery - blocking host commands


04/27  6:01:39: prRecGo: Ready to attempt recovery errLBA=3c0b81e on pd=00


04/27  6:01:39: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/27  6:01:39: prRecGo: dataErr found on ld 0 span 0 arm 0


04/27  6:01:39: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78170, stripe=78170, refBlk=1e, type=1


04/27  6:01:39: prRecGo: RAID1 - read from peer pd=01


04/27  6:01:41: REC:log MedErr on pd[1] #retries=0


04/27  6:01:41: LogSense: pd=01, cdb=2f 00 03 c0 72 d0 00 ff ff 00 


04/27  6:01:41:           sense=f0 00 03 03 c0 87 de 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  6:01:41: REC: MedErr on LD[1] BadLba=3c087de


04/27  6:01:41: prCallback: Medium Error on pd=01, StartLba=3c072d0, ErrLba=3c087de


04/27  6:01:41: prRecQueue: adding pd=01 to recovery wait queue


04/27  6:01:43: REC:log MedErr on pd[1] #retries=0


04/27  6:01:43: LogSense: pd=01, cdb=28 00 03 c0 b8 1e 00 00 01 00 


04/27  6:01:43:           sense=f0 00 03 03 c0 b8 1e 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  6:01:43: REC: MedErr on LD[1] BadLba=3c0b81e


04/27  6:01:43: prRecCallback: Error, cdb=28, pd=01, err=2


04/27  6:01:43: PatrolReadStart: Log MedErr pd=00 errLBA=3c0b81e in NVRAM_EVENT_LOG


04/27  6:01:43: prRecGo: Ready to attempt recovery errLBA=3c087de on pd=01


04/27  6:01:43: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/27  6:01:43: prRecGo: dataErr found on ld 0 span 0 arm 1


04/27  6:01:43: prRecGo: data NOT in cache; cacheLn=ffffffff, row=7810f, stripe=7810f, refBlk=5e, type=1


04/27  6:01:43: prRecGo: RAID1 - read from peer pd=00


04/27  6:01:45: REC:log MedErr on pd[0] #retries=0


04/27  6:01:45: LogSense: pd=00, cdb=28 00 03 c0 87 de 00 00 01 00 


04/27  6:01:45:           sense=f0 00 03 03 c0 87 de 0a 19 ce 48 eb 11 00 00 00 00 a5 


04/27  6:01:45: REC: MedErr on LD[0] BadLba=3c087de


04/27  6:01:45: prRecCallback: Error, cdb=28, pd=00, err=2


04/27  6:01:45: PatrolReadStart: Log MedErr pd=01 errLBA=3c087de in NVRAM_EVENT_LOG


04/27  6:01:48: REC:log MedErr on pd[0] #retries=0


04/27  6:01:48: LogSense: pd=00, cdb=2f 00 03 c0 b8 1f 00 ff ff 00 


04/27  6:01:48:           sense=f0 00 03 03 c0 ba 7e 0a 18 2e 48 f0 11 00 00 00 00 a5 


04/27  6:01:48: REC: MedErr on LD[0] BadLba=3c0ba7e


04/27  6:01:48: prCallback: Medium Error on pd=00, StartLba=3c0b81f, ErrLba=3c0ba7e


04/27  6:01:48: prRecQueue: starting pd=00 recovery - blocking host commands


04/27  6:01:48: prRecGo: Ready to attempt recovery errLBA=3c0ba7e on pd=00


04/27  6:01:48: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/27  6:01:48: prRecGo: dataErr found on ld 0 span 0 arm 0


04/27  6:01:48: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78174, stripe=78174, refBlk=7e, type=1


04/27  6:01:48: prRecGo: RAID1 - read from peer pd=01


04/27  6:01:50: REC:log MedErr on pd[1] #retries=0


04/27  6:01:50: LogSense: pd=01, cdb=28 00 03 c0 ba 7e 00 00 01 00 


04/27  6:01:50:           sense=f0 00 03 03 c0 ba 7e 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  6:01:50: REC: MedErr on LD[1] BadLba=3c0ba7e


04/27  6:01:50: prRecCallback: Error, cdb=28, pd=01, err=2


04/27  6:01:50: PatrolReadStart: Log MedErr pd=00 errLBA=3c0ba7e in NVRAM_EVENT_LOG


04/27  6:01:52: REC:log MedErr on pd[1] #retries=0


04/27  6:01:52: LogSense: pd=01, cdb=2f 00 03 c0 87 df 00 08 00 00 


04/27  6:01:52:           sense=f0 00 03 03 c0 8a 3e 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  6:01:52: REC: MedErr on LD[1] BadLba=3c08a3e


04/27  6:01:53: prCallback: Medium Error on pd=01, StartLba=3c087df, ErrLba=3c08a3e


04/27  6:01:53: prRecQueue: starting pd=01 recovery - blocking host commands


04/27  6:01:53: prRecGo: Ready to attempt recovery errLBA=3c08a3e on pd=01


04/27  6:01:53: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/27  6:01:53: prRecGo: dataErr found on ld 0 span 0 arm 1


04/27  6:01:53: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78114, stripe=78114, refBlk=3e, type=1


04/27  6:01:53: prRecGo: RAID1 - read from peer pd=00


04/27  6:01:54: REC:log MedErr on pd[0] #retries=0


04/27  6:01:54: LogSense: pd=00, cdb=28 00 03 c0 8a 3e 00 00 01 00 


04/27  6:01:54:           sense=f0 00 03 03 c0 8a 3e 0a 19 2e 48 ec 11 00 00 00 00 a5 


04/27  6:01:54: REC: MedErr on LD[0] BadLba=3c08a3e


04/27  6:01:54: prRecCallback: Error, cdb=28, pd=00, err=2


04/27  6:01:54: PatrolReadStart: Log MedErr pd=01 errLBA=3c08a3e in NVRAM_EVENT_LOG


04/27  6:01:59: REC:log MedErr on pd[0] #retries=0


04/27  6:01:59: LogSense: pd=00, cdb=2f 00 03 c0 c2 7f 00 ff ff 00 


04/27  6:01:59:           sense=f0 00 03 03 c0 d1 be 0a 19 ce 48 f1 11 00 00 00 00 a5 


04/27  6:01:59: REC: MedErr on LD[0] BadLba=3c0d1be


04/27  6:01:59: prCallback: Medium Error on pd=00, StartLba=3c0c27f, ErrLba=3c0d1be


04/27  6:01:59: prRecQueue: starting pd=00 recovery - blocking host commands


04/27  6:01:59: prRecGo: Ready to attempt recovery errLBA=3c0d1be on pd=00


04/27  6:01:59: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/27  6:01:59: prRecGo: dataErr found on ld 0 span 0 arm 0


04/27  6:01:59: prRecGo: data NOT in cache; cacheLn=ffffffff, row=781a3, stripe=781a3, refBlk=3e, type=1


04/27  6:01:59: prRecGo: RAID1 - read from peer pd=01


04/27  6:02:00: REC:log MedErr on pd[1] #retries=0


04/27  6:02:00: LogSense: pd=01, cdb=2f 00 03 c0 8a 3f 00 ff ff 00 


04/27  6:02:00:           sense=f0 00 03 03 c0 a3 de 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  6:02:00: REC: MedErr on LD[1] BadLba=3c0a3de


04/27  6:02:00: prCallback: Medium Error on pd=01, StartLba=3c08a3f, ErrLba=3c0a3de


04/27  6:02:00: prRecQueue: adding pd=01 to recovery wait queue


04/27  6:02:02: REC:log MedErr on pd[1] #retries=0


04/27  6:02:02: LogSense: pd=01, cdb=28 00 03 c0 d1 be 00 00 01 00 


04/27  6:02:02:           sense=f0 00 03 03 c0 d1 be 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  6:02:02: REC: MedErr on LD[1] BadLba=3c0d1be


04/27  6:02:02: prRecCallback: Error, cdb=28, pd=01, err=2


04/27  6:02:02: PatrolReadStart: Log MedErr pd=00 errLBA=3c0d1be in NVRAM_EVENT_LOG


04/27  6:02:02: prRecGo: Ready to attempt recovery errLBA=3c0a3de on pd=01


04/27  6:02:02: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/27  6:02:02: prRecGo: dataErr found on ld 0 span 0 arm 1


04/27  6:02:02: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78147, stripe=78147, refBlk=5e, type=1


04/27  6:02:02: prRecGo: RAID1 - read from peer pd=00


04/27  6:02:04: REC:log MedErr on pd[0] #retries=0


04/27  6:02:04: LogSense: pd=00, cdb=28 00 03 c0 a3 de 00 00 01 00 


04/27  6:02:04:           sense=f0 00 03 03 c0 a3 de 0a 1a 2e 48 ee 16 00 00 00 00 a6 


04/27  6:02:04: REC: MedErr on LD[0] BadLba=3c0a3de


04/27  6:02:04: prRecCallback: Error, cdb=28, pd=00, err=2


04/27  6:02:04: PatrolReadStart: Log MedErr pd=01 errLBA=3c0a3de in NVRAM_EVENT_LOG


04/27  6:02:07: REC:log MedErr on pd[0] #retries=0


04/27  6:02:07: LogSense: pd=00, cdb=2f 00 03 c0 d1 bf 00 ff ff 00 


04/27  6:02:07:           sense=f0 00 03 03 c0 eb 53 0a 1a c3 48 f3 11 00 00 00 00 a5 


04/27  6:02:07: REC: MedErr on LD[0] BadLba=3c0eb53


04/27  6:02:07: prCallback: Medium Error on pd=00, StartLba=3c0d1bf, ErrLba=3c0eb53


04/27  6:02:07: prRecQueue: starting pd=00 recovery - blocking host commands


04/27  6:02:07: prRecGo: Ready to attempt recovery errLBA=3c0eb53 on pd=00


04/27  6:02:07: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/27  6:02:07: prRecGo: dataErr found on ld 0 span 0 arm 0


04/27  6:02:07: prRecGo: data NOT in cache; cacheLn=ffffffff, row=781d6, stripe=781d6, refBlk=53, type=1


04/27  6:02:07: prRecGo: RAID1 - read from peer pd=01


04/27  6:02:09: REC:log MedErr on pd[1] #retries=0


04/27  6:02:09: LogSense: pd=01, cdb=28 00 03 c0 eb 53 00 00 01 00 


04/27  6:02:09:           sense=f0 00 03 03 c0 eb 53 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  6:02:09: REC: MedErr on LD[1] BadLba=3c0eb53


04/27  6:02:09: prRecCallback: Error, cdb=28, pd=01, err=2


04/27  6:02:09: PatrolReadStart: Log MedErr pd=00 errLBA=3c0eb53 in NVRAM_EVENT_LOG


04/27  6:02:19: REC:log MedErr on pd[1] #retries=0


04/27  6:02:19: LogSense: pd=01, cdb=2f 00 03 c0 ac 1f 00 ff ff 00 


04/27  6:02:19:           sense=f0 00 03 03 c0 b8 1d 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  6:02:19: REC: MedErr on LD[1] BadLba=3c0b81d


04/27  6:02:19: REC:log MedErr on pd[0] #retries=0


04/27  6:02:19: LogSense: pd=00, cdb=2f 00 03 c0 eb 94 00 ff ff 00 


04/27  6:02:19:           sense=f0 00 03 03 c1 01 f3 0a 18 c3 48 f5 11 00 00 00 00 a5 


04/27  6:02:19: REC: MedErr on LD[0] BadLba=3c101f3


04/27  6:02:19: prCallback: Medium Error on pd=01, StartLba=3c0ac1f, ErrLba=3c0b81d


04/27  6:02:19: prRecQueue: starting pd=01 recovery - blocking host commands


04/27  6:02:19: prCallback: Medium Error on pd=00, StartLba=3c0eb94, ErrLba=3c101f3


04/27  6:02:19: prRecQueue: adding pd=00 to recovery wait queue


04/27  6:02:19: prRecGo: Ready to attempt recovery errLBA=3c0b81d on pd=01


04/27  6:02:19: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/27  6:02:19: prRecGo: dataErr found on ld 0 span 0 arm 1


04/27  6:02:19: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78170, stripe=78170, refBlk=1d, type=1


04/27  6:02:19: prRecGo: RAID1 - read from peer pd=00


04/27  6:02:20: REC:log MedErr on pd[0] #retries=0


04/27  6:02:20: LogSense: pd=00, cdb=28 00 03 c0 b8 1d 00 00 01 00 


04/27  6:02:20:           sense=f0 00 03 03 c0 b8 1d 0a 18 cd 48 ef 11 00 00 00 00 a5 


04/27  6:02:20: REC: MedErr on LD[0] BadLba=3c0b81d


04/27  6:02:20: prRecCallback: Error, cdb=28, pd=00, err=2


04/27  6:02:20: PatrolReadStart: Log MedErr pd=01 errLBA=3c0b81d in NVRAM_EVENT_LOG


04/27  6:02:20: prRecGo: Ready to attempt recovery errLBA=3c101f3 on pd=00


04/27  6:02:20: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/27  6:02:20: prRecGo: dataErr found on ld 0 span 0 arm 0


04/27  6:02:20: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78203, stripe=78203, refBlk=73, type=1


04/27  6:02:20: prRecGo: RAID1 - read from peer pd=01


04/27  6:02:23: REC:log MedErr on pd[1] #retries=0


04/27  6:02:23: LogSense: pd=01, cdb=28 00 03 c1 01 f3 00 00 01 00 


04/27  6:02:23:           sense=f0 00 03 03 c1 01 f3 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  6:02:23: REC: MedErr on LD[1] BadLba=3c101f3


04/27  6:02:23: prRecCallback: Error, cdb=28, pd=01, err=2


04/27  6:02:23: PatrolReadStart: Log MedErr pd=00 errLBA=3c101f3 in NVRAM_EVENT_LOG


04/27  6:02:26: REC:log MedErr on pd[1] #retries=0


04/27  6:02:26: LogSense: pd=01, cdb=2f 00 03 c0 b8 1e 00 ff ff 00 


04/27  6:02:26:           sense=f0 00 03 03 c0 b8 1e 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  6:02:26: REC: MedErr on LD[1] BadLba=3c0b81e


04/27  6:02:26: prCallback: Medium Error on pd=01, StartLba=3c0b81e, ErrLba=3c0b81e


04/27  6:02:26: prRecQueue: starting pd=01 recovery - blocking host commands


04/27  6:02:26: prRecGo: Ready to attempt recovery errLBA=3c0b81e on pd=01


04/27  6:02:26: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/27  6:02:26: prRecGo: dataErr found on ld 0 span 0 arm 1


04/27  6:02:26: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78170, stripe=78170, refBlk=1e, type=1


04/27  6:02:26: prRecGo: RAID1 - read from peer pd=00


04/27  6:02:27: REC:log MedErr on pd[0] #retries=0


04/27  6:02:27: LogSense: pd=00, cdb=28 00 03 c0 b8 1e 00 00 01 00 


04/27  6:02:27:           sense=f0 00 03 03 c0 b8 1e 0a 18 ce 48 ef 11 00 00 00 00 a5 


04/27  6:02:27: REC: MedErr on LD[0] BadLba=3c0b81e


04/27  6:02:27: prRecCallback: Error, cdb=28, pd=00, err=2


04/27  6:02:27: PatrolReadStart: Log MedErr pd=01 errLBA=3c0b81e in NVRAM_EVENT_LOG


04/27  6:02:28: REC:log MedErr on pd[0] #retries=0


04/27  6:02:28: LogSense: pd=00, cdb=2f 00 03 c1 01 f4 00 08 00 00 


04/27  6:02:28:           sense=f0 00 03 03 c1 04 53 0a 18 23 48 f6 11 00 00 00 00 a5 


04/27  6:02:28: REC: MedErr on LD[0] BadLba=3c10453


04/27  6:02:28: prCallback: Medium Error on pd=00, StartLba=3c101f4, ErrLba=3c10453


04/27  6:02:28: prRecQueue: starting pd=00 recovery - blocking host commands


04/27  6:02:28: prRecGo: Ready to attempt recovery errLBA=3c10453 on pd=00


04/27  6:02:28: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/27  6:02:28: prRecGo: dataErr found on ld 0 span 0 arm 0


04/27  6:02:28: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78208, stripe=78208, refBlk=53, type=1


04/27  6:02:29: prRecGo: RAID1 - read from peer pd=01


04/27  6:02:31: REC:log MedErr on pd[1] #retries=0


04/27  6:02:31: LogSense: pd=01, cdb=28 00 03 c1 04 53 00 00 01 00 


04/27  6:02:31:           sense=f0 00 03 03 c1 04 53 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  6:02:31: REC: MedErr on LD[1] BadLba=3c10453


04/27  6:02:31: prRecCallback: Error, cdb=28, pd=01, err=2


04/27  6:02:31: PatrolReadStart: Log MedErr pd=00 errLBA=3c10453 in NVRAM_EVENT_LOG


04/27  6:02:33: REC:log MedErr on pd[1] #retries=0


04/27  6:02:33: LogSense: pd=01, cdb=2f 00 03 c0 b8 1f 00 08 00 00 


04/27  6:02:33:           sense=f0 00 03 03 c0 ba 7e 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  6:02:33: REC: MedErr on LD[1] BadLba=3c0ba7e


04/27  6:02:33: prCallback: Medium Error on pd=01, StartLba=3c0b81f, ErrLba=3c0ba7e


04/27  6:02:33: prRecQueue: starting pd=01 recovery - blocking host commands


04/27  6:02:33: prRecGo: Ready to attempt recovery errLBA=3c0ba7e on pd=01


04/27  6:02:33: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/27  6:02:33: prRecGo: dataErr found on ld 0 span 0 arm 1


04/27  6:02:33: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78174, stripe=78174, refBlk=7e, type=1


04/27  6:02:33: prRecGo: RAID1 - read from peer pd=00


04/27  6:02:34: REC:log MedErr on pd[0] #retries=0


04/27  6:02:34: LogSense: pd=00, cdb=28 00 03 c0 ba 7e 00 00 01 00 


04/27  6:02:34:           sense=f0 00 03 03 c0 ba 7e 0a 18 2e 48 f0 11 00 00 00 00 a5 


04/27  6:02:34: REC: MedErr on LD[0] BadLba=3c0ba7e


04/27  6:02:35: prRecCallback: Error, cdb=28, pd=00, err=2


04/27  6:02:35: PatrolReadStart: Log MedErr pd=01 errLBA=3c0ba7e in NVRAM_EVENT_LOG


04/27  6:02:36: REC:log MedErr on pd[0] #retries=0


04/27  6:02:36: LogSense: pd=00, cdb=2f 00 03 c1 04 54 00 ff ff 00 


04/27  6:02:36:           sense=f0 00 03 03 c1 5d 6c 0a 03 0c 48 fd 11 00 00 00 00 a5 


04/27  6:02:36: REC: MedErr on LD[0] BadLba=3c15d6c


04/27  6:02:36: prCallback: Medium Error on pd=00, StartLba=3c10454, ErrLba=3c15d6c


04/27  6:02:36: prRecQueue: starting pd=00 recovery - blocking host commands


04/27  6:02:36: prRecGo: Ready to attempt recovery errLBA=3c15d6c on pd=00


04/27  6:02:36: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/27  6:02:36: prRecGo: dataErr found on ld 0 span 0 arm 0


04/27  6:02:37: prRecGo: data NOT in cache; cacheLn=ffffffff, row=782ba, stripe=782ba, refBlk=6c, type=1


04/27  6:02:37: prRecGo: RAID1 - read from peer pd=01


04/27  6:02:39: REC:log MedErr on pd[1] #retries=0


04/27  6:02:39: LogSense: pd=01, cdb=28 00 03 c1 5d 6c 00 00 01 00 


04/27  6:02:39:           sense=f0 00 03 03 c1 5d 6c 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  6:02:39: REC: MedErr on LD[1] BadLba=3c15d6c


04/27  6:02:39: prRecCallback: Error, cdb=28, pd=01, err=2


04/27  6:02:39: PatrolReadStart: Log MedErr pd=00 errLBA=3c15d6c in NVRAM_EVENT_LOG


04/27  6:02:44: REC:log MedErr on pd[0] #retries=0


04/27  6:02:44: LogSense: pd=00, cdb=2f 00 03 c1 5d 6d 00 ff ff 00 


04/27  6:02:44:           sense=f0 00 03 03 c1 71 8d 0a 01 a2 48 fe 11 00 00 00 00 a5 


04/27  6:02:44: REC: MedErr on LD[0] BadLba=3c1718d


04/27  6:02:44: prCallback: Medium Error on pd=00, StartLba=3c15d6d, ErrLba=3c1718d


04/27  6:02:44: prRecQueue: starting pd=00 recovery - blocking host commands


04/27  6:02:44: prRecGo: Ready to attempt recovery errLBA=3c1718d on pd=00


04/27  6:02:44: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/27  6:02:44: prRecGo: dataErr found on ld 0 span 0 arm 0


04/27  6:02:44: prRecGo: data NOT in cache; cacheLn=ffffffff, row=782e3, stripe=782e3, refBlk=d, type=1


04/27  6:02:44: prRecGo: RAID1 - read from peer pd=01


04/27  6:02:45: REC:log MedErr on pd[1] #retries=0


04/27  6:02:45: LogSense: pd=01, cdb=2f 00 03 c0 c2 7f 00 ff ff 00 


04/27  6:02:45:           sense=f0 00 03 03 c0 d1 be 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  6:02:45: REC: MedErr on LD[1] BadLba=3c0d1be


04/27  6:02:45: prCallback: Medium Error on pd=01, StartLba=3c0c27f, ErrLba=3c0d1be


04/27  6:02:45: prRecQueue: adding pd=01 to recovery wait queue


04/27  6:02:47: REC:log MedErr on pd[1] #retries=0


04/27  6:02:47: LogSense: pd=01, cdb=28 00 03 c1 71 8d 00 00 01 00 


04/27  6:02:47:           sense=f0 00 03 03 c1 71 8d 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  6:02:47: REC: MedErr on LD[1] BadLba=3c1718d


04/27  6:02:47: prRecCallback: Error, cdb=28, pd=01, err=2


04/27  6:02:47: PatrolReadStart: Log MedErr pd=00 errLBA=3c1718d in NVRAM_EVENT_LOG


04/27  6:02:47: prRecGo: Ready to attempt recovery errLBA=3c0d1be on pd=01


04/27  6:02:47: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/27  6:02:47: prRecGo: dataErr found on ld 0 span 0 arm 1


04/27  6:02:47: prRecGo: data NOT in cache; cacheLn=ffffffff, row=781a3, stripe=781a3, refBlk=3e, type=1


04/27  6:02:47: prRecGo: RAID1 - read from peer pd=00


04/27  6:02:49: REC:log MedErr on pd[0] #retries=0


04/27  6:02:49: LogSense: pd=00, cdb=28 00 03 c0 d1 be 00 00 01 00 


04/27  6:02:49:           sense=f0 00 03 03 c0 d1 be 0a 19 ce 48 f1 11 00 00 00 00 a5 


04/27  6:02:49: REC: MedErr on LD[0] BadLba=3c0d1be


04/27  6:02:49: prRecCallback: Error, cdb=28, pd=00, err=2


04/27  6:02:49: PatrolReadStart: Log MedErr pd=01 errLBA=3c0d1be in NVRAM_EVENT_LOG


04/27  6:02:55: REC:log MedErr on pd[1] #retries=0


04/27  6:02:55: LogSense: pd=01, cdb=2f 00 03 c0 d1 bf 00 ff ff 00 


04/27  6:02:55:           sense=f0 00 03 03 c0 eb 53 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  6:02:55: REC: MedErr on LD[1] BadLba=3c0eb53


04/27  6:02:55: prCallback: Medium Error on pd=01, StartLba=3c0d1bf, ErrLba=3c0eb53


04/27  6:02:55: prRecQueue: starting pd=01 recovery - blocking host commands


04/27  6:02:55: prRecGo: Ready to attempt recovery errLBA=3c0eb53 on pd=01


04/27  6:02:55: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/27  6:02:55: prRecGo: dataErr found on ld 0 span 0 arm 1


04/27  6:02:55: prRecGo: data NOT in cache; cacheLn=ffffffff, row=781d6, stripe=781d6, refBlk=53, type=1


04/27  6:02:55: prRecGo: RAID1 - read from peer pd=00


04/27  6:02:56: REC:log MedErr on pd[0] #retries=0


04/27  6:02:56: LogSense: pd=00, cdb=28 00 03 c0 eb 53 00 00 01 00 


04/27  6:02:56:           sense=f0 00 03 03 c0 eb 53 0a 1a c3 48 f3 11 00 00 00 00 a5 


04/27  6:02:56: REC: MedErr on LD[0] BadLba=3c0eb53


04/27  6:02:56: prRecCallback: Error, cdb=28, pd=00, err=2


04/27  6:02:56: PatrolReadStart: Log MedErr pd=01 errLBA=3c0eb53 in NVRAM_EVENT_LOG


04/27  6:03:02: REC:log MedErr on pd[1] #retries=0


04/27  6:03:02: LogSense: pd=01, cdb=2f 00 03 c0 eb 54 00 ff ff 00 


04/27  6:03:02:           sense=f0 00 03 03 c1 01 f3 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  6:03:02: REC: MedErr on LD[1] BadLba=3c101f3


04/27  6:03:02: prCallback: Medium Error on pd=01, StartLba=3c0eb54, ErrLba=3c101f3


04/27  6:03:02: prRecQueue: starting pd=01 recovery - blocking host commands


04/27  6:03:02: prRecGo: Ready to attempt recovery errLBA=3c101f3 on pd=01


04/27  6:03:02: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/27  6:03:02: prRecGo: dataErr found on ld 0 span 0 arm 1


04/27  6:03:02: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78203, stripe=78203, refBlk=73, type=1


04/27  6:03:02: prRecGo: RAID1 - read from peer pd=00


04/27  6:03:03: REC:log MedErr on pd[0] #retries=0


04/27  6:03:03: LogSense: pd=00, cdb=28 00 03 c1 01 f3 00 00 01 00 


04/27  6:03:03:           sense=f0 00 03 03 c1 01 f3 0a 18 c3 48 f5 11 00 00 00 00 a5 


04/27  6:03:03: REC: MedErr on LD[0] BadLba=3c101f3


04/27  6:03:03: prRecCallback: Error, cdb=28, pd=00, err=2


04/27  6:03:03: PatrolReadStart: Log MedErr pd=01 errLBA=3c101f3 in NVRAM_EVENT_LOG


04/27  6:03:09: REC:log MedErr on pd[1] #retries=0


04/27  6:03:09: LogSense: pd=01, cdb=2f 00 03 c1 01 f4 00 ff ff 00 


04/27  6:03:09:           sense=f0 00 03 03 c1 04 53 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  6:03:09: REC: MedErr on LD[1] BadLba=3c10453


04/27  6:03:09: prCallback: Medium Error on pd=01, StartLba=3c101f4, ErrLba=3c10453


04/27  6:03:09: prRecQueue: starting pd=01 recovery - blocking host commands


04/27  6:03:09: prRecGo: Ready to attempt recovery errLBA=3c10453 on pd=01


04/27  6:03:09: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/27  6:03:09: prRecGo: dataErr found on ld 0 span 0 arm 1


04/27  6:03:09: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78208, stripe=78208, refBlk=53, type=1


04/27  6:03:09: prRecGo: RAID1 - read from peer pd=00


04/27  6:03:10: REC:log MedErr on pd[0] #retries=0


04/27  6:03:10: LogSense: pd=00, cdb=2f 00 03 c5 71 8a 00 ff ff 00 


04/27  6:03:10:           sense=f0 00 03 03 c6 4e 38 0a 1a a8 49 63 11 00 00 00 00 a5 


04/27  6:03:10: REC: MedErr on LD[0] BadLba=3c64e38


04/27  6:03:10: prCallback: Medium Error on pd=00, StartLba=3c5718a, ErrLba=3c64e38


04/27  6:03:10: prRecQueue: adding pd=00 to recovery wait queue


04/27  6:03:11: REC:log MedErr on pd[0] #retries=0


04/27  6:03:11: LogSense: pd=00, cdb=28 00 03 c1 04 53 00 00 01 00 


04/27  6:03:11:           sense=f0 00 03 03 c1 04 53 0a 18 23 48 f6 11 00 00 00 00 a5 


04/27  6:03:11: REC: MedErr on LD[0] BadLba=3c10453


04/27  6:03:12: prRecCallback: Error, cdb=28, pd=00, err=2


04/27  6:03:12: PatrolReadStart: Log MedErr pd=01 errLBA=3c10453 in NVRAM_EVENT_LOG


04/27  6:03:12: prRecGo: Ready to attempt recovery errLBA=3c64e38 on pd=00


04/27  6:03:12: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/27  6:03:12: prRecGo: dataErr found on ld 0 span 0 arm 0


04/27  6:03:12: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78c9c, stripe=78c9c, refBlk=38, type=1


04/27  6:03:12: prRecGo: RAID1 - read from peer pd=01


04/27  6:03:14: REC:log MedErr on pd[1] #retries=0


04/27  6:03:14: LogSense: pd=01, cdb=28 00 03 c6 4e 38 00 00 01 00 


04/27  6:03:14:           sense=f0 00 03 03 c6 4e 38 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  6:03:14: REC: MedErr on LD[1] BadLba=3c64e38


04/27  6:03:14: prRecCallback: Error, cdb=28, pd=01, err=2


04/27  6:03:14: PatrolReadStart: Log MedErr pd=00 errLBA=3c64e38 in NVRAM_EVENT_LOG


04/27  6:03:18: REC:log MedErr on pd[1] #retries=0


04/27  6:03:18: LogSense: pd=01, cdb=2f 00 03 c1 04 54 00 ff ff 00 


04/27  6:03:18:           sense=f0 00 03 03 c1 5d 6c 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  6:03:18: REC: MedErr on LD[1] BadLba=3c15d6c


04/27  6:03:18: prCallback: Medium Error on pd=01, StartLba=3c10454, ErrLba=3c15d6c


04/27  6:03:18: prRecQueue: starting pd=01 recovery - blocking host commands


04/27  6:03:18: prRecGo: Ready to attempt recovery errLBA=3c15d6c on pd=01


04/27  6:03:18: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/27  6:03:18: prRecGo: dataErr found on ld 0 span 0 arm 1


04/27  6:03:18: prRecGo: data NOT in cache; cacheLn=ffffffff, row=782ba, stripe=782ba, refBlk=6c, type=1


04/27  6:03:18: prRecGo: RAID1 - read from peer pd=00


04/27  6:03:19: REC:log MedErr on pd[0] #retries=0


04/27  6:03:19: LogSense: pd=00, cdb=2f 00 03 c6 4e 39 00 ff ff 00 


04/27  6:03:19:           sense=f0 00 03 03 c6 50 98 0a 1a 08 49 64 11 00 00 00 00 a5 


04/27  6:03:19: REC: MedErr on LD[0] BadLba=3c65098


04/27  6:03:19: prCallback: Medium Error on pd=00, StartLba=3c64e39, ErrLba=3c65098


04/27  6:03:19: prRecQueue: adding pd=00 to recovery wait queue


04/27  6:03:20: REC:log MedErr on pd[0] #retries=0


04/27  6:03:20: LogSense: pd=00, cdb=28 00 03 c1 5d 6c 00 00 01 00 


04/27  6:03:20:           sense=f0 00 03 03 c1 5d 6c 0a 03 0c 48 fd 11 00 00 00 00 a5 


04/27  6:03:20: REC: MedErr on LD[0] BadLba=3c15d6c


04/27  6:03:20: prRecCallback: Error, cdb=28, pd=00, err=2


04/27  6:03:20: PatrolReadStart: Log MedErr pd=01 errLBA=3c15d6c in NVRAM_EVENT_LOG


04/27  6:03:20: prRecGo: Ready to attempt recovery errLBA=3c65098 on pd=00


04/27  6:03:20: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/27  6:03:20: prRecGo: dataErr found on ld 0 span 0 arm 0


04/27  6:03:20: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78ca1, stripe=78ca1, refBlk=18, type=1


04/27  6:03:20: prRecGo: RAID1 - read from peer pd=01


04/27  6:03:22: REC:log MedErr on pd[1] #retries=0


04/27  6:03:22: LogSense: pd=01, cdb=28 00 03 c6 50 98 00 00 01 00 


04/27  6:03:22:           sense=f0 00 03 03 c6 50 98 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  6:03:22: REC: MedErr on LD[1] BadLba=3c65098


04/27  6:03:22: prRecCallback: Error, cdb=28, pd=01, err=2


04/27  6:03:22: PatrolReadStart: Log MedErr pd=00 errLBA=3c65098 in NVRAM_EVENT_LOG


04/27  6:03:26: REC:log MedErr on pd[1] #retries=0


04/27  6:03:26: LogSense: pd=01, cdb=2f 00 03 c1 5d 6d 00 ff ff 00 


04/27  6:03:26:           sense=f0 00 03 03 c1 71 8d 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  6:03:26: REC: MedErr on LD[1] BadLba=3c1718d


04/27  6:03:26: prCallback: Medium Error on pd=01, StartLba=3c15d6d, ErrLba=3c1718d


04/27  6:03:26: prRecQueue: starting pd=01 recovery - blocking host commands


04/27  6:03:26: prRecGo: Ready to attempt recovery errLBA=3c1718d on pd=01


04/27  6:03:26: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/27  6:03:26: prRecGo: dataErr found on ld 0 span 0 arm 1


04/27  6:03:26: prRecGo: data NOT in cache; cacheLn=ffffffff, row=782e3, stripe=782e3, refBlk=d, type=1


04/27  6:03:26: prRecGo: RAID1 - read from peer pd=00


04/27  6:03:27: REC:log MedErr on pd[0] #retries=0


04/27  6:03:27: LogSense: pd=00, cdb=2f 00 03 c6 50 99 00 ff ff 00 


04/27  6:03:27:           sense=f0 00 03 03 c6 64 d8 0a 18 a8 49 65 11 00 00 00 00 a5 


04/27  6:03:27: REC: MedErr on LD[0] BadLba=3c664d8


04/27  6:03:27: prCallback: Medium Error on pd=00, StartLba=3c65099, ErrLba=3c664d8


04/27  6:03:27: prRecQueue: adding pd=00 to recovery wait queue


04/27  6:03:28: REC:log MedErr on pd[0] #retries=0


04/27  6:03:28: LogSense: pd=00, cdb=28 00 03 c1 71 8d 00 00 01 00 


04/27  6:03:28:           sense=f0 00 03 03 c1 71 8d 0a 01 a2 48 fe 11 00 00 00 00 a5 


04/27  6:03:28: REC: MedErr on LD[0] BadLba=3c1718d


04/27  6:03:28: prRecCallback: Error, cdb=28, pd=00, err=2


04/27  6:03:28: PatrolReadStart: Log MedErr pd=01 errLBA=3c1718d in NVRAM_EVENT_LOG


04/27  6:03:28: prRecGo: Ready to attempt recovery errLBA=3c664d8 on pd=00


04/27  6:03:28: prGetLDInfo: MediaErr in ld=0, span=0, arm=0


04/27  6:03:28: prRecGo: dataErr found on ld 0 span 0 arm 0


04/27  6:03:28: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78cc9, stripe=78cc9, refBlk=58, type=1


04/27  6:03:28: prRecGo: RAID1 - read from peer pd=01


04/27  6:03:30: REC:log MedErr on pd[1] #retries=0


04/27  6:03:30: LogSense: pd=0����ƒ*��8 00 03 c6 64 d8 00 00 01 00 


04/27  6:03:30:           sense=f0 00 03 03 c6 64 d8 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  6:03:30: REC: MedErr on LD[1] BadLba=3c664d8


04/27  6:03:30: prRecCallback: Error, cdb=28, pd=01, err=2


04/27  6:03:30: PatrolReadStart: Log MedErr pd=00 errLBA=3c664d8 in NVRAM_EVENT_LOG


04/27  6:03:50: REC:log MedErr on pd[1] #retries=0


04/27  6:03:50: LogSense: pd=01, cdb=2f 00 03 c5 71 8a 00 ff ff 00 


04/27  6:03:50:           sense=f0 00 03 03 c6 4e 38 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  6:03:50: REC: MedErr on LD[1] BadLba=3c64e38


04/27  6:03:50: prCallback: Medium Error on pd=01, StartLba=3c5718a, ErrLba=3c64e38


04/27  6:03:50: prRecQueue: starting pd=01 recovery - blocking host commands


04/27  6:03:50: prRecGo: Ready to attempt recovery errLBA=3c64e38 on pd=01


04/27  6:03:50: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/27  6:03:50: prRecGo: dataErr found on ld 0 span 0 arm 1


04/27  6:03:50: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78c9c, stripe=78c9c, refBlk=38, type=1


04/27  6:03:50: prRecGo: RAID1 - read from peer pd=00


04/27  6:03:52: REC:log MedErr on pd[0] #retries=0


04/27  6:03:52: LogSense: pd=00, cdb=28 00 03 c6 4e 38 00 00 01 00 


04/27  6:03:52:           sense=f0 00 03 03 c6 4e 38 0a 1a a8 49 63 11 00 00 00 00 a5 


04/27  6:03:52: REC: MedErr on LD[0] BadLba=3c64e38


04/27  6:03:52: prRecCallback: Error, cdb=28, pd=00, err=2


04/27  6:03:52: PatrolReadStart: Log MedErr pd=01 errLBA=3c64e38 in NVRAM_EVENT_LOG


04/27  6:03:58: REC:log MedErr on pd[1] #retries=0


04/27  6:03:58: LogSense: pd=01, cdb=2f 00 03 c6 4e 39 00 ff ff 00 


04/27  6:03:58:           sense=f0 00 03 03 c6 50 98 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  6:03:58: REC: MedErr on LD[1] BadLba=3c65098


04/27  6:03:58: prCallback: Medium Error on pd=01, StartLba=3c64e39, ErrLba=3c65098


04/27  6:03:58: prRecQueue: starting pd=01 recovery - blocking host commands


04/27  6:03:58: prRecGo: Ready to attempt recovery errLBA=3c65098 on pd=01


04/27  6:03:58: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/27  6:03:58: prRecGo: dataErr found on ld 0 span 0 arm 1


04/27  6:03:58: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78ca1, stripe=78ca1, refBlk=18, type=1


04/27  6:03:58: prRecGo: RAID1 - read from peer pd=00


04/27  6:03:59: REC:log MedErr on pd[0] #retries=0


04/27  6:03:59: LogSense: pd=00, cdb=28 00 03 c6 50 98 00 00 01 00 


04/27  6:03:59:           sense=f0 00 03 03 c6 50 98 0a 1a 08 49 64 11 00 00 00 00 a5 


04/27  6:03:59: REC: MedErr on LD[0] BadLba=3c65098


04/27  6:03:59: prRecCallback: Error, cdb=28, pd=00, err=2


04/27  6:03:59: PatrolReadStart: Log MedErr pd=01 errLBA=3c65098 in NVRAM_EVENT_LOG


04/27  6:04:05: REC:log MedErr on pd[1] #retries=0


04/27  6:04:05: LogSense: pd=01, cdb=2f 00 03 c6 50 99 00 ff ff 00 


04/27  6:04:05:           sense=f0 00 03 03 c6 64 d8 0a 00 00 00 00 11 00 e4 00 00 8e 


04/27  6:04:05: REC: MedErr on LD[1] BadLba=3c664d8


04/27  6:04:05: prCallback: Medium Error on pd=01, StartLba=3c65099, ErrLba=3c664d8


04/27  6:04:05: prRecQueue: starting pd=01 recovery - blocking host commands


04/27  6:04:05: prRecGo: Ready to attempt recovery errLBA=3c664d8 on pd=01


04/27  6:04:05: prGetLDInfo: MediaErr in ld=0, span=0, arm=1


04/27  6:04:05: prRecGo: dataErr found on ld 0 span 0 arm 1


04/27  6:04:05: prRecGo: data NOT in cache; cacheLn=ffffffff, row=78cc9, stripe=78cc9, refBlk=58, type=1


04/27  6:04:05: prRecGo: RAID1 - read from peer pd=00


04/27  6:04:06: REC:log MedErr on pd[0] #retries=0


04/27  6:04:06: LogSense: pd=00, cdb=28 00 03 c6 64 d8 00 00 01 00 


04/27  6:04:06:           sense=f0 00 03 03 c6 64 d8 0a 18 a8 49 65 11 00 00 00 00 a5 


04/27  6:04:06: REC: MedErr on LD[0] BadLba=3c664d8


04/27  6:04:06: prRecCallback: Error, cdb=28, pd=00, err=2


04/27  6:04:06: PatrolReadStart: Log MedErr pd=01 errLBA=3c664d8 in NVRAM_EVENT_LOG


04/27  6:05:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  6:05:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  6:05:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  6:10:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  6:10:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  6:10:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  6:15:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  6:15:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  6:15:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  6:20:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  6:20:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  6:20:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  6:25:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  6:25:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  6:25:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  6:30:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  6:30:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  6:30:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  6:35:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  6:35:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  6:35:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  6:40:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  6:40:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  6:40:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  6:45:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  6:45:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  6:45:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  6:50:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  6:50:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  6:50:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  6:55:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  6:55:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  6:55:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  7:00:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  7:00:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  7:00:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  7:05:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  7:05:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  7:05:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  7:10:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  7:10:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  7:10:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  7:15:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  7:15:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  7:15:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  7:20:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  7:20:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  7:20:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  7:25:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  7:25:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  7:25:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  7:29:34: prCallback: PR completed for pd=00


04/27  7:29:34: prDiskDone: finishing PR on PD=00


04/27  7:30:58: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  7:30:58: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  7:30:58:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  7:32:37: prCallback: PR completed for pd=01


04/27  7:32:37: prDiskDone: finishing PR on PD=01


04/27  7:35:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  7:35:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  7:35:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  7:40:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  7:40:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  7:40:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  7:45:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  7:45:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  7:45:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  7:50:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  7:50:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  7:50:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  7:55:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  7:55:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  7:55:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  8:00:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  8:00:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  8:00:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  8:05:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  8:05:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  8:05:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  8:10:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  8:10:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  8:10:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  8:15:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  8:15:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  8:15:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  8:20:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  8:20:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  8:20:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  8:25:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  8:25:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  8:25:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  8:30:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  8:30:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  8:30:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  8:35:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  8:35:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  8:35:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


04/27  8:40:57: LogSense: ASC_PREDFAIL on dev 1 PredFailArry=0002


04/27  8:40:57: LogSense: pd=01, cdb=03 00 00 00 10 00 


04/27  8:40:57:           sense=70 00 00 00 00 00 00 0a 00 00 00 00 5d 00 10 00 00 00 


�������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������


_1235561749/DSET Report (050331).zip


READ ME before viewing this report!.txt



=======================================================

Instructions for the proper viewing of this DSET report

=======================================================





1) Decompress ALL files within this report into one folder using any ZIP utility 



2) When prompted enter the password "dell" (no quotes, all lowercase letters) to decrypt

   the report contents during decompression



3) Then open "dsetreport.hta" in Internet Explorer 5.0 or newer (which must support

   viewing XML data)





NOTE: Do NOT open the ZIP report file using the built-in Windows XP/2003 .ZIP expansion

      utility then double-clicking "dsetreport.hta". Other GUI support files must also

      be decompressed into the folder in order for the DSET graphical web interface to

      display properly. The DSET .ZIP report file must be entirely decompressed to a

      folder first before opening "dsetreport.hta".



NOTE: Dsetreport.hta is considered an "HTML Application" for purposes of secure local

      operation on Windows. Double-clicking the file will launch the HTML rendering engine

      of Windows which looks very similar to Internet Explorer. Do not rename the file

      extension to ".htm" as unpredictable behavior may result.



NOTE: Netscape and Mozilla are NOT supported at this time for displaying the DSET

      graphical interface



NOTE: Various log and configuration files can be found categorized within the "log"

      folder of this report




dsetreport.hta


